3GPP TSG-RAN WG2 Meeting #127-bis 	R2-2408171

Hefei, China, 14th – 18th Oct, 2024

Agenda Item:	8.2.5

Source:	Spreadtrum Communications

Title:	Discussion on Topology 2 issues

Document for:	Discussion and Decision

Introduction

In the SID [1], topologies 1 and 2 are the scope of AIoT. And the overall objective is to study a harmonized air interface design with minimized differences for different topologies.

RP-240826 Revised SID [1] General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm,DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. RAN1-led: …… For Topology 2, no difference in physical layer design from Topology 1.

This Tdoc mainly focuses on topology 2 related aspects between gNB and reader. In this contribution, we discuss the issues related to topology 2. And we provide our opinions.

Discussion on Topology 2 related issues

2.1 Compact Protocol stack

In the current SA2 progress [2], various solutions for Topology 2 have been identified. There are three types of solutions:

- Upper layer user plane based solutions, such as S #3, 4, 22, 32. There is an end to end layer between intermediate UE and CN, which is the upper layer above PDU layer. Figure 1 takes solution #3 as an example.

- Upper layer NAS based solutions, such as S #4, 11, 12, 21, 30. The end-to-end layer between the intermediate UE and core network is the NAS layer or new CP protocol layer above NAS layer. Figure 2 takes solution #4 as an example.

- RRC-based solutions, such as S#33. There is no direct end-to-end protocol layer between the intermediate UE and the core network for A-IoT services. Instead, the Uu RRC layer is used to forward the devices’ A-IoT NAS messages/A-IoT data, Figure 3 takes solution #33 as an example.

Figure 1 Solution #3 upper layer above PDU layer

Figure 2 Solution #4 upper layer above NAS layer

Figure 3 Solution #33 RRC based solution

Among those solutions, upper layer PDU layer based solution and upper layer NAS based solution can be seen as upper layer based solution. And those solutions have minimal RAN2 impact, and the major work is in SA2, potentially involving the CT group. For instance, in solution #3/4/22/32, the data for AIoT services are above PDU layer and it can be seen as normal data of intermediate UE. And in solution #4/11/12/21/30, the data for AIoT services is in NAS layer or above NAS layer and it can be seen as NAS messages of intermediate UE.

For RRC solution, AIoT data can be piggybacked in RRC messages. And the gNB receives the data from CN node and send the information to intermediate UE via dedicated Uu RRC messages. And the intermediate UE also receives the data from AIoT device and send the information to the gNB via dedicated Uu RRC messages. So RAN2 needs to specify this part. And the major spec impact is in RAN2 and the CN impact could be minimized.

But how to select those solutions may depend on SA2. So we can analyse the spec impact of different solutions and give our views.

Observation 1: According to SA2 TR, there are three types of protocol stacks: 1) UP based solutions; 2) NAS based solutions; 2) RRC based solutions.

Observation 2: For UP and NAS based solutions, there is minor RAN2 impact.

Observation 3: For RRC based solutions, it is necessary to support the transmission of AIoT data via Uu RRC messages between gNB and intermediate UE.

2.2 Uu link spec impact

For topology 2, the AIoT link is no difference in physical layer, RAN2 protocol stack from topology 1. For topology 2, since the UE acts as an intermediate node, the link between intermediate UE and gNB would reuse the legacy Uu interface as much as possible, with minimum improvements if deemed necessary.

Proposal 1: In topology 2, reuse the legacy Uu interface for the interface between the intermediate UE and gNB with minimum improvements.

2.2.1 Resource allocation for AIoT link

For resource allocation for AIoT link, in last RAN2 meeting, SL mode2-like resource allocation is discussed. In our view, SL mode-2 resource allocation requires device to perform sensing procedure to avoid interference, which bring RAN1 additional spec impact.

The transmission resource of AIoT link is allocated by gNB. In topology 1, gNB allocates the D2R transmission resources to AIoT device. In topology 2. How to allocate R2D and D2R resources is an issue that needs to be solved. For R2D and D2R transmission resources, gNB needs to allocate R2D and R2D resources used in AIoT link to intermediate UE. The above R2D and D2R resources can be either dynamically allocated or semi-persistent scheduling. The dynamic resource allocation incur a higher signalling overhead and also require the frequently Uu BSR reporting, which increases complexity. In addition, the gNB also can allocate a bulk of transmission resources to intermediate UE for R2D and D2R transmission, such as configuration grant in NR, which avoids frequent request and obtains resources from the gNB.

Proposal 2: The resource allocation for AIoT link, there are two options as following:

- Option 1: the gNB allocate a bulk of resources for R2D and D2R transmission at one time.

- Option 2: the gNB allocates R2D transmission resource and D2R transmission resource to intermediate UE for each transmission.

2.2.2 Traffic differentiate in Uu link

For topology 2, the traffic on intermediate UE mainly contains signalling for inventory and command, and the DL/UL data. the AIoT R2D traffic, when the intermediate UE reader receive the inventory/command request, it will trigger AIoT related procedure, such as paging, random access, data communication procedure. After RAN-related procedure, intermediate UE will forward AIoT link response data to gNB, such as device ID or command response.

According to SA2 progress, there are three types of data transmission solutions: 1) upper layer data transmission via user plane solutions; 2) upper layer data transmission via control plane solutions; 3) RRC layer based data transmission.

For upper layer data transmission via user plane solutions, the A-IoT messages is transmitted as user plane data. And gNB is unaware of the contents of the AIoT message. And upper layer data transmission via control plane solutions, AIoT messages is transmitted as NAS PDU. And gNB also is unaware of the contents of the AIoT messages. And for RRC-based solutions, AIoT message is transmitted as RRC signalling. If those AIoT messages are seen as normal data/NAS/RRC messages of intermediate UE, it will incur frequent BSR or SR report to gNB to request UL resources, especially for inventory response (amount of devices response the device ID to CN node). So the intermediate UE needs to differentiate the AIoT traffic from its own traffic based on the configuration from the gNB, for example, gNB configures different LCID/LCG ID/SR configuration for relayed AIoT traffic and the traffic of the intermediate node.

Proposal 3: gNB configures different LCID/LCG ID/SR configuration for relayed AIoT traffic and the traffic of the intermediate node.

2.2.3 The state of intermediate UE

Another issue that needs to be discussed is: what is the RRC status of the intermediate node UE?

The intermediate UE needs to receive the AIoT request from gNB and transmit the AIoT response to gNB. So the intermediate UE can work normally when the UE is in the RRC_Connected state during inventory or command procedure. Furthermore, if the UE is in the RRC_Inactive state, it needs to be warked up and moved to RRC_Connected state. If the intermediate UE is in the RRC_Idle state, when there is AIoT data, the intermediate UE need to frequently transfer to the RRC_Connected state, which increases additional signalling overhead. Thus, in our opinion, the intermediate UE should not be in the RRC_Idle state.

Proposal 4: In R19, the intermediate UE should be in RRC_Connected state during inventory/command procedures.

2.2.4 The selection of the intermediate UE

The selection of the intermediate UE can be performed based on device specific information (e.g. geographic location of the AIoT devices, the identity of the intermediate node from AF, AIoT device capability and the UE reader capability etc.) by CN or by RAN [2]. For the selection by CN option, the selected UE readers can be informed to gNB. There is no RAN2 spec impact. For the selection by RAN option, the assistance information from CN node may be needed to select UE reader. So RAN2 can wait for SA2 and RAN3 progress to discuss whether/how to select the intermediate UE.

Proposal 5: Discussion on whether/how to select the intermediate UE in RAN2 can wait for the progress of SA2/RAN3.

Conclusions

In this contribution, we discuss the Rel-19 Topology 2 related enhancements. And the proposals are given as follows:

Observation 1: According to SA2 TR, there are three types of protocol stacks: 1) UP based solutions; 2) NAS based solutions; 2) RRC based solutions.

Observation 2: For UP and NAS based solutions, there is minor RAN2 impact.

Observation 3: For RRC based solutions, it is necessary to support the transmission of AIoT data via Uu RRC messages between gNB and intermediate UE.

Proposal 1: In topology 2, reuse the legacy Uu interface for the interface between the intermediate UE and gNB with minimum improvements.

Proposal 2: The resource allocation for AIoT link, there are two options as following:

- Option 1: the gNB allocate a bulk of resources for R2D and D2R transmission at one time.

- Option 2: the gNB allocates R2D transmission resource and D2R transmission resource to intermediate UE for each transmission.

Proposal 3: gNB configures different LCID/LCG ID/SR configuration for relayed AIoT traffic and the traffic of the intermediate node.

Proposal 4: In R19, the intermediate UE should be in RRC_Connected state during inventory/command procedures.

Proposal 5: Discussion on whether/how to select the intermediate UE in RAN2 can wait for the progress of SA2/RAN3.

References

[1] RP-240826, Revised Rel-19 Ambient IoT SID, RAN #103 meeting.

[2] TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)