3GPP TSG-RAN WG2 Meeting #127-bis 	R2-2408173

Hefei, China, 14th – 18th Oct, 2024

Agenda Item:	8.5.3

Source:	Spreadtrum Communications

Title:	Discussion on on-demand SIB1 for NES

Document for:	Discussion and Decision

Introduction

Based on the outcome of RAN#105 meeting, on-demand SIB1 for case 2 was successfully transferred to WI phase for standardization. The following objectives were agreed in the WID (RP-242354):

The objectives of the work item are the following: 2. Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In this contribution, we will discussion the issues related to procedures and signaling method for case 2, e.g. which SIB includes UL WUS config for cell A and NES cell, access restriction for legacy UEs and provide our opinions.

Discussion

2.1 UL WUS configuration

Agreements on OD-SIB1:

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

In WI phase, RAN2 focus on the procedure and signalling design for case 2. In SI phase, RAN2 agreed that Cell A’s new SIB or the existing SIB can be used to configure OD SIB1 related configuration. One of the possible options is to include UL WUS configuration for NES cell into SIB1 or other existing SIBx of Cell A. Considering the design target of SIB1, SIB1 only carries the essential information concerning the cell in which it is transmitted. And the UL WUS configuration of NES cell cannot be considered essential information of Cell A. And for OD SIB1 provisioning for NES cell, the same cell A may contain multiple UL WUS configuration of multiple NES cell. The size of multiple UL WUS configuration may be very large. If the existing SIBx is used to carry the UL WUS configuration, the size of the existing SIBx will be significantly increased, and because the SIB has a maximum size limit, the size of the UL WUS configuration is also limited. So using new SIB to carry UL WUS configuration is more suitable.

Proposal 1: UL WUS configuration for the NES Cell is provided in a new SIB of Cell A.

2.2 The impact to legacy UE

Agreement：

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

For access restriction for legacy UEs, in SI phase, RAN2 analysis three example options and there is no consensus. For option 1, because legacy UE cannot access the OD-SIB1 cell without SIB1. Even emergency call service cannot access OD-SIB1 cell. So the legacy UE can directly use the cell barred in the MIB to bar the OD-SIB1 cell.

For option 2, RAN1 needs to first agree that ssb-SubcarrierOffset in MIB indicates no SIB1 of a cell. According to the current RAN1 discussion, there are two ways to indicate OD-SIB1 cell by PBCH payload of NES cell (the yellow part of the below table). For the first way, the meaning of the reserved k_SSB value needs to be reinterpreted. Not sure if all Legacy UE can understand the meaning of the reinterpreted value of k_SSB. For the second way, lecagy UE cannot understand the meaning of the new parameters. So in our view, it not sure that all legacy UEs can bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

### FL Proposal 7-1 RAN1 to further study UE identification of NES cell with on-demand SIB1 based on both of the following options: - Option 1: By WUS configuration - Cell(s) that the UL WUS configuration(s) applies to are NES cell(s) - Option 2: By PBCH payload of NES cell - FFS: How to use the (reserved) k_SSB values to indicate a NES cell with on-demand SIB1 - E.g. k_SSB for FR1 and k_SSB =14 for FR2 - FFS: Whether to introduce a new parameter to distinguish Case 1/2 (on-demand SIB1 from NES cell) and Case 3 (on-demand SIB1 from Cell A)

And for option 3, network needs to configure a list of excluded cells which does not support OD-SIB1. In current spec, exclude-listed cell in measurement object or SIB3/4 is used for measurement/cell reselection to exclude the neighbour cells [3]. And in TS 38.304, the spec stipulates that the UE shall not consider any exclude-listed cells as candidate for cell reselection [4]. If network to configure an OD-SIB1 cell to a list of excluded cells, the protocol description needs to be relaxed for R19 OD-SIB1 UE, for example, R19 UE is able to access excluded cell. Or add an excluded cell list for the OD-SIB1 cell in SIB3/4 or the measurement target, which has great spec impact. So in our opinion, this option is not a suitable solution.

Proposal 2: For access restriction for legacy UEs, Option 1 is preferred.

Conclusions

In this contribution, we discuss the Rel-19 on-demand SIB1 operation. And the proposals are given as follows:

Proposal 1: UL WUS configuration for the NES Cell is provided in a new SIB of Cell A.

Proposal 2: For access restriction for legacy UEs, Option 1 is preferred.

References

[1] RAN2 #125bis meeting chair notes.

[2] RAN2 #126 meeting chair notes.

[3] TS 38.331 v18.1.0, RRC specification.

[4] TS 38.304 v18.1.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state.