3GPP TSG-RAN WG2 Meeting #127bis                                                            R2-2408181

Hefei, China, Oct 14th – Oct 18th, 2024

Source:	CATT

Title:	Discussion on MAC CE impact for PL offset updates

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- 1. Introduction

In RAN1#118 meeting, RAN1 discussed the issue of PL offset configuration and update, and sent the LS [1] to RAN2 as follows:

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). “MAC-CE impact: MAC-CE updates the PL offset value(s) configured in one or more joint TCI states or UL TCI states”

In this contribution, we will analyze the MAC CE impact for PL offset updates.

- 2. Discussion

When to update the PL offset

Based on RAN1 agreement, PL offset associated with joint or UL TCI state(s) can be configured by RRC and updated by MAC CE. In current MAC Spec, the MAC CE can activate and deactivate the joint or separate TCI state(s). Thus, there are two options regarding when to update the PL offset.

- Option1: PL offset is updated together with the corresponding joint or separate TCI state(s) activation MAC CE

- Option2: PL offset update MAC CE is separate with the joint or separate TCI state(s) activation MAC CE

According to RAN1 discussion, PL offset is updated due to, e.g., UE movement, in the scenario of asymmetric DL sTRP/UL mTRP deployment. Different UE locations may cause different PL offset between DL PL and UL PL. Thus, the PL offset may be updated after activating the corresponding joint or UL TCI state(s). And it’s meaningless that PL offset is updated when there is no any activated joint or UL TCI state(s) for the UE. Thus, Option 2 is more flexible than Option1. We have following proposals.

Proposal 1: For PL offset updates, a new MAC CE is designed separately with the joint or separate TCI state(s) activation MAC CE(s).

Proposal 2: PL offset is updated after the corresponding joint or UL TCI state(s) activation.

Value range of PL offset

In RAN1#118 meeting, RAN1 discuss the value range of PL offset and made the following agreement.

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB - FFS: Extending the range to lower than -10dB - Step size is 4dB

The value range of PL offset includes [-10, 60]dB and Step size is 4dB. Whether or not to extend the range to lower than -10dB is still under discussing by RAN1. Based on the current RAN1 agreement, 5bits in MAC CE is enough to represent the value range of PL offset, even though RAN1 agree to extend the range to lower than -10dB.

Proposal 3: The length of PL offset field is 5bits in the new MAC CE.

The number of updated PL offset in the new MAC CE

RAN1 made following agreements related with PL offset in RAN1#116 and RAN1#116bis.

Agreement For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset: - When a UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this UL TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset. - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW. - Note: it does not intend to increase the number of maintained PLs per cell. - FFS: whether to support associating joint TCI state (if supported) with a PL offset. Further study whether/how to apply a PL offset on PDCCH-order PRACH transmission too. - FFS: how to determine the Tx beam of PRACH towards UL TRP - Note: this does not imply to support 2 TA for single-DCI based system. Agreement For FR1, a joint TCI state can be associated with a PL offset. - When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset. - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

According to the above RAN1 agreements, the PL offset is used for the asymmetric DL sTRP/UL mTRP deployment scenarios. UE shall calculate the Tx power of the UL TRP based on the DL PL RS sent by DL TRP and PL offset associated with this joint TCI state or UL TCI state. Thus, for each TCI state codepoint, there is one PL offset associated with the first/second joint TCI state or UL TCI state. The max number of updated PL offsets is 8, which is the same as the max number of activated TCI state codepoints. PL offset is updated due to, e.g., UE movement. Thus, not all of the 8 PL offsets are always updated simultaneously. The number of updated PL offset(s) in the MAC CE is depended on NW implementation and the max number is 8.

Proposal 4: The number of updated PL offsets in the new MAC CE is variable, which is depended on NW implementation.

Proposal 5: The max number of updated PL offsets in the new MAC CE is 8, which is the same as the max number of activated TCI state codepoints.

How to indicate the updated PL offset in the new MAC CE

Based on the above discussion, the PL offset is updated after the corresponding joint or UL TCI state(s) activation and the number of updated PL offsets in the new MAC CE is variable. Thus, a bit map is preferred to indicate which PL offset is updated associated with the corresponding codepoint of the joint or UL TCI state(s) which is activated by the last joint or separate TCI state(s) activation MAC CE. There is no need to include the TCI state ID, since it’s already indicated in the last joint or separate TCI state(s) activation MAC CE. According to Proposal 5, the length of the bit map is 8 bits.

Proposal 6: A bit map in the new MAC CE is used to indicate which PL offset is updated associated with the corresponding codepoint of the joint or UL TCI state(s) which is activated by the last joint or separate TCI state(s) activation MAC CE.

Proposal 7: The length of the bit map is 8 bits.

Proposal 8: There is no need to include the TCI state ID in the new MAC CE.

Other fields in the new MAC CE

Similarly with the joint or separate TCI state(s) activation/deactivation MAC CE, the Serving Cell ID is needed in the new MAC CE to indicate the identity of the Serving Cell for which the MAC CE applies. The length of the Serving Cell ID field is 5 bits. Moreover, DL/UL BWP ID is also needed in the new MAC CE to indicate a DL/UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212. DL BWP ID is referred to the joint TCI state(s) which PL offset associated with, while UL BWP ID is referred to the UL TCI state(s). However, the PL offset is updated after the corresponding joint or UL TCI state(s) activation. And a UE cannot be configured joint and separate TCI states simultaneously. Thus, the UE knows what kind of TCI state(s), i.e., joint or UL, is associated with the PL offset in the new MAC CE. Only including one field of DL/UL BWP ID is enough in the new MAC CE. The length of the DL/UL BWP ID field is 2 bits.

Proposal 9: Serving Cell ID is included in the new MAC CE to indicate the identity of the Serving Cell for which the MAC CE applies. The length of the Serving Cell ID field is 5 bits.

Proposal 10: One field of DL/UL BWP ID is included in the new MAC CE to indicate a DL/UL BWP for which the MAC CE applies. The length of the DL/UL BWP ID field is 2 bits.

The detail design of the new MAC CE

In Summary, according to the above analysis, we propose the new MAC CE for PL offset updates as Figure 1. In Figure 1, Fi, where i=1, 2,…, 8, is the bit map to indicate which PL offset is updated associated with the corresponding codepoint of the joint or UL TCI state(s) which is activated by the last joint or separate TCI state(s) activation MAC CE. If Fi field is set to 1, it indicates the i-th PL offset for codepoint i is present. If Fi field is set to 0, it indicates the i-th PL offset for codepoint i is absent. The maximum number of updated PL offset is 8.

Figure 1: The new MAC CE for PL offset updates

Proposal 11: Adopt Figure 1 as the new MAC CE for PL offset updates.

- 3. Conclusion

This paper discusses the MAC CE impact for PL offset updates. The contribution of this paper is summarized as follows.

Proposal 1: For PL offset updates, a new MAC CE is designed separately with the joint or separate TCI state(s) activation MAC CE(s).

Proposal 2: PL offset is updated after the corresponding joint or UL TCI state(s) activation.

Proposal 3: The length of PL offset field is 5bits in the new MAC CE.

Proposal 4: The number of updated PL offsets in the new MAC CE is variable, which is depended on NW implementation.

Proposal 5: The max number of updated PL offsets in the new MAC CE is 8, which is the same as the max number of activated TCI state codepoints.

Proposal 6: A bit map in the new MAC CE is used to indicate which PL offset is updated associated with the corresponding codepoint of the joint or UL TCI state(s) which is activated by the last joint or separate TCI state(s) activation MAC CE.

Proposal 7: The length of the bit map is 8 bits.

Proposal 8: There is no need to include the TCI state ID in the new MAC CE.

Proposal 9: Serving Cell ID is included in the new MAC CE to indicate the identity of the Serving Cell for which the MAC CE applies. The length of the Serving Cell ID field is 5 bits.

Proposal 10: One field of DL/UL BWP ID is included in the new MAC CE to indicate a DL/UL BWP for which the MAC CE applies. The length of the DL/UL BWP ID field is 2 bits.

Proposal 11: Adopt Figure 1 as the new MAC CE for PL offset updates.

- 4. Reference

- R2-2407906, LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5 (R1-2407285; contact: Samsung)