3GPP TSG-RAN WG2 Meeting #127bis                                                               R2-2408183

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

In RAN2#127 meeting, the following agreements were achieved on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE.

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

In this contribution, we give our further consideration on RRM measurement offloading/relaxation for LP-WUS UEs in RRC_IDLE/INACTIVE.

Discussion

Serving cell measurement offloading

- MR neighbor cell measurement

In RAN4#111 meeting, RAN4 agreed the MR serving cell measurement fully offloading case as follows.

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON.

According to the agreed case#1 fully offloading case in RAN4, it is not required to perform MR neighboring cell measurement either. In our understanding, MR neighbor cell measurements on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority are not required when MR serving cell measurement is offloading to LR. But whether MR neighbor cell measurements on NR inter-frequency cells of higher priority, or inter-RAT frequency cells of higher priority can be stopped depends on RAN4’s further discussion.

Proposal 1：When the criterion for serving cell measurement offloading is fulfilled (i.e., there is no serving cell measurement by MR), it is not required to perform measurement for MR neighbor cell on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

- Relationship between Serving cell measurement offloading and MR neighbor cell measurement off

In legacy, if the MR serving cell is good enough (e.g. in cell center), the UE may not perform neighbor cell measurements including [1]:

-	If the serving cell fulfills Srxlev > SIntraSearchP (i.e. s-IntraSearchP broadcast in SIB2) and Squal > SIntraSearchQ (i.e. s-IntraSearchQ broadcast in SIB2), the UE may not perform intra-frequency measurements.

-	If the serving cell fulfills Srxlev > SnonIntraSearchP (i.e. s-NonIntraSearchP broadcast in SIB2) and Squal > SnonIntraSearchQ (i.e. s-NonIntraSearchQ broadcast in SIB2), the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

In RAN2#117 meeting, RAN2 agreed that the entry conditions for serving cell measurement offloading (i.e., there is no serving cell measurement by MR) can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. The entry conditions for serving cell measurement offloading should not be worse than conditions for legacy MR neighbor cell measurement off on intra-frequency, NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority, otherwise it is against the legacy principle of MR neighbor cell measurement off. Therefore, MR RSRP threshold in the entry conditions for serving cell measurement offloading should be better than or equal to s-IntraSearchP and s-NonIntraSearchP, as shown in Figure 1. And before entry conditions for serving cell measurement offloading are fulfilled, the UE can follow legacy criteria for MR neighbor cell measurement off, i.e. if the MR serving cell fulfills Srxlev > SIntraSearchP (i.e. s-IntraSearchP broadcast in SIB2) and Squal > SIntraSearchQ (i.e. s-IntraSearchQ broadcast in SIB2), the UE may not perform MR intra-frequency measurements; If the MR serving cell fulfills Srxlev > SnonIntraSearchP (i.e. s-NonIntraSearchP broadcast in SIB2) and Squal > SnonIntraSearchQ (i.e. s-NonIntraSearchQ broadcast in SIB2), the UE may choose not to perform MR measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

Proposal 2: MR RSRP threshold in the entry conditions for serving cell measurement offloading (i.e., there is no serving cell measurement by MR) should be better than or equal to s-IntraSearchP and s-NonIntraSearchP.

Proposal 3: Before entry conditions for serving cell measurement offloading are fulfilled, the UE can follow legacy criteria for MR neighbor cell measurement on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

- Relationship between serving cell measurement offloading and LP-WUS monitoring

According to RAN2 agreement, entry/exit conditions for LP-WUS monitoring are similar to those for serving cell measurement offloading. For the relationship between serving cell measurement offloading and LP-WUS monitoring, the possible options are:

- Option 1: Entry/exit conditions for serving cell measurement offloading should be better than or equal to those for LP-WUS monitoring.

- Option 2: Entry/exit conditions for serving cell measurement offloading should be worse than or equal to those for LP-WUS monitoring.

Entry conditions for LP-WUS monitoring are MR serving cell quality above threshold(s) and LR-based thresholds can also be included. With option 2, if entry/exit conditions for serving cell offloading are worse than those for LP-WUS monitoring, it is unclear how entry conditions for LP-WUS monitoring can be fulfilled considering MR serving cell measurement is off during serving cell offloading. Therefore, option 2 is not feasible. Option 1 is preferred, and one example is shown in Figure 1.

Proposal 4: Entry/exit conditions for serving cell measurement offloading should be better than or equal to those for LP-WUS monitoring.

Figure 1: One example of the serving cell measurement offloading coverage

RRM measurement relaxation on MR

In RAN4#112 meeting, RAN4 agreed case #3 as follows.

Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

RAN2 can discuss the related measurement criteria of case 3. For the criteria of case 3, there are two possible alternatives:

-	Alt1: Same criteria are configured for both MR serving cell relaxation measurement and MR neighbor cell relaxation measurement.

For R16 RRM relaxation, network configures one set of parameters for ‘not at cell edge’ and/or ‘low mobility’.  The same criteria for both ‘not at cell edge’ and ‘low mobility’ are applied to neighbor cell RRM relaxation regardless of whether the frequency priority of neighbor cell is higher, equal or lower. The similar principle is reused in Alt1. With this alternative, one set of parameters for criteria of case 3 are broadcast in SIB. When corresponding criteria are fulfilled, both MR serving cell relaxation measurement and MR neighbor cell relaxation measurement can be performed.

-	Alt2: Separate criteria are configured for both MR serving cell relaxation measurement and MR neighbor cell relaxation measurement.

With this alternative, different criteria or same criteria may be applied to both MR serving cell relaxation measurement and MR neighbor cell relaxation measurement. Even if the same criteria (e.g. not at cell edge) are applied to both MR serving cell relaxation measurement and MR neighbor cell relaxation measurement, two possible separate sets of parameters for criteria of case 3 are broadcast in SIB, e.g. one set of parameters for criteria (e.g. not at cell edge) of MR serving cell relaxation measurement and another set of parameters for criteria (e.g. not at cell edge) of MR neighbor cell relaxation measurement. And it is possible that UE is not allowed to perform MR serving cell relaxation measurement while the UE is allowed to perform MR neighbor cell relaxation measurement, and vice versa. According to the above RAN4 discussion, these possible cases, e.g. the UE is not allowed to perform MR serving cell relaxation measurement (i.e. the UE perform normal MR serving cell measurement) while the UE is allowed to perform MR neighbor cell relaxation measurement, are not in the discussed in RAN4.

Alt1 is simple, while Alt2 provides the flexibility, which can ensure MR serving cell quality but the power saving gain is reduced if UE is not allowed to perform MR serving cell relaxation measurement while the UE is allowed to perform MR neighbor cell relaxation measurement. Suggest RAN2 to discuss which option is preferred.

Proposal 5: Suggest RAN2 to discuss which alternative is adopted for the criteria of MR serving cell relaxation measurement and MR neighbor cell relaxation measurement:

-	Alt1: Same criteria are configured for both MR serving cell relaxation measurement and MR neighbor cell relaxation measurement.

-	Alt2: Separate criteria are configured for both MR serving cell relaxation measurement and MR neighbor cell relaxation measurement.

Criteria of MR relaxation measurement:

For MR neighbor cell relaxation measurement, the following understandings were achieved in RAN2#126 meeting:

RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

RAN2 confirm criteria of ‘not at cell edge’ and ‘low mobility’ are reused as the criteria for MR neighbor cell relaxation measurement.

Proposal 6: RAN2 confirm the criteria of ‘not at cell edge’ and ‘low mobility’ are reused as the criteria for MR neighbor cell relaxation measurement (if the UE is using LR to measure the serving cell).

Similar to entry/exit conditions for serving cell measurement offloading and LP-WUS monitoring, the criterion of ‘not at cell edge’ for MR neighbor cell relaxation measurement includes:

- Entry conditions can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

- The exit condition is based on LR measurement results.

Proposal 7: For the criterion of ‘not at cell edge’ for MR neighbor cell relaxation measurement:

-	Entry conditions can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

-	The exit condition is based on LR measurement results.

And it is possible that both serving cell measurement offloading and RRM measurement relaxation on MR are configured in one cell. When both the criteria of serving cell measurement offloading and ‘not at cell edge’ for MR neighbor cell relaxation measurement are configured in one cell, it is straightforward that entry/exit conditions for serving cell measurement offloading should be better than those for ‘not at cell edge’ for MR neighbor cell relaxation measurement, as shown in Figure 2. When a LP-WUS UE moves into cell center of a cell, ‘not at cell edge’ for MR neighbor cell relaxation measurement maybe fulfilled at first. Then legacy criteria for MR neighbor cell measurement off on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority (i.e. s-IntraSearchP and s-NonIntraSearchP) may be fulfilled. Finally, serving cell measurement offloading maybe fulfilled. On the other hand, when the UE moves out of the coverage of serving cell measurement offloading, and legacy criteria for MR neighbor cell measurement on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority (i.e. s-IntraSearchP and s-NonIntraSearchP) may be fulfilled at first, then ‘not at cell edge’ for MR neighbor cell relaxation measurement maybe fulfilled.

Proposal 8: When both the criteria of serving cell measurement offloading and ‘not at cell edge’ for MR neighbor cell relaxation measurement are configured in one cell, entry/exit conditions for serving cell measurement offloading should be better than those for ‘not at cell edge’ for MR neighbor cell relaxation measurement.

Figure 2: One example of the ‘not at cell edge’ for MR neighbor cell relaxation measurement

The following is the R16 relaxed measurement criterion for UE with low mobility which is based on MR. The similar criterion can be used for ‘low mobility’ for MR neighbor cell relaxation measurement. But if ‘low mobility’ for MR neighbor cell relaxation measurement is still based on MR serving cell measurement, TSearchDeltaP for low mobility need to be longer enough considering MR serving cell measurement may be relaxed at the same time and UE mobility state cannot be updated timely. Therefore, it makes sense to consider ‘low mobility’ for MR neighbor cell relaxation measurement based on LR.

5.2.4.9.1 Relaxed measurement criterion for UE with low mobility The relaxed measurement criterion for UE with low mobility is fulfilled when: - (SrxlevRef – Srxlev) < SSearchDeltaP, Where: - Srxlev = current Srxlev value of the serving cell (dB). - SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows: - After selecting or reselecting a new cell, or - If (Srxlev - SrxlevRef) > 0, or - If the relaxed measurement criterion has not been met for TSearchDeltaP: - The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

Proposal 9: Criterion of ‘low mobility’ for MR neighbor cell relaxation measurement is based on LR.

Conclusion

According to the analysis in section 2, we propose:

Serving cell measurement offloading:

Proposal 1：When the criterion for serving cell measurement offloading is fulfilled (i.e., there is no serving cell measurement by MR), it is not required to perform measurement for MR neighbor cell on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Proposal 2: MR RSRP threshold in the entry conditions for serving cell measurement offloading (i.e., there is no serving cell measurement by MR) should be better than or equal to s-IntraSearchP and s-NonIntraSearchP.

Proposal 3: Before entry conditions for serving cell measurement offloading are fulfilled, the UE can follow legacy criteria for MR neighbor cell measurement on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Proposal 4: Entry/exit conditions for serving cell measurement offloading should be better than or equal to those for LP-WUS monitoring.

RRM measurement relaxation on MR:

Proposal 5: Suggest RAN2 to discuss which alternative is adopted for the criteria of MR serving cell relaxation measurement and MR neighbor cell relaxation measurement:

-	Alt1: Same criteria are configured for both MR serving cell relaxation measurement and MR neighbor cell relaxation measurement.

-	Alt2: Separate criteria are configured for both MR serving cell relaxation measurement and MR neighbor cell relaxation measurement.

Proposal 6: RAN2 confirm the criteria of ‘not at cell edge’ and ‘low mobility’ are reused as the criteria for MR neighbor cell relaxation measurement (if the UE is using LR to measure the serving cell).

Proposal 7: For the criterion of ‘not at cell edge’ for MR neighbor cell relaxation measurement:

-	Entry conditions can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

-	The exit condition is based on LR measurement results.

Proposal 8: When both the criteria of serving cell measurement offloading and ‘not at cell edge’ for MR neighbor cell relaxation measurement are configured in one cell, entry/exit conditions for serving cell measurement offloading should be better than those for ‘not at cell edge’ for MR neighbor cell relaxation measurement.

Proposal 9: Criterion of ‘low mobility’ for MR neighbor cell relaxation measurement is based on LR.

Reference

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state;