3GPP TSG-RAN WG2 Meeting #127bis	R2-2408189

Hefei, China, Oct. 14th – Oct. 18th, 2024

Agenda Item:	8.5.2

Source: 	Xiaomi

Title:  	Discussion on on-demand SSB

Document for:	Discussion and Decision

1	Introduction

In last RAN2 meeting, we had some initial discussion on OD-SSB and achieved the following agreement.

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

In this contribution, we would like to further discuss the procedures and signalling to support on-demand SSB and have corresponding observations and proposals.

2	Discussion

2.1 Activation/deactivation MAC CE

Scenario#2 (Scell added but not activated)

According to the following RAN1 agreement, MAC CE to indicate the activation of SSB transmission of the cell is supported, while deactivation/adaptation should be further discussed.

Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

In last RAN2 meeting, we discussed the L3 measurement impact with OD-SSB and agreed UE can skip measurement when SSB is deactivated for case 1. Therefore, deactivation of the SSB transmission should be indicated to UE as well.

Agreements on OD-SSB Measurement on OD-SSB: - For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated

Proposal 1: RAN2 to confirm both activation and deactivation of SSB transmission needs to be indicated to UE.

One byte/four bytes bitmap can be reused to indicate the activation/deactivation status of SSB transmission per SCell. In addition, RAN1 has agreed to indicate at least the applicable periodicity with MAC CE, therefore, besides bitmap, additional information associated with the OD-SSB transmission should be included in the MAC CE.  If only few parameters to be additionally indicated, these parameters can be explicated included while if there are multiple parameters, an ID may be used to associate with a set of applicable parameters and including the ID is sufficient. However, since the other parameters are still pending RAN1 discussion, how to indicate the additional information can be addressed later when RAN1 has final conclusion.

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

Therefore, the following MAC CE format can be utilized as a baseline and how to indicate the additional SSB information can be further discussed.

Figure 1: SSB activation/deactivation MAC CE with one octet Ci field

Figure 2: SSB activation/deactivation MAC CE with four octet Ci field

Proposal 2: SSB activation/deactivation MAC CE includes one byte/four bytes bitmap and additional SSB information field associated with each activated SSB transmission. The detailed design of the additional SSB information field depends on RAN1 conclusion.

Scenario#2A (Scell activation)

Another issue is whether to introduce any optimization for Scenario#2A, based on discussion from last meeting, there are two candidate options listed as below.

- Alternative 1: separate MAC CE for OD-SSB transmission activation/deactivation and SCell activation/deactivation

For this alternative, the MAC CE applied for Scenario#2 can be reused for Scenario#2A to achieve a unified solution. However, with separate MAC CE, it may need to rely on NW implementation to ensure these two MAC CE are transmitted in the same MAC PDU, otherwise, there may be some impact on the time requirement of SCell activation, which needs to further check with RAN4. In addition, if it is hard to always ensure simultaneous transmission of these two MAC CE, the corresponding UE behaviour needs to be specified when they are not received simultaneously, e.g., the OD-SSB activation/deactivation MAC CE arrives earlier than the SCell activation/deactivation MAC CE.

- Alternative 2: single MAC CE for joint OD-SSB transmission activation/deactivation and SCell activation/deactivation

For this alternative, single MAC CE is used to indicate both OD-SSB activation/deactivation and SCell activation/deactivation. In last meeting, there was proposal proposed to have two byte bitmap with one for OD-SSB transmission indication and the other for SCell status indication. However, we think two byte bitmap may not be necessary if the OD-SSB transmission always has the same activation/deactivation status as SCell, i.e., when SCell is activated/deactivated, the corresponding OD-SSB is activated/deactivated. Some companies may argue for case 2, OD-SSB transmission may not be activated upon SCell activation since there is always-on long period SSB. However, we think even for case 2, OD-SSB should be transmitted to accelerate the SCell activation procedure upon SCell activation. Therefore, we think single bitmap is sufficient, but we can also check with RAN1 if needed.

Based on the above analysis, we think for alternative 1, we need to check with RAN4 on the impact of SCell activation latency if these two MAC CEs are not transmitted in the same MAC PDU while for alternative 2, whether the status of SSB is always the same as that of SCell or not should be further check with RAN1. Based on the discussion in last meeting, a majority of companies prefer alternative 1, therefore, we think we can firstly check with RAN4 if separate MAC CE has any drawback on SCell activation or not. A draft LS attached can be used as a baseline.

Proposal 3: Send LS to RAN4 to check the impact on SCell activation latency if separate MAC CEs are defined and these two MAC CEs are not multiplexed in the same MAC PDU.

2.2 Dedicated deactivation MAC CE

As discussed in the previous section, deactivation of SSB transmission needs to be indicated to UE as well. To reduce signalling overhead, a dedicated deactivation MAC CE can be considered. This is because if the same MAC CE to indicate both the activation and deactivation, when the NW only needs to deactivate some SSB transmission, the indication bit associated with the activated SSB needs to be set to “1” and the corresponding additional information field needs to present, which is a waste of signalling especially for 32 CC aggregation.

Observation 1: With the same MAC CE to indicate both the activation and deactivation of SSB, it is a waste of signalling when only deactivation of SSB on some SCell is required.

Companies may argue for fast SCell activation/deactivation, additional field to indicate the TRS information is also defined, but no dedicated SCell deactivation was defined in the past. However, we think different from SCell activation/deactivation, the activation/deactivation of SSB may be triggered variously and quite dynamically/frequently, e.g., before/during/after SCell activation for L1/L3 measurement and/or DL sync etc., where the usage of this MAC CE is more often than SCell activation/deactivation and makes more sense to consider the signalling overhead.

With this dedicated deactivation MAC CE, when NW only performs deactivation of SSB transmission on some SCells, this MAC CE can be used while if both deactivation of SSB on some SCells and activation of SSB on some other SCells are required simultaneously, the activation/deactivation MAC CE as discussed in previous section can be used. In addition, for this dedicated deactivation MAC CE, one byte/four bytes bitmap is enough with the indication bit setting to “1” to indicate the deactivation of the corresponding SSB transmission while setting to “0” should be ignored.

Proposal 4: RAN2 to discuss the introduction of dedicated deactivation MAC CE when only deactivation of SSB transmission is performed.

2.3 RRC impact

No matter of which scenario and which trigger to be supported, UE should be informed of whether a SCell is a normal SCell or on-demand SSB SCell. This is because without this information, UE may derive incorrect measurement results with the assumption that the SCell is normal SCell and SSB is transmitted. It may cause problems on RRM procedure as the accuracy of measurement may be significantly degraded. Therefore, we think upon SCell addition, NW should indicates whether the SSB transmission on the SCell is on-demand manner.

Proposal 5: Upon SCell addition, NW indicates whether the SSB transmission on this SCell is on-demand manner via RRC signalling.

In last meeting, we agreed the RRC signalling can be used to indicate the initial activation/deactivation status of the SSB transmission. While whether to support reconfiguration via RRC or not is FFS.

RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

Actually this parameter is quite similar as sCellState, which is defined in R16 and included in SCellConfig to indicate the activation/deactivation status of the configured SCell during SCell configuration. Since SCellConfig can be utilized to modify/reconfigure a corresponding SCell configuration, during which, the sCellState can be reconfigured to indicate the SCell status.

1> for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification): 2> modify the SCell configuration in accordance with the sCellConfigDedicated; 2> if the sCellToAddModList was received in an RRCReconfiguration message including reconfigurationWithSync, or received in an RRCResume message, or received in an RRCReconfiguration message including reconfigurationWithSync embedded in an RRCResume message or embedded in an RRCReconfiguration message or embedded in an E-UTRA RRCConnectionReconfiguration message or embedded in an E-UTRA RRCConnectionResume message, or received in an RRCReconfiguration message embedded in an RRCReconfiguration message or embedded in an E-UTRA RRCConnectionReconfiguration message activating deactivated SCG: 3> if the sCellState is included: 4> configure lower layers to consider the SCell to be in activated state; 3> else: 4> configure lower layers to consider the SCell to be in deactivated state.

In addition, according to the following RAN1 agreement, the RRC indication for SSB is probably included in the SCellConfig to indicate the SSB transmission status when configures/activates the SCell. Therefore, similar as sCellState, we think this RRC indication should not be limited to only indicate the initial activation/deactivation status of SSB transmission on the cell, reconfiguration via this RRC signalling should be allowed at least when the NW reconfigures/modifies the SCell.

Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases.

Proposal 6: RRC based OD-SSB transmission indication can be used to reconfigure the activation/deactivation state of OD-SSB transmission.

2.4 L3 RRM measurement

In last RAN2 meeting, we discussed the impact of OD-SSB on L3 RRM measurement, e.g., UE cannot perform L3 RRM measurement towards the OD-SSB, if the OD-SSB is transmitted outside of the configured SMTC window. There was proposal to configure additional SMTC window for OD-SSB and UE performs adaptation between the configurations.

Measurement on OD-SSB: - RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration

Actually we think even in legacy, this issue may exist, since the SMTC is configured per frequency while the SSB transmission is configured per cell. When the transmission periodicity of SSB does not match with the SMTC window, some of the SSB transmission may fall outside of the SMTC window. For those SSB that are not covered by the SMTC window, UE shall not perform L3 measurement.

Observation 2: In legacy, due to the mismatch between the SMTC window and SSB transmission periodicity, some SSB may fall outside of the SMTC window.

With OD-SSB, which is triggered more dynamically, even this issue may happen more frequently but we can still rely on NW implementation to handle, since NW configures the SMTC and triggers the transmission of OD-SSB. One possible implementation is to configure a suitable SMTC value e.g., with shorter periodicity, to cover most of the always-on SSB and OD-SSB. For the OD-SSB outside of the SMTC, UE just ignores, as in legacy.

Observation 3: NW implementation may avoid this issue by configuring suitable SMTC value to cover most of the always-on SSB and OD-SSB.

In addition, RAN1 is discussing the higher layer configuration of the OD-SSB. During the discussion of the parameters, there was proposal to consider SMTC, if companies think it is hard to rely on NW implementation to avoid this issue, SMTC can be considered as one parameter associated with a OD-SSB configuration. Details can be up to RAN1 to discuss. Therefore, we think RAN2 can wait for more RAN1 progress.

## [Moderator’s note #1] Regarding contents of on-demand SSB configuration/indication, company views are summarized as follows. - Details on higher layer parameter signaling - Frequency of the on-demand SSB - Explicit RRC signaling: Huawei, Google, vivo, Samsung - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Explicit RRC signaling: Google, vivo, Samsung (also in on-demand SSB indication signaling) - Same as always-on SSB: Samsung - MAC CE: vivo - Huawei: New pattern? - Periodicity of the on-demand SSB - Explicit RRC signaling: Google, Samsung, NEC - MAC CE: vivo - Huawei: New value - Sub-carrier spacing of the on-demand SSB - Explicit RRC signaling: Huawei, Spreadtrum, vivo, Samsung - Same as always-on SSB: CMCC, Samsung - Physical Cell ID of the on-demand SSB - Explicit RRC signaling: Huawei, Spreadtrum, Samsung - Same as always-on SSB: Samsung - MAC CE: vivo - Location of on-demand SSB burst (or half-frame index of the on-demand SSB) - Implicitly known: LG Electronics - Explicit signaling: vivo, LG Electronics, Samsung (Up to RAN2?) - Downlink transmit power of on-demand SSB - Explicit RRC signaling: Huawei, CMCC, vivo, Samsung - Same as always-on SSB: Samsung - Others - # of SSB bursts: CMCC, ZTE, LG Electronics, Samsung (in on-demand SSB indication signaling), MediaTek - Transmission duration/window: ZTE, NEC, CMCC - On/Off status of on-demand SSB: CATT, NEC, Google (in on-demand SSB indication signaling) - Time gap: CMCC - Offset between always-on SSB and on-demand SSB: NEC - SMTC and timing reference cell: LG Electronics

Proposal 7: RAN2 to wait for more RAN1 progress on the configuration of SMTC for OD-SSB.

2.5 UE based trigger

As captured in the WID, there are three potential triggering methods for on-demand SSB:

- Method 1: Cell on/off indication via backhaul

- Method 2: Scell activation/deactivation signalling

- Method 3: UE uplink wake-up-signal

Both method 1 and method 2 are up to gNB to trigger the transmission of SSB without considering the requirement of UE while method 3 is based on UE demand. As agreed by RAN1, NW trigger is supported in Scenario#2/2A.

However, network-only mechanism cannot perfectly fit the requirement or conditions at UE side. For example, after SCell being activated (Scenario#3A/B), it is up to UE implementation to use SSB for DL sync when necessary. Some company may assume periodically transmitted SSB after SCell activation, however, this of course will reduce the NW power saving gain, which is not aligned with the WID.

Observation 4: Network-only mechanism cannot perfectly fit the requirement or conditions at UE side. UE based trigger is a better trade-off between energy saving and QoS.

Given that, it is reasonable to allow UE to send wake-up-signal for requesting SSB so as a better trade-off between energy saving and QoS. Therefore, we think both NW trigger and UE trigger should be supported.

Proposal 8: RAN2 to agree to support UE-based trigger of on-demand SSB transmission.

In addition, both method 1 and method 2 belong to NW-based trigger, however we think there are some difference between these two methods. For method 1, it relies on ideal backhaul indication from PCell to SCell, e.g., between CU and DU to trigger the SSB transmission on the SCell and meanwhile, PCell would inform UE about the transmission of on-demand SSB on SCell. Therefore, in our understanding, method 1 aims to introduce additional indication via F1 message on top of method 2, which should be firstly discussed by RAN3. From RAN2 point of view, method 2 and method 3 should be prioritized and discussed firstly.

Proposal 9: Method of cell on/off indication via backhaul should be firstly discussed by RAN3.

2.6 Impact on RRC_IDLE/INACTIVE UE

According to the WID, on-demand SSB serving cell cannot operate as a PCell, which means this kind of serving cell should prevent UEs from camping on. Candidate solutions are listed as below.

- Option 1: cellbarred in MIB is set to barred

- Option 2: on demand SSB cell is not visible to RRC_IDLE/INACTIVE UEs

Option 1 is the most straightforward solution and applies to both cell selection and cell reselection. For option 2, some details should be further considered, e.g., if the on-demand SSB cell is deployed per frequency, then this frequency should not be included in cell selection/reselection frequency while if the on-demand SSB cell is deployed per cell, then this cell should be included in the excluded cell list in SIB3/4. As long as UE camps on a normal cell, UE will not reselect to an on-demand SSB serving cell since this cell is not visible to UE based on the broadcasted selection/reselection frequency or excluded cell list. In this case, option 2 is only applicable to cell reselection.

However, even without any solution, there seems no serious problem. This is because if UE cannot obtain the SSB on this cell, UE will not camp on this cell and if UE obtains the SSB by chance and determines to camp on this cell, for example, this cell is transmitting SSB when UE performs cell selection/reselection, UE can just camp on. After camping, if this cell stops SSB transmission, UE will perform cell reselection according to the existing conditions.  Therefore, we suggest RAN2 to further discuss whether and how to prevent RRC_IDLE/INACTIVE UEs from camping on the on-demand SSB cell.

Proposal 10: RAN2 to discuss whether and how to prevent RRC_IDLE/INACTIVE UEs from camping on the on-demand SSB cell.

3	Conclusion

In this contribution, we discussed the procedures and signalling to support on-demand SSB and have corresponding observations and proposals:

Observation 1: With the same MAC CE to indicate both the activation and deactivation of SSB, it is a waste of signalling when only deactivation of SSB on some SCell is required.

Observation 2: In legacy, due to the mismatch between the SMTC window and SSB transmission periodicity, some SSB may fall outside of the SMTC window.

Observation 3: NW implementation may avoid this issue by configuring suitable SMTC value to cover most of the always-on SSB and OD-SSB.

Observation 4: Network-only mechanism cannot perfectly fit the requirement or conditions at UE side. UE based trigger is a better trade-off between energy saving and QoS.

Proposal 1: RAN2 to confirm both activation and deactivation of SSB transmission needs to be indicated to UE.

Proposal 2: SSB activation/deactivation MAC CE includes one byte/four bytes bitmap and additional SSB information field associated with each activated SSB transmission. The detailed design of the additional SSB information field depends on RAN1 conclusion.

Proposal 3: Send LS to RAN4 to check the impact on SCell activation latency if separate MAC CEs are defined and these two MAC CEs are not multiplexed in the same MAC PDU.

Proposal 4: RAN2 to discuss the introduction of dedicated deactivation MAC CE when only deactivation of SSB transmission is performed.

Proposal 5: Upon SCell addition, NW indicates whether the SSB transmission on this SCell is on-demand manner via RRC signalling.

Proposal 6: RRC based OD-SSB transmission indication can be used to reconfigure the activation/deactivation state of OD-SSB transmission.

Proposal 7: RAN2 to wait for more RAN1 progress on the configuration of SMTC for OD-SSB.

Proposal 8: RAN2 to agree to support UE-based trigger of on-demand SSB transmission.

Proposal 9: Method of cell on/off indication via backhaul should be firstly discussed by RAN3.

Proposal 10: RAN2 to discuss whether and how to prevent RRC_IDLE/INACTIVE UEs from camping on the on-demand SSB cell.

4	Reference

[1] RAN1#118 Chair notes.

[2] RAN2#127 meeting minutes

5	Annex

3GPP TSG-RAN WG2 Meeting #127bis	R2-240xxxx

Hefei, China, Oct. 14th – Oct. 18th, 2024

Title:	LS on MAC CE indication of OD-SSB

Response to:	-

Release:	Release 19

Work Item:	Netw_Energy_NR_enh-Core

Source:	RAN2

To:	RAN4

Cc:	RAN1

Contact Person:

#### Name:	Li Zhao

E-mail Address:	zhaoli6@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

During RAN2#127bis, RAN2 discussed the MAC CE indication of OD-SSB and RAN2 assumes a new MAC CE is used to indicate the activation/deactivation of OD-SSB for Scenario#2 and Scenario#2A. However, for Scenario#2A, if this new MAC CE is not multiplexed in the same MAC PDU as the legacy SCell activation/deactivation MAC CE, RAN2 is not sure if there is any impact on SCell activation latency and would like to check with RAN4.

2. Actions:

To RAN WG4

ACTION: 	RAN2 would like to ask RAN4 to provide answer to the above question.

3. Date of Next TSG-RAN WG2 Meeting:

TSG-RAN WG2 Meeting #128	November 2024		Orlando, US

TSG-RAN WG2 Meeting #129	February 2025		Athens, Greece