3GPP TSG-RAN WG2 Meeting #127bis	R2-2408194

Hefei, China, Oct 14th – 18th, 2024

Source: 	Xiaomi

Title:  	Discussion on conditional LTM

Agenda Item:	8.6.4

Document for:	Discussion and Decision

Introduction

A Revised WID on NR mobility enhancements Phase 4 [1] has been approved. In the WID, one of the objectives is to specify support for conditional Intra-CU LTM, as shown below.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this contribution, we provide our understandings on conditional Intra-CU LTM.

Discussion

  - Stage-2 signalling procedure

The signalling procedure of Rel-18 intra-CU LTM is divided into four key components, which are LTM preparation, Early synchronization, LTM cell switch execution and LTM cell switch completion, as specified in TS 38.300.

To design the signalling procedure of intra-CU conditional LTM, the flow chart of Rel-18 LTM can be taken as a baseline. In essence, the main difference between LTM and conditional LTM is the entity to trigger the decision on LTM cell switch. In LTM, the gNB-DU decides whether to execute LTM and which LTM candidate cell to be triggered based on the L1 measurement report from UE. In conditional LTM, the UE decides to execute the LTM cell switch based on the evaluation for conditions pre-configured by the network.

Hence, based on the signalling procedure of Rel-18 LTM, we modify the LTM cell switch execution phase, including removing steps for the L1 measurement report and the transmission of LTM cell switch command MAC CE, and adding the step for conditional LTM evaluation in the UE. The signalling procedure for intra-CU conditional LTM, as illustrated in the Figure 1 below, can be considered.

Figure 1. Signalling procedure for intra-CU conditional LTM

Proposal 1: For Rel-19 intra-CU conditional LTM in SA case, the signalling procedure of Rel-18 intra-CU LTM is taken as the baseline. The signalling procedure for intra-CU conditional LTM, as illustrated in Figure 1, can be considered with the following modifications compared to the Rel-18 intra-CU LTM:

- Remove the step: L1 measurement report

- Remove the step: LTM cell switch command (MAC CE)

- Add a new step: Evaluate the conditions of conditional LTM in the UE

  - Conditions for triggering LTM

For L3 conditional mobility, e.g., CHO, the execution condition can consist of one or two trigger event(s) (CHO events A3/A4/A5). And the similar mechanism can be considered to configure the execution conditions of conditional LTM.

For the network triggered LTM, the network can trigger the LTM cell switch based on the beam-level measurement results included in L1 measurement report. And the beam-level LTM events (Event LTM2~5) have been supported for the event triggered L1 measurement report. Hence, beam-level measurement results shall be considered for the triggering of LTM and the beam-level LTM events can be reused as the evaluation condition.

In addition, the cell-level measurement result is very important for the cell-level mobility performance and only considering the beam-level measurement result to trigger the mobility may increase handover frequency and the probability of ping-pong handover. Furthermore, LTM and L1 measurement are decoupled, and Rel-18 LTM can be triggered based on Layer 3 measurements by the network. Therefore, cell-level measurement results can be used for conditional LTM evaluation.

And then, we will discuss how to configure the condition based on the cell-level measurements. A simple method is to reuse the L3 CHO events (condEventA3/A4/A5). Another method is to introduce the new cell-level LTM events. RAN2 has discussed whether to introduce the cell-level LTM events and it is FFS on the conditional LTM case.

Proposal 2: The execution condition for triggering conditional LTM can consist of one or two trigger event(s) and the following options can be considered for conditional LTM:

- Option 1: Beam-level measurement results can be used for the evaluation of conditional LTM, and the condition consists of beam-level event(s) (e.g., Event LTM3/4/5)

- Option 2: Cell-level measurement results can be used for the evaluation of conditional LTM, and the condition consists of cell-level event(s) (e.g., L3 CHO events (condEventA3/A4/A5) or new cell-level LTM events)

In L3 RRM measurement, L3 CHO events (condEventA3/A4/A5) cannot be used for the L3 measurement report. The restriction is also applicable for the LTM event if it is associated with the conditional LTM configuration.

Proposal 3: The LTM measurement event associated with the conditional LTM configuration cannot be used to trigger the measurement report.

  - L2 handling

In Rel-18 intra-CU LTM, security keys are maintained upon an LTM cell switch. And whether to perform L2 reset is based on the NoResetID. i.e., when performing the LTM cell switch procedure, if the value of field ltm-NoResetID of the target cell is not equal to the value of ltm-ServingCellNoResetID of the serving cell, the UE performs PDCP data recovery and RLC re-establishment. The L2 handling procedure for Rel-18 intra-CU LTM can be reused for intra-CU conditional LTM.

Proposal 4：For intra-CU conditional LTM, L2 handling of Rel-18 LTM can be reused and security keys are maintained upon an LTM cell switch.

  - Early sync

As specified in step 4a and step 4b of Rel-18 intra-CU LTM in TS 38.300, both early DL synchronization and early UL synchronization are supported. To minimize the interruption of conditional LTM, both early DL synchronization and early UL synchronization can be considered.

Proposal 5：For conditional LTM, both early DL synchronization and early UL synchronization can be considered.

In the early UL synchronization procedure of Rel-18 LTM, the UE performs UL synchronization with LTM candidate cell(s) before receiving the cell switch command, by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell for early TA acquisition, as triggered by the gNB. As in intra-CU LTM, both early TA acquisition (via PDCCH ordered early RACH) and UE-based TA measurement can be supported for intra-CU conditional LTM.

Proposal 6:  Both early TA acquisition and UE-based TA measurement can be supported for intra-CU conditional LTM.

For UE-based TA measurement, the mechanism of Rel-18 LTM is sufficient.

However, in the procedure PDCCH ordered early RACH, UE only transmits a preamble towards the candidate cell and the TA value of the target candidate cell is included in the LTM cell switch command MAC CE. As we described in Chapter 2.1, the UE decides to execute the cell switch of conditional LTM based on the evaluation of conditions without the LTM cell switch command MAC CE. Therefore, how to provide the TA value of candidate cell(s) to UE needs to be discussed and there are two options can be considered:

- Option 1: RAR-based TA acquisition can be considered, and the TA value is provided by the candidate cell in RAR

- Option 2: As in Rel-18 LTM, the TA value is also provided by the serving cell. But, for conditional LTM, the serving cell shall provide the TA value of candidate cells to UE before conditional LTM execution.

Proposal 7: For early TA acquisition, the following options can be considered to provide the TA value of the candidate cell to UE:

- Option 1: the TA value of the candidate cell is provided by the candidate cell in RAR

- Option 2: the TA value of the candidate cell is provided by the serving cell

The gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. But for conditional LTM, the UE performs cell switch autonomously when the condition of candidate cells is fulfilled. How the UE acquires target cell’s DL/UL TCI states needs to be discussed. One option is for the network to indicate the DL/UL TCI states to UE before cell switch and a new MAC CE needs to be design to serve as the indication. Besides, UE may need to perform CSI report to assist with the TCI state selection in network, which would increase signaling interactions and overhead. A simple method is for the UE to select the UL/DL TCI states of candidate cells for conditional LTM.

Proposal 8: The UE can select the UL/DL TCI states of candidate cells for conditional LTM.

Conclusions

According to the analysis given above, we have the following proposals:

Proposal 1: For Rel-19 intra-CU conditional LTM in SA case, the signalling procedure of Rel-18 intra-CU LTM is taken as the baseline. The signalling procedure for intra-CU conditional LTM, as illustrated in Figure 1, can be considered with the following modifications compared to the Rel-18 intra-CU LTM:

- Remove the step: L1 measurement report

- Remove the step: LTM cell switch command (MAC CE)

- Add a new step: Evaluate the conditions of conditional LTM in the UE

Proposal 2: The execution condition for triggering conditional LTM can consist of one or two trigger event(s) and the following options can be considered for conditional LTM:

- Option 1: Beam-level measurement results can be used for the evaluation of conditional LTM, and the condition consists of beam-level event(s) (e.g., Event LTM3/4/5)

- Option 2: Cell-level measurement results can be used for the evaluation of conditional LTM, and the condition consists of cell-level event(s) (e.g., L3 CHO events (condEventA3/A4/A5) or new cell-level LTM events)

Proposal 3: The LTM measurement event associated with the conditional LTM configuration cannot be used to trigger the measurement report.

Proposal 4：For intra-CU conditional LTM, L2 handling of Rel-18 LTM can be reused and security keys are maintained upon an LTM cell switch.

Proposal 5：For conditional LTM, both early DL synchronization and early UL synchronization can be considered.

Proposal 6:  Both early TA acquisition and UE-based TA measurement can be supported for intra-CU conditional LTM.

Proposal 7: For early TA acquisition, the following options can be considered to provide the TA value of the candidate cell to UE:

- Option 1: the TA value of the candidate cell is provided by the candidate cell in RAR

- Option 2: the TA value of the candidate cell is provided by the serving cell

Proposal 8: The UE can select the UL/DL TCI states of candidate cells for conditional LTM.

Reference

[1] RP-242356, “Revised WID: NR mobility enhancements Phase 4”, Apple Inc, China Telecom