3GPP TSG-RAN WG2 Meeting #127bis                                               R2-240XXXX

Hefei, China, Oct 14th – Oct 18th, 2024

Source:	CATT

Title:	Discussion on MRO Enhancements for Rel-18 Mobility

Agenda Item:	8.10.2

Document for:	Discussion and Decision

1. Introduction

In RAN2#127 meeting, MRO enhancements for LTM, CHO with candidate SCGs and subsequent CPAC were discussed and achieved the following agreements [1].

LTM Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC. introduce a new field in RLF report to indicate the LTM recovery cell id. RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled. We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command. RA-based access will not be a new triggering condition for the SHR report. HO interruption time will not be a new triggering condition for the SHR report Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios. We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases. We will not log interruption time in SON reports (e.g. SHR) for LTM. We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit. Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec. CHO with candidate SCGs UE includes following information in RLF report: - b. Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled. - c. Measurement results of PCells and PSCells.

In this contribution, we continue the discussion on MRO enhancements for LTM, CHO with candidate SCGs and subsequent CPAC, and show our views.

2. Discussion

2.1 LTM

- Enhance RLF report for LTM MRO

RLF report should be enhanced to record the LTM failure information. When the UE is configured with LTM configuration, UE performs L1 measurement and reporting, based on which the network makes a handover decision and sends LTM cell switch command to UE to perform LTM execution. In RAN2#127 meeting, RAN2 agreed that we aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit. Similar as SHR, UE should also record the LTM candidate cell(s) configured by network (explicit or implicit) in RLF report, e.g., include an indicator to indicate whether a neighbor cell is an LTM candidate cell or not, or include a candidate cell(s) list (similar like choCandidateCellList), for providing more information to network for optimization of LTM failure.

Proposal 1: Enhance RLF report for LTM MRO to include the LTM candidate cell(s), FFS if explicit or implicit.

- Enhance SHR for LTM MRO

For successful LTM execution but it’s still close to failure, legacy SHR trigger conditions (i.e., T310 threshold, T312 threshold, T304 threshold) can be applied to LTM.

Proposal 2: The following legacy SHR trigger conditions can also be applied to LTM.

- T310 threshold;

- T312 threshold;

- T304 threshold.

SHR should be enhanced to record the successful LTM information. Similar as L1 and L3 measurement of serving cell, target cell and other LTM candidate cell(s) are included in RLF report upon RLF or mobility failure, L1 and L3 measurement of serving cell, target cell and other LTM candidate cell(s) should also be recorded in SHR for successful LTM execution.

Proposal 3: Enhance SHR for LTM MRO to include at least the following information:

- L1 measurement results of serving cell, target cell and other LTM candidate cell(s);

- L3 measurement results of serving cell, target cell and other LTM candidate cell(s);

- Enhance RACH report for LTM MRO

RAN2 has agreed that enhance RACH report for MCG LTM SON, raPurpose should be enhanced to cover successful RACH-based LTM execution, FFS whether legacy raPurpose can be reused or a new raPurpose for LTM execution needs to be introduced.

Proposal 4: Enhance raPurpose in RACH report to support successful RACH-based LTM execution. FFS whether reuse legacy raPurpose or introduce a new raPurpose.

UE performs RACH-less LTM cell switch execution if UE has valid TA value when receives LTM cell switch command, the TA value can be received in LTM cell switch command or be UE measured TA value. For the TA value in LTM cell switch command (network-based TA acquisition), the early RACH procedure is needed, i.e., the network needs to send the PDCCH order command to UE, based on which the UE sends the preamble towards the indicated candidate cell, the candidate cell acquires the TA value when receives the preamble from UE and sends the TA value to the source cell, source cell includes the TA value in the LTM cell switch command to UE. The RACH procedure can be triggered multiple times towards same or different candidate cells before receiving LTM cell switch command. A new raPurpose can be introduced to indicate the network-based early TA acquisition.

Proposal 5: A new raPurpose in RACH report is introduced to indicate the network-based early TA acquisition.

Based on current spec, the UE may include up to 8 RACH report in RACH report list. If the RACH report is enhanced to support logging the RACH information of network-based early TA acquisition, the legacy RACH information, e.g., RACH information of handover will be overridden by frequent network-based early TA acquisition, which will cause the legacy RACH information loss, this is not be expected as these RACH information is important for RACH optimization. RAN2 needs to discuss how to log the RACH information of network-based early TA acquisition to avoid overriding the legacy RACH information.

Proposal 6: RAN2 to discuss how to address the issue of frequent network-based early TA acquisition RACH information override the legacy RACH information (e.g., RACH information for handover).

- Other

The LTM configuration will not be released after successful LTM execution, which means subsequent LTM can continue to be performed. However, LTM execution is based on the L1 measurement results which vary very rapidly and may cause the frequent LTM execution. Considering that the legacy RLF report/SHR only includes the latest failure/successful handover information, for LTM, there is high probability that the old RLF report/SHR will be overrode by new RLF report/SHR due to the frequent LTM execution, this will cause the information loss for network optimization. In order to address the issue, recording multiple RLF report/SHR can be considered as a candidate solution.

Proposal 7: RAN2 to discuss how to address the issue of frequent overriding report for subsequent LTM, and recording multiple reports can be considered as a candidate solution.

2.2 CHO with candidate SCGs

- Scenario

In Rel-18, CHO with candidate SCGs was supported for accessing a best PSCell when the UE accesses the target PCell. It is the enhancement for CHO or CHO with target SCG. In Rel-18 SON/MDT, during the discussion of SPR, the scenario “HO with SN change” was proposed to postpone and didn’t discussed until Rel-18 SON/MDT WID is completed. In Rel-19 SON/MDT, it has been agreed to consider the enhancement for CHO with candidate SCGs, we think RAN2 should clarify whether the leftover scenario (i.e., CHO with target SCG) from R18 is in the scope of Rel-19 WID. From our perspective, this scenario (i.e., CHO with target SCG) can be supported with less effort if the CHO with candidate SCGs is supported. Therefore, we propose to consider both CHO with target SCG and CHO with candidate SCGs scenarios in R19 SON/MDT WID.

Proposal 8: For bullet “CHO with candidate SCG” in MRO enhancement for mobility, also consider the R18 leftover scenario of “CHO with target SCG”.

- RLF report enhancement

In RAN2#127 meeting, RAN2 has discussed and achieved the following agreement.

UE includes following information in RLF report: - b. Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled. c. Measurement results of PCells and PSCells.

Based on the agreement above, time information regarding condition fulfilment for CHO with candidate SCGs is supported in RLF report, details are FFS. Both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled need to be considered.

For the case that both CHO and CPAC execution conditions are fulfilled, the time elapsed between the CHO/CPAC execution condition is fulfilled and the CPAC/CHO execution condition is fulfilled can be included in RLF report for mobility failure optimization.

For the case that one of execution condition (CHO or CPAC) is fulfilled but another execution condition (CPAC or CHO) is not fulfilled, the time elapsed between CHO/CPAC execution condition fulfilled and failure occur (e.g., RLF occurs at source PCell, RLF occurs at source PSCell) can be included in RLF report for mobility failure optimization.

In addition, in RAN3#125 meeting, the type of the first fulfilled execution condition e.g. CPAC or CHO was agreed to be included in RLF report as follow. RAN2 can further confirm that including the type of the first fulfilled execution condition e.g., CPAC or CHO in RLF report.

The RLF report includes: - The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met. - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered - FFS whether to optimize the association of the PCell and PSCell.

For the CHO with candidate SCG(s) mobility failure, it is high possibility the CHO and/or CPAC execution condition configurations are not properly configured, considering CHO candidate cells and associated execution conditions have been included in RLF report for CHO only mobility failure optimization, thus, for CHO with candidate SCG(s), besides the information mentioned above, some CPAC configuration information (e.g., CPAC candidate cells and associated execution conditions etc.) should also be included in RLF report together with CHO configuration information for the CHO with candidate SCG(s) failure optimization. Furthermore, the association between CHO and CPAC should also be recorded and informed to network.

Proposal 9: Enhance RLF report for CHO with candidate SCGs to include at least the following information:

- the type of the first fulfilled execution condition e.g. CPAC or CHO;

- time duration between two fulfilled execution conditions;

- time elapsed between CHO/CPAC execution condition fulfilled and failure occur;

- CPAC configuration information (e.g., CPAC candidate cells and associated execution conditions) and the association between CHO and CPAC;

- SHR enhancement

In case of CHO with candidate SCGs, both PCell and PSCell are configured with execution conditions, and only the PCell and PSCell execution conditions are fulfilled at the same time, the UE triggers the CHO with candidate SCGs execution. If one of the execution condition (e.g., the execution condition of PSCell) is not fulfilled in a long time but another execution condition (e.g., the execution condition of PCell) is fulfilled, the UE will also not execute the CHO with candidate SCGs, which will cause the unexpected situation, i.e., the CPAC hinders the CHO execution, and vice versa. The ideal result of execution condition evaluation for CHO with candidate SCGs is the execution condition of PCell and PSCell should be fulfilled as closely as possible in terms of time. A new trigger condition can be considered for successful CHO with candidate SCGs execution.

Proposal 10: A new trigger condition that “the time difference between PCell execution condition being fulfilled and PSCell execution condition being fulfilled exceeds a configured threshold” is introduced for successful CHO with candidate SCGs execution.

2.3 Subsequent CPAC

- SPR enhancement

In Rel-18 MRO for CPAC, the UE indicates in the neighbor cell measurement results that whether a neighbor cell is a CPAC candidate cell or not in SPR (if triggered) for MRO optimization. For subsequent CPAC, the candidate cells change after each CPAC execution, the actual evaluated candidate cells can be a sub-set of all the candidate cells in VarConditionalReconfig when near failure occurs. RAN2 needs to clarify for subsequent CPAC the UE records whether a neighbor cell is a CPAC candidate cell only for evaluated candidate cells when near failure occurs (not all candidate cells in VarConditionalReconfig).

Proposal 11: For subsequent CPAC, clarify UE records whether a neighbor cell is a CPAC candidate cell only for evaluated candidate cells when near failure occurs (not all candidate cells in VarConditionalReconfig).

In Rel-18 MRO for CPAC, a time elapsed between the initiation of the last conditional reconfiguration execution towards the target cell and the reception of the latest conditional reconfiguration for this target cell (i.e., timeSinceCHO-Reconfig) was introduced in SPR for network optimization to avoid wasting of resources due to CPAC configuration is provided to UE too early. However for subsequent CPAC, the CPAC execution may be performed many times with the provided CPAC configuration, when a SPR is triggered by one CPAC execution, there can be multiple times CPAC execution before current CPAC execution, the timeSinceCHO-Reconfig will become useless. Therefore, we suggest the UE does not include the timeSinceCHO-Reconfig in SPR when the SPR is triggered due to subsequent CPAC execution.

Proposal 12: UE does not include the timeSinceCHO-Reconfig in SPR when the SPR is triggered due to subsequent CPAC execution.

- Other

For subsequent CPAC, frequent configuration of candidate PSCell add/update/release may occur which will introduce much XN interface signaling overhead, e.g., for MN-initiated subsequent CPAC, the UE initial is configured with 5 candidate cells (i.e., candidate cell 1~candidate cell 5), if the MN expects to add a candidate cell 6, the MN needs to request all candidate cell 1~5 to configure the execution condition for candidate cell 6, and request the candidate cell 6 to configure the execution condition for candidate cell 1~5. In order to reduce such overhead, it is better to configure suitable initial subsequent CPAC configuration without follow-up configuration update. The subsequent CPAC configuration optimization can be performed based on SON/MDT enhancement.

Proposal 13: RAN2 to consider initial subsequent CPAC configuration optimization for reducing the follow-up configuration update.

3. Conclusion

According to the discussion in section 2, we propose:

LTM

Proposal 1: Enhance RLF report for LTM MRO to include the LTM candidate cell(s), FFS if explicit or implicit.

Proposal 2: The following legacy SHR trigger conditions can also be applied to LTM.

- T310 threshold;

- T312 threshold;

- T304 threshold.

Proposal 3: Enhance SHR for LTM MRO to include at least the following information:

- L1 measurement results of serving cell, target cell and other LTM candidate cell(s);

- L3 measurement results of serving cell, target cell and other LTM candidate cell(s);

Proposal 4: Enhance raPurpose in RACH report to support successful RACH-based LTM execution. FFS whether reuse legacy raPurpose or introduce a new raPurpose.

Proposal 5: A new raPurpose in RACH report is introduced to indicate the network-based early TA acquisition.

Proposal 6: RAN2 to discuss how to address the issue of frequent network-based early TA acquisition RACH information override the legacy RACH information (e.g., RACH information for handover).

Proposal 7: RAN2 to discuss how to address the issue of frequent overriding report for subsequent LTM, and recording multiple reports can be considered as a candidate solution.

CHO with candidate SCGs

Proposal 8: For bullet “CHO with candidate SCG” in MRO enhancement for mobility, also consider the R18 leftover scenario of “CHO with target SCG”.

Proposal 9: Enhance RLF report for CHO with candidate SCGs to include at least the following information:

- the type of the first fulfilled execution condition e.g. CPAC or CHO;

- time duration between two fulfilled execution conditions;

- time elapsed between CHO/CPAC execution condition fulfilled and failure occur;

- CPAC configuration information (e.g., CPAC candidate cells and associated execution conditions) and the association between CHO and CPAC;

Proposal 10: A new trigger condition that “the time difference between PCell execution condition being fulfilled and PSCell execution condition being fulfilled exceeds a configured threshold” is introduced for successful CHO with candidate SCGs execution.

Subsequent CPAC

Proposal 11: For subsequent CPAC, clarify UE records whether a neighbor cell is a CPAC candidate cell only for evaluated candidate cells when near failure occurs (not all candidate cells in VarConditionalReconfig).

Proposal 12: UE does not include the timeSinceCHO-Reconfig in SPR when the SPR is triggered due to subsequent CPAC execution.

Proposal 13: RAN2 to consider initial subsequent CPAC configuration optimization for reducing the follow-up configuration update.

4. Reference

- Draft_RAN2_127_Meeting_Report_v1