3GPP TSG-RAN WG2 Meeting #127bis	R2-2408206

Hefei, China, October 14th – 18th, 2024

Source:	CATT, Turkcell

Title:	Simulation results of RRM Measurement Prediction

Agenda Item:	8.3.2.1

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, 3 scenarios have been agreed as high priority [1] for RRM measurement prediction:

Table 1: Prioritization of evaluation scenarios

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

And the following 3 cell level sub cases were agreed in RAN2#125bis [2]:

1 For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results

In this document, we focus on the scenarios with high priority, and the simulation results of cell level results prediction based on the cell level results (sub case 2) are provided.

Discussion

Before the discussion on the simulation results, some clarification on the definition of Average RSRP Difference in Case 2/3/4 simulation is provided. In the simulation, one universal AI model addressing all UEs/cells is used, and the cell level RSRP (per UE per cell at each sample time) is used as AI inputs to predict the cell level RSRP (per UE per cell in the future time sample). Then, the predicated RSRP RSRP_Predicated (per UE per cell at the predicted sample time) and RSRP obtained from system simulation RSRP_Simulated (per UE per cell corresponding to the simulated sample time) are compared to derive the RSRP Difference. Finally, Average RSRP difference is calculated as the average absolute RSRP Difference. The formula is illustrated in Formula (1):

Average RSRP Difference=∑Total_Predicated_Samples(|RSRP_Predicted-RSRP_Simulated|)Total_Predicated_Samples                  (1)

2.1	Case 2: FR1 to FR1 intra-frequency temporal domain case B

Temporal domain case B can be used to predict the measurement result(s) in the future time instance by historical measurement(s). Based on the Post email discussion [3], two examples are provided as shown below. In our simulation, we use Example 2 for Case B.

Figure 5.2.1-2 Example 1 of intra-frequency temporal domain case B Figure 5.2.1-3 Example 2 of intra-frequency temporal domain case B

Figure 1: 2 Examples of intra-frequency temporal domain case B

For Case B, the input are a series of measurement results (with a given measurement periodicity), whereas the output is the prediction of RSRP values at a point of time which does not and will not coincide with any measurement. ResNet is one residual learning framework to ease the training of the networks which can be used to reformulate the layers. For temporal domain case B, ResNet shows its good feature on prediction with different window lengths for predication window and observation window. Hence, we use ResNet for FR1 to FR1 intra-frequency temporal domain case B.

In the Post email [3], it was agreed that both sliding L1/L3 filtering and non-sliding L1/L3 filtering options can be used for evaluation. In Case 2, non-sliding L1/L3 filtering is used for simulation.

In Section 2.1.1-2.1.2, we compare RSRP difference from speed and MRRT perspective. And in Section 2.1.3, we import RSRP predicated by AI model into the system simulation platform and see whether HO performance is greatly impacted by the inaccuracy brought by AI predication.

- 2.1.1	Speed

For Case 2, system level simulations are performed and the simulation results are summarized in Table 2.

Table 2: Simulation report table for FR1 to FR1 intra-frequency temporal domain case B considering different UE speed

| Report parameters |  | CATT |  |  |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |  |
|  | UE speed (30,60,90,120 Km/h) | 30 | 60 | 90 |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |  |  |
|  | Whether LOSsoft is modeled or not | Yes |  |  |
|  | spatial consistency option (A or B) | Procedure A |  |  |
|  | Number of TX beams | 4 |  |  |
|  | Number of RX beams | 1 |  |  |
|  | Measurement reduction rate(50%,…Note2) | 66.78% |  |  |
|  | Observation window(Note3) | N/A |  |  |
|  | Prediction window (Note3) | N/A |  |  |
|  | Any other parameters (Note 4) | N/A |  |  |
| Data Size (Number of Samples) | Training/validity | 13000/1625 |  |  |
|  | Testing | 1625 |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |  |  |
|  | Model output(Note 6) | Cell level RSRP of all cells |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ResNet |  |  |
|  | Model complexity in a number of parameters(M) | 0.024M |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 0.093Mbytes |  |  |
|  | Computational complexity [FLOPs] | 292.66K |  |  |
| Metrics | Average L3 cell level RSRP difference (dBm) | 0.512 | 0.917 | 1.344 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |  |  |

We draw the RSRP difference trend along with the UE speed. It can be seen that when the UE speed grows, the RSRP difference increases. This is because as the speed increases, the moving distance increases between the two measurement samples. And the uncertainty of the channel condition deteriorates which brings higher inaccuracy and complexity on AI/ML prediction. As the RSRP difference grows due to the UE increasing UE speed, the accuracy of the predicated best cell decreases. But it is kept above 90% even the UE speed exceeds 90Km/h when the MRRT is 66.78%.

Observation 1: The RSRP difference increases as the UE speed increases in Case 2: FR1 to FR1 intra-frequency temporal domain case B.

Observation 2: The accuracy of the predicated best cell decreases as the UE speed increases in Case 2: FR1 to FR1 intra-frequency temporal domain case B.

Figure 2: The trend of RSRP difference and accuracy of the predicated best cell considering UE speed

- 2.1.2	MRRT

It is predicable that when the observation window increases, more information can be used to predication. And the simulation results are shown in Table 3.

Table 3: Simulation report table for FR1 to FR1 intra-frequency temporal domain case B considering different OW vs. PW

| Report parameters |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |  |  |
|  | UE speed (30,60,90,120 Km/h) | 60Km/h |  |  |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |  |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |  |  |  |
|  | Whether LOSsoft is modeled or not | Yes |  |  |  |
|  | spatial consistency option (A or B) | Procedure A |  |  |  |
|  | Number of TX beams | 4 |  |  |  |
|  | Number of RX beams | 1 |  |  |  |
|  | Measurement reduction rate(50%,…Note2) | 50% | 66.7% | 75% | 80% |
|  | Observation window(Note3) | N/A |  |  |  |
|  | Prediction window (Note3) | N/A |  |  |  |
|  | Any other parameters (Note 4) | N/A |  |  |  |
| Data Size (Number of Samples) | Training/validity | 13000/ 1625 |  |  |  |
|  | Testing | 1625 |  |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |  |  |  |
|  | Model output(Note 6) | Cell level RSRP of all cells |  |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ResNet |  |  |  |
|  | Model complexity in a number of parameters(M) | 0.024M |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 0.093Mbytes |  |  |  |
|  | Computational complexity [FLOPs] | 292.66K |  |  |  |
| Metrics | Average L3 cell level RSRP difference (dBm) | 0.602 | 0.917 | 1.092 | 1.486 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |  |  |  |

And the RSRP difference trend and the accuracy of the predicated best cell are shown in Figure 3. As MRRT increases, the information for AI model predication decreases. Hence, the RSRP difference should increase. And the simulation results verify this predication. It can be seen that as the MRRT increases, the RSRP difference increases which also leads the accuracy of the predicated best cell decreases.

Observation 3: The RSRP difference increases as the MRRT increases in Case 2: FR1 to FR1 intra-frequency temporal domain case B.

Observation 4: The accuracy of the predicated best cell decreases as the MRRT increases in Case 2: FR1 to FR1 intra-frequency temporal domain case B.

Figure 3: RSRP difference and accuracy of the predicated best cell trend considering MRRT

- 2.1.3	HO performance

The target study goal of Case 2 is to reduce the measurement efforts in temporal domain by using predicted measurements which is beneficial on reduction on power consumption and measurement overhead. To achieve this goal, it is also essential to make sure that the HO performance is not obviously impacted.

In this section, we import the predicted RSRP into the system platform and compare the HO performance between predicted RSRP and simulated RSRP imported into the system platform respectively. The corresponding definitions of PingPong handover and short of stay are based on TR 36.839, and the basic simulation assumption is shown in Table 4.

Table 4: Simulation assumption for HO performance in Case 2

| Report parameters |  | CATT |  |
| --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |
|  | UE speed (30,60,90,120 Km/h) | 60Km/h |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |  |
|  | Whether LOSsoft is modeled or not | Yes |  |
|  | spatial consistency option (A or B) | Procedure A |  |
|  | Number of TX beams | 4 |  |
|  | Number of RX beams | 1 |  |
|  | Measurement reduction rate(50%,…Note2) | 0%(non-AI) | 50% |
|  | Observation window(Note3) | N/A |  |
|  | Prediction window (Note3) | N/A |  |
|  | Any other parameters (Note 4) | N/A |  |
| Data Size (Number of Samples) | Training/validity | 13000/ 1625 |  |
|  | Testing | 1625 |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |  |
|  | Model output(Note 6) | Cell level RSRP of all cells |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ResNet |  |
|  | Model complexity in a number of parameters(M) | 0.024M |  |
|  | Model complexity in model size (e.g. Mbyte) | 0.093Mbytes |  |
|  | Computational complexity [FLOPs] | 292.66K |  |
| Metrics | HO Rate(Aver HO/UE/s) | 0.0628 | 0.0643 |
|  | Ping-Pong Rate(%) | 2.81 | 4.25 |
|  | Short ToS rate(%) | 5.45 | 6.87 |

It can be seen that with AI predication:

- The HO rate for non-AI and different MRRT are around 0.06 times/UE/s;

- The Ping-Pong rate or non-AI and different MRRT are below 5%;

- Short ToS rate ranges from 5.45% to 6.87%.

There is some fluctuation for the simulation results due to the randomness but there is no obvious deterioration introduced for the HO performance.

Observation 5: The HO performance of the AI predication does not deteriorate obviously compared with non-AI baseline in Case 2: FR1 to FR1 intra-frequency temporal domain case B.

2.2	Case 3: FR1 to FR1 inter-frequency (frequency domain)

It is agreed in last RAN2#127 meeting that companies are encouraged to considers both prediction from low-frequency cell to high-frequency cell and prediction from high-frequency cell to low-frequency cell, but only low to high is expected. Only one UE speed is considered but other speeds are not excluded. Therefore we perform the simulation assuming UE speed is 0 or 90Km/h, and both prediction from 2G to 4G and prediction from 4G to 2G are considered. The simulation results are shown in Table 5 as below. Average L3 cell level RSRP difference refers to the average value of the absolute RSRP difference of all cells corresponding to the UE (where the RSRP difference refers to the difference between the predicted RSRP value and the RSRP value obtained based on system simulation) based on the formula in the beginning of section 2.

Table 5: Simulation report table for FR1 to FR1 cell-level inter-frequency prediction

| Report parameters |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  | N/A |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  | N/A |  |
|  | UE speed (30,60,90,120 Km/h) | 90 |  | 0 |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | No |  |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | Full |  |  |  |
|  | Whether LOSsoft is modeled or not | Yes |  |  |  |
|  | spatial consistency option (A or B) | Procedure A |  | N/A |  |
|  | Number of TX beams | 8 |  |  |  |
|  | Number of RX beams | 2 |  |  |  |
|  | Measurement reduction rate(50%,…Note2) | N/A |  |  |  |
|  | Observation window(Note3) | N/A |  |  |  |
|  | Prediction window (Note3) | N/A |  |  |  |
|  | Any other parameters (Note 4) | N/A |  |  |  |
| Data Size (Number of Samples) | Training/validity | 13000/1625 |  |  |  |
|  | Testing | 1625 |  |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell-level measured RSRP of all cells on 2G | Cell-level measured RSRP of all cells on 4G | Cell-level measured RSRP of all cells on 2G | Cell-level measured RSRP of all cells on 4G |
|  | Model output(Note 6) | Cell-level predicted RSRP of all cells on 4G | Cell-level predicted RSRP of all cells on 2G | Cell-level predicted RSRP of all cells on 4G | Cell-level predicted RSRP of all cells on 2G |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ResNet |  |  |  |
|  | Model complexity in a number of parameters(M) | 0.017M |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 0.065MB |  |  |  |
|  | Computational complexity [FLOPs] | 285.60K |  |  |  |
| Metrics | Average L3 cell level RSRP difference (dB) | 0.2056 | 0.1997 | 0.2083 | 0.1963 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |  |  |  |

The following figure shows the CDF  curves of inter-frequency AI/ML inference for different UE speeds and frequency prediction direction (low-frequency cell to high-frequency cell/low-frequency cell to high-frequency cell):

Figure 4: CDF of RSRP-Diff for cell-level inter-frequency prediction (unit in dB)

From the simulation results, we observe that similar performance (RSRP difference) can be achieved by RSRP prediction from 2G to 4G and from 4G to 2G.

Based on the simulation results, it can be seen that:

- the Average L3 cell level RSRP difference is around 0.2 dB;

- the proportion of sample points for RSRP difference below 1dB is higher than 97.8%.

This shows that the cell-level inter-frequency RRM measurement prediction has excellent performance, which proves that the AI/ML could achieve high prediction accuracy for cell-level inter-frequency RRM measurement prediction.

Observation 6: AI/ML could achieve high prediction accuracy regarding the cell-level inter-frequency RRM measurement prediction.

Observation 7: The prediction accuracy is almost the same for prediction from low-frequency cell to high-frequency cell and for prediction from high-frequency cell to low-frequency cell.

Observation 8: The prediction accuracy of inter-frequency prediction is almost unaffected by different UE speed.

2.3	Case 4: FR2 to FR2 intra-frequency temporal domain case A

Temporal domain case A is mainly used to predict measurement result in the future of cell. Based on the predicted measurement result, some procedures, e.g., measurement event or RLF can also be predicated. One of the benefits of the predication is to improve the HO performance. For example, the target cell can be predicated in advance based on the predicated measurement result, which helps to facilitate network mobility control.

For AI model, long short-term memory (LSTM) based neural networks have played an important role in the field of Natural Language Processing. In addition, they have been used widely for sequence modeling. The reason why LSTMs have been used widely for this is because the model connects back to itself during a forward pass of the samples, and thus benefits from context generated by previous predications when predication for any new sample.

For Case 4, system level simulations are performed and we use LSTM model to predict the measurement result.

In the Post email [3], it was agreed that both sliding L1/L3 filtering and non-sliding L1/L3 filtering options can be used for evaluation. In Case 4, sliding L1/L3 filtering is used for simulation.

In Section 2.3.1-2.3.2, we compare RSRP difference from speed, OW vs. PW perspective.

- 2.3.1	Speed

For Case 4, we perform the simulation assuming UE speed is 60, 90 and 120Km/h and compare the RSRP difference trend between (OW vs. PW = 2N: N) and (OW vs. PW = 2N: 2N). The simulation results are shown in Table 5.

Table 5: Simulation report table for FR2 to FR2 intra-frequency temporal domain case A considering UE speed and different OW vs. PW

| Report parameters |  | CATT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |  |  |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |  |  |  |  |
|  | UE speed (30,60,90,120 Km/h) | 60 | 90 | 120 | 60 | 90 | 120 |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |  |  |  |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |  |  |  |  |  |
|  | Whether LOSsoft is modeled or not | Yes |  |  |  |  |  |
|  | spatial consistency option (A or B) | Procedure A |  |  |  |  |  |
|  | Number of TX beams | 32 |  |  |  |  |  |
|  | Number of RX beams | 4 |  |  |  |  |  |
|  | Measurement reduction rate(50%,…Note2) | N/A |  |  |  |  |  |
|  | Observation window(Note3) | 160ms |  |  |  |  |  |  |
|  | Prediction window (Note3) | 80ms |  |  | 160ms |  |  |
|  | Any other parameters (Note 4) | N/A |  |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | 208656/23184 |  |  |  |  |  |
|  | Testing | 33810 |  |  |  |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |  |  |  |  |  |
|  | Model output(Note 6) | Cell level RSRP of all cells |  |  |  |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |  |  |  |  |  |
|  | Model complexity in a number of parameters(M) | 0.87M |  |  |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 10.5MB |  |  |  |  |  |
|  | Computational complexity [FLOPs] | 1300.08K |  |  |  |  |  |
| Metrics | Average L3 cell level RSRP difference (dBm) | 0.422 | 0.601 | 0.67 | 0.619 | 0.855 | 1.082 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |  |  |  |  |  |

It can be seen that the RSRP difference increases as the UE speed increases for both 2N: N and 2N: 2N (N=80ms). Like Case 2, when the UE speed increases, the channel condition varies more dramatically. This brings larger RSRP difference when the UE speed increases.

Observation 9: The RSRP difference increases as the UE speed increases in Case 4: FR2 to FR2 intra-frequency temporal domain case A.

Figure 5: RSRP difference trend considering UE speed

- 2.3.2	OW vs. PW

It is predicable that when the observation window length increases, more information can be used for AI prediction. Hence, the RSRP difference decreases as the observation window increases. The simulation is provided in Table 6

Table 6: Simulation report table for FR2 to FR2 intra-frequency temporal domain case A considering UE speed and different OW vs. PW

| Report parameters |  | CATT |  |  |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |  |
|  | UE speed (30,60,90,120 Km/h) | 60 |  |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |  |  |
|  | Whether LOSsoft is modeled or not | Yes |  |  |
|  | spatial consistency option (A or B) | Procedure A |  |  |
|  | Number of TX beams | 32 |  |  |
|  | Number of RX beams | 4 |  |  |
|  | Measurement reduction rate(50%,…Note2) | N/A |  |  |
|  | Observation window(Note3) | 80ms | 160ms | 240ms |
|  | Prediction window (Note3) | 80ms |  |  |
|  | Any other parameters (Note 4) | N/A |  |  |
| Data Size (Number of Samples) | Training/validity | 208656/23184 |  |  |
|  | Testing | 33810 |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |  |  |
|  | Model output(Note 6) | Cell level RSRP of all cells |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |  |  |
|  | Model complexity in a number of parameters(M) | 0.87M |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 10.5MB |  |  |
|  | Computational complexity [FLOPs] | 1300.08K |  |  |
| Metrics | Average L3 cell level RSRP difference (dB) | 0.802 | 0.43 | 0.405 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |  |  |

And the RSRP difference trend is shown in Figure 6. It can be seen that as the observation window increases, the RSRP difference decreases.

Observation 10: The RSRP difference decreases as the observation window increases in Case 4: FR2 to FR2 intra-frequency temporal domain case A.

It also can be seen that the RSRP difference does not decrease rapidly (0.43dB to 0.405dB) when the observation window increases from 2N to 3N (160ms to 240ms). That is because the information to predict the RSRP is almost sufficient when the observation window is above 2N in this case. Hence, proper observation window length can be configured for RRM AI model to balance the accuracy and complexity of AI model.

Observation 11: Proper observation window length can be configured to balance the accuracy and complexity of AI model in Case 4: FR2 to FR2 intra-frequency temporal domain case A.

Figure 7: RSRP difference and accuracy of the predicated best cell trend considering the observation window in Case 4

We also simulate the case 3N:3N in Case 4. One example is shown in Figure 8.

Figure 8: RSRP predication for Case 4 with observation window with M measurement periods (M>1) vs. prediction window with M measurement periods (M>1)

And the separate RSRP difference for each time instance which equals to m*(measurement periods) (1≤m≤M and m is integer)in predication window is shown in Figure 9. It can be seen that the RSRP difference increases as the m increases. That is the predication time which is closer to the simulated results in the observation window shows smaller RSRP difference. As the m goes greater, the RSRP difference increases. This implies that as predication window increases, the predication accuracy decreases.

Observation 12: as the sample index increases in prediction window, the RSRP difference increases accordingly.

OW vs. PW = 3N: 3N

Figure 9: RSRP difference trend in predication window

Conclusion

In this document, we shows the simulation results for Case 2, Case 3 and Case 4 for RRM measurement, and it can be seen that:

Case 2: FR1 to FR1 intra-frequency temporal domain case B:

Observation 1: The RSRP difference increases as the UE speed increases in Case 2: FR1 to FR1 intra-frequency temporal domain case B.

Observation 2: The accuracy of the predicated best cell decreases as the UE speed increases in Case 2: FR1 to FR1 intra-frequency temporal domain case B.

Observation 3: The RSRP difference increases as the MRRT increases in Case 2: FR1 to FR1 intra-frequency temporal domain case B.

Observation 4: The accuracy of the predicated best cell decreases as the MRRT increases in Case 2: FR1 to FR1 intra-frequency temporal domain case B.

Observation 5: The HO performance of the AI predication does not deteriorate obviously compared with non-AI baseline in Case 2: : FR1 to FR1 intra-frequency temporal domain case B.

Case 3: FR1 to FR1 inter-frequency (frequency domain)

Observation 6: AI/ML could achieve high prediction accuracy regarding the cell-level inter-frequency RRM measurement prediction.

Observation 7: The prediction accuracy is almost the same for prediction from low-frequency cell to high-frequency cell and for prediction from high-frequency cell to low-frequency cell.

Observation 8: The prediction accuracy of inter-frequency prediction is almost unaffected by different UE speed.

Case 4: FR2 to FR2 intra-frequency temporal domain case A

Observation 9: The RSRP difference increases as the UE speed increases in Case 4: FR2 to FR2 intra-frequency temporal domain case A.

Observation 10: The RSRP difference decreases as the observation window increases in Case 4: FR2 to FR2 intra-frequency temporal domain case A.

Observation 11: Proper observation window length can be configured to balance the accuracy and complexity of AI model in Case 4: FR2 to FR2 intra-frequency temporal domain case A.

Observation 12: as the sample index increases in prediction window, the RSRP difference increases accordingly.

And we propose that:

Proposal: Capture the observations and the simulation tables for Case 2: FR1 to FR1 intra-frequency temporal domain case B, Case 3: FR1 to FR1 inter-frequency (frequency domain), and Case 4: FR2 to FR2 intra-frequency temporal domain case A for RRM measurement prediction into TR.

Reference

- R2-2406202, RAN2#126 chairman notes;

- R2-2404102, RAN2#125bis chairman notes;

- R2-2407885, Summary of [POST127][030][AI mobility] RRM simulation assumptions (OPPO), Oppo