3GPP TSG RAN WG2 #127bis                                                                           R2-2408210

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Consideration on NW side data collection

Agenda Item:	8.1.3

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting [1], some agreements have been achieved for NW side data collection in the aspect of BM use case:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

In this contribution, we further discuss the data collection for NW-side model training for BM and Pos use cases.

- Beam management

  - Mechanism of NW side data collection

It has been agreed that immediate MDT is the baseline framework for OAM-centric training data collection. In legacy, immediate MDT reuse the measConfig field in RRC message of RRCReconfiguration in the air-interface to send the MDT configuration. To continue and enhance this field for AI/ML purpose, additional parameters are necessary:

- Enhancement of configuration for AI/ML training data collection;

For immediate MDT, the UE measurement and report mechanism is entirely under NW control. The NW could configure a set of MeasObjectNR(s) and a set of ReportConfigNR(s), only the MeasObjectNR and ReportConfigNR associated by a measID could be actually performed for measurement. Multiple measId(s) could be configured to the UE for different measurements. Therefore based on the immediate MDT, it is easy for the NW to configure multiple AI/ML purposed measurements, and the activation/deactivation of measurements can be controlled by the presence/absence of measId in RRCReconfiguration.

Proposal 1: Support multiple configurations provided to UE for AI/ML training data collection.

Proposal 2: Activation/deactivation of measurements can be controlled by the presence/absence of measId in RRCReconfiguration.

The gNB should configure UE to perform data logging and reporting based on the potential AI/ML training requirements, such as:

- trigger conditions of Measurement result logging;

- trigger conditions of Measurement result reporting.

Based on the legacy immediate MDT, the reference signal related information in the serving/neighboring cell(s) needs to be measured could be included in the MeasObjectNR IE, and the logging trigger conditions and reporting trigger conditions could be introduced in the ReportConfigNR IE which including the logging type, reporting type, logging/reporting quantity, report Interval/Amount .etc.

Proposal 3: For AI/ML specific NW side training data collection, network configures data logging/reporting trigger conditions related information in the ReportConfigNR IE, and configures the reference signal related information in the MeasObjectNR IE.

- Enhanced data logging in UE;

The measurement result recording trigger conditions may include the periodically, event-based, event-based periodically.

For periodically recording, it is more like combination of immediate MDT and logged MDT. The UE can record one or multiple AL/ML related items of UE measurement results in one reporting period;

For event-based recording, UE could only record one item if the pre-configured event is triggered, e.g. the HOF occurred, the UE measurement has fulfill the event A3 threshold. etc.

For event-based periodically recording, the UE periodically records the measurement results on the UE side according to the network-configured recording trigger conditions and interval parameters after the recording trigger conditions are met.

To compare among these recording mechanism, different triggering condition may apply to different data collection content requirement, and periodically recording could reduce the frequency of reporting.

Proposal 4: Support periodic and event-based data logging.

In the logged MDT, an area scope will be configured to the UE. The area scope indicates the area for which UE is requested to perform measurement logging. If not configured, measurement logging is not restricted to specific cells or tracking areas but applies as long as the RPLMN is contained in the MDT PLMN list. For the AI/ML specific NW side training data collection, an area scope could also be used to restrict the data collection geographical area, to avoid unnecessary logging and power waste. The area scope could be one cell, a cell list or TA/PLMN wide (as in logged MDT).

Proposal 5: Introduce area scope to restrict the geographical area in which the AI/ML specific NW side training data collection is allowed. The area scope could be one cell, a cell list or TA/PLMN wide (as in logged MDT).

- Enhanced data reporting by UE;

The measurement result reporting trigger conditions may include the periodically, on-demand request or event-based .

For periodically reporting which is already considered as a baseline for AI/ML training data collection, the UE can report the AL/ML related UE measurement results based on reporting interval configured by the network. Since the frequent reporting may lead to waste of resources, the reporting interval can be lengthened and the UE could record multiple AL/ML related items of UE measurement results in one reporting period which seems a combination of immediate MDT and logged MDT;

It has been agreed to support the on-demand request mechanism in last RAN2 meeting, but whether to support the available indication is FFS. The available indicator of legacy logged MDT will be sent to the network to indicate whether the UE stores the available measurement result. Since the UE logging time may quite long to avoid frequent reporting, the UE may move to other cells and the logging could be continued. To make the new cell to know the available measurement result in UE, it is necessary to introduce the available indicator.

Proposal 6: Availability indicator in uplink RRC message is supported for AI/ML training data collection.

For event-based reporting, it is FFS whether AS buffer event based reporting is supported, and it is. FFS whether the UE only stops autonomously or if it reports to the network. We think the event-based reporting should not be supported since the frequency report of relative small data should be avoid in the air-interface for AI/ML training data collection. But the UE could indicate to the NW the power state condition is fulfilled and the measurement has been stopped. In a similar way, if the logged AI/ML measurement results reached the AS buffer limitation, the available indicator could also be sent to the NW. After receiving the available indicator, NW could request for the measurement result reporting, and the legacy messages of UEInformationRequest/ UEInformationResponse for logged MDT reporting could be used.

Proposal 7: No need to support event-based reporting.

Proposal 8: Support event-based (e.g. Fulfill the power state, AS buffer condition) availability indicator for AI/ML training data collection.

Proposal 9: Legacy messages of UEInformationRequest/UEInformationResponse could be used for on-demand reporting of AI/ML training data collection.

  - Transmission priority of NW side training data

The legacy logged MDT should be transmitted on SRB2. SRB2 has a lower priority than SRB1, and could be configured with higher/lower/equal priority with DRB or other SRBs. It has been agreed that no standardized RRC segmentation procedure is needed, and the UE could  determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. Since it is same as for the logged MDT measurements, SRB2 could be used to transmit the AI/ML training data.

Proposal 10: To use SRB2 to transmit the AI/ML training data.

  - Memory for NW side training data

It is agreed that UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. But it is FFS on the memory size. For logged MDT, the minimum memory size of MDT logged measurements is 64KB. Since the AI/ML memory size is across all use cases, we could assume 2*64KB to be the minimum memory size of AI/ML training data.

Proposal 11: Assume the minimum memory size of AI/ML training data logging is 2*64KB.

  - Type of data required to be collected

The data required to be collected includes the training data for NW sided model and/or for UE sided model. In the R18 Data Collection Requirements and Assumptions reply LS [2] from RAN1, for training data of BM use case:

| LCM purpose | UE-side/NW-side models | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | UE-side, NW-side | L1-RSRPs and/or beam-IDs | See Note 1 for L1-RSRPs | Relaxed |  |

It is summarized by RAN1 that for both NW sided model and UE sided model, the data content include L1-RSRPs and/or beam-IDs. Therefore, the data required to be collected for BM is common for NW sided model and UE sided model.

Observation 1: The data required to be collected for BM is common for NW sided model and UE sided model which including L1-RSRPs and/or beam-IDs.

- Positioning accuracy enhancements

  - Mechanism of NW side data collection

Immediate MDT involves UE, gNB, and TCE node. Some CN node may also be utilized for MDT configuration transmission without information change, i.e. for signaling based MDT. Since LMF is not a related node of MDT, immediate MDT could not be directly reused for Pos use case. The training data collection for Pos use case could be performed between UE/gNB and LMF, and the mechanism could be designed similar as the collection mechanism of BM:

1) Current LPP message can support periodic information transfer procedures, i.e. location information reports, periodic assistance data delivery. Take location information reports as an example, in the commonIEsRequestLocationInformation of RequestLocationInformation message, periodicalReporting field could be included to indicate the reportingAmount and reportingInterval by the location server’s request. Therefore for periodic reporting of AI/ML related measurement, current periodic procedure could be utilized. If other reporting mechanism such as on-demand reporting could be introduced for BM, similar new procedure could also be considered in TS37.355.

2) Current NRPPa message can support periodic and on-demand information transfer related procedures:

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

But this on-demand report is triggered by the LMF, and no available indicator is sent from the measurement collection node to trigger network-side retrieval. Therefore for the AI/ML related measurement result collection from gNB to LMF, current periodic procedure could be utilized. If other reporting mechanism such as on-demand reporting could be introduced for BM, similar new procedure could also be considered in TS38.455.

Proposal 12: The similar training data collection mechanism agreed for BM can be applied for Pos use case between UE/gNB and LMF.

  - Data collection per sub-case

Among all 5 Pos cases, UE side model cases include:

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

Among all 5 Pos cases, NW side model cases include:

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

Since it is approved to focus on the first priority issues until end of 2024 in RAN#105 meeting [3], we only discuss the data collection of case 1/3a/3b.

For training data collection of AI/ML based positioning, the collected data sample can include two parts defined by RAN1:

| Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement | Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label |
| --- | --- |

And based on agreements in RAN1#116bis and RAN1#117, for the 3 first priority cases, the information for training could be generated by different nodes:

Table 1 Training data generate node per case

|  | Part A (measurement and its related data) | Part B (labels related) |
| --- | --- | --- |
| Case 1 | PRU/UE | PRU/UE/LMF |
| Case 3a | TRP/gNB | LMF |
| Case 3b | TRP/gNB | PRU/UE/LMF |

If we assumed LMF may be used for gNB-side model training as noted in TR38.843, all data collected for NW sided model training should be transmitted from UE/gNB to LMF.

Collection of Part A

For Part A defined by RAN1, channel measurements include time (DP) and time+power (PDP), but whether to support time+power+phase (CIR) is FFS and can be further discussed by RAN1. These channel measurements are considered as model input in inference and should be collected by the training node for all cases base on the R18 Data Collection Requirements and Assumptions reply LS [2] from RAN1, for training data of PoS use case:

| LCM purpose | Case | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | All Cases | Measurements (corresponding to model input): timing, power, and/or phase info See Note 2 | Size depends on number of PRS/SRS resources, measurement type (timing, power, and/or phase info) and report format: ~100 bits to 1000s bits per PRS/SRS resource See Note 3 | Relaxed |  |
|  | Direct AI/ML positioning | Label: Location coordinates as model output | 56 to 144 bits See Note 3 | Relaxed |  |
|  | AI/ML assisted positioning | Label: Intermediate positioning measurement (timing info, LOS/NLOS indicator) as model output See Note 2 | 10s bits to 100s bits per PRS/SRS resource See Note 3 | Relaxed |  |

Note 2: For measurements as model input, no agreement on measurement types (i.e., time, power, and/or phase) in RAN1 for all cases (i.e., Case1 to Case3b). Measurement types (including their necessity) and sizes/dimension needs to be further discussed. Candidate measurement types discussed/evaluated for model input include CIR (contains timing, power and phase information), PDP (contains timing and power information), DP (contains timing information). For labels (i.e., model output) of AI/ML assisted positioning (Case2a, Case3a), RAN1 identified an initial listing of candidates that provide performance benefits (i.e., timing info, LOS/NLOS indicator). RSRP/RSRPP is for further discussion.

Therefore, report of the channel measurements cannot be covered by traditional UE positioning procedures, and should be introduced for reporting for all cases.

Observation 2: For RAN1 defined Part A (measurement and its related data), DP, PDP, or CIR (FFS) should be collected by the training node for both UE sided model training (case 1) and NW sided model training (case 3a/3b).

Since the transmission format of the DP/PDP/CIR(FFS) measurement should be provided by RAN1, RAN2 could first discuss in which message(s) such channel measurement could be carried.

- For case 1, the AI/ML related channel measurement should be transferred from UE/PRU to the training node. If the LMF is assumed as the training node of case 1, the LPP message could be used to carry them. Such AI/ML related channel measurement could be included into the legacy measurement related message, e.g. ProvideLocationInformation message, and new fields may be introduced for AI/ML usage;

- For case 3a/3b, it is the RAN3 responsibility to design the signalling enhancement. Other than the SRS configuration parameter values which may be enhanced in RAN1, no training related specification impact is foreseen in RAN2.

Proposal 13: Introduce new fields in LPP messages for transmission of the AL/ML related channel measurement if the LMF is assumed as the training node of case 1, and wait for more RAN1 progress on detailed channel measurement content and format, e.g. DP/PDP/CIR(FFS).

Proposal 14: For case 3a and 3b, no training related discussion is needed in RAN2 until triggered by RAN1 input about SRS configuration parameter if any.

Collection of Part B

For Part B (labels related) :

- Case 1: The label is the UE’s location. The label and its related data (e.g. quality indicator, time stamp) can be generated by UE implementation, or generated by LMF based on some measurement transmitted by UE and TRP’s location coordinate.

- Case 3a: The label includes LOS/NLOS indicator and/or timing information (UL RTOA or gNB Rx-Tx time difference). The label and its related data (e.g. quality indicator, time stamp) can be generated by at least LMF based on UE’s location and gNB/TRP coordinates, where the UE’s location may be:

  - provided by UE/PRU;

  - estimated by LMF using intermediate positioning measurement provided by gNB/TRP(s).

- Case 3b: The label is the UE’s location. The label and its related data (e.g. quality indicator, time stamp) can be generated by UE/PRU and transmitted to LMF, or the LMF can generate ground truth label based on measurement transmitted by gNB/TRP.

Observation 3: For RAN1 defined Part B (labels and its related data), the data required to be collected includes the labels for different cases as below and the possible related quality indicators and time stamps:

- UE side model Case 1: UE’s location;

- gNB side model Case 3a: LOS/NLOS indicator and/or timing information (UL RTOA or gNB Rx-Tx time difference);

- LMF side model Case 3b: UE’s location.

If the ground truth label (UE’s location) is provided by UE/PRU, traditional UE positioning method via LPP procedures could be used; if the UE location is estimated by LMF using intermediate positioning measurement provided by UE/PRU, the UE/PRU should provide the intermediate positioning measurement based on the traditional positioning method via LPP procedures; if the UE location is estimated by LMF using intermediate positioning measurement provided by gNB/TRP, the gNB/TRP should provide the intermediate positioning measurement based on the traditional positioning method via NRPPa procedures.

Proposal 15: Reuse legacy LPP/NRPPa procedures and UE positioning method for ground truth label (UE’s location) collection for both UE sided model training (case 1) and NW sided model training (case 3a/3b), if LMF is assumed as the training node.

- Conclusion

In this contribution, we provide our views on the training data collection for NW side model, and the observation and proposals are summarized as follows:

For beam management

- Mechanism:

Proposal 1: Support multiple configurations provided to UE for AI/ML training data collection.

Proposal 2: Activation/deactivation of measurements can be controlled by the presence/absence of measId in RRCReconfiguration.

Proposal 3: For AI/ML specific NW side training data collection, network configures data logging/reporting trigger conditions related information in the ReportConfigNR IE, and configures the reference signal related information in the MeasObjectNR IE.

Proposal 4: Support periodic and event-based data logging.

Proposal 5: Introduce area scope to restrict the geographical area in which the AI/ML specific NW side training data collection is allowed. The area scope could be one cell, a cell list or TA/PLMN wide (as in logged MDT).

Proposal 6: Availability indicator in uplink RRC message is supported for AI/ML training data collection.

Proposal 7: No need to support event-based reporting.

Proposal 8: Support event-based (e.g. Fulfill the power state, AS buffer condition) availability indicator for AI/ML training data collection.

Proposal 9: Legacy messages of UEInformationRequest/UEInformationResponse could be used for on-demand reporting of AI/ML training data collection.

- Priority:

Proposal 10: To use SRB2 to transmit the AI/ML training data.

- Memory:

Proposal 11: Assume the minimum memory size of AI/ML training data logging is 2*64KB.

- Data required:

Observation 1: The data required to be collected for BM is common for NW sided model and UE sided model which including L1-RSRPs and/or beam-IDs.

For positioning accuracy enhancements

- Mechanism:

Proposal 12: The similar training data collection mechanism agreed for BM can be applied for Pos use case between UE/gNB and LMF.

- Data required for Part A:

Observation 2: For RAN1 defined Part A (measurement and its related data), DP, PDP, or CIR (FFS) should be collected by the training node for both UE sided model training (case 1) and NW sided model training (case 3a/3b).

Proposal 13: Introduce new fields in LPP messages for transmission of the AL/ML related channel measurement if the LMF is assumed as the training node of case 1, and wait for more RAN1 progress on detailed channel measurement content and format, e.g. DP/PDP/CIR(FFS).

Proposal 14: For case 3a and 3b, no training related discussion is needed in RAN2 until triggered by RAN1 input about SRS configuration parameter if any.

- Data required for Part B:

Observation 3: For RAN1 defined Part B (labels and its related data), the data required to be collected includes the labels for different cases as below and the possible related quality indicators and time stamps:

- UE side model Case 1: UE’s location;

- gNB side model Case 3a: LOS/NLOS indicator and/or timing information (UL RTOA or gNB Rx-Tx time difference);

- LMF side model Case 3b: UE’s location.

Proposal 15: Reuse legacy LPP/NRPPa procedures and UE positioning method for ground truth label (UE’s location) collection for both UE sided model training (case 1) and NW sided model training (case 3a/3b), if LMF is assumed as the training node.

- References

- Chairman’s Notes RAN2#127, Maastricht, Netherlands, Aug 19th – 23rd, 2024

- R1-2310681 Reply LS on Data Collection Requirements and Assumptions RAN1

- RP-242387	Moderator's summary on offline discussion about non-RAN2 aspects on REL-19 WI NR_AIML_Air	RAN1 vice-chair (CMCC)