3GPP TSG-RAN2 Meeting #127-bis	R2-2408213

# Hefei, China,  Oct 14th - Oct 18th

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0516 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on GNSS-AlmanacSupport and GNSS-UTC-ModelSupport in A-GNSS positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Issue 1: In GNSS-AlmanacElement in A-GNSS-ProvideAssistanceData, LMF can provide model-1 to model-8 to UE, where model-8 refers to the almanac assistance data of NavIC introduced in Rel-16; In GNSS-AlmanacReq in A-GNSS-RequestAssistanceData, UE can request model-1 to model-8 to LMF, where model-8 refers to the almanac assistance data of NavIC introduced in Rel-16: GNSS-ID to modelID relation (in GNSS-AlmanacReq) GNSS-ID; modelID. gps; 2. sbas; 6. qzss; 2. galileo; 1. glonass; 5. bds; 7. navic; 8. However in GNSS-AlmanacSupport in A-GNSS-ProvideCapabilities, there is only model-1 to model-7, model-8 is missing. This means that UE cannot indicate its support of NavIC almanac assistance data to LMF. Issue 2: In GNSS-UTC-Model in A-GNSS-ProvideAssistanceData, LMF can provide model-1 to model-5 to UE. In GNSS-UTC-ModelReq in A-GNSS-RequestAssistanceData, UE can request model-1 to model-5 to LMF, where model-2 refers to the GNSS-UTC-Model assistance data of NavIC introduced in Rel-16: GNSS-ID to modelID relation (in GNSS-UTC-ModelReq) GNSS-ID; modelID. gps; 1. sbas; 4. qzss; 1. galileo; 1. glonass; 3. bds; 5. navic; 2. However in the field description of GNSS-UTC-ModelSupport in A-GNSS-ProvideCapabilities, if UE supports NavIC, the mandatory support of model-2 GNSS-UTC-Model assistance data is missing. |
| Summary of change: |  | Change 1: Add model-8 in GNSS-AlmanacSupport, and add in field description that ‘if the target device supports NavIC and GNSS-Almanac assistance, it shall support Model-8’; Change 2: Add in field description of GNSS-UTC-ModelSupport that ‘if the target device supports NavIC and GNSS-UTC-Model assistance, it shall support Model-2’. Impact analysis Impacted 5G architecture options: SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: A-GNSS positioning Inter-operability: - If the UE is implemented according to this CR while the network is not, network will not understand model-8 reported in GNSS-AlmanacSupport; - If the network is implemented according to this CR while the UE is not, UE will report a UE capability that does not align with network’s understanding, which further leads to unwanted/not-supported assistance data provision to UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot indicate its support of some specific NavIC assistance data. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# START OF CHANGE

6.5.2	A-GNSS Positioning

<omitted>

6.5.2.10	GNSS Capability Information Elements

<omitted>

–	GNSS-AlmanacSupport

-- ASN1START

GNSS-AlmanacSupport ::=	SEQUENCE {

almanacModel		BIT STRING {	model-1		(0),

model-2		(1),

model-3		(2),

model-4		(3),

model-5		(4),

model-6		(5),

model-7		(6),

model-8-v16xy     (7) } (SIZE (1..8))		OPTIONAL,

...

}

-- ASN1STOP

GNSS-AlmanacSupport field descriptions

almanacModel This field specifies the almanacModel choice(s) in GNSS-Almanac IE supported by the target device for the GNSS indicated by GNSS-ID. This is represented by a bit string, with a one-value at the bit position means the particular almanac model is supported; a zero-value means not supported. If the target device supports GPS and GNSS-Almanac assistance, it shall support Model-2. If the target device supports SBAS and GNSS-Almanac assistance, it shall support Model-6. If the target device supports QZSS and GNSS-Almanac assistance, it shall support Model-2. If the target device supports Galileo and GNSS-Almanac assistance, it shall support Model-1. If the target device supports GLONASS and GNSS-Almanac assistance, it shall support Model-5. If the target device supports BDS and GNSS-Almanac assistance, it shall support Model-7. If the target device supports NavIC and GNSS-Almanac assistance, it shall support Model-8. If this field is absent, the target device supports the mandatory (native) almanacModel choice only as listed above for the GNSS indicated by GNSS-ID.

–	GNSS-UTC-ModelSupport

-- ASN1START

GNSS-UTC-ModelSupport ::=	SEQUENCE {

utc-Model		BIT STRING {	model-1		(0),

model-2		(1),

model-3		(2),

model-4		(3),

model-5		(4) } (SIZE (1..8))		OPTIONAL,

...

}

-- ASN1STOP

GNSS-UTC-ModelSupport field descriptions

utc-Model This field specifies the GNSS-UTC-Model choice(s) in GNSS-UTC-Model IE supported by the target device for the GNSS indicated by GNSS-ID. This is represented by a bit string, with a one-value at the bit position means the particular UTC model is supported; a zero-value means not supported. If the target device supports GPS and GNSS-UTC-Model assistance, it shall support Model-1. If the target device supports SBAS and GNSS-UTC-Model assistance, it shall support Model-4. If the target device supports QZSS and GNSS-UTC-Model assistance, it shall support Model-1. If the target device supports Galileo and GNSS-UTC-Model assistance, it shall support Model-1. If the target device supports GLONASS and GNSS-UTC-Model assistance, it shall support Model-3. If the target device supports BDS and GNSS-UTC-Model assistance, it shall support Model-5. If the target device supports NavIC and GNSS-UTC-Model assistance, it shall support Model-2. If this field is absent, the target device supports the mandatory (native) utc-Model choice only as listed above for the GNSS indicated by GNSS-ID.

# END OF CHANGE