3GPP TSG-RAN2 Meeting #127-bis	R2-2408217

# Hefei, China,  Oct 14th - Oct 18th

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0520 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on remaining issues in LPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Issue 1: Rel-18 has introduced a new location information type as locationEstimateAndMeasurementsRequired, the PRU should return both location estimate and measurements upon receiving this request. For UE-based A-GNSS positioning request location information, LMF can request UE return location estimate using high accuracy RTK/PPP methods or high accuracy GNSS positioning metrics, but the corresponding Conditional presence fails to include the newly introduced locationEstimateAndMeasurementsRequired. Issue 2: LMF provides PRU’s location together with PRU’s measurements to the target UE for calibration usage using NR-PRU-DL-Info IE. Currently in this IE, the PRU location is set to ‘OPTIONAL’. However, PRU’s location should be compulsively provided to target UE for the calibration. If PRU location is unknown by the target UE, then the theoretical true value between PRU and TRP cannot be known, it is then useless/meaningless for the target UE to get PRU’s real measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: Add locationEstimateAndMeasurementsRequired in the conditional presence UEB in GNSS-PositioningInstructions. Change 2: Add ‘In this version of specification, the location server shall always include this field’ in the field description of nr-PRU-LocationInfo. Impact analysis Impacted 5G architecture options: SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: A-GNSS positioning, carrier phase positioning Inter-operability: - If the UE is implemented according to this CR while the network is not, PRU will not be able to report location estimate using high accuracy RTK/PPP methods or high accuracy GNSS positioning metrics; UE may not get PRU’s location for calibration. - If the network is implemented according to this CR while the UE is not, there is no inter-operability issues foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PRU will not be able to report location estimate using high accuracy RTK/PPP methods or high accuracy GNSS positioning metrics; UE may not get PRU’s location for calibration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.8, 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# START OF CHANGE

6.5.2.8	GNSS Location Information Request Elements

–	GNSS-PositioningInstructions

The IE GNSS-PositioningInstructions is used to provide GNSS measurement instructions.

-- ASN1START

GNSS-PositioningInstructions ::= SEQUENCE {

gnss-Methods				GNSS-ID-Bitmap,

fineTimeAssistanceMeasReq	BOOLEAN,

adrMeasReq					BOOLEAN,

multiFreqMeasReq			BOOLEAN,

assistanceAvailability		BOOLEAN,

...,

[[

ha-GNSS-Req-r15			ENUMERATED { true }		OPTIONAL	-- Cond UEB

]],

[[

ha-GNSS-MetricsReq-r17	ENUMERATED { true }		OPTIONAL	-- Cond UEB

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is optionally present, need OP, if the locationInformationType is set to locationEstimateRequired, locationEstimatePreferred, locationEstimateAndMeasurementsRequired or locationMeasurementsPreferred; oltherwise it is not present. |

6.4.3	Common NR Positioning Information Elements

–	NR-PRU-DL-Info

The IE NR-PRU-DL-Info is used by the location server to provide the carrier phase measurements with associated measurements and additional information reported by a PRU for UE-based DL-TDOA to a target UE.

-- ASN1START

NR-PRU-DL-Info-r18 ::= SEQUENCE {

nr-PRU-LocationInfo-r18			LocationCoordinates						OPTIONAL, -- Need ON

nr-PRU-DL-TDOA-MeasInfo-r18		NR-DL-TDOA-SignalMeasurementInformation-r16

OPTIONAL, -- Need ON

nr-PRU-DL-AoD-MeasInfo-r18		NR-DL-AoD-SignalMeasurementInformation-r16

OPTIONAL, -- Need ON

nr-PRU-RSCP-MeasInfo-r18		NR-PRU-RSCP-MeasurementInformation-r18

OPTIONAL, -- Need ON

...

}

NR-PRU-RSCP-MeasurementInformation-r18 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF

NR-PRU-RSCP-MeasElement-r18

NR-PRU-RSCP-MeasElement-r18 ::= SEQUENCE {

dl-PRS-ID-r18					INTEGER (0..255),

nr-PhysCellID-r18				NR-PhysCellID-r16						OPTIONAL, -- Need ON

nr-CellGlobalID-r18				NCGI-r15								OPTIONAL, -- Need ON

nr-ARFCN-r18					ARFCN-ValueNR-r15						OPTIONAL, -- Need ON

nr-DL-PRS-ResourceID-r18		NR-DL-PRS-ResourceID-r16				OPTIONAL, -- Need ON

nr-DL-PRS-ResourceSetID-r18		NR-DL-PRS-ResourceSetID-r16				OPTIONAL, -- Need ON

nr-TimeStamp-r18				NR-TimeStamp-r16,

nr-los-nlos-Indicator-r18		CHOICE {

perTRP					LOS-NLOS-Indicator-r17,

perResource				LOS-NLOS-Indicator-r17

}																		OPTIONAL, -- Need ON

nr-RSCP-r18						INTEGER (0..3599)						OPTIONAL, -- Need ON

nr-DL-PRS-RSRP-Result-r18			INTEGER (0..126)					OPTIONAL, -- Need ON

nr-DL-PRS-FirstPathRSRP-Result-r18	INTEGER (0..126)					OPTIONAL, -- Need ON

nr-PhaseQuality-r18				NR-PhaseQuality-r18						OPTIONAL, -- Need ON

nr-PRU-RSCP-AddSampleMeasurements-r18	SEQUENCE (SIZE (1..nrNumOfSamples-1-r18 )) OF

NR-RSCP-AdditionalMeasurements-r18		OPTIONAL, -- Need ON

nr-PRU-RSRP-AddSampleMeasurements-r18	SEQUENCE (SIZE (1..nrNumOfSamples-1-r18)) OF

INTEGER (0..61)							OPTIONAL, -- Need ON

nr-PRU-FirstPathRSRP-ResultDiff-AddSampleMeasurements-r18

SEQUENCE (SIZE (1..nrNumOfSamples-1-r18 )) OF INTEGER (0..61)																				OPTIONAL, -- Need ON

nr-PRU-RSCP-AdditionalMeasurements-r18

NR-PRU-RSCP-AdditionalMeasurements-r18	OPTIONAL, -- Need ON

...

}

NR-PRU-RSCP-AdditionalMeasurements-r18 ::= SEQUENCE (SIZE (1..3)) OF

NR-PRU-RSCP-AdditionalMeasurementElement-r18

NR-PRU-RSCP-AdditionalMeasurementElement-r18 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r18			NR-DL-PRS-ResourceID-r16			OPTIONAL, -- Need ON

nr-DL-PRS-ResourceSetID-r18			NR-DL-PRS-ResourceSetID-r16			OPTIONAL, -- Need ON

nr-DL-PRS-RSRP-ResultDiff-r18		INTEGER (0..61)						OPTIONAL, -- Need ON

nr-DL-PRS-FirstPathRSRP-ResultDiff-r18	INTEGER (0..61)					OPTIONAL, -- Need ON

nr-PRU-RSCP-AdditionalMeasurementsList-r18	SEQUENCE (SIZE (1..nrNumOfSamples-r18 )) OF

NR-RSCP-AdditionalMeasurements-r18		OPTIONAL, -- Need ON

nr-PRU-RSRPDiff-AdditionalMeasurementsList-r18	SEQUENCE (SIZE (1..nrNumOfSamples-r18 )) OF											INTEGER (0..61)							OPTIONAL, -- Need ON

nr-PRU-FirstPathRSRP-ResultDiff-AdditionalMeasurementsList-r18

SEQUENCE (SIZE (1..nrNumOfSamples-r18 )) OF INTEGER (0..61)			OPTIONAL, -- Need ON

...

}

-- ASN1STOP

NR-PRU-DL-Info field descriptions

nr-PRU-LocationInfo This field provides the location coordinates of the PRU. In this version of specification, the location server shall always include this field.

nr-PRU-DL-TDOA-MeasInfo This field specifies the list of carrier phase measurement RSCPD together with the other measurement information in DL-TDOA by the PRU.

nr-PRU-DL-AoD-MeasInfo This field specifies the list of other measurement information in DL-AoD by the PRU.

nr-PRU-RSCP-MeasInfo This field specifies the list of carrier phase measurement RSCP measured by the PRU, together with DL-PRS RSRP, and/or DL-PRS RSRPP measurement(s) associated with the RSCP measurements.

# END OF CHANGE