3GPP TSG-RAN WG2 Meeting #127-bis                                     R2-2408218

Hefei, China, Oct 14th – 18th, 2024

Source:        ZTE Corporation

Title:           Discussion on LCM for positioning use case

Agenda item:   8.1.2.3

Document for:  Discussion and Decision

- Introduction

The revised WID [1] of Rel-19 NR_AIML_air related to AI/ML positioning is given as follows:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

This contribution provides our view on LCM for AI/ML positioning use case, focusing on use case 1/2a/2b (high priority from RAN2 perspective) and use case 3a/3b (low priority from RAN2 perspective).

- Discussion

The inference procedure of 5 AI/ML positioning use cases described in the WID are briefly depicted as follows:

Figure 1. Use case 1, model inference is at UE side (1st priotrity)

Figure 2. Use case 2a, model inference is at UE side (2st priotrity)

Figure 3. Use case 2b, model inference is at LMF side (2st priotrity)

Figure 4. Use case 3a, model inference is at gNB side (1st priotrity)

Figure 5. Use case 3b, model inference is at LMF side (1st priotrity)

It can be seen that among the 5 use cases, only use case 1, use case 2a and use case 2b are involved with LMF-UE signaling, wherein use case 1 is the 1st priority.

Observation 1: Use case 1, use case 2a and use case 2b have clear RAN2 LPP specification impacts, wherein use case 1 is the 1st priority.

Although use case 2b is not UE-side model, use case 2b requires UE to report PRS channel measurement (which is a different kind of report compared with use case 1 and 2a), so it is worth to discuss how use case 2b works by the UE, similar like use case 1 and 2a.

Proposal 1: Support RAN2 to discuss the LCM or the spec impact of use case 1, 2a and 2b together, even if use case 2b is not UE-side model.

LCM of UE involved use case (use case 1, 2a and 2b)

UE-side model, Functionality/AI model activation

Based on the discussion so far, a comprehensive understanding of functionality and AI model is, the functionality can be recognized per use case, different functionality can be distinguished by different UE capability and different UE applicable functionality. Under each functionality, there can be multiple AI models, different models can be distinguished by different NW/UE additional conditions.

Figure 6. An example relationship between functionality and AI model

RAN2#125-bis [2] has made the following agreement for UE side model:

For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19.

Based on the above understanding of functionality and AI model, there can have following approaches to control the LCM:

- Approach 1: LMF activates/deactivates the functionality, and LMF activates/deactivates the AI model of the functionality.

  - Approach 1 means whether UE activates/deactivates use case 1, use case 2a or use case 2b is determined and indicated by LMF. For use case 1 and use case 2a where the AI model is at UE side, LMF should further determine which AI model under the indicated use case should be activated/deactivated. That is to say, LMF needs to get UE’s applicable functionality to know which functionalities is available/unavailable to UE, and LMF should also know the relationship between UE side additional condition and AI model, so that LMF knows which AI model is available/unavailable to UE.

- Approach 2: LMF activates/deactivates the functionality, and LMF allows UE to activate/deactivate the AI model of the functionality

- Approach 2 means whether UE activates/deactivates use case 1, use case 2a or use case 2b is determined and indicated by LMF. For use case 1 and use case 2a where the AI model is at UE side, LMF can allow UE to activate/deactivate the AI model of the indicated use case according to UE’s own additional condition and already provided NW side additional conditions. That is to say, LMF needs to get UE’s applicable functionality to know which functionalities is available/unavailable to UE.

- Approach 3: LMF allows UE to activate/deactivate the functionality, and LMF allows UE to activate/deactivate the AI model of the functionality, and UE reports the activated/deactivated functionality to LMF

- Approach 3 means LMF can allow UE to activate/deactivate use case1 use case 2a or use case 2b according to UE’s applicable functionalities. That is to say, LMF can allow UE to report AI UE location (using use case 1), to report AI intermediate feature (using use case 2a) or to report PRS channel measurement (using use case 2b). Then UE should let LMF know which functionalities is activated/deactivated. Further, LMF can allow UE to activate/deactivate the AI model of the indicated use case according to UE’s own additional condition and already provided NW side additional conditions.

- Letting LMF know UE’s decision of functionality may not need new explicit signaling since in AI/ML positioning, different functionalities has different kinds of reporting content. LMF can differentiate different kinds of functionality according to different kinds of reporting content that UE reports.

As we analysed above, NW side additional condition is already provided to UE. It is not necessary and not efficient to let UE report UE additional condition to LMF, then LMF decides the AI model and feedback to UE, i.e., approach 1 is not preferred in AI/ML positioning. Also as mentioned above, the relationship between additional condition and AI model is hard to specify, so LMF is hard to know such information. Since UE knows the UE side additional condition and NW side additional condition, it is more efficient to let UE decide the AI model under each functionality, i.e., go with approach 2 or approach 3.

Observation 2: Since NW side additional condition is already provided to UE, it is not necessary and not efficient to let UE report UE additional condition to LMF, then LMF decides the AI model and feedback to UE.

Proposal 2: For AI/ML positioning use case 1, 2a and 2b, RAN2 to consider the following approaches for functionality-LCM:

- LMF activates/deactivates the functionality

- LMF allows UE to activate/deactivate the functionality

Proposal 3: For AI/ML positioning use case 1 and 2a, RAN2 to consider that LMF allows UE to activate/deactivate the AI model of the activated functionality for model-LCM.

Functionality management as positioning method

For AI/ML positioning, the functionality can be recognized as use case 1, use case 2a and use case 2b.

Proposal 4: For AI/ML positioning, the functionality can be recognized as use case 1, use case 2a and use case 2b.

In legacy LPP specification, LMF can schedule different positioning methods to UE, wherein each positioning method has different kinds of reporting content. Different positioning method can be scheduled in parallel or in series, but they are scheduled independently from each other. Also each legacy positioning method has the type of UE-assisted positioning or UE-based positioning, where UE-assisted positioning means UE can report measurement and LMF is to calculate the UE location, UE-based positioning means UE can make the measurement and calculate the location by the UE itself. The table from TS38.305 [3] describes the mapping of the type and the positioning method:

Table 4.3.1-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, LMF-based | NG-RAN node assisted | SUPL Note 8 |
| --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | Yes |
| OTDOA Note1, Note 2 | No | Yes | No | Yes |
| E-CID Note 4, Note 7 | No | Yes | Yes | Yes for E-UTRA |
| Sensor | Yes | Yes | No | No |
| WLAN | Yes | Yes | No | Yes |
| BluetoothNote 9 | Yes | Yes | No | No |
| TBS Note 5 | Yes | Yes | No | Yes (MBS) |
| DL-TDOA | Yes | Yes | No | Yes |
| DL-AoD | Yes | Yes | No | Yes |
| Multi-RTT | No | Yes | Yes | Yes |
| NR E-CID | No | Yes | Yes | Yes (DL NR E-CID) |
| UL-TDOA | No | No | Yes | Yes |
| UL-AoA | No | No | Yes | Yes |
| NOTE 1: This includes TBS positioning based on PRS signals. NOTE 2: In this version of the specification only OTDOA based on LTE signals is supported. NOTE 3: Void NOTE 4: This includes Cell-ID for NR method when UE is served by gNB. NOTE 5: In this version of the specification only for TBS positioning based on MBS signals. NOTE 6: Void NOTE 7: Enhanced Cell ID based on LTE signals. NOTE 8: This shows whether the positioning method is supported by SUPL ULP [16]. NOTE 9: UE-based positioning mode is supported only for Bluetooth AoD positioning method. |  |  |  |  |

In AI/ML positioning, it is similar that different functionality (i.e., different use case) has different kinds of reporting content, i.e., the reporting content of use case 1 is AI UE location, the reporting content of use case 2a is AI intermediate feature, the reporting content of use case 2b is PRS channel measurement. So from signaling structure perspective, AI/ML positioning functionality can be managed as positioning method, i.e., activating a positioning method means activating the corresponding AI/ML positioning functionalities.

Proposal 5: From RAN2 signaling structure perspective, AI/ML positioning functionality (use case) can be managed as positioning method, i.e., activating a positioning method means activating the corresponding AI/ML positioning functionalities (use cases).

There can be following options to map the functionality to the positioning method:

- Option 1: Introduce a new AI/ML positioning method for use case 1, 2a and 2b;

  - This is most straightforward way of mapping. To be specific, UE-based positioning can be UE reporting AI UE location (use case1), UE-assisted positioning can be UE reporting AI intermediate feature (use case 2a) or PRS channel measurement (use case 2b), depending on LMF’s scheduling or UE’s decision.

- Option 2: Introduce a new AI/ML positioning method for use case 1 and 2a;

- This is to differ UE-side model and LMF-side model. use case 1 and 2a are UE-side model, so it is reasonable to introduce a new positioning method dedicated for the UE who has the AI model capability. To be specific, UE-based positioning can be UE reporting AI UE location (use case 1), UE-assisted positioning can be UE reporting AI intermediate feature (use case 2a).

- Option 3: Introduce a new AI/ML positioning method for use case 1 and 2b;

- To be specific, UE-based positioning can be UE reporting AI UE location (use case 1), UE-assisted positioning can be UE reporting PRS channel measurement (use case 2b), depending on LMF’s scheduling or UE’s decision. That means UE can use model or not use model in this new positioning method.

- Under this approach, AI intermediate feature can be reported together with other existing positioning methods (like DL-TDOA, DL-AoD or multi-RTT) since if a UE supports AI intermediate feature such as AI RSTD, the UE may also support the corresponding legacy measurement such as RSTD since the calculation is the same. So it will be no problem for UE to report legacy measurement and corresponding AI intermediate feature together.

- Option 4: Introduce a new AI/ML positioning method for use case 2b;

- This is to differ UE-side model and LMF-side model. use case 2b is LMF-side model and requires UE to report PRS channel measurement. The reason why use case 2b deserves a new positioning method rather than letting UE report PRS channel measurement together with the measurement report of existing positioning methods(like DL-TDOA, DL-AoD or multi-RTT) is because the Rel-19 UE may only have the UE capability of measuring/reporting PRS channel measurement. Such UE does not have the capability to make and report legacy measurement such as RSTD or Rx Tx time difference measurement which is a mandatory reporting in existing positioning methods.

Based on above analysis, the above four options are more reasonable than other kinds of mappings, so RAN2 can take these options as baseline for now, and make down-scope if needed in the future discussion.

Proposal 6: RAN2 to consider the following options to map the AI/ML positioning functionality (use case) to the positioning method in TS37.355:

- Option 1: Introduce a new AI/ML positioning method in LPP for use case 1, 2a and 2b;

- Option 2: Introduce a new AI/ML positioning method in LPP for use case 1 and 2a;

- Option 3: Introduce a new AI/ML positioning method in LPP for use case 1 and 2b;

- Option 4: Introduce a new AI/ML positioning method in LPP for use case 2b.

Next we will discuss what should be included in the newly introduced AL/ML positioning method.

In 37.355 logic, one positioning method contains the following 5 basic procedures:

Figure 7. LPP signaling in a positioning method in current spec

RAN2#126 [4] has agreed to reuse Capability transfer procedure to indicate supported AI/ML positioning capabilities, and agreed to reuse Location Information transfer procedure to provide the results of the UE sided model inference:

RAN2#126 agreement The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements.

For usecase 1 and usecase 2a, UE needs to get training data set for training. The training data set can be UE’s own channel PRS measurement (with the label of UE’s own location), or can be other PRU’s PRS channel measurement (with the label of other PRU’s location/intermediate feature).

To acquire the UE’s own PRS channel measurement and the UE’s own location, UE needs to get PRS configuration and/or TRP information in advance. So, the LMF should be able to provide PRS configuration and/or TRP information to a UE, LMF should also be able to provide other UE/PRU’s PRS channel measurement with PRU/UE location/intermediate feature to a UE, for AI/ML training (see green arrows in the figure 8).

Figure 8. LMF provides PRS configuration, TRP information and other UE/PRU’s PRS channel measurement with PRU/UE location/intermediate feature to target UE for AI training

We think the provision of such assistance data for AI/ML training can be embedded in ProvideAssistanceData message. Further, UE can make assistance data request for AI training using RequestAssistanceData message.

Proposal 7: For usecase 1 and usecase 2a, UE can request assistance data for AI/ML training using RequestAssistanceData message, and LMF can provide assistance data for AI/ML training using ProvideAssistanceData message.

RAN1#116-bis [5] has the following agreement on performance monitoring:

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Based on the agreement, there are 4 kinds of performance monitoring control procedure under RAN1’s decision:

- Option 1: UE generates monitoring metric, UE makes monitoring decision;

- Option 2: LMF generates monitoring metric, LMF makes monitoring decision;

- Option 3: UE generates monitoring metric, LMF makes monitoring decision;

- Option 4: LMF generates monitoring metric, UE makes monitoring decision.

Option 1 and option 2 appear to have no LPP specification on performance monitoring.

If option 3 is adopted, LMF needs to request UE to perform monitoring, and UE needs to report monitoring metric to LMF.

If option 4 is adopted, LMF needs to indicate monitoring metric to UE.

For the above potential spec impact, RAN2 needs to discuss whether a new set of signaling/procedure is necessary, or to embed it in other existing procedures (e.g. Location Information transfer procedure).

Proposal 8: RAN2 needs to discuss whether to introduce a new signaling procedure in LPP for performance monitoring metric transfer.

In current LPP specification, the Abort message is to allow the UE or LMF to abort an ongoing procedure due to some unexpected event (e.g., cancellation of a location request by an LCS client). It can also be used to stop an ongoing procedure (e.g., periodic location reporting from the UE).

Therefore, the abort procedure can be reused in this new AI positioning method for LMF to deactivate a functionality of UE. For example, if LMF makes the monitoring decision and decides the current AI/ML positioning result is bad, LMF can send the Abort message to abort the on-going PRS measurement/report of the AI positioning method.

Proposal 9: The Abort procedure can be reused in the new AI/ML positioning method, e.g., for LMF to deactivate an AI/ML functionality of UE.

In current LPP specification, the Error message is to indicate an error occurs at LMF or UE side during an on-going positioning method. For this new AI/ML positioning method, the Error message can be reused with some new error cause indication dedicated for AI/ML positioning use cases.

Proposal 10: The Error message can be reused in the new AI/ML positioning method, for LMF or UE to indicate AI/ML-specific errors.

Consistency between training and inference

There is a discussion on how to ensure the consistency between training and inference. The basic assumption is that, each AI model that UE trains should be under some certain condition, and the trained AI model should also be used for inference under the same or similar condition to achieve higher accuracy. From specification perspective, this means the set of attribute/parameter/condition of the training data input of an AI model should be same or similar as the set of attribute/parameter/condition of the inference data input.

For AI/ML positioning usecase 1, UE receives and measures TRP’s PRS for training and inference, so the set of attribute/parameter/condition of the training data input or inference data input can include NW additional conditions such as TRP location, TRP synchronization error, TRP number, PRS configuration, etc. It can be seen that for AI/ML positioning, the aforementioned NW additional conditions can already be indicated to the UE in current LPP spec since Rel-16.

Observation 3: For AI/ML positioning, the NW additional conditions (e.g., TRP location, TRP synchronization error, TRP number, PRS configuration) can already be indicated to the UE in current LPP spec.

So, ensuring the consistency can be based on UE implementation. When the inference configuration is provided to UE, UE will naturally know the current NW side additional conditions for inference. So UE will ensure the consistency by choosing the owned AI model which was trained under same/similar NW side additional condition.
In addition, it is hard to set up explicit relationship between NW additional conditions and AI models, for example, an AI model can have good generalization and it can work well under several different additional conditions.

Proposal 11: For AI/ML positioning usecase 1 and 2a where the AI model is at UE side, ensuring the consistency between training and inference can be based on UE implementation. No stage-3 impact is needed.

LCM of gNB involved use case (use case 3a and 3b)

Similar like section 2.1, the gNB side functionality can also be recognized as use case 3a where gNB reports AI intermediate feature to LMF, and as use case 3b where gNB reports SRS channel measurement to LMF.

For use case 3a, gNB’s model input is SRS’s channel measurement derived from the SRS that UE sends, the ground truth label is the AI intermediate features. Therefore UE additional condition such as UE location, UE velocity, etc., can also have impact on gNB’s model performance. That is to say, different gNB-side AI models can correspond to different UE or gNB additional conditions;

For use case 3b, LMF’s model input is SRS’s channel measurement derived from the SRS that UE sends, the ground truth label is the UE’s location. Therefore UE additional condition such as UE location, UE velocity, etc., can also have impact on LMF’s model performance. That is to say, different LMF-side AI models can correspond to different UE or gNB additional conditions.

Observation 4: For use case 3a, different gNB-side AI models can correspond to different UE or gNB additional conditions; for use case 3b, different LMF-side AI models can correspond to different UE or gNB additional conditions.

Based on the above understanding of functionality and AI model, there can have following approaches to control the LCM:

- Approach 1: LMF activates/deactivates the gNB functionality, and LMF activates/deactivates the AI model of the functionality.

  - Approach 1 means whether gNB activates/deactivates use case 3a or 3b is determined and indicated by LMF. For use case 3a where the AI model is at gNB side, LMF should further determine and indicate to gNB that which AI model should be activated/deactivated. That is to say, LMF needs to get gNB’s applicable functionality to know which functionality is currently available/unavailable to gNB, and LMF should also know the relationship between gNB/UE side additional condition and the AI model, so that LMF knows which AI model is currently available/unavailable to gNB.

- Approach 2: LMF activates/deactivates the gNB functionality, and LMF can allow gNB to activate/deactivate the AI model of the functionality

- Approach 2 means whether gNB activates/deactivates use case 3a or 3b is determined and indicated by LMF. For use case 3a where the AI model is at gNB side, LMF can allow gNB to activate/deactivate the AI model according to gNB/UE’s additional condition. That is to say, gNB needs to get UE’s additional condition to know which AI model should be activated/deactivated.

- Approach 3: LMF can allow gNB to activate/deactivate the functionality, and LMF can allow gNB to activate/deactivate the AI model of the functionality, and gNB reports the activated/deactivated functionality to LMF

- Approach 3 means LMF can allow gNB to activate/deactivate use case3a or use case 3b according to gNB’s own applicable functionalities. That is to say, LMF can allow gNB to report AI intermediate feature (using use case 3a) or to report SRS channel measurement (using use case 3b). Then gNB should let LMF know which functionalities is activated/deactivated. Further, LMF can allow gNB to activate/deactivate the AI model according to gNB/UE’s additional condition.

- Letting LMF know gNB’s decision of functionality may not need new explicit signaling since in AI/ML positioning, different functionalities has different kinds of reporting content. LMF can differentiate different kinds of functionality according to different kinds of reporting content that gNB reports.

As mentioned above, the relationship between additional condition and AI model is hard to specify, so LMF is hard to know such information.

It can be seen that approach 2 and 3 both require gNB to know the UE’s additional condition to make AI model decision. If the UE is PRU then we think the problem can be easily solved, i.e., gNB can get the PRU’s current additional condition and get the SRS sent by the PRU, gNB can activate/deactivate the AI model according to PRU/gNB additional conditions.

Since gNB can know the PRU side additional condition and gNB side additional condition, it is more efficient to let gNB decide the AI model under each functionality, i.e., go with approach 2 or approach 3.

Proposal 12: For AI/ML positioning use case 3a and 3b, RAN2 assumes the following approaches can be used for functionality-LCM:

- LMF activates/deactivates the functionality

- LMF allows gNB to activate/deactivate the functionality

Proposal 13: For AI/ML positioning use case 3a, RAN2 assumes that LMF can allow gNB to activate/deactivate the AI model of the functionality for model-LCM.

Proposal 14: For use case 3a, RAN2 assumes that gNB should to know the PRU side additional condition to determine an AI model. FFS on how gNB get such information.

This section should be low priority in RAN2 discussion as the main signaling is between gNB and LMF, which is RAN3’s scope. However RAN3 will start their discussion in October meeting with only 0.5 TU, it will be more efficient if RAN2 can make some recommendation or assumptions on use case 3a and 3b from RAN2 perspective, and send LS to them. Also, it is not hard to make such recommendation or assumptions, since the principle of LMF-UE and LMF-gNB use cases are very similar, we can make the recommendation or assumptions of LMF-gNB use cases according to RAN2’s agreement on LMF-UE use cases.

Proposal 15: Send LS to RAN3 about RAN2’s agreement of LCM for use case 3a and use case 3b.

- Conclusion

In this contribution, we propose the following observation and proposals:

Observation 1: Use case 1, use case 2a and use case 2b have clear RAN2 LPP specification impacts, wherein use case 1 is the 1st priority.

Proposal 1: Support RAN2 to discuss the LCM or the spec impact of use case 1, 2a and 2b together, even if use case 2b is not UE-side model.

UE-side model, Functionality/AI model activation:

Observation 2: Since NW side additional condition is already provided to UE, it is not necessary and not efficient to let UE report UE additional condition to LMF, then LMF decides the AI model and feedback to UE.

Proposal 2: For AI/ML positioning use case 1, 2a and 2b, RAN2 to consider the following approaches for functionality-LCM:

- LMF activates/deactivates the functionality

- LMF allows UE to activate/deactivate the functionality

Proposal 3: For AI/ML positioning use case 1 and 2a, RAN2 to consider that LMF allows UE to activate/deactivate the AI model of the activated functionality for model-LCM.

Functionality management as positioning method:

Proposal 4: For AI/ML positioning, the functionality can be recognized as use case 1, use case 2a and use case 2b.

Proposal 5: From RAN2 signaling structure perspective, AI/ML positioning functionality (use case) can be managed as positioning method, i.e., activating a positioning method means activating the corresponding AI/ML positioning functionalities (use cases).

Proposal 6: RAN2 to consider the following options to map the AI/ML positioning functionality (use case) to the positioning method in TS37.355:

- Option 1: Introduce a new AI/ML positioning method in LPP for use case 1, 2a and 2b;

- Option 2: Introduce a new AI/ML positioning method in LPP for use case 1 and 2a;

- Option 3: Introduce a new AI/ML positioning method in LPP for use case 1 and 2b;

- Option 4: Introduce a new AI/ML positioning method in LPP for use case 2b.

Proposal 7: For usecase 1 and usecase 2a, UE can request assistance data for AI/ML training using RequestAssistanceData message, and LMF can provide assistance data for AI/ML training using ProvideAssistanceData message.

Proposal 8: RAN2 needs to discuss whether to introduce a new signaling procedure in LPP for performance monitoring metric transfer.

Proposal 9: The Abort procedure can be reused in the new AI/ML positioning method, e.g., for LMF to deactivate an AI/ML functionality of UE.

Proposal 10: The Error message can be reused in the new AI/ML positioning method, for LMF or UE to indicate AI/ML-specific errors.

Consistency between training and inference:

Observation 3: For AI/ML positioning, the NW additional conditions (e.g., TRP location, TRP synchronization error, TRP number, PRS configuration) can already be indicated to the UE in current LPP spec.

Proposal 11: For AI/ML positioning usecase 1 and 2a where the AI model is at UE side, ensuring the consistency between training and inference can be based on UE implementation. No stage-3 impact is needed.

LCM of gNB involved use case (use case 3a and 3b):

Observation 4: For use case 3a, different gNB-side AI models can correspond to different UE or gNB additional conditions; for use case 3b, different LMF-side AI models can correspond to different UE or gNB additional conditions.

Proposal 12: For AI/ML positioning use case 3a and 3b, RAN2 assumes the following approaches can be used for functionality-LCM:

- LMF activates/deactivates the functionality

- LMF allows gNB to activate/deactivate the functionality

Proposal 13: For AI/ML positioning use case 3a, RAN2 assumes that LMF can allow gNB to activate/deactivate the AI model of the functionality for model-LCM.

Proposal 14: For use case 3a, RAN2 assumes that gNB should to know the PRU side additional condition to determine an AI model. FFS on how gNB get such information.

Proposal 15: Send LS to RAN3 about RAN2’s agreement of LCM for use case 3a and use case 3b.

- Reference

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 2024-09

- Report of 3GPP TSG RAN WG2 meeting #125-bis, Changsha, China, 2024-04

- TS 38.305-i10, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN, 2024-03

- Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, 2024-05

- R1-2403741, Summary of specification support for positioning accuracy enhancement, Changsha, China, 2024-04