3GPP TSG-RAN WG2 Meeting #127-bis                                     R2-2408221

Hefei, China, Oct 14th – 18th, 2024

Source:        ZTE Corporation

Title:           Introduction of BDS B2b in A-GNSS positioning

Agenda item:   8.16

Document for:  Discussion and Decision

- Introduction

China has published a BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2b in [1]. The WID [2] of Rel-19 BDS_B2b is as follows:

Specify A-BDS enhancements in Rel-19, including: – Supporting new signal of BDS B2b in A-GNSS for both E-UTRA and NR [RAN2]; – Specify corresponding assistant information in dedicated signaling [RAN2].

This contribution provides our view on the specification impact of introducing BDS B2b in A-GNSS positioning in TS37.355.

- Discussion

TS37.355 can contain the information of up to 16 GNSS systems wherein BDS is one of them. Each GNSS system can have up to 16 satellites, and each GNSS system can have up to 16 GNSS signals. B2b is a new signal of BDS.

The whole signaling structure of A-GNSS positioning in TS37.355 is as follows:

Figure 1. Signaling structure of A-GNSS positioning in TS37.355

The above signaling differs at different level with respect to different GNSS IDs, different satellite IDs and/or different signal IDs. To be specific:

For A-GNSS-ReqeustCapabilities, it only contains 3 request signalings with the format of BOOLEAN, so there is no specification impact on A-GNSS-ReqeustCapabilities when introducing BDS B2b signal in TS37.355.

For A-GNSS-ProvideCapabilities, UE sends the supported GNSSs (using gnss-ID) and supported GNSS signals (using GNSS-SignalIDs) to LMF. So BDS B2b signal should be introduced in GNSS-SignalIDs.

For A-GNSS-RequestAssistanceData, introducing BDS B2b signal will only impact gnss-GenericAssistDataReq, since it is a per-GNSS request. In the GNSS-GenericAssistDataReqElement, UE requests AD for each GNSS on data bit assistance, almanac assistance or differential correction assistance, etc. Current BDS-DifferentialCorrectionsReq only suits for B1I and B3I signal. RAN2 should discuss whether B2b signal has a different differential correction parameter to be requested independently.

For A-GNSS-ProvideAssistanceData, introducing BDS B2b signal will only impact gnss-GenericAssistData, since it is a per-GNSS AD provision. Each GNSS, e.g., BDS, is associated with a set of assisatance data IEs in gnss-GenericAssistData. According to our observation, at least the following IEs in gnss-GenericAssistData should consider the specification impact of introducing BDS B2b signals:

- gnss-NavigationModel: this IE provides navigation model assistance data of each satellite of BDS, wherein the issue of data(iod), health, clock model and orbit model may need to be updated regarding BDS B2b.

- gnss-DataBitAssistance: this IE provides data bit assistance data of each satellite, each GNSS signal of BDS, wherein the GNSS-SignalID should introduce BDS B2b, and the field description of gnss-DataBits should include reference of BDS B2b as [1].

- gnss-Almanac: this IE provides the coarse, long-term model of the satellite positions and clocks to UE. The week number of BDS B2b should be referenced to [1]; The BDS B2b has independent reduced almanac and Midi almanac as in section 7.9 and 7.8 of [1], so GNSS-AlmanacElement should make corresponding reference of BDS B2b.

- gnss-UTC-Model: there are 5 models for GNSS-UTC-Model, where model-2 is for B1C and B2a, model-5 is for B1I. RAN2 should discuss which UTC model the B2b corresponds to.

- gnss-AuxiliaryInformation: it contains the BDS B1C and BDS B2a satellite orbit type of each BDS satellite. The satellite orbit type of B2b should also be allowed to indicate in this IE. The field description of satType-r16 should increase B2b signal.

- bds-DifferentialCorrections: this assistance data is currently only for B1I and B3I. RAN2 should discuss whether this also suits for BDS B2b, or a new differential corrections is needed.

- bds-GridModel: this assistance data is currently only for B1I and B3I. RAN2 should discuss whether this IE also suits for BDS B2b, or a new differential correction is needed.

For A-GNSS-RequestLocationInformation, the IE does not contain any GNSS signal-specific request. So there is no specification impact on A-GNSS-RequestLocationInformation when introducing BDS B2b signal in TS37.355.

For A-GNSS-ProvideLocationInformation, the IE provides measurements per satellite (i.e., SV-ID), per GNSS signal (i.e., GNSS-SignalID) and per GNSS system (i.e., gnss-ID). So the only specification impact is to introduce BDS B2b in GNSS-SignalID.

Observation 1: There is no additional spec impact in A-GNSS-RequestLocationInformation and A-GNSS-ProvideLocationInformation when introducing BDS_B2b in A-GNSS.

There is also a GNSS-FrequencyID, which is to indicate different values corresponding to different centre frequency of different GNSS signals, see below figure:

Value & Explanation relation (below GNSS-FrequencyID)

| System | Value | Explanation |  |
| --- | --- | --- | --- |
|  |  | Link | Centre Frequency [MHz] |
| GPS | 0 | L1 | 1575.42 |
|  | 1 | L2 | 1227.60 |
|  | 2 | L5 | 1176.45 |
|  | 3-7 | reserved |  |
| SBAS | 0 | L1 | 1575.42 |
|  | 1 | L5 | 1176.45 |
|  | 2-7 | reserved |  |
| QZSS | 0 | L1 | 1575.42 |
|  | 1 | L2 | 1227.60 |
|  | 2 | L5 | 1176.45 |
|  | 3-7 | reserved |  |
| GLONASS k = -7..13 | 0 | G1 | 1602+k×0.5625 |
|  | 1 | G2 | 1246+k×0.4375 |
|  | 2 | G3 | 1202.025 |
|  | 3-7 | reserved |  |
| Galileo | 0 | E1 | 1575.420 |
|  | 1 | E6 | 1278.750 |
|  | 2 | E5a | 1176.450 |
|  | 3 | E5b | 1207.140 |
|  | 4 | E5 | 1191.795 |
|  | 5-7 | reserved |  |
| BDS | 0 | B1I | 1561.098 |
|  | 1 | B1C | 1575.420 |
|  | 2 | B2 | 1207.140 |
|  | 3 | B3 | 1268.520 |
|  | 4 | B2a | 1176.450 |
|  | 5-7 | reserved |  |
| NavIC | 0 | L5 | 1176.450 |
|  | 1-7 | reserved |  |

However the BDS B2b signal has a centre frequency as 1207.140 MHz, which is same to the B2 in the figure. So there is no need to change the figure of Value & Explanation relation in GNSS-FrequencyID.

Observation 2: There is no need to introduce BDS B2b in GNSS-FrequencyID.

Based on above analysis, the most basic change is to introduce BDS B2b in GNSS-SignalID and GNSS-SignalIDs. Example changes are given in Annex.

Except for this change, the introduction of BDS B2b only have impact on A-GNSS-RequestAssistanceData and A-GNSS-ProvideAssistanceData. Since A-GNSS-RequestAssistanceData is based on A-GNSS-ProvideAssistanceData, RAN2 should focus on some detailed IE in A-GNSS-ProvideAssistanceData to see whether these IEs should be provided independently/specifically for BDS B2b signal, or only adding reference of BDS B2b in current IE field description is enough. At least the following IEs can be considered: gnss-NavigationModel, gnss-DataBitAssistance, gnss-Almanac, gnss-UTC-Model, gnss-AuxiliaryInformation, bds-DifferentialCorrections and bds-GridModel.

Proposal 1: RAN2 to introduce BDS B2b signal in 37.355 IE GNSS-SignalID and IE GNSS-SignalIDs.

Proposal 2: RAN2 should focus on some detailed IE in A-GNSS-ProvideAssistanceData to see whether these IEs should be provided independently for BDS B2b signal, or only adding reference of BDS B2b in current IE field description is enough.

- Conclusion

In this contribution, we propose the following observation and proposals:

Observation 1: There is no additional spec impact in A-GNSS-RequestLocationInformation and A-GNSS-ProvideLocationInformation when introducing BDS_B2b in A-GNSS.

Observation 2: There is no need to introduce BDS B2b in GNSS-FrequencyID.

Proposal 1: RAN2 to introduce BDS B2b signal in 37.355 IE GNSS-SignalID and IE GNSS-SignalIDs.

Proposal 2: RAN2 should focus on some detailed IE in A-GNSS-ProvideAssistanceData to see whether these IEs should be provided independently for BDS B2b signal, or only adding reference of BDS B2b in current IE field description is enough.

- Reference

- BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2b (Version 1.0) , China Satellite Navigation Office, 2020-07

- RP-242413, New WID on Introduction of BDS B2b Signal in A-GNSS, CAICT, CATT, 2024-09

- Annex: Changes highlighted in GNSS-SignalID and GNSS-SignalIDs

System to Value & Explanation relation (below GNSS-SignalID)

| System | Value | Explanation |
| --- | --- | --- |
| GPS | 0 | GPS L1 C/A |
|  | 1 | GPS L1C |
|  | 2 | GPS L2C |
|  | 3 | GPS L5 |
|  | 4 | GPS L1 P |
|  | 5 | GPS L1 Z-tracking |
|  | 6 | GPS L2 C/A |
|  | 7 | GPS L2 P |
|  | 8 | GPS L2 Z-tracking |
|  | 9 | GPS L2 L2C(M) |
|  | 10 | GPS L2 L2C(L) |
|  | 11 | GPS L2 L2C(M+L) |
|  | 12 | GPS L5 I |
|  | 13 | GPS L5 Q |
|  | 14 | GPS L5 I+Q |
|  | 15 | GPS L1 L1C(D) |
|  | 16 | GPS L1 L1C(P) |
|  | 17 | GPS L1 L1C(D+P) |
|  | 18-23 | Reserved |
| SBAS | 0 | L1 C/A |
|  | 1 | L5 I |
|  | 2 | L5 Q |
|  | 3 | L5 I+Q |
|  | 4-7 | Reserved |
| QZSS | 0 | QZS-L1 C/A |
|  | 1 | QZS-L1C |
|  | 2 | QZS-L2C |
|  | 3 | QZS-L5 |
|  | 4 | QZS-LEX S |
|  | 5 | QZS-LEX L |
|  | 6 | QZS-LEX S+L |
|  | 7 | QZS-L2 L2C(M) |
|  | 8 | QZS-L2 L2C(L) |
|  | 9 | QZS-L2 L2C(M+L) |
|  | 10 | QZS-L5 I |
|  | 11 | QZS-L5 Q |
|  | 12 | QZS-L5 I+Q |
|  | 13 | QZS L1 L1C(D) |
|  | 14 | QZS L1 L1C(P) |
|  | 15 | QZS L1 L1C(D+P) |
|  | 16-23 | Reserved |
| GLONASS | 0 | GLONASS G1 C/A |
|  | 1 | GLONASS G2 C/A |
|  | 2 | GLONASS G3 |
|  | 3 | GLONASS G1 P |
|  | 4 | GLONASS G2 P |
|  | 5 | GLONASS G1a(D) |
|  | 6 | GLONASS G1a(P) |
|  | 7 | GLONASS G1a (D+P) |
|  | 8 | GLONASS G2a(I) |
|  | 9 | GLONASS G2a(P) |
|  | 10 | GLONASS G2a(I+P) |
|  | 11 | GLONASS G3 I |
|  | 12 | GLONASS G3 Q |
|  | 13 | GLONASS G3 I+Q |
|  | 14-23 | Reserved |
| Galileo | 0 | Galileo E1 |
|  | 1 | Galileo E5A |
|  | 2 | Galileo E5B |
|  | 3 | Galileo E6 |
|  | 4 | Galileo E5A + E5B |
|  | 5 | Galileo E1 C No data |
|  | 6 | Galileo E1 A |
|  | 7 | Galileo E1 B I/NAV OS/CS/SoL |
|  | 8 | Galileo E1 B+C |
|  | 9 | Galileo E1 A+B+C |
|  | 10 | Galileo E6 C |
|  | 11 | Galileo E6 A |
|  | 12 | Galileo E6 B |
|  | 13 | Galileo E6 B+C |
|  | 14 | Galileo E6 A+B+C |
|  | 15 | Galileo E5B I |
|  | 16 | Galileo E5B Q |
|  | 17 | Galileo E5B I+Q |
|  | 18 | Galileo E5(A+B) I |
|  | 19 | Galileo E5(A+B) Q |
|  | 20 | Galileo E5(A+B) I+Q |
|  | 21 | Galileo E5A I |
|  | 22 | Galileo E5A Q |
|  | 23 | Galileo E5A I+Q |
| BDS | 0 | B1 I |
|  | 1 | B1 Q |
|  | 2 | B1 I+Q |
|  | 3 | B3 I |
|  | 4 | B3 Q |
|  | 5 | B3 I+Q |
|  | 6 | B2 I |
|  | 7 | B2 Q |
|  | 8 | B2 I+Q |
|  | 9 | B1C(D) |
|  | 10 | B1C(P) |
|  | 11 | B1C(D+P) |
|  | 12 | B2a(D) |
|  | 13 | B2a(P) |
|  | 14 | B2a(D+P) |
|  | 15 | B2b |
|  | 16-23 | Reserved |
| NavIC | 0 | NavIC L5 SPS |
|  | 1-23 | Reserved |

Interpretation of the bit map in gnssSignalIDs-Ext

| GNSS | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | L2 Z | L2C(M) | L2C(L) | L2C(M+L) | L5 I | L5 Q | L5 I+Q | L1C(D) |
| SBAS |  |  |  |  |  |  |  |  |
| QZSS | L2C(L) | L2C(M+L) | L5 I | L5 Q | L5 I+Q | L1C(D) | L1C(P) | L1C(D+P) |
| GLONASS | G2a(I) | G2a(P) | G2a(I+P) | G3 I | G3 Q | G3(I+Q) |  |  |
| Galileo | E1 B+C | E1 A+B+C | E6C | E6A | E6B | E6 B+C | E6 A+B+C | E5B I |
| BDS | B2 I+Q | B1C(D) | B1C(P) | B1C(D+P) | B2a(D) | B2a(P) | B2a(D+P) | B2b |
| NavIC |  |  |  |  |  |  |  |  |

| GNSS | Bit 9 | Bit 10 | Bit 11 | Bit 12 | Bit 13 | Bit 14 | Bit 15 | Bit 16 (LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | L1C(P) | L1C(D+P) |  |  |  |  |  |  |
| SBAS |  |  |  |  |  |  |  |  |
| QZSS |  |  |  |  |  |  |  |  |
| GLONASS |  |  |  |  |  |  |  |  |
| Galileo | E5B Q | E5B I+Q | E5(A+B) I | E5(A+B) Q | E5(A+B) I+Q | E5A I | E5A Q | E5A I+Q |
| BDS |  |  |  |  |  |  |  |  |
| NavIC |  |  |  |  |  |  |  |  |