3GPP TSG-RAN WG2 Meeting #127bis		R2-2408222

Hefei, China, 14th – 18th Oct 2024

Source:	vivo

Title:          Discussion on LCM for UE-sided model for Beam Management

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

# Introduction

At RAN2#127 meeting, RAN2 studied and agreed on signalling procedure on applicable functionality reporting for beam management UE-sided model. To enable RAN2 further progress, LS R2-2407848 was sent to RAN1 to request for essential input from RAN1.

In this contribution, we will further discuss the functionality-based LCM for UE-sided model for Beam Management use case, focus on issues not dependent on RAN1 reply.

# Discussion

In the latest RAN2 meeting, RAN2 reached the following agreements regarding the applicable functionality reporting procedure:

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

For applicable functionality reporting, we think that RAN2 can consider an unified and complete signaling procedure for proactive reporting and reactive reporting. The unified procedure is illustrated in Figure 1:

Figure 1: unified and complete procedure (including proactive reporting and reactive reporting)

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

Step 3:  Network configures UE via RRCReconfiguration including OtherConfig that UE is requested to provide its applicable functionalities (and also is requested to update the applicable functionalities when the applicable functionalities change).

Step 4-5:  Upon receiving the RRCReconfiguration including OtherConfig, UE responses the RRCReconfigurationComplete message, and also UEAssistanceInformation message including the applicable functionalities.

Step 6: Start inference/monitoring based on network/UE activation/deactivation.

Step 7-8: UE sends updated applicable functionalities to network upon change of applicable functionalities.

Step 9: Based on updated applicable functionalities, start inference/monitoring based on network/UE activation/deactivation.

For reactive reporting, one remaining issue is how to respond the applicable functionality, i.e., via UAI or RRCReconfigurationComplete message. To support proactive reporting, the UEAssistanceInformation message needs to be enhanced anyway to include the applicable functionalities, therefore it is better to use the unified message for reactive and proactive reporting.

For handover scenario, the unified message, i.e., UEAssistanceInformation message, can also work well. Specifically, if the handover command includes the OtherConfig that UE is requested to provide its applicable functionalities, the UE can send the UEAssistanceInformation message including the applicable functionalities after sending the RRCReconfigurationComplete message to target gNB.

- RAN2 to discuss a unified and complete signaling procedure (including proactive reporting and reactive reporting), and use the unified message, i.e., UEAssistanceInformation message, for reactive and proactive reporting.

In the previous email discussion [2], there is an open question for further discussion:

- Whether neighbouring cell’s applicability needs to be considered in applicable functionality reporting to serving cell?

The issue was raised due to AI mobility use case. However, AI mobility is out of the scope of this WI. Beam management use case is only for serving cell. Therefore, we think that neighbouring cell’s applicability does not need to be considered in this WI.

- Neighbouring cell’s applicability is not considered, as it is not in the scope of this WI.

For the training of the UE-sided model, the UE needs to obtain the training configuration (e.g., associated ID(s)) of the serving cell. For beam management use case, the UE is in connected state when collecting the training data. Therefore, there may be two possible methods to obtain the training configuration of the serving cell:

- Alt 1: via the system information of the serving cell

Figure 2: via system information

- Alt 2: via the RRC dedicated signaling, e.g., after requested by the UE

Figure 3: via RRC dedicated signaling

With Alt 1, the broadcast associated ID can also be used for model inference. It means that Step 3 in the above Figure 1 does not need to include the associated ID any more. With Alt 2, the request for training configuration from the UE may be needed, otherwise, the gNB does not know whether the UE wants to perform the data collection for training. RAN2 can discuss which alternative(s) is adopted.

- For the training of the UE-sided model, RAN2 to discuss how UE obtains the training configuration (e.g., associated ID(s)) of the serving cell, i.e., via the system information or via the RRC dedicated signaling, or both.

# Conclusion

In the contribution, we have the following observations and proposals:

- RAN2 to discuss a unified and complete signaling procedure (including proactive reporting and reactive reporting), and use the unified message, i.e., UEAssistanceInformation message, for reactive and proactive reporting.

- Neighbouring cell’s applicability is not considered, as it is not in the scope of this WI.

- For the training of the UE-sided model, RAN2 to discuss how UE obtains the training configuration (e.g., associated ID(s)) of the serving cell, i.e., via the system information or via the RRC dedicated signaling, or both.

# Reference

- RP-240774 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R2-2406381 Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2