3GPP TSG-RAN WG2 Meeting #127bis		R2-2408229

Hefei, China, 14th – 18th Oct, 2024

Source:	vivo

Title:	Discussion on Topology 2 related aspects

Agenda Item:	8.2.5

Document for:	Discussion

# Introduction

Topology 2 with UE as intermediate node under NW control is in the scope of the ambient IoT SID [1]. In previous meetings RAN2 achieved below topology 2 related agreements [2]:

RAN2#127 Agreements

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

- A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

- RAN2 assumes that intermediate UE authorization is performed by upper layers.  RAN2 impact can be studied after further SA/RAN3 progress.

- RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

- NW is in control of resources to be used by AIoT air interface.

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

In this contribution, we will discuss topology 2 related aspects, listed as follows:

- UE Reader RRC state;

- AIoT radio resource allocation;

- AIoT data transport;

- Uu Paging;

- QoS handling.

# Discussion

Based on the basic AS procedures between AIoT device and reader, as Figure 1 shows, we may consider the potential impact on the interface between UE Reader and the network (BS/CN):

- UE reader is paged for AIoT service when the UE reader is in RRC IDLE or INACTIVE state.

- AIoT radio resources are allocated before UE reader initiates the AIoT procedure towards the AIoT device.

- UE Reader transports the AIoT data between the network and the AIoT device.

- Possible UE reader mobility, e.g. HO.

- Overall AS procedures in Topology 2

## 2.1 UE Reader RRC state

RAN2 has agreed to study cases for reader “temporarily” out of connection (e.g. RLF, HO). We analyze the Deployment Scenario 2 with Topology 2 firstly. The location of the UE reader is indoor while BS is outdoor; and the UE reader’s indoor mobility is possible.

We assume the UE Reader is in-coverage and configured with AIoT radio resources for AIoT service (e.g., inventory, command) by the network. When the UE Reader moves, RLF and reestablishment failure temporarily may occur, and then the UE has to enter RRC_IDLE if T311 expires. This scenario is a kind of “temporary” out of connection.

Observation 1.	UE reader may enter RRC_IDLE state after RLF recovery failure while performing AIoT AS procedure.

The AIoT service efficiency is an important KPI for user experience. During “temporarily” out of connection, we believe it is beneficial that the AIOT AS procedures (if triggered) will not stop but continue, such that the UE reader can complete the AS procedures even out of Uu connection.

We propose RAN2 to achieve this basic consensus.

Proposal 1:  	RAN2 to study how UE reader can continue AIoT AS procedures when the UE Reader is temporarily in RRC_IDLE state.

If the above temporary RRC_IDLE state is supported to perform AIoT AS procedure, more benefits are possible, e.g. UE power saving, and less interference between AIoT radio and Uu. In this case, UE can continue using the AIoT radio resources in RRC_IDLE state temporarily under network control.

What’s more, reader may be requested to perform periodic AIoT inventory service. In this scenario, it is better for the UE reader to use the AIoT radio resources configured by the network to periodically perform the AIoT service, e.g. the network may release the UE reader to RRC_IDLE or RRC_INACIVE state with AIoT radio resource allocation for potential periodic AIoT service execution. Most importantly, the periodical using of AIoT radio resources should comply with the network control and there is a little work to be done on the basis of the support of temporary RRC_IDLE state.

Proposal 2:  	In addition to RRC_CONNECTED state, UE reader in RRC IDLE/INACTIVE is also supported.

Proposal 3:  	RAN2 to study how UE reader can continue AIoT AS procedures when the UE Reader is released to RRC_IDLE/INACTIVE state.

- When UE reader is in RRC IDLE/INACTIVE,  the AIoT data collected by the UE during out-of-connection can still be reported to the network as soon as it recovers from the temporary out of connection or RRC_IDLE./INACTIVE state.

Proposal 4:  	The UE Reader can trigger the AIoT data reporting if needed as soon as it enters RRC_CONNCTED, if the UE Reader is previously in RRC_IDLE/INACTIVE state.

## 2.1 AIoT radio resource allocation for UE reader

Regarding the AIoT radio resource allocation for UE reader, some potential reference can to sidelink resource allocation schemes. As specified in TS38.300[3, sec5.7.2], two sidelink resource allocation modes are supported for sidelink communication: SL Mode-1 and SL Mode-2.

- SL Mode-1: the SL transmission resources are provided by the network via DCI.

- SL Mode-2: the sidelink resource pool(s) is provided by the network via RRC signaling. UE decides the SL transmission resources in the resource pool(s).

Due to the different scenario requirement, it may not be straightforward to reuse the SL mode-1/2. For example, the potential resource collisions need to be avoided by the sidelink UE to perform sensing of occupyed SL radio resources before resource selection. However, it is not suitable for the UE reader to perform sensing steps before occupying the SL mode-2 like radio resources. Similar to SL modes, two potential solutions for AIoT resource allocation and feaisbility in different RRC states are shown in below Table 1.

Table 1. AIoT resource allocation scheme

| AIoT resource allocation scheme | RRC_CONNECTED | Out of CONNECTED (i.e. RRC_IDLE/INACTIVE ) |
| --- | --- | --- |
| Option-1: Dynamic resource scheduling | Applicable each R2D/D2R resource is provided by the network via DCI | NOT Applicable |
| Option-2: Semi-static pre-configured resource | Applicable - the AIoT radio resource is configured by the network via dedicated RRC - UE uses the configured AIoT resources for AIoT radio. | Applicable - UE keeps the pre-configured AIoT resource. - UE uses the pre-configured AIoT resources in RRC_IDLE/INACTIVE. |

We can observe that dynamic resource scheduling (Option-1) supports the network to control the R2D/D2R resources via DCI, which adds more complexity to the UE Reader as well as the network. If UE Reader is “temporarily” out of connection, the AIoT resource is not available. It can only be used in RRC_CONNECTED state. On the contrary, Semi-static pre-configured resource (Option-2) applies to all RRC states of UE.

Observation 2.	Semi-static pre-configured resources applies to all RRC states of UE, while dynamic resource scheduling applies only to RRC_CONNECTED state.

With semi-static pre-configured resource mechanism, under the control of the network, the AIoT radio resource allocation in Topology 2 is considered as follows:

- The BS configures AIoT radio resource per UE reader via Uu when the UE Reader is in RRC_CONNECTED state, wherein the AIoT radio resource configuration contains at least the following information:

  - Time and frequency resources for AIoT air interface

  - Indication whether the AIoT radio resource is applicable or allowed to be used when UE Reader is out-of-connection. The signalling details on how to indicate can be left to WI phase.

- The UE reader can trigger R2D control and schedule AIoT time&frequency resources in the AIoT radio resource configuration for R2D/D2R communications as long as the AIoT resource configuration is still applicable.

In this way, dedicated AIoT radio resource allocation per UE is used in the control of network, where it can be possible to achieve a common solution for all RRC states of UE reader. Therefore,

Proposal 5:  	To achieve common solution for all RRC states of UE Reader, dedicated AIoT radio resource allocation per UE via dedicated RRC is considered as baseline in Topology 2.

There are further open questions regarding controlling & using of AIoT radio resources.

Firstly,  how to judge whether the allocated AIoT radio source is applicable? For example, validity area of the configured dedicated resource can be defined:

- Per cell: intra-cell using. dedicated resouces can only be used with the cell;

- Per gNB: intra-gNB using (among different cells of the same gNB);

- Per multiple gNB: inter-gNB using (among different gNB).

Secondly, to address the concern of allowing UE reader continuously occupying/using the dedicated resource, validity time can be defined. It also clarifies the meaning of “temporarily out of connection” in the previous agreement. In another words, UE reader is allowed to continue using the allocated AIoT resources within the validity time when it goes to temporarily out of connection.

Furthermore, it should allowed to consider the UE reader’s mobility. Even when UE reader moves within an indoor range, temporarily out of connection (e.g. HO, RLF) are possible.

- In case of HO, the AIoT radio resource may be reconfigured if UE moves out of  the validity area. Continuing the AIoT AS procedure during HO should be also considered.

- In case of RLF recovery failure and going to RRC_IDLE temporarily, the UE reader can only use the pre-configured resources within the defined or configured validily time. Validily time can also be applied after the UE reader is released to RRC_IDLE/RRC_INACTIVE by the network.

Proposal 6:  	RAN2 to study the controling & using of AIoT radio resources:

      - Option-1: the AIoT radio resource is applicable within validity area, e.g. per-cell, per-gNB or multiple gNB area.

      - Option-2: the AIoT radio resource is applicable within validity time.

      - Option-3: the AIoT radio resource can be reconfigured during HO procedure.

## 2.3 AIoT data Transport

Another key issue in Topology 2 is how UE reader transports AIoT upper layer information. In RAN3, potential solutions below are to be studied [4]:

6.3.2.1 Solutions for Topology 2 To support Topology 2, the following solutions are to be studied for conveying AIoT upper layer information: - RRC based solution. With this solution, AIoT CN applies AIoT upper layer information explicitly over XXAP signaling. AIoT upper layer information is then relayed explicitly to/from the AIoT-enabled UE via NR Uu RRC. - NAS based solution. With this solution, there is no explicit termination of AIoT upper layer information at AIoT-enabled gNB. AIoT upper layer information is transmitted over AIoT-enabled UE's NAS. - UP based solution. With this solution, there is no explicit termination of AIoT upper layer information at AIoT-enabled gNB. AIoT upper layer information is transmitted as AIoT-enabled UE's user plane data.

Potential RAN2 impacts due to each solution may be:

- RRC-based solution: AIoT data is transported via UE reader RRC message. This is a kind of control plane transport, i.e. SRB is used to transport AIoT data. In this way, it requires gNB to decode RRC message and further transport AIoT data to AIoT CN. It seems not advisable to expose too many AIoT data details to gNB.

- NAS-based solution:  AIoT data is transported via UE reader NAS container. CN handles the received AIoT data from UE reader NAS. In this way, the data of all AIoT devices are transferred through the UE NAS NG without distinction. And, gNB needs not to understand the details of AIoT data.This solution is more easier to support the UE reader to continue the AIoT AS procedure when temporarily out of connection.

- UP-based solution: AIoT data is transported via UE’s user plane. Tunnel establishment and AIoT data routing is needed to transfer the AIoT data to AIoT CN finally. It brings more complex handling in the network side than NAS-based and RRC-based solution.

Comparing the above options, NAS-based solution is the simplest and has less impact on RAN. Therefore, we propose RAN2 to assume AIoT upper layer information is conveyed over UE reader’s NAS container.

Proposal 7:  	RAN2 assumes UE reader’s NAS container is used to convey AIoT upper layer information for Topology2 if NAS-based solution is adopted.

## 2.4 Uu Paging

In Topology 2, IDLE/INACTIVE UE will be paged if it is selected as UE reader to perform AIoT service. Regarding how to page the selected UE reader, this question is related to the result of UE reader selection, which may be characterized in two cases as:

- Case 1: exact UE reader(s) are selected

For example, AIoT CN selects the exact UE reader(s) and provides one or multiple UE reader ID(s) to gNB to further initiate the paging, the Uu Paging message can include the exact UE reader ID(s).

- Case 2: a group of UE readers are selected

For example, if the network wants to inventory all AIoT devices, it may need help from a group of UE or even all the UE readers within the range of cells or tracking areas. In this case, a group of UE readers are selected but without any specific UE reader ID(s). We need to consider how to trigger all UE readers to perform AIoT service efficiently, e.g., UE group ID instead of exact UE ID(s) is contained in the Uu Paging message.

Therefore, we propose to study the Uu Paging for AIoT service considering the selected UE reader(s) is/are explicitly or ambiguously indicated.

Proposal 8:  	RAN2 to study the Uu Paging for AIoT service considering UE ID based paging or UE group based paging.

## 2.5 QoS handling

According to SA1 TS 22.369[5], the max allowed end-to-end latency requirement has been defined for performance service requirements of Ambient IoT. Generally, the end-to-end latency requirement needs to be met no matter which topology is deployed. Moreover, it is assumed that there is a radio latency requirement corresponding to the max allowed end-to-end latency requirement, and RAN2 can further study how to guarantee the radio latency requirement from AS perspective. Considering Topology 2 includes the AIoT radio interface and Uu radio interface, the end-to-end latency requirement needs to be split into Uu radio latency and AIoT radio latency requirements. Some detailed issues need further investigation, e.g.:

- Which entity is responsible for splitting the E2E latency requirement, e.g., AIoT CN, gNB, or UE Reader.

- How to apply the AIoT radio latency requirement in the subsequent inventory/command procedures e.g., in terms of resource allocation/scheduling, procedure success/failure, etc.

Hence, we suggest to study how to guarantee the end-to-end latency requirement for topology 2, and propose that:

Proposal 9:  	RAN2 to study how to guarantee the end-to-end radio latency requirement, which corresponds to the max allowed end-to-end latency requirement as specified in SA1 TS 22.369.

Proposal 10:  	QoS split functionality is required to define Uu radio latency and AIoT radio latency requirements for AIoT data transmission performance.

# Conclusion

Based on the discussion, we have the following observations and proposals:

Observation 1.	UE reader may enter RRC_IDLE state after RLF recovery failure while performing AIoT AS procedure.

Observation 2.	Semi-static pre-configured resources applies to all RRC states of UE, while dynamic resource scheduling applies only to RRC_CONNECTED state.

UE Reader RRC state

Proposal 1:  	RAN2 to study how UE reader can continue AIoT AS procedures when the UE Reader is temporarily in RRC_IDLE state.

Proposal 2:  	In addition to RRC_CONNECTED state, UE reader in RRC IDLE/INACTIVE is also supported.

Proposal 3:  	RAN2 to study how UE reader can continue AIoT AS procedures when the UE Reader is released to RRC_IDLE/INACTIVE state.

Proposal 4:  	The UE Reader can trigger the AIoT data reporting if needed as soon as it enters RRC_CONNCTED, if the UE Reader is previously in RRC_IDLE/INACTIVE state.

UE Reader resource allocation

Proposal 5:  	To achieve common solution for all RRC states of UE Reader, dedicated AIoT radio resource allocation per UE via dedicated RRC is considered as baseline in Topology 2.

Proposal 6:  	RAN2 to study the controling & using of AIoT radio resources:

- Option-1: the AIoT radio resource is applicable within validity area, e.g. per-cell, per-gNB or multiple gNB area.

- Option-2: the AIoT radio resource is applicable within validity time.

- Option-3: the AIoT radio resource can be reconfigured during HO procedure.

Data transport

Proposal 7:  	RAN2 assumes UE reader’s NAS container is used to convey AIoT upper layer information for Topology2 if NAS-based solution is adopted.

Uu paging

Proposal 8:  	RAN2 to study the Uu Paging for AIoT service considering UE ID based paging or UE group based paging.

QoS handling

Proposal 9:  	RAN2 to study how to guarantee the end-to-end radio latency requirement, which corresponds to the max allowed end-to-end latency requirement as specified in SA1 TS 22.369.

Proposal 10:  	QoS split functionality is required to define Uu radio latency and AIoT radio latency requirements for AIoT data transmission performance.

# References

- RP-240826 Study on solutions for Ambient IoT (Internet of Things) in NR.

- RAN2#127 Chair Notes.

- TS38.300-i20 NR; NR and NG-RAN Overall Description

- R3-244837 (TP for TR 38.769) RAN architecture aspects. Huawei.

- 3GPP TS 22.369, Service requirements for ambient power-enabled IoT; Stage 1 (Release 19).