3GPP TSG-RAN WG2 Meeting #127bis                                           	R2-2408244

Hefei, 14.-18.10.2024

Source:	Telit Communications S.p.A., Novamint , Sateliot, Thales

Title:	Further considerations on S&F operation from device perspective

Agenda Item:	8.9.2

Document for:	Discussion and Decision

# Introduction

The present document is an update of R2-2407487, it intends to further discuss some of the points agreed or indicated as FFS in the last RAN2 meeting. In addition, it also discusses whether S&F to real time or vice versa switching point should be indicated.

In the last RAN2 meeting following agreements were made related to S&F Satellite operation.

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

Even though in many areas RAN2 needs to wait until discussion in other groups has further progressed, some important decisions were already made especially by SA2, which are reflected hereafter:

Extract of SA2 [2] conclusions on KI#2 (Store and Forward):

1)	When feeder link is not available and the network supports S&F operation, the network shall be able to inform UE(s) whether S&F Satellite operation is applied, (e.g. eNB broadcast support of S&F operation as part of System Information).

NOTE 1:	The trigger for the eNB to broadcast support of S&F operation is based on the decision of RAN WGs. From system perspective the expectation is that if the network does not support S&F operation and the feeder link is not available then eNB switches off and does not broadcast any signal.

……

7) The MME may indicate to UE the estimated delivery time in NAS messages (Attach accept or TAU accept message or service accept). How UE uses this information is left for UE implementation.

# Discussion

A. Indication whether S&F Satellite operation is applied and indication whether S&F capability is supported

In accordance with previous RAN2 agreement #4 and SA2 agreements, the NW shall be able to inform UE(s) whether S&F Satellite operation is applied, i.e., a UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast. Let’s refer to this indication as S&F Activated indication.

Observation 1: UE should receive an indication via system information broadcast whether S&F satellite operation is currently activated in an NTN cell or not (as per RAN2 and SA2 agreements). This indication is referred to as S&F Activated indication in this paper.

Furthermore, in RAN2, there was discussion whether, in addition to the S&F Activated indication, we also need a static indication that in general the NW supports the feature. No decision was taken and the point was left FFS. Let’s refer to this static indication as S&F Capability indication.

In our view, when an NTN cell is currently operating in S&F mode, and accordingly, the S&F Activated indication is broadcast, providing an S&F Capability indication in addition to the S&F Activated indication is not needed since the mere presence of the S&F Activated indication is already an indication that the capability is supported.

On the other hand, when an NTN cell supporting S&F capability is currently operating in Real-Time (RT) mode (i.e. normal/default mode), providing an S&F Capability indication does not seem necessary either because all UE(s), irrespective of whether they support or not S&F capability, are expected to operate in the same way as they would do in a NTN cell with no support of S&F capability. In other words, the behavior of a UE is expected to be the same in the case of being served by an NTN cell that do not support S&F capability (e.g. pre-Rel-19 NTN cell) and in the case of being served by a NTN cell that does support the S&F capability though it is not currently activated/operational.

Proposal 1: If a S&F Activated indication is not provided in a NTN cell (i.e. either no S&F Satellite operation indication is provided via system broadcast or the S&F Satellite operation mode is explicitly indicated as not activated), the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal mode).

Proposal 2: From a UE behavior standpoint, there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but S&F Activated indication is not provided.

Proposal 3: Providing a S&F Capability indication, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the S&F Activated indication, is not needed.

B. Cell barring

About cell barring in a S&F network, the following agreements were reached:

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

The support of S&F satellite operation is not expected to introduce any modification or extension of the legacy cell barring mechanisms (i.e. use of legacy cellBarred and/or cellBarred-NTN bit), which should remain unchanged for legacy UE and Rel-19 UE. Hence, by using cellBarred and/or cellBarred-NTN, this should also imply for a Rel-19 UE, that the NTN cell is considered to be barred when the satellite does not provide a S&F Activated indication. In addition, this would also result in the fact that for a UE not supporting the S&F feature no changes to existing barring behaviour would be needed.

However, in case the NTN cell indicates the momentary support or activity of S&F (i.e. the NTN cell provides the S&F Activated indication), the UE supporting S&F shall behave as indicated by a Rel.-19 S&F indication element. A separate S&F barring indication is not required. We would maintain the general barring concept and have separate behaviour for S&F feature support. In such context, the scenario, considered by SA1, where an NTN cell supports S&F Satellite operation, and there may be a need to prevent temporary from S&F Satellite operation, i.e. when the storage is full, could also be handled by such approach.

Proposal 4: A Rel-19 UE (regardless whether supporting S&F or not) shall consider an NTN cell as barred if cellBarred-NTN is activated and no indication about support of S&F operation mode is given (i.e. the S&F Activated indication is not broadcast).

Proposal 5: In the case that the NTN cell is barred (cellBarred-NTN bit activated), a Rel-19 S&F-capable UE could still treat the NTN cell as not barred only for S&F satellite operation if S&F Activated indication is provided.

Proposal 6: The presence or absence of the S&F indication element shall be interpreted as access regulation/barring for Rel.-19 S&F operation.  Hence a S&F separate barring may not be needed in addition.

Below table summarises the possible situations:

| cellBarred-NTN | S&F Activated indication | Situation |
| --- | --- | --- |
| True | Not present | The cell operates in normal/default mode. The cell is barred for all NTN devices |
| True | Present | The cell operates in S&F mode. The cell is barred for all NTN devices except for S&F-capable devices. |
| False | Not present | The cell operates in normal/default mode. The cell is not barred. |
| False | Present | The cell operates in S&F mode. The cell is not barred. |

Proposals above would also cope for scenarios where a Rel.-19 UE would camp on a Rel.-18 NTN cell or where a NTN cell simply would not support S&F operation mode for whatever reason. As also for UE implementations for Rel.-19 not supporting S&F leaving barring behavior unchanged.

C. Additional indications/information for S&F Satellite operation

S&F is especially suited for delay-tolerant traffic, however additional indication on expected delivery delay would be important for the device.

Delivery delay is at least the time it requires to reach the next feeder link connection opportunity but may even be larger if not all data can be offloaded during visibility of the feeder link from the satellite. Hence an upper boundary indication of S&F expected delivery time is important in general for the device.

The UE could also use the expected delivery time to evaluate whether the data transfer service using S&F mode is suitable or not for the service needs.

Proposal 7: For S&F Satellite operation expected maximum delivery time should be indicated to the UE. FFS if an indication of the expected maximum delivery time can be provided via system information or signaling via NAS suffices.

A SIB indication could be a coarse indication informing on minimum delivery delay for a UE acquiring the S&F service, the NAS indication is also considering specific UE aspects. The tradeoffs should be further discussed by RAN2.

In addition, it was discussed in the last meeting whether switching times from real-time support to S&F support and vice versa should be also indicated by the satellite. It could be important for the UE to know, and especially for the application on the UE, which of the two modes, i.e. S&F mode or RT mode, is available at a given time. The knowledge on S&F is important also for applications on the device, e.g. whether to use TCP/IP or UDP protocol for the transfer.

Depending on the actual scenario it may be beneficial or not, if such information is provided. For instance, if next switching time from S&F to RT is provided, a UE connected to a NTN cell with S&F mode activated may refrain from transmission during the S&F period and wait for RT operation to become available. In the same way, if next switching time from RT to S&F is provided, a UE involved in bi-directional data exchange session (e.g. a TCP session) may decide to smoothly tore down the session before S&F operation starts.

On the other hand, given that an indication on mode switching time has no meaning for devices not supporting Rel-10 S&F operation, those devices may be impacted as the devices supporting both modes may adapt their behavior and hence are also impacted. In addition such an indication is only meaningful, when the service switching takes place while the UE is still in coverage of said satellite/NTNcell. (E.g. smaller than tservice for earth fixed cells.)

Proposal 8: It is up to the network to provide an indication on operation mode switching times.

Proposal 9: If network operation switching times are provided, UE behavior is up to UE implementation and application needs.

In case of switching the operation mode between RT and S&F or vice-versa the handling of ongoing or started service needs to be further discussed, i.e. whether the mode transition should keep in-order delivery of data packets for a started session or how to handle started TCP/IP sessions.

Scenarios to be clarified, i.e. when a session was started in S&F and the satellite switches to RT, is the in-order packet delivery ensured, or when a TCP/IP session was started in RT but not finalized until switching point is reached, how behavior is considered in those scenarios.

Of note is that the switching point between the S&F and RT modes should not necessarily coincide with the time that the feeder link gets activated or deactivated. For instance, in case of switching between S&F and RT mode, at the time that the feeder link gets activated, the satellite may download all previous stored data before switching to RT operation.

Proposal 10: RAN2 to further discuss expected behavior for ongoing sessions during mode transition.

FFS to consider a situation in which the satellite data storage is full (i.e. the satellite cannot accept more MO data from UEs) but the satellite can still perform control plane procedures (e.g. Attach, TAU) and MT traffic delivery.

Proposal 11: In the case that the satellite data storage is full (i.e. the satellite cannot accept more MO data from UEs) but the satellite can still perform control plane procedures (e.g.; Attach, TAU) and MT traffic delivery, an indication can be provided for UEs to refrain from sending MO traffic. FFS is the indication on maximum delivery time can be used also for this purpose.

D. Proposal for an Information Element

In light of above discussions, a possible approach to provide the S&F Activated indication along with additional information on switching times, maximum delivery time and MO data not allowed could be in the form of a new optional IE to be included in SIB31 (TBC):

- SIB31 (TBC)

  - (Optional) S&F_Operational_Status:

    - S&F Activated Flag: TRUE: S&F Activated, FALSE: S&F Deactivated, which is equivalent to RT/Default mode)

    - (Optional) S&F Switching Time: When activated flag=FALSE, indicates the time instant at which S&F mode will be activated. When activation flag=TRUE, signals the time instant at which the S&F mode will be deactivated.

    - (Optional) S&F Maximum Delivery Time

    - (Optional) S&F MO Data Barred Flag: TRUE: MO data is barred, not allowed, FALSE or IE not present: MO is allowed.

Proposal 12: Indication of S&F operational status can be an optional IE broadcast in SIB31 and include the following information:

  - (Optional IE in SIB31) S&F_Operational_Status:

    - S&F Activated Flag: TRUE: S&F Activated, FALSE: S&F Deactivated, which is equivalent to RT/Default mode)

    - (Optional) S&F Switching Time: When activated flag=FALSE, indicates the time instant at which S&F mode will be activated. When activation flag=TRUE, signals the time instant at which the S&F mode will be deactivated.

    - (Optional) S&F Maximum Delivery Time:

    - (Optional) S&F MO Data Barred Flag: TRUE: MO data is not allowed, FALSE or IE not present: MO is allowed.

# Conclusion

Observation 1: UE should receive an indication via system information broadcast whether S&F satellite operation is currently activated in an NTN cell or not (as per RAN2 and SA2 agreements). This indication is referred to as S&F Activated indication in this paper.

Proposal 1: If a S&F Activated indication is not provided in a NTN cell (i.e. either no S&F satellite operation indication is provided via system broadcast or the S&F satellite operation mode is explicitly indicated as not activated), the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal mode).

Proposal 2: From a UE behavior standpoint, there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but S&F Activated indication is not provided.

Proposal 3: Providing a S&F Capability indication, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the S&F Activated indication, is not needed.

Proposal 4: A Rel-19 UE (regardless whether supporting S&F or not) shall consider an NTN cell as barred if cellBarred-NTN is activated and no indication about support of S&F operation mode is given (i.e. the S&F Activated indication is not broadcast).

Proposal 5: In the case that the NTN cell is barred (cellBarred-NTN bit activated), a Rel-19 S&F-capable UE could still treat the NTN cell as not barred only for S&F satellite operation if S&F Activated indication is provided.

Proposal 6: The presence or absence of the S&F indication element shall be interpreted as access regulation/barring for Rel.-19 S&F operation.  Hence a S&F separate barring may not be needed in addition.

Proposal 7: For S&F Satellite operation expected maximum delivery time should be indicated to the UE. FFS if an indication of the expected maximum delivery time can be provided via system information or signaling via NAS suffices.

Proposal 8: It is up to the network to provide an indication on operation mode switching times.

Proposal 9: If network operation switching times are provided, UE behavior is up to UE implementation and application needs.

Proposal 10: RAN2 to further discuss expected behavior for ongoing sessions during mode transition.

Proposal 11: In the case that the satellite data storage is full (i.e. the satellite cannot accept more MO data from UEs) but the satellite can still perform control plane procedures (e.g.; Attach, TAU) and MT traffic delivery, an indication can be provided for UEs to refrain from sending MO traffic. FFS is the indication on maximum delivery time can be used also for this purpose.

Proposal 12: Indication of S&F operational status can be an optional IE broadcast in SIB31 and include the following information:

  - (Optional IE in SIB31) S&F_Operational_Status:

    - S&F Activated Flag: TRUE: S&F Activated, FALSE: S&F Deactivated, which is equivalent to RT/Default mode)

    - (Optional) S&F Switching Time: When activated flag=FALSE, indicates the time instant at which S&F mode will be activated. When activation flag=TRUE, signals the time instant at which the S&F mode will be deactivated.

    - (Optional) S&F Maximum Delivery Time:

    - (Optional) S&F MO Data Barred Flag: TRUE: MO data is barred, not allowed, FALSE or IE not present: MO is allowed.

# Reference

[1] R2-240XXX Report from Break-Out Session on NR NTN and IoT NTN

[2] TR 23.700-29 v1.0.0 [FS_5GSAT_Ph3_Arch]