3GPP TSG-RAN WG2 Meeting #127bis	R2-2408251

Hefei, China, Oct 14th -18th, 2024

Agenda Item:	8.1.2.2

Source:	NEC

Title:	Discussion on LCM for UE-sided model

Document for:	Discussion, Decision

Introduction

During last meeting, RAN2 has discussed a lot on LCM for UE sided model, some agreements have been made for applicability definition, LCM procedure and consistency issue.

In this paper, we will have some further discussion on these issues.

Discussion

During last meeting, RAN2 has made significant progress on the procedure related to UE applicability/capability reporting, as well as network configuration of network side additional condition. One comprehensive LS has been sent to RAN1 to announce RAN2’s agreement [1]. In addition, there are some leftover issues which needs to be resolved by RAN1. Meanwhile, in order to avoid repetitive discussion, RAN2 shall wait for RAN1’s reply LS before making any decision regarding to the FFS issues in the current agreement.

Skipping of Model training

Currently RAN2 has some discussion on model/functionality training, i.e., network side additional condition can be coupled with data set configuration so that can ensure consistency between model training and model inference. Yet, one will doubt whether it is necessary for each UE to go through the complete model training procedure. From our understanding, once the model has been well trained by one UE, it can be shared towards other UE for direct using, unless some of the following conditions have been met:

- Network side additional condition is changed, since according to RAN1’s intention, the model shall be available for specific network side additional condition, once the network side additional condition is changed, UE shall use updated data set to train the model again.

- UE side additional condition is changed, similar to network side additional condition, the model shall also be applicable to specific UE side additional condition, e.g. UE’s mobility speed, once the speed is changed, the previous model may no longer be applicable.

- Poor performance result, the well-trained model does not have a good performance result, which needs to be further trained.

From our observation, the UE does not need to perform model training if it has received one well trained model, unless some of the above-mentioned condition is met.

UE does not need to perform model training if it has received one well trained model, unless some of the following condition is met:

- Network side additional condition is changed

- UE side additional condition is changed

- Poor performance result is detected

In addition, even though UE is configured to perform model training, it also needs to fulfil the following two conditions:

- UE capability is fulfilled, since for UE sided model, the model training procedure is performed at either UE side or UE’s OTT server side, the procedure will consume quite a lot of resource for either UE or UE’s OTT server. Thus, model training cannot always be supported at UE side

- UE side additional condition is limited, even though the capability is good enough, UE side additional condition is dynamically changed, which cannot always ensure UE to perform model training, e.g., UE’s temperature is too high, UE’s battery life or current memory is too low.

Therefore, it is suggested some UE can be configured to skip the model training procedure

UE can only be configured to perform model training if its capability and UE side additional condition can fulfil the request.

It is suggested that for some UEs, the model training procedure can be configured to skip.

Common LCM framework

During last meeting, for UE-sided model, for the functionality management, the network decision and network-initiated AI/ML management is supported as a baseline. Both UE autonomous, reported to network and network decision, UE initiated can be further considered. While UE autonomous, UE decision and not reported to the network will not be considered in this release.

Considering whether UE can perform AI/ML based operation for certain functionality is not only rely on UE’s capability, but also rely on network environment. For example, when both model training and model transfer cost quite a lot radio resource which is unknown at UE side, whether UE can perform AI/ML based operation may closely rely on the evaluation of current cell capacity. Therefore, UE cannot fully make an accurate decision on functionality management and only reported to network of its decision. On the other hand, network involvement for LCM decision is really necessary. Therefore, it is suggested to only keep the following two principle:

- Network decision and network-initiated AI/ML management (fully controlled by network)

- Network decision and UE initiated AI/ML management (partially controlled by network)

UE cannot make reasonable decisions on AI/ML management due to its lack of the knowledge of radio resource condition and network capacity.

For LCM functionality management, it is suggested to keep the following two principles:

- Network decision and network-initiated AI/ML management (fully controlled by network)

- Network decision and UE initiated AI/ML management (partially controlled by network)

The fundamental difference between the two principles is that for fully network control, network will decide which AI/ML functionality can be activated and when to activate this functionality. For partial network control, network will decide which AI/ML functionality can be activated and UE will decide when to activate this functionality, optionally be based on a set of configured or specified criteria.

For the partially controlled AI/ML management principle, UE would decide to initiate the activation of AI/ML models based on a set of configured/specified criteria.

No matter for the functionality is fully controlled or partially controlled by network, model monitoring will be performed at both network side and UE side. When the model monitoring is performed at network side, network does not need to inform UE about the model monitoring performance result except further LCM functionality management.

When the model monitoring is performed at network side, network does not need to inform UE about the model monitoring performance result except further LCM functionality management instructions.

When the model monitoring is performed at UE side, UE should report the model monitoring performance result towards network. In order to improve the monitoring accuracy, UE should perform model monitoring procedures for certain times as a whole model monitoring procedure. In details, every time UE get the inference output, UE should be based on the inference output to perform model monitoring. Therefore, RAN2 should define model monitoring cycle. For each model monitoring cycle, UE will generate one model monitoring result. UE should report multiple model monitoring results towards network. In details, there are two reporting principles as listed in below:

- Dynamic reporting: for each monitoring cycle, as long as UE get the monitoring performance, it should feedback towards network immediately

- One time reporting: after the model monitoring is accomplished, UE will collect the monitoring result for all monitoring cycles as an output set and report towards network.

The pros and cons of the two performance feedback approaches are obviously observed, which is listed in the following table:

|  | Dynamic reporting | One time reporting |
| --- | --- | --- |
| Pros | Network can be aware of the model performance in a real-time way so that to perform LCM management more accurate | Decrease the complex of signalling exchange between UE and network |
| Cons | Every time UE get the monitoring result, it needs to report towards network, which needs extra procedure for uplink resource request. | Network cannot be aware of the model performance on time. |

According to the pro/con analysis, RAN2 should discuss which reporting principle of monitoring performance should be adopted.

The model monitoring accuracy can be improved by repeating multiple times of model monitoring procedure.

If model monitor is performed at UE side, RAN2 should define model monitoring cycle with regard to each model inference output.

If model monitor is performed at UE side, UE should report the performance result towards network for each model monitoring cycle.

If model monitor is performed at UE side, RAN2 can consider two reporting principles for performance reporting:

- Dynamic reporting: for each monitoring cycle, as long as UE get the monitoring performance, it should feedback towards network immediately

- One time reporting: after the model monitoring is accomplished, UE will collect the monitoring result for all monitoring cycles as an output set and report towards network.

The pros and cons of dynamic reporting and one time reporting are listed in the below table:

|  | Dynamic reporting | One time reporting |
| --- | --- | --- |
| Pros | Network can be aware of the model performance in a real-time way so that to perform LCM management more accurate | Decrease the complex of signalling exchange between UE and network |
| Cons | Every time UE get the monitoring result, it needs to report towards network, which needs extra procedure for uplink resource request. | Network cannot be aware of the model performance on time. |

RAN2 should determine which reporting principle should be adopted.

Considering that AI/ML based functionality activation will cause severe pressure on UE’s power, radio resource consumption and memory. So that UE has to perform fallback even if the model performance is still good enough. Similar as model monitoring, the AI/ML fallback strategy can also be performed at network side or UE side. If the AI/ML fallback decision is performed at network side, network does not need to inform UE about the strategy, e.g., fallback cause value, except further LCM management instructions.

However, if the fallback decision is performed at UE side, then network needs to provide AI/ML fallback configuration towards the UE. So that the UE can be based on the corresponding configuration to deactivate AI/ML functionality.

Network does not need to inform the UE about AI/ML fallback strategy, e.g., fallback cause value if the LCM fallback decision is performed at network side.

UE needs to be based on network configuration to perform AI/ML fallback decision if the decision is made at UE side.

Consistency issue

So far according to the progress of both RAN1 and RAN2, which are shown in below:

RAN1 #117 Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study) RAN1 #116bis Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions B: UE(s) collects the data corresponding to the associated ID(s) C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., Alt.1: NW assigns Model ID Alt.2: UE assigns/reports Model ID Alt.3: Associated ID(s) is assumed as model ID(s) “Model ID is determined/assigned for each AI/ML model” in D is not needed Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

The general principal usage of associated ID is quite clear. Basically, associated ID would be in relation with different sets of network side additional condition, of which the detailed condition will not be specified so that UE may not know. In order to keep the network side additional condition, set UE used for model inference is the same as the network side additional condition set UE used for model training, of which so called consistency, an associated ID would be indicated towards the UE. The procedure would be firstly network signals the data collection related configuration and it/their associated ID(s). Then UE(s) collects the data corresponding to the associated ID(s) and the AI/ML model are developed at UE side based on the collected data corresponding to the associated ID(s). Finally, UE reports information of its AI/ML model corresponding to associated IDs to the network and model ID is determined/assigned for each AI/ML model.

It can be clearly observed that there would be a mapping relationship between associated ID and model ID, where RAN1 has discussed whether the mapping relationship shall be one-to-one, one-to-many, many-to-one or many-to-many.

There would be a mapping relationship between associated ID and model ID, where RAN1 has discussed whether the mapping relationship shall be one-to-one, one-to-may, many-to-one or many-to-many.

Furthermore, considering the purpose of associated ID is to ensure the consistency, if there is the one-to-one mapping between associated ID and model ID, one can simply use model ID to indicate the associated ID so that consistency issue is no longer existing. So RAN2 should only consider to have one-to-many, many-to-one or many-to-many mapping relationship between model ID and associated ID.

If one-to-one mapping is adopted between model ID and associated ID, one can simply use model ID to indicate to associated ID as long as model ID is determined/assigned, and consistency issue is no longer existing.

RAN2 should consider to have one-to-many, many-to-one or many-to-many mapping relationship between model ID and associated ID.

Conclusion

In this contribution, we have discussed the opening issues for LCM at UE sided model, a brunch of observations and proposals are given out:

Observation 1	UE does not need to perform model training if it has received one well trained model, unless some of the following condition is met:

-	Network side additional condition is changed

-	UE side additional condition is changed

-	Poor performance result is detected

Observation 2	UE can only be configured to perform model training if its capability and UE side additional condition can fulfil the request.

Observation 3	UE cannot make reasonable decisions on AI/ML management due to its lack of the knowledge of radio resource condition and network capacity.

Observation 4	The model monitoring accuracy can be improved by repeating multiple times of model monitoring procedure.

Observation 5	The pros and cons of dynamic reporting and one time reporting are listed in the below table:

Observation 6	There would be a mapping relationship between associated ID and model ID, where RAN1 has discussed whether the mapping relationship shall be one-to-one, one-to-may, many-to-one or many-to-many.

Observation 7	If one-to-one mapping is adopted between model ID and associated ID, one can simply use model ID to indicate to associated ID as long as model ID is determined/assigned, and consistency issue is no longer existing.

Proposal 1	It is suggested that for some UEs, the model training procedure can be configured to skip.

Proposal 2	For LCM functionality management, it is suggested to keep the following two principles:

-	Network decision and network-initiated AI/ML management (fully controlled by network)

-	Network decision and UE initiated AI/ML management (partially controlled by network)

Proposal 3	For the partially controlled AI/ML management principle, UE would decide to initiate the activation of AI/ML models based on a set of configured/specified criteria.

Proposal 4	When the model monitoring is performed at network side, network does not need to inform UE about the model monitoring performance result except further LCM functionality management instructions.

Proposal 5	If model monitor is performed at UE side, RAN2 should define model monitoring cycle with regard to each model inference output.

Proposal 6	If model monitor is performed at UE side, UE should report the performance result towards network for each model monitoring cycle.

Proposal 7	If model monitor is performed at UE side, RAN2 can consider two reporting principles for performance reporting:

-	Dynamic reporting: for each monitoring cycle, as long as UE get the monitoring performance, it should feedback towards network immediately

-	One time reporting: after the model monitoring is accomplished, UE will collect the monitoring result for all monitoring cycles as an output set and report towards network.

Proposal 8	RAN2 should determine which reporting principle should be adopted.

Proposal 9	Network does not need to inform the UE about AI/ML fallback strategy, e.g., fallback cause value if the LCM fallback decision is performed at network side.

Proposal 10	UE needs to be based on network configuration to perform AI/ML fallback decision if the decision is made at UE side.

Proposal 11	RAN2 should consider to have one-to-many, many-to-one or many-to-many mapping relationship between model ID and associated ID.

Reference

[1]R1-2407604	LS on applicable functionality reporting for beam management UE-sided model 	Intel