3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408253

Hefei, China, 14th – 18th October, 2024

Agenda Item:	8.2.2

Source:	Huawei, HiSilicon

Title:	A-IoT functionalities

Document for: Discussion and Decision

1	Introduction

In RAN2#127 meeting, the following agreements on functionality were achieved:

Agreements - Send an LS to RAN1 regarding TBS size. Ask SA2 on the max and typical upper layer packet size. - RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages. FFS details on signaling - RAN2 will study the need and use of a simple message “size” reporting to the reader - At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional - RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

Based on the current RAN2 progress and focusing on the issues without conclusions, this contribution discusses segmentation, protocol stack, AS scheduling temporary ID, information visible to reader, message size reporting, etc.

2	Discussion

2.1	Segmentation

In RAN2#127, the following agreement was achieved:

RAN2#127 agreements on segmentation - Send an LS to RAN1 regarding TBS size. Ask SA2 on the max and typical upper layer packet size.

It is uncertain when other WGs will arrive at their conclusions. From RAN2 perspective, RAN2 can discuss some fundamental technical principles related to the feasibility if segmentation is supported.

- No need of sequence number, re-ordering, and segment retransmission:

The legacy sequence number (SN) of a RLC PDU is mainly used for retransmission-related procedures and out-of-order issues, such as:

- Using SN to check which RLC PDU transmission is failed, and the corresponding SDU with the missing SN can be retransmitted by ARQ procedures.

As for A-IoT, the functionalities of HARQ, ARQ, and AS-layer retransmission/repetition were precluded, according to RAN SID [1] and RAN2#126. There is no need to check which R2D/D2R message is missing since the reader implementation can address the issue of missing messages.

- Using SN to resolve the issues of out-of-order RLC PDUs, which is caused by lower-layer retransmission.

As for A-IoT, HARQ is not supported. Each D2R transmission is triggered by a R2D transmission, one device’s multiple D2R messages will be always transmitted in order. There is no out-of-order issue of R2D/D2R transmission.

As analysed above, the functionalities of SN and re-ordering are not needed for A-IoT.

Besides, since AS-layer retransmission/repetition is not supported, the retransmission of segments should not be supported either.

RAN SID General Scope - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ.

RAN2#126 agreements on functionality 4. AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed)

- To study “Almost-buffer-less” segmentation:

- First, it is critical to understand that the segmentation functionality requires some additional AS buffer/register. The AS buffer needs to store data to be transmitted and the segmentation position information (or un-transmitted segments, e.g. about 500 bits for half split segmentation and about 1000 bits for the worst case, if an approximate 1000-bit message needs to be segmented and buffered.).

Therefore, the legacy segmentation requires additional AS buffer in the A-IoT device, which, together with its operations, requires unnecessary and not negligible complexity in the A-IoT device.

Observation 1:	Additional AS buffer in the A-IoT device for temporarily storing the data to be transmitted (e.g. about 1000bits) requires unnecessary and not negligible complexity in the A-IoT device.

Observation 2:	Segmentation requiring temporarily storing the data to be transmitted (e.g. about 1000bits) is not acceptable to A-IoT devices from complexity and cost perspectives.

To develop a feasible segmentation feature, we need to find an alternative which can avoid requiring much AS buffer. For example, the data to be read or device ID is stored in the memory bank (a non-volatile memory (NVM)). The device just needs to store the current location information of the segmented data to be sent (e.g., data address pointer in the memory bank) instead of buffering all the data to be transmitted (up to almost 1000bits). By this way, it only requires a few bits to store this address information, which can be considered to be "almost-buffer-less".

- Only D2R segmentation is considered:

First, to support R2D segmentation, a device should anyway buffer the previous segment(s) before re-assembling. This inevitably adds buffer overheads to the A-IoT devices. We should reduce as much as possible the additional complexity requirement, if segmentation is supported.

Besides, a R2D message in the data transmission procedure, e.g., a DL Command, is usually not too large (e.g., not larger than 100 bits). Take RFID as an example, the main information provided by Write Command includes:

- Command (8-bit identifier of command types).

- MemBank (2-bit memory bank pointer).

- WordPtr (variable-bit word address). WordPtr uses format EBV (extensible bit vectors), which can indicate the word (16 bits) address in the memory bank specified by MemBank. It can use less bits to map a large range of index. For example, 128 addresses need 8 bits, and 16383 addresses only needs 16 bits.

- Data (16-bit data to be written after being XORed by RN16). Because, for ultra-low power consumption devices, the write latency is even longer than other processing time (e.g. R2D/D2R message processing). RFID only considers supporting Write Command including 16-bit data to be written each time as a mandatory command.

- RN16 (16-bit random number).

As analysed above, in RFID, the Write Command is usually not more than 100 bits.

In RFID, a Read Command includes 8-bit Command, 2-bit MemBank, WordPtr (16 bits as an example), 8-bit WordCont (number of words to read), 16-bit RN. The Read Command is also not more than 100 bits.

Observation 3:	The typical size for one R2D command is not large, e.g. below 100bits.

Hence, R2D transmission does not need segmentation.

We propose the limitations of the candidate segmentation for a feasible solution of A-IoT.

Proposal 1:	There should be the following restrictions on the simplified segmentation function, if needed based on the replies from other WGs (it is still proposed to not support segmentation at all):

No sequence number, no re-ordering, and no re-transmission of the segments (e.g. just 1bit indication of the segment);

Only “almost-buffer-less” segmentation is considered for the candidate solution, i.e. device is not required to buffer the un-transmitted segments or the original SDU in the AS buffer;

Only D2R segmentation to be considered (i.e. do not consider the R2D segmentation).

2.2	Protocol stack

In RAN2#125bis, RAN2 agreed the following for A-IoT protocol stack:

RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC.

It can be seen that two options are considered for A-IoT, i.e. “a new AS protocol on top of A-IoT MAC layer” or “A-IoT MAC layer”.

Currently, the AS higher layer functionalities include paging, random access, and data transmission (for device ID reporting, R2D command and D2R reply). For random access, it’s clear a A-IoT MAC layer function. For data transmission, it’s also easy to be considered as MAC PDU payload. Then for paging, based on the current RAN2 study, paging message is used to trigger device(s) to perform access, which should include paging ID/MUSK/ID filter and other essential information for device to determine RACH type and access occasions. To define paging message in MAC layer is feasible and straightforward. Then we also notice that there is scheduling relate control information required by L1 transmission/reception, which can also be included in MAC CE if needed, which is quite like legacy MAC function. Then we can arrive at the following observation that from RAN2 perspective, it is sufficient to include higher layer functionalities in A-IoT MAC layer and also carry the A-IoT control signalling for scheduling, if any, by MAC CE.

Observation 4:	A-IoT MAC layer is sufficient to contain AS higher layer functionalities (paging, random access and data transmission) as well as to provide MAC CE to carry L2 control signalling (e.g. for scheduling if any).

As discussed in 2.1 above, even if segmentation is supported, only a simplified segmentation function is feasible and can be adopted. Compared with legacy RLC segmentation where segment number is defined and retransmission of segment is supported, the simplified segmentation (i.e. “almost-buffer-less” segmentation) does not require any SN, or segment number. Instead, only one bit in the packet to indicate this is a sequent and there is a follow-up sequent should be sufficient. Therefore, there is no need to introduce a new protocol layer to support just one-bit indication, which means the simple segmentation function can be achieved by using the MAC layer.

Observation 5:	Even if segmentation is needed, the simplified segmentation without SN can be directly implemented in MAC layer, which will be quite simple and not worth a separate layer/protocol.

Proposal 2:	For A-IoT air interface protocol stack, it is assumed there is no other L2 AS layer (i.e. A-IoT MAC layer only).

2.3	AS scheduling temporary ID

In RAN2#125-bis, it was proposed to discuss whether a radio network temporary identifier (e.g. C-RNTI like) is needed for A-IoT device. This RNTI-like ID concept can be used to indicate that the data transmission is targeted to a specific UE in case other UEs decode the packet. Similarly, this mechanism can be also reused by A-IoT device. However, the RNTI-like design should be in RAN1 scope. By using this, the device can know that the data is transmitted to itself (not others) in physical layer without the need of decoding the data in MAC layer.

With respect of the A-IoT scheduling, RAN1 discussed the D2R scheduling and R2D scheduling in RAN1#118 meeting, and achieved the following agreement:

RAN1#118 meeting agreement For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used For R2D reception, the following information potentially can be explicitly/implicitly indicated to the device via PRDCH: - ID associated with device(s) intended for the reception of R2D, potentially including all devices (if supported) - FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

It is not clear about the detailed meaning of "ID associated with device(s)", we will wait for RAN1 further progress on this. But we can get the information that there will be some ID(s) for AS scheduling. From RAN2 perspective, some assumptions can be analysed. For example, with respect to the generation/allocation of the AS temporary ID, it is more reasonable to reuse a current random ID for random access or leave the reader to generate the temporary ID for device(s) considering different scenarios. Specifically:

- For contention-based case, RAN2#126 agreed that for 4 step RA “A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).” It is simpler to reuse this existing ID than to introduce a new temporary ID, to reduce the complexity of device implementation (e.g. on random generation). Therefore, after contention resolution, the device can reuse the existing random ID in Msg1 as the temporary ID for further R2D reception and scheduling.

- For contention-free case, for better scheduling, reader can generate and allocate an AS temporary ID to each device at the beginning, so that the device can directly use the allocated temporary ID for R2D reception. In addition, if different devices performing contention-free access generate the AS temporary ID by themselves, it may occur an “AS temporary ID conflict” between different devices, which leads to the case where a device incorrectly receives data of another device. Therefore, it is more reasonable for the reader to generate and allocate the AS temporary ID for each device in this case.

Even though the above analysis explains the usage of the AS scheduling ID, it should be still RAN1 discussion on the need of this AS ID.

Proposal 3:	It should be RAN1 to final decide on whether this AS scheduling ID is needed in D2R scheduling and R2D reception, while RAN2 only attempts to study some assumptions.

2.4	Information visible to reader from CN

In RAN2#127 meeting, the following agreements on functionality were achieved:

RAN2#127 Agreements on Visibility of A-IoT Information: 1 At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command). FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional 2 RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

In RAN2#126 meeting, the following agreements on functionality were achieved:

RAN2#126 agreements on functionality: - Legacy NR BSR/SR is not needed for A-IoT communication - FFS whether further indication of device message size/status is needed

To better schedule the next D2R message, a reader needs to know the (rough) D2R message size, especially when the size of D2R message is variable due to the following factors:

- For inventory use cases, the size of D2R message (e.g. device ID) may be not unified. As described in SA1 TR 22.840, for inventory, the size of a D2R message is primarily determined by the length of EPC (Electronic Product Code). The EPC is a unique identifier for an item that can be used in supply chain management systems to track and manage products, particularly in RFID (Radio Frequency Identification) systems where EPCs are often used. The EPC length can vary from 96 to 496 bits according to EPC Tag Data standard [5], and directly impacts the information capacity about the product. For example, longer EPC can include more details like manufacturer and product type, and can also be used to associate a larger number of products.

- For command use cases, the size of D2R message (e.g. response to different commands) is also variable. As described in SA1 TR 22.840, in different scenarios, D2R message size varies due to the different stored information. In RFID systems, specific memory banks or variable data can be indicated by a Read Command due to the flexibility and customization options.

As analysed above, the D2R message size, e.g., the device ID or the response to command, is variable. Hence, it is useful for the reader to be told about the D2R message size of the next D2R message for scheduling.

Proposal 4a:	It should be useful and visible to the reader: the expected (rough) message size of the following D2R transmission.

Except for the above information, other information may be useful for AS procedure if visible to the reader:

- Command type: So far, there is no conclusion on how a device responds to a command (e.g., write, read, disable). It is not sure if there is a command that does not need a mandatory response, which is up to SA2 discussion. For example, if a device receives a Disable Command, whether there is a response depends on SA2 design.

- Service request periodicity:

In RAN3 discussion on RAN-CN information, service request periodicity was mentioned:

RAN3#124 meeting discussion on RAN-CN interface impact: Paging: In the Inventory Request from the AIoT CN towards the AIoT RAS, it is FFS whether the following information may be included: Inventory Session ID (e.g., to allow multiple Inventory procedure instances to run in parallel): the purpose, scope and handling of this information in AIoT RAS are FFS. Estimated Number of expected responses from devices, or Minimum number of required responses from devices. FFS the usage of these information. Periodicity: to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN

If it is supported, there is a possible case: a reader needs to periodically initialize inventory AS procedure (e.g., paging and subsequent RA) after receiving a single inventory request from CN. It is useful for reader to know the service periodicity, e.g., inventory periodicity, to determine when to initialize the next service to devices.

So far, there is no conclusion on periodic inventory procedures yet. The detailed conclusion is up to RAN3/SA2 discussion. In addition, there is no further RAN2 impact if it is supported, since RAN2 anyway allows the reader to send repeat/multiple paging message.

- Device type/capability: Different types/capabilities of devices may support different features. However, the detailed physical designs for different device types are not finished. Therefore, RAN2 should wait for the completion of physical layer specification design. After that, RAN2 can check whether device type/capability impacts on the AS procedure, and whether device type/capability is needed by reader.

Proposal 4b:	Whether the following information should be visible to the reader and useful for AS procedure can be decided in Stage-3 phase:

Information 1: command type, which is pending on whether the command response from device is optional or mandatory in different command types;

Information 2: periodicity to execute the service request, which is up to RAN3/SA2 discussion;

Information 3: device type/capability, which can be discussed/checked after the completion of physical layer specification design.

2.5	Message size reporting/information

As analysed above, it is useful for the reader to know the D2R message size. The reader can obtain the message size through device report or CN indication:

- CN indication: As an alternative approach, CN can indicate the expected data size of the following D2R transmission to the reader when sending DL command. Then the reader can schedule the corresponding D2R message according to the CN’s indication, e.g., indicating TBS together with transmitting the DL command. Compared with device reporting-based approach, this can eliminate the need of two additional steps (e.g., BSR-like and scheduling MAC CEs) during the command procedures.

- Device report: The device can indicate the (rough) size of the next D2R data to be transmitted. As for command use cases, a device can know the data size to be transmitted only after receiving the command message. In such case, at least one interaction between device and reader is needed for scheduling the next D2R message. For this purpose, two additional steps are needed before R2D trigger and D2R transmission, i.e., one for message size reporting (e.g., via BSR-like MAC CE) and one for triggering message size reporting (e.g., scheduling MAC CE). This will increase the additional signalling overheads and reduce the transmission efficiency. Furthermore, size granularity of message size reporting impacts the D2R transmission overhead. For example, a more accurate message size is better for scheduling by reader, but takes more overheads.

By comparison between the above two methods, as shown in Figure 2.5.1, if the CN can provide message size information for both inventory and command use cases, CN indication will be sufficient.

Figure 2.5.1 Comparison between CN indication and Device report

As described in TR 38.848[2], the A-IoT use cases defined by SA1 can be classified into 8 parts:

These two groupings are then used to form representative use cases (rUCs) as follows, which are used in Clause 4.2 – Deployment scenarios and connectivity topologies. - rUC1: Indoor inventory - rUC2: Indoor sensors - rUC3: Indoor positioning - rUC4: Indoor command - rUC5: Outdoor inventory - rUC6: Outdoor sensors - rUC7: Outdoor positioning - rUC8: Outdoor command

See the mapping from RAN indoor command to the SA1 use cases below in TR 38.848.

| rUC | Applicable SA1 UCs / traffic scenarios |
| --- | --- |
| rUC1: Indoor inventory | 5.1 Automated warehousing 5.2 Medical instruments inventory management and positioning 5.4 Non-Public Network for logistics 5.5 Automobile manufacturing 5.7 Airport terminal / shipping port 5.15 Smart laundry 5.16 Automated supply chain distribution 5.18 Fresh food supply chain 5.27 End-to-end logistics 6.1 Flower auction 6.3 Electronic shelf label |
| rUC2: Indoor sensor | 5.6 Smart homes 5.13 Base station machine room environmental supervision 5.15 Smart laundry 5.20 Smart agriculture 5.23 Smart pig farm 6.2 Cow stable |
| rUC4: Indoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.29 Device Permanent Deactivation 6.3 Electronic shelf label |

Inventory, sensors, positioning, and command are defined as four independent use cases.

- Inventory: For inventory use cases, the device needs to reply its device ID to reader. Since the device ID format is defined by CN, the (rough) device ID size can be known by CN.

- Command:

1) For Read Command, the size of data to be read should be indicated by Read Command. Otherwise, the device needs to determine how much data it will transmit by itself. A micro-controller is usually required to complete this operation, and the information such as the data size needs to be recorded in the non-volatile memory (NVM). As a result, more complexity will be added by additional micro-controller, which is challenging to device 1. Besides, considering the size of data to be read is variable in different scenarios, inevitably and frequently updating information in NVM is required. However, as clarified in RAN1 LS reply to SA2 in R1-2407399, RAN1 thinks that frequent or recurring writing to NVM should be avoided. Therefore, it is better to indicate the size of data to be read by Read Command.

RAN1 answer to the second question from SA2 is the following: Whether an Ambient IoT device will be able to update its non-volatile memory based on its energy status and energy storage capabilities at some point after receiving a trigger from the Reader RAN1 would like to provide the following answer: - An Ambient IoT device will be able to update its non-volatile memory at some point after receiving a trigger from the Reader. Writing may not always be possible at all times. - The power consumption during writing to the NVM is higher than during reading. - RAN1 thinks that frequent or recurring writing to NVM should be avoided. - RAN1 has not discussed energy status and energy storage capabilities in relation to an Ambient IoT device ability to update its non-volatile memory.

Furthermore, if the Command format is defined by CN, in the Read Command, the CN needs to indicate the size of data to be read. If the Command format is defined by the application layer, this will impact the scale of production since Command use cases lack 3GPP specification. Even if by the application layer (e.g., AF) defines the Command format, the CN can also acquire the data size from the AF. Therefore, the CN can knows/decides the (rough) expected size of a Read Command (by itself or obtains it from AF).

2) For Write Command, if a response is needed, a fixed format of D2R message which indicates acknowledgment/non-acknowledgment is sufficient. Hence, the CN can also know the response size of a write/disable command.

- Sensor (out of R19 SI scope): For sensor use cases, if the data collected by sensor is variable, the exact data size may only be known by devices. According to RAN SID, only rUC1 (indoor inventory) and rUC4 (indoor command) are considered into R19 study scope [1]. Sensor use case is not in the R19 scope.

General Scope E. Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command).

Observation 6:	For inventory case, the CN has the knowledge of (rough) device ID size.

Observation 7:	For read command, the CN knows/decides the range in the device memory to be read. “Sensor data collection” use case is not in R19 scope, in which case only device knows the D2R data size.

Proposal 5:	CN can provide the information on the expected (rough) message size of the following D2R transmission.

2.6	Energy status reporting

In RAN2#127 meeting, the energy status reporting was discussed:

RAN2#127 agreements on energy status report - RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages. FFS details on signaling

First of all, to determine the remaining energy level, an additional energy management function is needed. Implementation of this function will introduce additional complexity/hardware requirements to an A-IoT device. That will be detrimental to the ultra-low power consumption devices (e.g., device 1). The more accurate remaining energy information detected by device, the more complexity will be added.

Observation 8:	Some device (e.g. device 1) is not able to determine the remaining energy level.

Then, devices from by different manufacturers may have different fabrication processes, so the time or power consumption of processing/transmitting/receiving one message will be different. Hence, it is hard to specify a unified criteria of energy information for processing/transmitting/receiving the follow-up messages. Hence, it should be up to device implementation on when/the criteria to send this information.

Observation 9:	It is not realistic to specify a common criterion to determine “not having energy for the follow up messages” for different devices.

In A-IoT, if a device does not have enough energy for the follow-up messages, there can be the following cases and corresponding solutions:

- Device is powered off, and cannot keep any temporary information in its register. In this case, the device will even forget any AS operation, e.g., if it has been paged/accessed or not. To solve this problem, the device can harvest enough energy then receive paging retransmission. Paging retransmission was agreed to be supported in RAN2#127 meeting. Specifically, harvesting energy depends on device implementation, and paging retransmission depends on reader implementation.

RAN2#127 agreements on paging triggers/retransmission: 1 Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

- Device starts to harvest energy before being powered off, and can keep some temporary information in its register. In this case, reusing the above solution can also work, i.e., the device receives paging retransmission to re-star the AS procedure, after harvesting enough energy. Besides, there can be other optimization. For example, the device can remember some AS operations, e.g., if it has been paged but has not been accessed successfully. It can perform re-access procedure after it harvests enough energy. This also depends on device implementation.

Therefore, based on device and reader implementation, the issue of “not have energy for the follow up messages” can be solved, without the energy status reporting.

Proposal 6a:	The energy status reporting is not suitable/necessary to be specified as an essential function for all devices.

If RAN2 finally agrees to support energy status reporting, it should be up to network implementation on whether to use this reporting. In addition, how to use the energy status information is up to reader implementation, e.g., sending more paging retransmission or allocating more re-access opportunities. To minimize additional impacts, the most simplified method to support energy status reporting is to report 1-bit information to indicate whether the energy is enough for the follow up messages. This is also because different devices manufactories may have different implementation, and it’s impossible to limit/specify the criteria to determine the remaining energy level. In this case, a 1-bit information with default value in energy status reporting can be used. This way can achieve a harmonized design for of devices who can support this feature. For example, if value 1 indicates that energy is enough, device 1 can set value=1 as a default value to continue the follow-up procedure.

Proposal 6b:	If RAN2 agrees on the need of energy status reporting:

it is up to deployment whether to use/implement this feature;

it is up to device implementation on when and the criteria to send this information;

it is up to reader implementation on how to use this assistant information;

only 1 bit information in piggyback manner is considered.

2.7	Positioning and proximity objectives

In RAN SID, proximity determination was proposed to be studied as shown below [1]:

RAN SID: - Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects).

As clarified in the RAN SID, the proximity determination aims to determine whether BS/intermediate UE and ambient IoT device are near each other or not [1].

RAN1#118 discussed the proximity determination issues and agreed that proximity determination is feasible using the two solutions below:

Conclusion Proximity determination is concluded to be feasible with either of the two solutions below. Potential specification impact or not will not be determined in the Rel-19 study item. Solution 1 For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission - then the device is determined as near to the reader based on measurements at the reader side Solution 2 For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission then the device is determined as near to the reader

Based on the above RAN1#118 agreement, it can be seen that the proximity determination work during SI phase is completed, which is also confirmed by RAN#105 meeting

Conclusion 7: no additional discussion in RAN1 on proximity determination in the Rel-19 SI.

Observation 10:	The proximity determination work during SI phase is completed in RAN1.

Furthermore, from E2E procedure perspective, the service request initiating the proximity determination should be an upper layer request (either from core network or from server). Otherwise, it is not clear how the reader will use the proximity determination result or where to report the proximity determination result. Considering SA2 and SA3 will take RAN SID into account during their study, RAN2 can assume the proximity determination is triggered by core network/server behind the core network.

SA3 SID: 1. Identify security threats introduced by AIoT services for use cases captured in TS 22.369, for topologies captured in RP-234058, and for architecture captured in TR 23-700-13. 2. Identify security requirements to address the identified threats. 3. Develop potential solutions that fulfill the security requirements, taking into account AIoT device constraints agreed upon in other 3GPP working groups.

SA2 SID: For this study, the types of devices, traffic assumptions and connectivity topologies are used from the RAN study item (please see RP-234058).

Observation 11:	RAN2 assumes the proximity determination use case is initiated/triggered by the core network/server behind the core network (e.g. by the inventory request).

For positioning/locating, RAN3 agreed the following:

RAN3#123bis agreement Use cases for locating an AIoT device: - Find an appropriate “reader” close to the A-IoT device; - Find where the A-IoT device is. Support locating the A-IoT device at “reader” granularity. RAN3#124 agreement AIoT device location information may be used for the following purposes: (a) improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the AIoT service. Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

As agreed by RAN3, the positioning/locating for A-IoT device can be performed at “reader” granularity. It can be seen that the “reader ID granularity” is enough for locating an A-IoT device in current SI phase. Considering that the location of reader in topology 1 is knowable to CN, and the UE reader’s position in topology 2 can be obtained using the current NR positioning procedure, there is no needs for RAN2 to further study the positioning/location issues for A-IoT device.

Proposal 7:	RAN2 understands there is no additional RAN2 impact for the positioning/locating and the proximity in study phase, i.e. it can be basically supported by the current procedure design.

2.8	DO-A gap assessment

In RAN SID, traffic types for A-IoT was proposed to be studied as shown below:

RP-240826: - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case.

It can be seen from the SID description that RAN2 first focus on DO-DTT and DT traffic type, and design a harmonized air interface for all the traffic types, after which RAN2 should further identify and analyse which part of the current harmonized air interface design is not sufficient to support the DO-A use case.

Observation 12:	Based on SID, RAN2 can assess which part(s) of the current design are not sufficient for the DO-A use case.

RAN2#126 discussed the A-IoT paging and agreed the following. The A-IoT paging message can indicate the device ID information and the resources information corresponding to current A-IoT service, based on which the device(s) can be triggered to perform the A-IoT service.

RAN2#126 agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

It can be seen from the above agreement that current A-IoT paging message is insufficient to support DO-A traffic. From RAN2 perspective, to perform the DO-A traffic, A-IoT paging message needs to be further enhanced to support the indication/filter related to DO-A traffic, e.g., new indication to indicate that any device with DO-A traffic can perform the A-IoT access as the response to this paging. With such indication, device with DO-A traffic could perform the A-IoT access, otherwise, device with DO-A traffic doesn't have resources for access or is even not allowed to perform the access. The A-IoT paging message with such enhancement can be sent periodically, or just by reader implementation.

Proposal 8:	It is observed that at least the A-IoT paging is an aspect/part of the current design which is not sufficient for the DO-A use case.

2.9	Multiplexing

In existing Uu MAC layer, MAC layer supports multiplexing of data from multiple logical channels, and multiplexing of data from logical channel and MAC CE. With respect of multiplexing of data from multiple logical channels, RAN2#126 agreed that “Multiple “AIoT logical channels” for upper layer data are not supported”. Based on the agreement, it is obvious that multiplexing of data from different A-IoT logical channels is not needed for A-IoT. With respect of multiplexing of upper layer data, i.e. MAC SDU, and control information, i.e., MAC CE, although it may cause additional A-IoT device complexity, it is feasible for A-IoT devices to support such multiplexing if needed.

Observation 13:	From A-IoT device complexity perspective, it is feasible to support the multiplexing (e.g. MAC SDU and MAC CE, if any), while it may cause additional device complexity.

2.10	Scheduling signaling

In legacy NR system, PDCCH can carry DL control information for UL data transmission. For A-IoT, in RAN1 agreement, for R2D, only one physical channel, i.e. PRDCH, is supported:

RAN1#117 agreement: For R2D, the only physical channel is PRDCH. - PRDCH carries any higher-layer payload - PRDCH carries L1 R2D control information (if defined) - FFS details of device behaviour(s) for receiving PRDCH

In RAN2 agreement, the A-IoT MAC layer is under study:

RAN2#125-bis agreements: - PDCP layer is not needed. FFS how to handle AS security (if needed pending SA3 discussion) and any other really needed functionalities. - SDAP is not supported for UP protocol stack. - RLC layer is not needed. FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size). RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed. - RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

From RAN2 respective, the L1 R2D control/scheduling information (if defined) can be put into A-IoT MAC payload, which is similar to legacy MAC CE. This can simplify the physical layer design, considering that only one R2D physical channel is supported. There should be no difference whether the R2D control information is specified in MAC layer or PHY layer, e.g., on latency and efficiency perspectives.

Proposal 9:	RAN2 considers that the A-IoT MAC layer signalling can be the solution to carry R2D scheduling/control information.

# 3	Conclusion

This contribution makes the following proposals:

Observation 1:	Additional AS buffer in the A-IoT device for temporarily storing the data to be transmitted (e.g. about 1000bits) requires unnecessary and not negligible complexity in the A-IoT device.

Observation 2:	Segmentation requiring temporarily storing the data to be transmitted (e.g. about 1000bits) is not acceptable to A-IoT devices from complexity and cost perspectives.

Observation 3:	The typical size for one R2D command is not large, e.g. below 100bits.

Observation 4:	A-IoT MAC layer is sufficient to contain AS higher layer functionalities (paging, random access and data transmission) as well as to provide MAC CE to carry L2 control signalling (e.g. for scheduling if any).

Observation 5:	Even if segmentation is needed, the simplified segmentation without SN can be directly implemented in MAC layer, which will be quite simple and not worth a separate layer/protocol.

Observation 6:	For inventory case, the CN has the knowledge of (rough) device ID size.

Observation 7:	For read command, the CN knows/decides the range in the device memory to be read. “Sensor data collection” use case is not in R19 scope, in which case only device knows the D2R data size.

Observation 8:	Some device (e.g. device 1) is not able to determine the remaining energy level.

Observation 9:	It is not realistic to specify a common criterion to determine to determine “not having energy for the follow up messages” for different devices.

Observation 10:	The proximity determination work during SI phase is completed in RAN1.

Observation 11:	RAN2 assumes the proximity determination use case is initiated/triggered by the core network/server behind the core network (e.g. by the inventory request).

Observation 12:	Based on SID, RAN2 can assess which part(s) of the current design are not sufficient for the DO-A use case.

Observation 13:	From A-IoT device complexity perspective, it is feasible to support the multiplexing (e.g. MAC SDU and MAC CE, if any), while it may cause additional device complexity.

Segmentation

Proposal 1:	There should be the following restrictions on the simplified segmentation function, if needed based on the replies from other WGs (it is still proposed to not support segmentation at all):

No sequence number, no re-ordering, and no re-transmission of the segments (e.g. just 1bit indication of the segment);

Only “almost-buffer-less” segmentation is considered for the candidate solution, i.e. device is not required to buffer the un-transmitted segments or the original SDU in the AS buffer;

Only D2R segmentation to be considered (i.e. do not consider the R2D segmentation).

Protocol stack

Proposal 2:	For A-IoT air interface protocol stack, it is assumed there is no other L2 AS layer (i.e. A-IoT MAC layer only).

AS scheduling temporary ID

Proposal 3:	It should be RAN1 to final decide on whether this AS scheduling ID is needed in D2R scheduling and R2D reception, while RAN2 only attempts to study some assumptions.

Information visible to reader from CN

Proposal 4a:	It should be useful and visible to the reader: the expected (rough) message size of the following D2R transmission.

Proposal 4b:	Whether the following information should be visible to the reader and useful for AS procedure can be decided in Stage-3 phase:

Information 1: command type, which is pending on whether the command response from device is optional or mandatory in different command types;

Information 2: periodicity to execute the service request, which is up to RAN3/SA2 discussion;

Information 3: device type/capability, which can be discussed/checked after the completion of physical layer specification design.

Message size reporting/information

Proposal 5:	CN can provide the information on the expected (rough) message size of following D2R transmission.

Energy status reporting

Proposal 6a:	The energy status reporting is not suitable/necessary to be specified as an essential function for all devices.

Proposal 6b:	If RAN2 agrees on the need of energy status reporting:

it is up to deployment whether to use/implement this feature;

it is up to device implementation on when and the criteria to send this information;

it is up to reader implementation on how to use this assistant information;

only 1 bit information in piggyback manner is considered.

Positioning and proximity objectives

Proposal 7:	RAN2 understands there is no additional RAN2 impact for the positioning/locating and the proximity in study phase, i.e. it can be basically supported by the current procedure design.

DO-A gap assessment

Proposal 8:	It is observed that at least the A-IoT paging is an aspect/part of the current design which is not sufficient for the DO-A use case.

Scheduling signaling

Proposal 9:	RAN2 considers that the A-IoT MAC layer signalling can be the solution to carry R2D scheduling/control information.

# 4	References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, Mar., 2024

- TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, Sep., 2023

- TR 22.840, Study on Ambient power-enabled Internet of Things, 2023

- EPC™ Radio-Frequency Identity Protocols Class-1 Generation-2 UHF RFID Protocol for Communications at 860 MHz – 960 MHz

- EPC Tag Data Standard, version 2.0.0, available at https://ref.gs1.org/standards/tds/2.0.0/