3GPP TSG RAN WG2 #127-bis	R2-2408257

Hefei, China, October 14th – 18th, 2024

Source:	Panasonic

Title:	SIB33/SIB33-NB multi-beam signalling

Agenda Item:	8.8.6     LTE to NR-NTN mobility

Document for:	Discussion, decision

# 1. Introduction

The Work Item Description for LTE to NR-NTN mobility is this, see [2]:

The [RAN2] work item aims at specifying support for the following objectives:

Idle mode mobility based on cell reselection from E-UTRA-TN to NR- NTN, where E-UTRA-TN provides satellite information for NR-NTN neighbour cells in a System Information Block

Furthermore, we reached the following nine agreements regarding agenda item “8.8.6 LTE to NR-NTN mobility” during the most recent RAN2 meeting #127 in Maastricht – see [3]:

Agreements:

1.	Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33 (can come back to this if we find serious issue with this).

2.	The ntn-PolarizationalDL is optional.

3.	Reuse the SatelliteId-r18 to identify either an NR- or an IoT-NTN satellite.

4.	Consider a solution that avoids repeating the ephemeris for a satellite which provides both IoT- and NR-NTN cells.

5.	maxSat-r17 (4) is reused for the maximum number of NR satellites.

6.	RAN2 will not do further work to introduce multiple SMTCs in LTE.

7.	NR-NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA-TN to NR-NTN cell.

8.	RAN2 confirms that measurements of NR-NTN cells for a UE in E-UTRAN-TN RRC_INACTIVE are supported, with the understanding that UE moves to RRC idle upon selecting the NR-NTN cell.

9.	Introduce a new UE capability without signalling for LTE-TN to NR-NTN mobility.

Table 1: List of RAN2#127 agreements regarding LTE to NR-NTN mobility

This paper proposes an irrelevance-saving solution covering agreements 1 to 5 above, in particular.

# 2. Discussion

# 2.1 Background

# It is possible to refer from a terrestrial E-UTRA cell to NTN neighbour cells – be it NR-NTN cells or (NB-)IoT-NTN cells. This task is overtaken by SIB33(-NB) [1] – see table 2 below outlining the coding.

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

...

}

NeighSatelliteInfoList-r18 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfo-r18 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18,

ephemerisInfo-r18				CHOICE {

stateVectors-r18				EphemerisStateVectors-r17,

orbitalParameters-r18			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18		SEQUENCE {

nta-Common-r18					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18			INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r18					SEQUENCE {

startSFN-r18					INTEGER (0..1023),

startSubFrame-r18				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r18						INTEGER (1..512)				OPTIONAL,	-- Need OP

t-ServiceStartNeigh-r18		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

-- ASN1STOP

Table 2: SIB33(-NB) coding

Analysing the character of the fields making up SIB33(-NB), we can detect that all fields apart from one are satellite-specific. The one exception is t-ServiceStartNeigh-r18, which is beam/cell-specific (that field is present only in case of NTN quasi-Earth-fixed neighbour cell(s)). So as long as the satellite emits a single beam or multiple beams covering the same area, the coding is fine, but as soon as the satellite emits multiple beams covering different areas the entire SIB33(-NB) has to be repeated for each beam. That leads to a pretty significant amount of irrelevance due to the currently unavoidable repetition of fields.
Furthermore, SIB33(-NB) doesn’t consist of a polarization indication for the downlink. The need for such an indication had been agreed during the last meeting #127, because it saves time for finding the SSB of the signal covering the neighbour cell.

Figure 1 below illustrates the different multi-beam situations. On the left hand side the two beams are covering the same area, on the right hand side the two beams are covering separate areas.

Figure 1: Satellite emitting two beams covering the same (left) or different area(s) (right)

Observation 1:	SIB33(-NB) mainly features satellite-specific fields – apart from one that is beam-specific. That structure leads to a significant signalling inefficiency when it comes to multi-beam satellites, because the entire IE SIB33(-NB) needs to be repeated – one SIB33(-NB) instance per beam.

Observation 2:	SIB33(-NB) does not yet consist of an optional polarization indication for each DL beam as required with agreement 2 listed – see table 1.

# 2.2 Proposal for SIB33(-NB) extension

# 2.2.1 Adding a beam loop to IE SIB33(-NB)

The proposed way out of this signalling dilemma is the addition of a beam loop to SIB33(-NB). That loop assigns a beam identifier – beamId-r19 – to each beam emitted by the satellite identified with satelliteId-r18 and consists of the field t-ServiceStartNeigh-r19. The latter field is defined in the same way as its Rel.18 counterpart already.
Also an optional ntn-PolarizationDL field becomes part of the named loop, because in the absence of that indication the UE may need more time to search the SSB of the satellite. Since it is still possible to find the SSB, that field can be an optional one. Also agreement 2 from last meeting requires the addition of that field in optional form.
Furthermore, the geographical area illuminated by the beam in question is described with a circular area consisting of the longitude and latitude of a related reference location as well as the radius of the circle.

Observation 3:	For efficient coding a beam-related loop featuring the characteristics of multiple beams needs to be added to SIB33(-NB).

Proposal 1:	Add a beam-related loop – enabling the indication of the characteristics of multiple beams – to SIB33(-NB).

# Coding

For the exact coding of the new fields see table 3 – basically an excerpt from SIB33(-NB) coding described with table 2 plus added highlighted new fields.

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

NeighSatelliteBeamList-r19 ::=	SEQUENCE (SIZE(1..maxBeam-r19)) OF NeighSatelliteBeams-r19

NeighSatelliteBeams-r19 ::=	SEQUENCE {

satelliteId-r18					SatelliteId-r18,

beamId-r19						BeamId-r19,

t-ServiceStartMBNeigh-r19		TimeOffsetUTC-r17			OPTIONAL	-- Need OR

ntn-polarizationDL-r17			ENUMERATED{rhcp,lhcp,linear}OPTIONAL	-- Need R

footprintInfo-r19			SEQUENCE {

referencePoint-r17		SEQUENCE {

longitude-r17			INTEGER (-131072..131071),

latitude-r17			INTEGER (-131072..131071)

} 														OPTIONAL,	-- Need OR

radius-r17					INTEGER (1..256)

},

lateNonCriticalExtension		OCTET STRING					OPTIONAL,	...

}

Table 3: beamId loop and fields therein (SIB33(-NB))

satelliteID:	The satellite ID applicable to a specific cell, used to associate with the satellite assistance information for neighbour cells.

beamId:		The beam ID applicable to a specific cell illuminated by the satellite identified with satelliteID, used to associate with the beam assistance information for neighbour cells.

t-ServiceStartMBNeigh:	Indicates the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s) served by the satellite beam indicated by satelliteId and beamId, see 5.5.3.1, 5.5.8 and 36.304 [4].

ntn-PolarizationDL:	If present, this parameter indicates polarization information for downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization.

latitude:	Latitude of the reference point. Unit in degree. Step of 360/262144 degree. Actual value = field value * (360/262144).

longitude:		Longitude of the reference point. Unit in degree. Step of 360/262144 degree. Actual value = field value * (360/262144).

radius:		Distance between the reference point and the edge of the satellite or beam coverage. Unit in km. Step of 10 km. Actual value = field value * 10.

Observation 4:	A fully backwards-compatible extension of SIB33(-NB) for indicating the characteristics of multiple beams is feasible.

Proposal 2:	Adopt the coding proposal outlined with clause 2.2.2 of this document R2-2408257 for an efficient coding of multiple beam characteristics as part of SIB33(-NB) – a fully backwards-compatible extension.

# 3. Conclusion

In this document, we discussed how to resolve the multi-beam signalling dilemma characterising SIB33(-NB) in its Rel.18 version.

The following observations and proposals were made:

Observation 1:	SIB33(-NB) mainly features satellite-specific fields – apart from one that is beam-specific. That structure leads to a significant signalling inefficiency when it comes to multi-beam satellites, because the entire IE SIB33(-NB) needs to be repeated – one SIB33(-NB) instance per beam.

Observation 2:	SIB33(-NB) does not yet consist of an optional polarization indication for each DL beam as required with agreement 2 listed – see table 1.

Observation 3:	For efficient coding a beam-related loop featuring the characteristics of multiple beams needs to be added to SIB33(-NB).

Observation 4:	A fully backwards-compatible extension of SIB33(-NB) for indicating the characteristics of multiple beams is feasible.

Proposal 1:	Add a beam-related loop – enabling the indication of the characteristics of multiple beams – to SIB33(-NB).

Proposal 2:	Adopt the coding proposal outlined with clause 2.2.2 of this document R2-2408257 for an efficient coding of multiple beam characteristics as part of SIB33(-NB) – a fully backwards-compatible extension.

- References

[1]	3GPP TS 36.331 V18.3.0 (2024-09)	3rd Generation Partnership Project; Technical Specification
Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 18)

[2]	RP-240776	Revised WID on Non-Terrestrial Networks (NTN) for NR-NTN
Phase 3

[3]	R2-127 NR-NTN-IoT-NTN (Sergio)_ EOM	NTN Chair’s minutes of meeting #127 in August 2024