3GPP TSG-RAN WG2 Meeting #127-bis	R2- 2408258

Hefei, China, Oct 14th - Oct 18th, 2024

Agenda Item:	7.4.2

Source:	MediaTek Inc.

Title:	Discussion and correction on RRC and UE capability for LTM

Document for:	Discussion, Decision

Introduction

This paper discusses the leftover UE capability issue for FG45-3a and provides correction to LTM fast RRC processing.

Discussion

Leftover UE capability issue for FG45-3a

In Rel-18 LTM, FG45-3a indicates the UE capability related to MAC-CE activated joint LTM TCI states for early TCI states activation, with following signalling structure:

-- R1 45-3a: MAC-CE activated joint LTM TCI states ltm-MAC-CE-JointTCI-r18 SEQUENCE { qcl-Resource-r18 ENUMERATED {ssb, trs, both}, maxNumberJointTCI-PerCell-r18 INTEGER (1..16), maxNumberJointTCI-AcrossCells-r18 ENUMERATED {n1,n2,n3,n4,n8,n16,n32} }

However, there is ambiguity with respect to the third component maxNumberJointTCI-AcrossCells-r18. Therefore, clarification and correction are needed.

The scope of maxNumberJointTCI-AcrossCells in FG45-3a

Regarding the third component maxNumberJointTCI-AcrossCells-r18, in the description of RAN1 Feature list and also TS 38.306, it is described as:

maxNumberJointTCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated joint LTM TCI states across candidate cells and serving cells.

However, this sentence can be interpreted into two understanding and may need further clarification:

- Understanding 1: The volume of this component includes the legacy TCI state indicated by FG23-1-1 UnifiedJointTCI-r17, which represent a total maximum number of activated TCI states that UE can support.

- Understanding 2: The volume of this component only includes “LTM TCI states” for the candidate cells and serving cell. Given that the LTM TCI states contains TCI states of candidate cells and also serving cell, this sentence is not wrong.

Proposal 1: RAN2 to clarify the understanding of maxNumberJointTCI-AcrossCells in FG45-3a:

- Understanding 1: The volume of this component includes the legacy TCI state indicated by FG23-1-1 UnifiedJointTCI-r17, which represent a total maximum number of activated TCI states that UE can support.

- Understanding 2: The volume of this component only includes LTM TCI states indicated in LTM-TCI-info. The value of UnifiedJointTCI-r17 is not taken into account.

Proposal 2: No matter which understanding, add a note in TS 38.306 for clarification.

Further potential corrections for understanding 1

If we go understanding 1, an issue arises where the values for a large number of that component are inconsistent, leading to UEs being unable to report precise supported values.

For example, if a UE supports 16 activate TCI states for legacy TCI state in FG23-1-1 UnifiedJointTCI-r17, and 8 activate TCI states for LTM TCI state in FG45-3a. The absence of the value 24 in ENUMERATED {n1, n2, n3, n4, n8, n16, n32} means the UE cannot identify a suitable value to report.

Observation 1: The ENUMERATED range of the component maxNumberJointTCI-AcrossCells in FG45-3a becomes discontinuous for large values, potentially causing UEs to struggle in finding an appropriate value to report.

Therefore, several options of corrections can be made to this feature:

- Option 1: Change the value of maxNumberJointTCI-AcrossCells in FG45-3a from ENUMERATED {n1, n2, n3, n4, n8, n16, n32} to INTEGER (1..32).

With option1, UE can indicate any value within this range, and the issue is fixed. Alternatively, option 2 can be considered to indicate the maximum number of activated joint LTM TCI states across candidate cells.

- Option 2.1: Add an new component in this IE to represent the maximum number of MAC-CE activated joint LTM TCI states across candidate cells.

- Option 2.2: Add an new IE (new UE capability) to represent the maximum number of MAC-CE activated joint LTM TCI states across candidate cells.

Proposal 3: If understanding 1 is the common understanding, RAN2 should further discuss the solution to address the discontinuous value issue for maxNumberJointTCI-AcrossCells in FG45-3a.

- Option 1: Change the value of maxNumberJointTCI-AcrossCells in FG45-3a from ENUMERATED {n1, n2, n3, n4, n8, n16, n32} to INTEGER (1..32).

- Option 2.1: Add an new component in this IE to represent the maximum number of MAC-CE activated joint LTM TCI states across candidate cells.

- Option 2.2: Add an new IE (new UE capability) to represent the maximum number of MAC-CE activated joint LTM TCI states across candidate cells.

The TPs of these three options can be found as below:

Option1:

---- Begin TP ----

-- R1 45-3a: MAC-CE activated joint LTM TCI states

ltm-MAC-CE-JointTCI-r18                                         SEQUENCE {

qcl-Resource-r18                                                ENUMERATED {ssb, trs, both},

maxNumberJointTCI-PerCell-r18                                   INTEGER (1..16),

maxNumberJointTCI-AcrossCells-r18                              INTEGER (1..32)

}

---- End TP ----

Option2:

---- Begin TP ----

-- R1 45-3a: MAC-CE activated joint LTM TCI states

ltm-MAC-CE-JointTCI-r18                                         SEQUENCE {

qcl-Resource-r18                                                ENUMERATED {ssb, trs, both},

maxNumberJointTCI-PerCell-r18                                   INTEGER (1..16),

maxNumberJointTCI-AcrossCandidateCells-r18					INTEGER (1..16),

maxNumberJointTCI-AcrossCells-r18                              ENUMERATED {n1,n2,n3,n4,n8,n16,n32}

}

---- End TP ----

Option3:

---- Begin TP ----

-- R1 45-3a: MAC-CE activated joint LTM TCI states

ltm-MAC-CE-JointTCI-r18                                         SEQUENCE {

qcl-Resource-r18                                                ENUMERATED {ssb, trs, both},

maxNumberJointTCI-PerCell-r18                                   INTEGER (1..16),

maxNumberJointTCI-AcrossCells-r18                              ENUMERATED {n1,n2,n3,n4,n8,n16,n32}

}

…

lTM-MAC-CE-JointTCI-acrossCandidateCells-r18				INTEGER (1..16) 			OPTIONAL,

---- End TP ----

Correction to LTM fast RRC processing

In LTM, the 'fast RRC processing' means the UE functionality to perform ASN.1 decoding and validity/compliance check for the RRCReconfiguration message, which will applied upon the LTM cell switch, before reception of the LTM Cell Switch Command MAC CE.

The conditions under which the UE performs the fast RRC processing are defined in TS 38.133 and the related UE capabilities are defined in TS 38.306, shown in CR cover sheet in Annex A.

As captured in the yellow highlights in the 38.133 and 38.306 clips in Annex A, the UE needs to know the number of serving cells (SpCell + SCell(s)) configured by each LTM candidate configuration, in order to know whether the fast RRC processing conditions are fulfilled or not.

However, with the current ASN.1, the UE does not know the number of serving cells in each LTM candidate configuration, which creates a chicken-egg problem for the UE - the UE needs to ASN.1 decode the LTM candidate configuration(s) in order to know whether or not to ASN.1 decode the LTM candidate configuration(s). This is very counter-intuitive and breaks the fundamental logic of the fast RRC processing functionality.

Therefore, we proposed to add a new field in LTM-Candidate IE to indicate the number of serving cells configured by this LTM candidate configuration.

Corresponding TP is in Annex A.

Proposal 4: Add a new field in LTM-Candidate IE to indicate the number of serving cells configured by this LTM candidate configuration. Endorse the TP in Annex A.

Conclusion

In this contribution, the following observations and proposals are made:

Leftover UE capability issue for FG45-3a

Proposal 1: RAN2 to clarify the understanding of maxNumberJointTCI-AcrossCells in FG45-3a:

- Understanding 1: The volume of this component includes the legacy TCI state indicated by FG23-1-1 UnifiedJointTCI-r17, which represent a total maximum number of activated TCI states that UE can support.

- Understanding 2: The volume of this component only includes LTM TCI states indicated in LTM-TCI-info. The value of UnifiedJointTCI-r17 is not taken into account.

Proposal 2: No matter which understanding, add a note in TS 38.306 for clarification.

Observation 1: The ENUMERATED range of the component maxNumberJointTCI-AcrossCells in FG45-3a becomes discontinuous for large values, potentially causing UEs to struggle in finding an appropriate value to report.

Proposal 3: If understanding 1 is the common understanding, RAN2 should further discuss the solution to address the discontinuous value issue for maxNumberJointTCI-AcrossCells in FG45-3a.

- Option 1: Change the value of maxNumberJointTCI-AcrossCells in FG45-3a from ENUMERATED {n1, n2, n3, n4, n8, n16, n32} to INTEGER (1..32).

- Option 2.1: Add an new component in this IE to represent the maximum number of MAC-CE activated joint LTM TCI states across candidate cells.

- Option 2.2: Add an new IE (new UE capability) to represent the maximum number of MAC-CE activated joint LTM TCI states across candidate cells.

LTM fast RRC processing

Proposal 4: Add a new field in LTM-Candidate IE to indicate the number of serving cells configured by this LTM candidate configuration. Endorse the TP in Annex A.

References

- R2-2407600-feMob-UE-caps-38306-post-127-final

- R2-2407859-femob-UE-cap-38331-post-127-final

- R1-2403703 Updated RAN1 UE features list for Rel-18 NR after RAN1_116bis

- R2_127_R18 SL_MOB_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM

- Annex A – TP for Correction to LTM fast RRC processing

| Reason for change: | In LTM, the 'fast RRC processing' means the UE functionality to perform ASN.1 decoding and validity/compliance check for the RRCReconfiguration message, which will applied upon the LTM cell switch, before reception of the LTM Cell Switch Command MAC CE. The conditions under which the UE performs the fast RRC processing are defined in TS 38.133 and the related UE capabilities are defined in TS 38.306, shown below. As captured in the yellow highlights in the 38.133 and 38.306 clips below, the UE needs to know the number of serving cells (SpCell + SCell(s)) configured by each LTM candidate configuration, in order to know whether the fast RRC processing conditions are fulfilled or not. However, with the current ASN.1, the UE does not know the number of serving cells in each LTM candidate configuration, which creates a chicken-egg problem for the UE - the UE needs to ASN.1 decode the LTM candidate configuration(s) in order to know whether or not to ASN.1 decode the LTM candidate configuration(s). This is very counter-intuitive and breaks the fundamendal logic of the fast RRC processing functionality. 38.306, clause 4.2.7.2: ltm-FastProcessingConfig-r18 Indicates whether the UE supports fast processing of LTM candidate cell RRC configuration. This capability signalling comprises the following parameters: - maxNumberStoredConfigCells-r18 indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTM candidate configurations and Scell(s) in LTM candidate configurations for MCG and SCG, that UE can store the configurations. - maxNumberConfigs-r18 represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133 [5]. A UE supporting this capability shall also indicate support of ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18. UE shall set the capability values for maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18 consistently for all bands. These capability values represent the maximum number across all the supported bands. NOTE: The conditions for fast processing of an LTM candidate cell RRC configuration is defined in clause 6.3 in TS 38.133 [5]. 38.133, clause 6.3.1.3: The interruption time TLTM-interrupt is the time between the end of the last TTI containing the MAC-CE command for LTM cell switch until the time the UE transmits the first UL message on the target cell, excluding Tcmd stated in section 6.3.1.2. TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms, Where: TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command. TLTM-RRC-processing = 0, if the UE supports ltm-FastProcessingConfig-r18 capability, and: - The number of candidate cells in the LTM candidate cell configuration does not exceed maxNumberConfigs-r18 and the number of the configured serving cells and candidate cells does not exceed maxNumberStoredConfigCells-r18, or - The following conditions are fulfilled for the target cell: - UE has received LTM candidate cell TCI state activation command for the target cell at least THARQ + 13 ms before the LTM cell switch command, and/or - UE has received PDCCH order for early RACH for the target cell at least NT,2+10 ms before the LTM cell switch command, where NT,2 is defined in section 8.1 of TS 38.213 [3], and - The number of candidate cells for which TCI state(s) were activated or PDCCH order was received before the cell switch command does not exceed maxNumberConfigs-r18, and the number of serving cells and candidate cells for which TCI state(s) were activated or PDCCH order was received before the cell switch does not exceed maxNumberStoredConfigCells-r18. - [FFS further conditions] - [FFS: SCell is not part of the cell switch.] Otherwise TLTM-RRC-processing = 10 ms. |
| --- | --- |
| Summary of change: | A new field is added in LTM-Candidate IE to indicate the number of serving cells configured by this LTM candidate configuration. Impact analysis Impacted 5G architecture options: NR standalone, NR-DC, NE-DC Impacted functionality: Fast RRC processing in LTM Inter-operability: If the network implements the change and the UE does not, there is no interoperability issue. If the UE implements the change and the network does not, the UE does not have required information for the fast RRC processing functionality. Consequently, the network may assume that TLTM-RRC-processing is 0 ms whereas it actually is 10 ms. |
| Consequences if not approved: | The fast RRC processing functionality for LTM is fundamentally broken and does not work as intended. |

- –	LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...

[[

ltm-NrOfServingCells-r18                       INTEGER (1..maxNrofServingCells)                      OPTIONAL     -- Need M

]]

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

-- TAG-LTM-CANDIDATE-STOP

-- ASN1STOP

LTM-Candidate field descriptions

ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate configuration.

ltm-CandidatePCI This field identifies the PCI of the SpCell of the LTM candidate configuration contained in ltm-CandidateConfig.

ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.

ltm-NoResetID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config.

ltm-NrOfServingCells If ltm-ConfigComplete is configured, this field indicates the number of serving cells (SpCell and SCell(s)) configured in ltm-CandidateConfig. Otherwise, this field indicates the total number of serving cells (SpCell and SCell(s)) configured in ltm-ReferenceConfiguration and ltm-CandidateConfig. The network always configures this field.

ltm-UE-MeasuredTA-ID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID. This field is absent if tag2 is present for this LTM candidate configuration.