3GPP TSG-RAN WG2 Meeting #127-bis	R2- 2408260

Hefei, China, Oct 14th - Oct 18th, 2024

Agenda Item:	8.6.4

Source:	MediaTek Inc.

Title:	Measurement reporting discussion

Document for:	Discussion, Decision

Introduction

In the last meeting, RAN2 discussed the report container for event triggered L1 measurement reporting. This contribution further discusses report pattern, report content and subsequent measurement reporting.

Discussion

Report pattern

The report content is highly related to the report pattern. We suggest that RAN2 discuss the report pattern first before discussing the detailed report content.

In the last meeting, RAN2 have made an agreement that, the beam level measurement result, but not cell level measurement result, is used for LTM event evaluation. However, during the discussion, it is still not clear the definition of the cell level or beam level measurement/ report, and the scope of the agreement is still vague.

From companies understanding, there are different options for the report pattern:

- Option 1: Beam level measurement, beam level report. Trigger event is configured for single beam. TTT is reset if the event condition does not meet the threshold for that beam.

- Option 2: Beam level measurement, cell level report. The measurement is performed at beam level, but TTT will continue if the beam index change within the same cell. Only one report is sent for one cell.

Although beam level report may cause more signalling overhead due to the fluctuations, it still provides more detailed beam quality information to the network. As LTM is a kind of cross cell beam switching, network need to know the specific quality of the indicated beam. It is suggested that the result is reported at beam level and not be impacted by other beams. Therefore, we prefer option1.

Proposal 1: The event triggered L1 MR is reported at beam level. Individual event is defined for each beam and no cross-beam TTT is introduced.

Report content

In legacy L3 measurement report, the physical cell ID, measurement quantity, RS type and related RS index are involved. We can consider these as starting point to further discuss the component of the event triggered L1 measurement report.

No matter the report pattern, at least the following components should be included in the event triggered L1 MR:

- MeasQuantity: The measurement quantity (e.g., value of RSRP, RSRQ) is needed to represent the link quality of the measured beam/cell. It is a must in the report. RAN1 may further discuss the type of measurement quantity(e.g., RSRP, RSRQ, etc.) to be used in event triggered L1 MR.

- RS type: RS type is included in the legacy L3 measurement report, and it is also included in the WID of Rel-19 mobility (to introduce CSI-RS measurement). Therefore, it should be agreeable to introduce the RS type to represent the measured resource RS in the report, since the measured value of different RS could be very different.

- Triggering event type: When multiple events are configured simultaneously, it could be beneficial to include the event type in the measurement report for network judgement. It is also included in the legacy L3 measurement report.

- RS index: RS index is needed for the network to know the measured target. It should be a must if the report is at beam level.

Proposal 2: The RS type, RS index, measurement quantity and Triggering event type are included in the report of event triggered L1 MR.

For beam level report pattern, the following components can be further discussed:

Time label

Considering the TTT during the event, there are two options for the report content:

- Option 1a: UE reports each measured result during the event evaluation period (e.g., TTT) of the RS.

- Option 1b: Remove time label from option 1a. UE only reports one latest value for the measured result for the target RS.

Figure 1 Time label (included or not) of the report

In our understanding, the triggering of LTM depends on the latest measured beam quality and the event. If the event condition is met, it should be enough for UE to report the latest measured beam quality and the event type. Reporting each measured quality during TTT seems unnecessary and cause more signalling overhead for the MAC-CE.

Proposal 3: UE only reports the latest measured value for the target RS. No time label is introduced.

Multiple beam information

Given that the event triggered L1 MR is sent via MAC-CE, it is likely that multiple events for multiple beams can be reported in the same container. In L3 MR, beams are measured per MO, and beams in the same frequency are measured at the same time. For those measured beam, it is likely that multiple different events are triggered at the same time. For example:

- T1: Beam 1 of cell 1 meet the event 1 (enter condition).

- T2: Beam 2 of cell 1 meet the event 1 (enter condition), Beam 1 of cell 1 leave the condition.

- T3: Beam 3 of cell 2 meet the event 2 (enter condition), Beam 2 of cell 1 leave the condition.

In T1, Beam 1 meet the event, and UE send a report to the network.

In T2, Beam 2 meet the event, and UE send a report to the network. In the same MO, Beam 1 of the same cell meet the leaving condition, and another report may be needed. It should be possible that these two reports can be sent together in the same container.

In T3, Beam 3 meet the event, and UE send a report to the network. In the same MO, Beam 2 of a different cell meet the leaving condition. It should also be possible that these two reports can be sent together, even if report is for different cells.

Proposal 4: Multiple events (for the same or different cells) can be reported in one MAC-CE if triggered simultaneously.

Other information

Since there is a paralleled agenda for conditional LTM, the event triggered L1 MR aligns well with conditional LTM and may assist in informing UE decisions to the network. E.g., UE’s decision for LTM cell switch. It is recommended to study the inclusion of conditional LTM information in event-triggered L1 MR.

Proposal 5: FFS the information of UE decisions in conditional LTM can be included in event triggered L1 MR.

Subsequent reporting

Another discussion point is whether the event-triggered L1 measurement reporting is a one-shot report or if multiple reports are introduced. We understand that this is more related to the design of the report container. Given that the MAC-CE based report container is adopted, the reliability is ensured by the HARQ mechanism and it is unnecessary to introduce multiple reporting to address reliability concerns.

Observation 1: Given that MAC-CE is adopted for the report container, there is no need to introduce retransmissions/multiple reporting to address reliability concerns.

On the other hand, some may worry about the fluctuation for a single report. We understand the fluctuation could be smoothed by adjusting the parameters of hysteresis, TTT, and offset. The introduction of retransmissions or multiple reporting will not bring benefits but instead weaken the benefit of signalling overhead reduction achieved by the event-triggered mechanism.

Observation 2: Fluctuation concern can be address by adjusting the parameter of hysteresis, TTT and offset. No need to introduce retransmissions/multiple reporting to address this concern.

Therefore, we propose:

Proposal 6: The event triggered L1 MR is a one-shot report. Multiple reporting for one trigger event is not introduced. (This does not preclude multiple report by multiple trigger events configuration)

Conclusion

In this contribution, the following observations and proposals are made:

Report pattern

Proposal 1: The event triggered L1 MR is reported at beam level. Individual event is defined for each beam and no cross-beam TTT is introduced.

Report content

Proposal 2: The RS type, RS index, measurement quantity and Triggering event type are included in the report of event triggered L1 MR.

Proposal 3: UE only reports the latest measured value for the target RS. No time label is introduced.

Proposal 4: Multiple events (for the same or different cells) can be reported in one MAC-CE if triggered simultaneously.

Proposal 5: FFS the information of UE decisions in conditional LTM can be included in event triggered L1 MR.

Subsequent reporting

Observation 1: Given that MAC-CE is adopted for the report container, there is no need to introduce retransmissions/multiple reporting to address reliability concerns.

Observation 2: Fluctuation concern can be address by adjusting the parameter of hysteresis, TTT and offset. No need to introduce retransmissions/multiple reporting to address this concern.

Proposal 6: The event triggered L1 MR is a one-shot report. Multiple reporting for one trigger event is not introduced. (This does not preclude multiple report by multiple trigger events configuration)

References

- R2_127_R18 SL_MOB_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM

- R2_126_R18 SL_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM

- RP-240299-Revised-WID-R19-Mobility-Enhancements