3GPP TSG-RAN WG2 Meeting #127b	                                          R2-2408265

Hefei, China, 14 – 18, Oct, 2024

Source: 	Xiaomi

Title:  	Discussion on RRM prediction

Agenda Item:	8.3.2

Document for:	Discussion and Decision

Introduction

RAN2 agreed two goals for RRM prediction, i.e. measurement reduction and handover performance enhancement. Following cases are identified to evaluate the prediction accuracy.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we provide simulation result of cell level RRM measurement prediction of case 2 and 4. Some observations are given. Furthermore, we discuss whether and how to do system level simulation.

Discussion

RRM prediction simulation result discussion

Common simulation assumption

In last meeting, two L1/L3 filtering options are proposed, i.e. sliding or non-sliding. In our simulation, we use the sliding window based L1/L3 filtering option. In this option, the measurement changes in a smoother way, which may be easier for AI to predict. In the L1 filtering, we average the 5 L1 measurements within one measurement period to obtain 1 L1 filtered measurement.

| Parameters | Value |
| --- | --- |
| UE trajectory option (option 1,2,3 in[4]) | 1 |
| UE trajectory boundary processing option (option 1,2,3 in[4]) | Circle-bouncing |
| L1/L3 filtering | Sliding window |
| Whether LOSsoft is modeled or not | No |
| spatial consistency option (A or B) | A |

For cell level measurement prediction model, RAN2 agreed to study the following prediction cases:

Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

Case 2: To directly predict cell level results based on cell level results.

Case 3: To directly predict cell level results based on beam level results

We did simulation of case 3 to evaluate the prediction accuracy. The other AI simulation parameters are shown as following,

| Input/Output | Model input/Output data type | Historic filtered L3 Cell Level RSRP /Future filtered L3 Cell Level RSRP |
| --- | --- | --- |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | GRU |

FR1 to FR1 intra-frequency temporal domain case B

FR1 specific radio simulation parameters are shown as following,

| Parameters | Value |
| --- | --- |
| UE speed (30,60,90 Km/h) | 30 km/h |
| Number of TX beams | 1 |
| Number of RX beams | 1 |

In this contribution, we focus on the 30km/h UE speed for case B. For other UE speed, similar trends can be expected according to [1].

We use the temporal domain observation and prediction pattern depicted in following fig. In this pattern, the measurement in OW and PW are consecutive. The advantage of this pattern is the consecutive measurements in the observation window can provide more information to AI to improve the prediction accuracy.

Fig 1 Temporal domain observation and prediction pattern

The goal of case B is to reduce the measurement. We evaluate different OW/PW length to check how the OW/PW impacts the prediction accuracy. The prediction accuracy simulation results are shown in following table,

| No. | OW (ms) | PW (ms) | MRRT | Avg RSRP difference (dB) |
| --- | --- | --- | --- | --- |
| 1 | 40 | 40 | 50% | 0.658 |
| 2 | 80 | 80 | 50% | 0.799 |
| 3 | 120 | 120 | 50% | 1.001 |
| 4 | 80 | 160 | 66% | 1.202 |
| 5 | 200 | 400 | 66% | 1.885 |

If the MRRT is fixed as 50%, the length of OW and PW is the same. From scenario {1 to 3}, the length of OB and PW increases from 40 to 120 ms. 40 is the minimum window length since it’s the L1 measurement period. The prediction accuracy reduces along with the window length increase. If the MRRT is fixed as 66%, the OB is half as the PW. From scenario {4 to 5}, similar trend is observed. This is because longer PW/OW would reduce the dependency between the measurement result in PW and OW.

Observation 1: If MRRT is fixed, increasing of OW and PW length reduces the prediction accuracy.

In scenario {2, 4}, the OW is fixed as 80ms and PW increases from 80 to 160. The prediction accuracy reduces along with the PW or MRRT length increase. This is because longer PW would reduce the dependency between the measurement result in PW and OW.

Observation 2: Increase of MRRT would reduce the prediction accuracy.

The detailed simulation results can be found in the spreadsheets along with this contribution.

Proposal 1: Capture the above observation and the simulation results in the spreadsheets into TR for case 2.

FR2 to FR2 intra-frequency temporal domain case A

FR2 specific radio simulation parameters are shown as following,

| Parameters | Value |
| --- | --- |
| UE speed (30,60,90 Km/h) | 120 km/h |
| Number of TX beams | 8 |
| Number of RX beams | 4 |

In this contribution, we focus on the 120km/h UE speed for case A, which is the most challenging scenario. For other UE speed, similar trends can be expected according to [1].

It’s agreed the ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5}. However, the goal of case A is to improve the mobility performance. The prediction accuracy is more important than power saving. There is no need to use short observation window, as the UE would always perform measurement. So, we simulate the following cases,

| No. | OW/PW | OW (ms) | PW (ms) | Avg RSRP diff (dB) |
| --- | --- | --- | --- | --- |
| 1 | 1 | 80 | 80 | 1.424 |
| 2 | 1 | 160 | 160 | 1.446 |
| 3 | 2 | 160 | 80 | 0.901 |
| 4 | 2 | 320 | 160 | 1.351 |
| 5 | 3 | 240 | 80 | 0.841 |
| 6 | 3 | 480 | 160 | 1.285 |

In scenario {1, 3, 5}, the PW is fixed as 80ms and OW increases from 80ms to 240ms. As the measurement period is 80ms in FR2, the number of measurements as input increases from 1 to 3. In general, the prediction accuracy improves when the number of input increases. This is because more input can improve the prediction accuracy. But the gain is not linear correlated to the input number. When the OW increase from 80ms to 160ms, the RSRP difference decreases by 0.523dB. When the OW further increase to 240ms, the RSRP difference further decreases by 0.06dB. This is because the earlier measurements may have little correlation with the measurements in PW. If OW is further increased, the gain would be less. There would be a maximum length. The gain would be neglectable, if the PW is increased over the maximum length.

Observation 3: If PW is fixed, increase of OW can improve the prediction accuracy.

Observation 4: Increase of OW within a maximum length can improve the prediction accuracy. Increase of OW over the maximum length can’t further improve the prediction accuracy.

If the OW/PW rate is fixed, the increase of OW and PW length would reduce the prediction accuracy. This can be seen by comparison of scenario {1, 2}, {3,4}, {5,6}.

Observation 5: if the OW/PW rate is fixed, increase of OW and PW length would reduce the prediction accuracy.

The detailed simulation results can be found in the spreadsheets along with this contribution.

Proposal 2: Capture the above observation and the simulation results in the spreadsheets into TR for case 4.

Whether system level simulation is needed

We can discuss whether system level simulation is needed case by case, since two cases aim at different goals.

For case 2, the goal is to reduce the measurement. The legacy handover procedure can still be followed, i.e. UE reports the MR and NW sends handover command. The MR even may be fulfilled during the PW. UE can acknowledge the MR event fulfilment at OW. If UE can report MR at OW, which is earlier than legacy, the handover performance may be improved. However, we understand this is in the scope of event prediction. So, in case 2, UE shall not report MR earlier. We can check the impact on the measurement reduction alone.

Proposal 3: For case 2, UE shall only report MR when MR event is fulfilled, even if UE can predict the MR event fulfilled in PW.

RAN2 can discuss whether system level simulation is needed.

Proposal 4: For case 2, RAN2 can discuss whether system level simulation is needed to evaluate the mobility performance.

For case 4, the situation is different. UE or NW can predict the measurements of neighbour or serving cell after MR. Theoretically, the prediction can help NW to make decision. However, it’s not clear how this prediction impacts the handover procedure. For example, if the reported cell is predicted to be poor, whether NW still selects it as target cell? How to decide prediction result is good or poor? We understand this is up to gNB implementation and difficult to reach consensus in RAN2.

Observation 5: It’s up to gNB implementation how to use the prediction result in case 4.

Based on above observation, we believe it’s unnecessary to do system level simulation for case 4. The prediction can be seen as a assistance information for NW to consider during handover.

Proposal 5: No need to do system level simulation for case 4.

System level simulation assumption

If we want to do the system level simulation, basic assumptions should be clear. We can reuse the modelling in [2] for HOF and ping-pong handover. The exact parameter value can be further discussed for FR2.

Proposal 6: Reuse the modelling in 36.839 for HOF and ping-pong handover. The exact parameter value can be further discussed for FR2.

Interruption time as mobility performance metric is not defined in [2]. This metric may not be reflected by HOF numbers. Because NW may decide not to perform handover if target cell is predicted to be poor and serving cell is predicted to be good. Therefore, we may need to define the interruption metric explicitly. It’s clear the communication is interrupted during T304 running. T304 is not implemented in the simulation. Therefore, we can set fixed interruption time for each handover to simplify the simulation. If handover is failed, UE would trigger reestablishment, which results in longer interruption. Therefore, failed handover shall count longer interruption time than successful handover.

Proposal 7: Fixed interruption time is set for successful and failed handover. Failed handover shall count longer interruption time than successful handover.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Case 2 simulation result

Observation 1: If MRRT is fixed, increasing of OW and PW length reduces the prediction accuracy.

Observation 2: Increase of MRRT would reduce the prediction accuracy.

Proposal 1: Capture the above observation and the simulation results in the spreadsheets into TR for case 2.

Case 4 simulation result

Observation 3: If PW is fixed, increase of OW can improve the prediction accuracy.

Observation 4: Increase of OW within a maximum length can improve the prediction accuracy. Increase of OW over the maximum length can’t further improve the prediction accuracy.

Observation 5: if the OW/PW rate is fixed, increase of OW and PW length would reduce the prediction accuracy.

Proposal 2: Capture the above observation and the simulation results in the spreadsheets into TR for case 4.

System level simulation discussion

Proposal 3: For case 2, UE shall only report MR when MR event is fulfilled, even if UE can predict the MR event fulfilled in PW.

Proposal 4: For case 2, RAN2 can discuss whether system level simulation is needed to evaluate the mobility performance.

Observation 5: It’s up to gNB implementation how to use the prediction result in case 4.

Proposal 5: No need to do system level simulation for case 4.

Proposal 6: Reuse the modelling in 36.839 for HOF and ping-pong handover. The exact parameter value can be further discussed for FR2.

Proposal 7: Fixed interruption time is set for successful and failed handover. Failed handover shall count longer interruption time than successful handover.

Reference

[1] R2-2406703	Discussion on RRM prediction simulation result	Xiaomi	discussion

[2] TR 36.839