3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408273

# Hefei, China, 14 - 28 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4996 | rev | - | Current version: | V18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CE-only BWP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core, NR_cov_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | On a initial BWP, a resource set associated with MSG3 repetition and a resource set not associated with MSG3 repetition can be both configured in the same RACHConfigCommon for different purposes. - If SI request without repetition is also configured on RedCap specific initial BWP, the resource set not associated with MSG3 repetition configured by RACHConfigCommon in additionalRACH-ConfigList-r17 is used for parameter initiatization for SI request without repetition. To improve resource efficency, RAN2 confirm that the overlapping preamble between different resource sets is allowed, i.e. the preamble index of both resource set starts from 0. RAN2 confirm that in RedCap specific initial uplink BWP where SI-request without Msg1 repetition is enabled, the network can configure “startPreambleForThisPartition-r17” equal to 0 for a RACH partition associated with a feature (or feature combination), and this RACH partition shares the ROs of another RACH partition that not associated with any feature. No spec impact is needed.. On dedicated BWP other than initial BWP (e.g. CE only BWP), there is no MSG1 based SI request initiated from UE and RAN2 agreed that the RACHConfigCommon in additionalRACH-ConfigList-r17 is used to configure CE-only BWP and the R15 RACHConfigCommon is absent in such case. Cov Enh R2-2308063 Clarification on CE-only BWP ZTE Corporation, Sanechips discussion Rel-17 NR_cov_enh-Core - MTK wonder if 1.2 invovles TS change. ZTE think maybe, as this is largely unclear. Use featureCombinationPreamblesList-r17 in additionalRACH-ConfigList-r17 to configure CE-only BWP, and the legacy RACHConfigCommon is absent in such case. Current spec doesn’t support CFRA for CE-only BWP. Generally in CE only BWP, a resource set associated with MSG3 repetition is configured which the preamble index can start from either 0 or other values. Observation 1: there should not have a resource set not assocated with MSG3 repetition on CE only BWP. However, currently for each RACH-ConfigCommon in additionalRACH-ConfigList-r17, a resource set with at least 4 preambles starting from preamble index 0 is mandatorily configured through the following parameter CB-PreamblesPerSSB (i.e. not configured with FeatureCombination) due to the association between SSB and RO (i.e. ssb-perRACH-Occasion) must be configured, both ssb-perRACH-Occasion and CB-PreamblesPerSSB are in the same RRC parameter and cannot be alone configured: ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { oneEighth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneFourth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneHalf ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, one ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, two ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32}, four INTEGER (1..16), eight INTEGER (1..8), sixteen INTEGER (1..4). - Note based on TS 38.321, the resource set configured by CB-PreamblesPerSSB (i.e. not configured with FeatureCombination) is considerred as the set of Random Access resources to not associated with any feature. TS 38.321 1> if a set of Random Access resources is not configured with FeatureCombination: 2> consider the set of Random Access resources to not associated with any feature.. Based on existing RRC parameter CB-PreamblesPerSSB, there is a resource set not assocated with MSG3 repetition configured from RRC perspective. We also checked the field description which indicates: BWP-UplinkCommon field descriptions. additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). The network does not configure this list to have more than 16 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.. In our understanding, the above requirement is made for mandatorily configure feature indication inside featureCombinationPreamblesList-r17. Hence even with the above, UE still does not know how to deal with the resource set configured by the another RRC parameter CB-PreamblesPerSSB. Observation 2: UE may not know whether CE-only BWP is configured if it does not know how to deal with the resource set configured by CB-PreamblesPerSSB (i.e. not associated with any feature). In TS 38.321, if there is resource set not associated with any feature, the UE will never met the below MAC condition for CE only BWP checking (i.e. which indicates only resource set associated with MSG3 repetition configured) as bellow: 1> if the BWP selected for Random Access procedure is configured with both set(s) of Random Access resources with msg3-Repetitions set to true and set(s) of Random Access resources without msg3-Repetitions set to true and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg3; or 1> if the BWP selected for Random Access procedure is only configured with the set(s) of Random Access resources with msg3-Repetitions set to true: 2> assume Msg3 repetition is applicable for the current Random Access procedure. 1> else: 2> assume Msg3 repetition is not applicable for the current Random Access procedure.. To avoid the UE confusion, we think that a clarification is needed. Even the overlapping resource set is allowed, the resource set not associated with any feature in additionalRACH-ConfigList-r17 of an initial BWP are modelled as separate valid resource set in MAC layer and may be used for SI request without repetition. However, the MSG1 based SI request is only configured on initial BWP, for dedicated BWP there is no MSG1 based SI request and another story may be needed, i.e. the resource set not associated with any feature is considered as invalid in additionalRACH-ConfigList-r17 of a dedicated BWP for configuring CE-only BWP. In CE-only BWP, network can configure RA resource set associated with MSG3 repetition for which preamble index may start from 0. In Rel-18 CE only BWP with MSG1 repetition, same issue also exists. If a change is needed for Rel-17 CE only BWP with MSG3 repetition, same principe should be applied for Rel-18 CE only BWP with MSG1 repetition. 1> else if the BWP selected for Random Access procedure is configured only with Random Access resources with msg1-Repetitions set to true: 2> assume Msg1 repetition is applicable for the current Random Access procedure; 2> if at least one of rsrp-ThresholdMsg1-RepetitionNumX is configured: 3> if rsrp-ThresholdMsg1-RepetitionNum8 is configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum8; 4> assume Msg1 repetition number applicable for the current Random Access procedure includes 8. 3> if rsrp-ThresholdMsg1-RepetitionNum4 is configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum4: 4> assume Msg1 repetition number applicable for the current Random Access procedure includes 4. 3> if rsrp-ThresholdMsg1-RepetitionNum2 is configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum2: 4> assume Msg1 repetition number applicable for the current Random Access procedure includes 2. 3> else if the RSRP of the downlink pathloss reference is not less than any configured rsrp-ThresholdMsg1-RepetitionNumX: 4> assume Msg1 repetition number applicable for the current Random Access procedure is the lowest Msg1 repetition number configured for this BWP. 2> else (none of rsrp-ThresholdMsg1-RepetitionNumX is configured): 3> assume Msg1 repetition number applicable for the current Random Access procedure is the Msg1 repetition number that configured for this BWP.. |
| Summary of change: |  | Clarify in FD of additionalRACH-ConfigList that on UE considers the resource set without feature associated as invalid in additionalRACH-ConfigList-r17 of a dedicated BWP. Impact analysis Impacted 5G architecture options: NR SA, NR-DC, NE-DC Impacted functionality: MSG3 repetition Inter-operability: - If the network is implemented according to the CR and the UE is not, then UE cannot perform RA on CE only BWP. - If the UE is implemented according to the CR and the network is not, then there are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RA cannot be performed on CE only BWP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

6.3.2	Radio resource control information elements

–	BWP-UplinkCommon

The IE BWP-UplinkCommon is used to configure the common parameters of an uplink BWP. They are "cell specific" and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

[Deleted]

BWP-UplinkCommon field descriptions

additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). UE considers the RA resource set not associated with any feature indication as invalid in an additional RACH configuration of a dedicated BWP (e.g. CE only BWP). The network does not configure this list to have more than 16 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.

enableRA-PrioritizationForSlicing Indicates whether or not the ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep should override the ra-PrioritizationForAccessIdentity. The field is applicable only when the UE is configured by upper layers with both NSAG and Access Identity 1 or 2. If value TRUE is configured, the UE should only apply the ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep. If value FALSE is configured, the UE should only apply ra-PrioritizationForAccessIdentity. If the field is absent, whether to use ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep or ra-PrioritizationForAccessIdentity is up to UE implementation.

mcs-Msg3-Repetitions Configuration of eight candidate MCS indexes for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. Only the first 4 configured or default MCS indexes are used for PUSCH transmission scheduled by RAR UL grant. This field is only applicable when the UE selects Random Access resources indicating Msg3 repetition in this BWP. If this field is absent when the set(s) of Random Access resources with MSG3 repetition indication are configured in the BWP-UplinkCommon, the UE shall apply the values {0, 1, 2, 3, 4, 5, 6, 7} (see TS 38.214 [19], clause 6.1.4).

END OF CHANGE