3GPP RAN WG2 Meeting #127bis                                                     R2-2408274

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.5.3

Source:	HONOR

Title:	Discussion on on-demand SIB1 for NES

Document for:	Discussion and Decision

1. Introduction

In RAN2#127, the following aspects were agreed to be specified in the on-demand SIB1 work item [1]:

Agreements on OD-SIB1

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

In this contribution, we analyse how to bar the legacy UE and the details of UE behaviours related to triggering UL-WUS and SIB1 acquisition failure for case2.

2 Discussion

2.1 Barring for Legacy UE

As specified, the UE shall not consider any exclude-listed cells as candidate for cell reselection. Therefore, it could be used to bar the legacy UE (i.e. UEs not supporting OD-SIB1) through including NES cells in the excluded cell list of Cell A with the following advantages:

- Avoid evaluation of NES cell. NES cell is not considered as candidate cell by legacy UE and there will be no cell reselection evaluation procedure.

- Minor specs impact. No impact to legacy UE. NES UE should ignore the excluded cell list if it is used to prevent legacy UE to reselect to NES cell.

Proposal 1:  Cell A includes the NES cell in the excluded cell list to bar the legacy UE. RAN2 to discuss how to indicate NES UE to ignore the excluded cell list used for barring the legacy UE.

2.2 Triggering of UL-WUS

In NES cell, it is possible that SIB1 is broadcasting triggered by UL-WUS from the other UE. The new coming UE should acquire the broadcasting SIB1 and the NES cell will also enjoy more energy saving gains without triggering of UL-WUS again.

Proposal 2:  The UE should check the SIB1 broadcasting status before initiating UL-WUS for SIB1 acquisition.

One option is the UE to determine SIB1 broadcasting status via detecting SIB1 based on the scheduling information. This may introduce camping delay with long scheduling period and poor quality. The user experience will be impacted if there are data packets waiting for transmission. So, it is proposed that the SIB1 broadcasting status is indicate explicitly.

Proposal 3: RAN2 to discuss how to indicate the SIB1 broadcasting status to UE.

2.3 SIB1 Acquisition Failure

R19 NES UE will bar the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell which includes:

- RA procedure failure: no RAR received

In general, the RA procedure failure means no RAR received after several times preamble transmission. This is also applied in OD-OSI that the UE is permitted to perform the preamble transmission for several times until achieve maximum number of Preamble transmission. In R18, preambleTransMax-Msg1-Repetition is introduced for coverage enhancement.

As SIB1 is essential for camping, the preambleTransMax and preambleTransMax-Msg1-Repetition should be used for SIB1 request to improve the SIB1 acquisition probability.

Proposal 4: For OD-SIB1, the UE should consider it as the SIB1 acquisition failure if there is no RAR responding received after several times preamble transmission with configurable repetition.

- SIB1 acquisition failure

Even if SIB1 is broadcasted more than one times, there are still UEs that could not be able to acquire SIB1 successfully. It should be clearly how to identify the acquisition failure to let UE reselect to the other cells quickly, e.g. configure limited detection duration.

RAN1 concluded to introduce the time window for SIB1 broadcasting as depicted below, which could be helpful for SIB1 acquisition failure detection [2].

Proposal 5: RAN2 to consider it as SIB1 acquisition failure if SIB1 can’t be acquired successfully after expiration of the configured time window.

3 Conclusions

In this contribution, we discussed several detailed on-demand SIB1 issues, and made the following proposals:

Proposal 1:  Cell A includes the NES cell in the excluded cell list to bar the legacy UE. RAN2 to discuss how to indicate NES UE to ignore the excluded cell list used for barring the legacy UE.

Proposal 2:  The UE should check the SIB1 broadcasting status before initiating UL-WUS for SIB1 acquisition.

Proposal 3: RAN2 to discuss how to indicate the SIB1 broadcasting status to UE.

Proposal 4: For OD-SIB1, the UE should consider it as the SIB1 acquisition failure if there is no RAR responding received after several times preamble transmission with configurable repetition.

Proposal 5: RAN2 to consider it as SIB1 acquisition failure if SIB1 can’t be acquired successfully after expiration of the configured time window.

4. References

[1]	R2-24xxxxx Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands

[2]	Draft Report of 3GPP TSG RAN WG1 #118 v0.3.0