3GPP TSG-RAN WG2 Meeting #127bis	R2-2408277

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.4

Source:	HONOR

Title: 	Discussion on A-IoT random access

Document for:	Discussion and decision

1	Introduction

The RAN finished the SI for Ambient IoT and approved the TR 38.848 as the outcome of the SI. During the initial discussion related to the Rel-19 Ambient IoT, RAN confirmed the general scope of this SI with some common understandings and limitations related to device type, deployment scenarios and traffic types [1].

During RAN2#7 meeting, RAN2 discussed the AIoT random access and made several initial agreements as:

Agreements - A-IoT Random Access - For 3-step CBRA Support fixed random ID size is 16 bit. The ID is randomly generated. - Failure/success indication of D2R will be studied. FFS if it would be implicit or explicit and for which use case it is needed. FFS whether it is applied only to some cases. - for 2step CBRA, RAN2 design will support msg2. Whether it is needed it is up to the reader. FFS when it is needed. For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2. FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices. - In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2). FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes. - FFS if reader assigns the AS ID for scheduling purposes

In this contribution, we analyze the remaining issues of the A-IoT random access based on the use cases and assumptions mentioned above and provide proposals related to A-IoT random access procedure and the failure handling.

- Discussion

Ambient IoT, which is introduced as a new 3GPP IoT technology, aims to rely on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications and address some representative use cases. AIoT random access is not the same as the NR random access or RFID access procedure since the overall device assumption and the design target are different based on the previous discussions in 3GPP working groups.

In the current NR specification [2], different kinds of random-access procedure are supported: 4-step CBRA as shown in Figure 1(a), 2-step CBRA as shown in Figure 1(b), 4-step CFRA as shown in Figure 1(c), 2-step CFRA as shown in Figure 1(d).

Figure 1: Random Access Procedures in NR

Slotted-ALOHA is employed in RFID to reduce the access collision. The general access procedure for RFID comprises of basic operations like select, inventory, access.

\

Figure 2: General access procedure for RFID

RAN2 confirmed that slotted-ALOHA is the baseline for Ambient IoT random access in the first meeting and analysed the basic steps for 3-step random access CBRA, 2-step CBRA and contention-free access.

2.1 A-IoT random access steps

3-step CBRA, as the baseline access procedure for A-IoT, is similar to the 4 step CBRA in NR and the inventory procedure in RFID. During the RAN2#126 meeting, RAN2 agreed that the first two messages were for contention resolution and the device sends the Msg3 to complete the random-access procedure. To take an example of inventory use case, we provide the general procedure for 3-step CBRA based on the current agreements.

Figure 3: General procedure of 3-step CBRA

For the use case of “inventory only”, the random-access procedure is completed from the point view of the device when it sends Device ID and/or any other upper layer data in Msg3. As agreed before, to handle the Msg3 transmission failure, subsequent R2D transmission after Msg3 could be used to confirm the failure/success reception of the Msg3. For example, the present of the subsequent R2D transmission indicates the failure/success reception of Msg3 while the absent of it indicates the success/failure reception of Msg3. That is, the subsequent R2D transmission after Msg3 is not always present in 3 Step CBRA for the use case of “inventory only”.

Based on our understanding, the device will not perform re-access in the following rounds if it is not aware of whether the Msg3 is not well-received by the reader. The reader needs to trigger another procedure to find the failed device which results in more signalling overhead and longer latency to complete the service request. In addition, if the failure is informed timely, the failure handling could be performed with less latency. Thus, the R2D transmission after Msg3 could be used to inform and handle the Msg3 transmission failure and provide some performance improvement. For example, the R2D transmission after Msg3 could confirm the failure reception of the Msg3 and the absent of it indicates the success/failure reception of Msg3. The success indication of the Msg3 could be explicitly provided by the start of the next occasion without one failure indication ahead.

Proposal 1: RAN2 confirms that the subsequent R2D transmission after Msg3 is not always present in 3 Step CBRA for success/failure indication of Msg3.

Proposal 2: Present of the subsequent R2D transmission is to confirm the failure reception of the Msg3 while the absent of it before the start of the next occasion indicates the success reception of Msg3 from device’s point of view.

For the use case of “inventory and command”, R2D data transmission will be sent to send the command to the device after the Msg3. Thus, the subsequent R2D transmission after Msg3 could be the scheduling information for the following command (e.g., new RN as RFID) or higher layer data (e.g., command).

Proposal 3: RAN2 confirms that the subsequent R2D transmission after Msg3 is present in 3 Step CBRA for the scheduling/transmission for the following higher layer data.

Based on the above discussion about subsequent R2D transmission, the present and function of it are shown as in the following table:

Table: the present and function of the subsequent R2D transmission

|  | Msg3 failure and with following data | Msg3 failure and without following data | Msg3 success and without following data | Msg3 success and with following data |
| --- | --- | --- | --- | --- |
| Present of the subsequent R2D transmission | Yes | Yes | No | Yes |
| Function of the subsequent R2D transmission | Scheduling and failure indication | failure indication |  | Scheduling |

Like the 3-step CBRA, the first two steps are also used for contention resolution for the 2-Step CBRA. In another word, the step 2 in 2-step CBRA needs to be mandatory to confirm the reception of the Msg1. Thus, the present of Msg2 means the Msg1 is well received from the perspective of device.

Proposal 4: RAN2 to confirm that, the Msg2 is always present for 2 Step CBRA to confirm the reception of the Msg1 with the same random ID.

2.2 Failure Detection and handling

As discussed before, for 3-step CBRA, the device considers the contention resolution as successful only when the reader ack the same ID received in Msg1. Similarly, the device considers the contention resolution as failed, if the Msg2 including the same ID in Msg1 is not received before the next occasion/slot. It could not distinguish the cause of the failure: collision with other devices or Msg1 decoding problem. From the perspective of reader, it may consider the Msg2 transmission as failed if there is no Msg3 from the device. Thus, to complete the whole random-access procedure, the failure determination needs to be from perspective of both reader and device.

Proposal 5: RAN2 to study the failure detection from perspective of both reader and device.

In addition to contention resolution, the subsequent R2D transmission after Msg3 for 3-step CBRA or Msg1 for CFRA could be detected as failure. From perspective of both reader, there may be contention resolution failure for the following two cases: 1) more than one device selects the same access occasion; 2) the decoding failure of the received D2R message. For the subsequent R2D transmission after the contention resolution, only the second failure case is valid. What’s more, considering the limited power for the AIoT devices, the failure may be caused by the un-availability. The handling should be studied for all the failure cases mentioned above.

Proposal 6: RAN2 to consider the following failure cases to handle: contention resolution failure, the subsequent R2D transmission failure.

3	Conclusions

In this contribution, we discussed the issues related to the random access procedure and failure handling aspects on Ambient IoT. Based on the  discussion, the following observation and proposals are concluded:

Proposal 1: RAN2 confirms that the subsequent R2D transmission after Msg3 is not always present in 3 Step CBRA for success/failure indication of Msg3.

Proposal 2: Present of the subsequent R2D transmission is to confirm the failure reception of the Msg3 while the absent of it before the start of the next occasion indicates the success reception of Msg3 from device’s point of view.

Proposal 3: RAN2 confirms that the subsequent R2D transmission after Msg3 is present in 3 Step CBRA for the scheduling/transmission for the following higher layer data.

Proposal 4: RAN2 to confirm that, the Msg2 is always present for 2 Step CBRA to confirm the reception of the Msg1 with the same random ID.

Proposal 5: RAN2 to study the failure detection from perspective of both reader and device.

Proposal 6: RAN2 to consider the following failure cases to handle: contention resolution failure, the subsequent R2D transmission failure.

4	References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR.

- TR 38.300, NR and NG-RAN Overall description; Stage-2; version 18.1.0 Release 18