3GPP RAN WG2 Meeting #127bis	R2-2408278

# Hefei, China, Oct 14th–18th, 2024

Agenda Item:	8.2.3

Source:	HONOR

Title:	Discussion on A-IoT Paging

Document for:	Discussion and Decision

1. Introduction

In RAN2#127, the following agreement was agreed [1]:

Agreements 1 Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

In this contribution, we focus on the remaining issues of A-IoT paging.

2 Discussion

2.1 A-IoT paging procedure

In the RFID, the select and access procedure is as following [2]:

Interrogator (reader-like entity) sends Select command to inventory, and tags initialize its slot counter when receives Query command. Tags decrement their slot counter every time they receive a QueryRep with matching session and backscatter an RN16 when their slot counter reaches 0000h. Similar to RFID procedure, we need to discuss whether the Query and QueryRep are needed in the paging and access procedure.

During the last meeting, it was agreed that reader could trigger multiple/subsequent A-IoT paging messages that are associated with the same request from the CN. The mainly reason to send subsequent A-IoT paging messages is to resolve the conflict or to give the unavailable devices other opportunities to access. Since the subsequent A-IoT paging messages could realize the functions of Round Trigger message, the Round Trigger message is needed only when we identify some specific demands. There are two methods provided by RAN1 for devices to synchronize with the reader, one is time-based method and the other is signalling-based method. For the signalling-based method, the slot-start indication is required. For the time-based method, a timer will be maintained in both the device and reader sides. However, it is up to RAN1 to decide which method to be adopted. If the signalling-based method is adopted, the entire paging and random access procedure is as following, the re-paging message following the initial paging message is associated with the same CN request:

Proposal 1: A-IoT Paging message can be used as round trigger message.

As shown above, the re-paging is sent after the initial paging message associated with the same CN request.

Proposal 2: The subsequent paging messages follow the initial paging message associated with the same CN request.

2.2 Content of A-IoT paging message

- Resources related

For inventory or DL command procedure, the reader needs to send A-IoT paging message to find devices. After receiving A-IoT paging message, device may send random ID or device ID to respond reader. Since RAN1 has agreed to use FDM for D2R message, there may be multiple frequency resources in one access slot. Unlike RFID, dedicated resources could be multiple access occasions in one slot. Thus, the A-IoT paging message can include scheduling information to indicate frequency/time resources for device(s) to respond. To support CBRA and CFRA, the resource information included in A-IoT paging message can be shared resources or dedicated resources. Since RAN1 is discussing the design of the resource, RAN2 can further discuss the parameters included in AIoT paging message after receiving RAN1 conclusions on resource.

Proposal 3: A-IoT paging message includes dedicated resources or shared resources for device(s) to initiate RA procedure. Detailed resource-related parameters depend on RAN1.

The following agreement was made in the RAN2 #126 meeting:

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

The CFRA applies to single and multi-device case. For one device case, dedicated resource(s) could be included in one paging message for this device. For multiple device case, multiple dedicated resources could be included in one paging message for these devices.

Proposal 4: For CFRA, dedicated resource(s) of one or more devices can be included in one A-IoT paging message.

For the case of dedicated resources for multiple devices, device needs to determine which access occasion is specified for it. Since device uses the device ID to identify itself, device ID can be used to determine the specific access occasion. For example, the configuration of an access occasion associated with a device ID is included in A-IoT paging message; device uses the device ID to determine the specific access occasion according to the association in such configuration.

Proposal 5: For CFRA, A-IoT paging message includes a mapping between each device ID and dedicated resource.

- RA type related

It is agreed that CBRA and CFRA are supported and CBRA may contain 2-step RA/3-step RA procedure. If a device is not be able to determine which type of RA procedure should be triggered, there should be an indication of which RA procedure needs to be initiated in A-IoT paging message. As the A-IoT paging message format is not determined for now, this indication can be implicit or explicit.

Proposal 6: The RA type, e.g., 2-step/3-step CBRA or CFRA, is provided implicitly or explicitly in the A-IoT paging message.

3 Conclusions

Proposal 1: A-IoT Paging message can be used as round trigger message.

Proposal 2: The subsequent paging messages follow the initial paging message associated with the same CN request.

Proposal 3: A-IoT paging message includes dedicated resources or shared resources for device(s) to initiate RA procedure. Detailed resource-related parameters depend on RAN1.

Proposal 4: For CFRA, dedicated resource(s) of one or more devices can be included in one A-IoT paging message.

Proposal 5: For CFRA, A-IoT paging message includes a mapping between each device ID and dedicated resource.

Proposal 6: The RA type, e.g., 2-step/3-step CBRA or CFRA, is provided implicitly or explicitly in the A-IoT paging message.

4. References

[1]	RAN2 #127 Chairman notes

[2]    EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID