3GPP TSG-RAN WG2 Meeting #127bis	R2-2408279

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.2

Source:	HONOR

Title: 	Further discussion on inter-CU LTM

Document for:	Discussion and Decision

1	Introduction

In Rel-19 Mobility enhancements WI, the following objective was achieved:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured. - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged. - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released. - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling. - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In RAN2#127, the following agreements were made:

Reference configuration: - Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported. - CSI resource and report configuration: - The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration. - The source CU is responsible to generate the common CSI resource configuration. Early DL sync: - For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused. Inter-CU LTM switch: - For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell. RLC and PDCP re-establishment in inter-CU LTM: - If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it. - Handling of candidate configuration after inter-CU LTM cell switch: - As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network. - DRB/PDU session mismatch in subsequent inter-CU LTM: - RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421. LTM and L3 HO: - L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM): - - When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network. - - The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations. - - For the execution order between CHO and LTM, Rel-18 principle is applied. Inter-CU SCG LTM: - Inter-CU SCG LTM preparation can be initiated by source SN. - The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. - For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. - RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. - Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. Inter-CU MCG LTM: - SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). - Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). - For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

In this contribution, we will take into consideration on the remaining issues in inter-CU LTM in both non-DC scenario and DC scenario.

2. Discussion

2.1 Inter-CU LTM with non-DC

2.1.1 Early DL sync

During UL synchronization with LTM candidate cell(s), after performing TA acquisition based on CFRA, the source-DU acquires TA via the interaction between CU-CU and CU-DU nodes. In detail, the candidate DU notifies the associated CU of the acquired TA. The candidate CU forwards the TA to the source CU. Finally, the source DU acquires the TA from the source CU. Therefore, it is naturally for source DU to manage TA validity. If not, while the TA is about to be used, the source DU has to acquire TA from source CU, which will have negative impacts on cell switch latency.

Upon cell switch, if the TA associated with candidate cell can continue to be maintained by NW side, the signalling overhead might be reduced, since it might not be necessary for UE to re-acquire TA in the following period of time. Therefore, the TA associated with candidate cell should be kept in serving DU.

Proposal 1: After performing UL synchronization with candidate cell(s), the source-DU manages the TA instead of source CU, and after cell switch, the TA associated with candidate cell(s) can continue to be kept in the following serving DU(s).

2.1.2 Security about inter CU LTM

In Rel-18, security update is not supported, which will not result in serious consequences due to the restriction of intra-CU. However, for inter-CU LTM, security update is necessary. For supporting security update in inter-CU scenario, how to keep alignments between source node and target node should be taken into consideration as a start point.

The approach of security change is seen as an essential issue. There are four potential approaches which are shown as follows:

Option 1: Use a new information in MAC CE to deliver the security information.

In L3 HO, the security update parameter (i.e., NCC) is included in the HO command. Refer to the mechanism of security update in L3 HO. The security update parameter (i.e., NCC) can be included in LTM cell switch command. It needs to be assessed whether there are security issues, since a MAC CE message is not protected by integrity protection and ciphering.

Option 2: Similar to Rel-18 S-CPAC key update mechanism, the UE is preconfigured from the source gNB with a list of NCC values per CU using RRC signalling.

Similar to SCPAC, a list of NCC associated with one candidate cell is configured for UE. Upon cell switch, the UE shall choose one unused NCC to be applied in the target cell.

Option 3: After the execution of inter-CU LTM cell switch, the participating gNBs are expected to be updated with new K-gNB* to be used for the next inter-CU LTM cell switch.

UE is pre-configured by the CN with a list of NCC values and the UE determines NCC value upon cell switch. The CN needs to be aware of how the UE would use the next NCC value. Therefore, the signalling interaction between gNB and CN is needed, which will result in increased complexity.

Option 4: After every inter-CU LTM cell switch execution, the UE is provided via RRC signalling with the NCC value to be used by the UE for key derivation at the next inter-CU LTM cell switch.

This scheme is very similar to legacy handover with key update via RRC reconfiguration message. It is not aligned with the intention of LTM.

Proposal 2: It is preferred to take the Rel-18 S-CPAC key update mechanism (option 2) as the baseline：the UE is preconfigured from the source gNB with a list of NCC values per CU using RRC signalling.

2.2 Inter-CU LTM with DC

2.2.1 Inter-CU LTM of MN with SN released or unchanged

In the previous meeting, it was agreed that SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). That is, SCG configuration can be kept or released based on NW side decision. In addition, we also need to consider how to indicate UE on this decision. For the handling on SCG configuration, there are two options as follows:

Option 1: RRC pre-configuration

Whether SCG remains unchanged or released can be indicated via RRC pre-configuration. In candidate cell pre-configuration, such indicator can be included.

Option 2: cell switch MAC CE

A cell switch MAC CE can be used to indicate the operation on SCG. Although it is flexible, the other issue on how to define the MAC CE should be taken into account.

Currently, we do not have an obvious preference on the two options, and both of the two options can be discussed in further study.

Proposal 3: For the inter-MN LTM in DC scenario, there are two options to handle the SCG for further study:

- Option 1: RRC pre-configuration

- Option 2: cell switch MAC CE

In the previous meeting, it was agreed that, for the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure. That is, the legacy R15 RRC reconfiguration with sync shall be applied once inter-CU MN LTM is performed. However, if subsequent inter-MN LTM is supported, such SN security update mechanism is not suitable. Therefore, a new mechanism for SN security update may be needed for supporting subsequent inter-MN LTM.

Proposal 4: RAN2 to discuss whether SN security update is an issue to be considered for subsequent inter-MN LTM.

2.2.2 Inter-CU LTM of SN with MN unchanged

In the signalling procedure of L3 SN initiated SN Change, source SN initiates the SN change procedure and performs inter-CU SCG configuration. The MN includes SCG configuration generated by the target SN into its MN RRC configuration message. The UE applies the new configuration after completing SN change. It is straightforward to take L3 SN initiated SN Change as baseline.

Proposal 5: For the procedure of inter-CU LTM of SN with MN unchanged, take L3 SN initiated SN Change as a baseline.

In SN-initiated inter-SN LTM, source SN is responsible to initial early DL/UL synch and LTM execution. During UL synchronization with LTM candidate SN, after performing TA acquisition, the candidate SN transfers TA to source SN and source SN starts to maintain TA.

Proposal 6: In inter-CU LTM of SN with MN unchanged, source SN is responsible to maintain the TA validity.

3 Conclusions

In this contribution, we discussed the issues related to inter-CU LTM. Based on the discussion, the following observations and proposals are concluded:

Proposal 1: After performing UL synchronization with candidate cell(s), the source-DU manages the TA instead of source CU, and after cell switch, the TA associated with candidate cell(s) can continue to be kept in serving DU.

Proposal 2: It is preferred to take the Rel-18 S-CPAC key update mechanism (option 2) as the baseline：the UE is preconfigured from the source gNB with a list of NCC values per CU using RRC signalling.

Proposal 3: For the inter-MN LTM in DC scenario, there are two options to handle the SCG for further study:

- Option 1: RRC pre-configuration

- Option 2: cell switch MAC CE

Proposal 4: RAN2 to discuss whether SN security update is an issue to be considered for subsequent inter-MN LTM.

Proposal 5: For the procedure of inter-CU LTM of SN with MN unchanged, take L3 SN initiated SN Change as a baseline.

Proposal 6: In inter-CU LTM of SN with MN unchanged, source SN is responsible to maintain the TA validity.

4 References

- RP-241515, Revised Work Item: NR mobility enhancements Phase 4.

- TS 37.340, Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2

- TS 38.401, Technical Specification Group Radio Access Network; NG-RAN; Architecture description

- S3-241898, Draft reply LS on security handling for inter-CU LTM in non-DC cases