3GPP TSG-RAN WG2 Meeting #127bis	R2-2408281

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.4

Source:	HONOR

Title: 	Discussion conditional LTM

Document for:	Discussion and decision

1	Introduction

Based on the discussion during RAN#105 meeting, the mobility enhancement WID in the Release 19[1] related to the conditional LTM and the measurement enhancements for LTM are updated as below.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

For the conditional LTM, UE evaluates conditions for triggering LTM where the design of the evaluation condition could refer to that of event triggered L1 measurement reporting. The discussion about measurements related enhancements for purpose of supporting LTM was already started several meetings ago and the following agreements were made during last RAN2 meeting.

- MR event evaluation - Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. - MAC layer handles the event evaluation and measurement report triggering. - Measurement report - Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

In this contribution, we discuss the relation between the event triggered L1 measurement discussed before and conditional LTM, the general concept of the conditional LTM like execution trigger events and the early UL synchronization. Then we propose some initial solutions/way-forwards accordingly.

- Discussion

In release 19, event triggered L1 measurement enhancements are introduced for triggering L1 measurement reporting and triggering conditional LTM execution. During the study of Conditional handover (CHO) and other conditional mobility procedures (CPAC, SCPAC), the L3 measurement event is reused to trigger the layer 3 mobility. To be more specifically, conditional events A3/A5 are configured for the evaluation of CHO execution condition of a single candidate and the UE would detach from the source gNB if HO candidate cell satisfies the corresponding CHO execution condition. Similarly, the execution event for the conditional LTM could take the L1 measurement event of event triggered L1 measurement as the baseline and avoid repeating the similar discussions. During the previous discussion, RAN2 agreed to support several LTM events as following.

- RAN2#126: - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

By comparing the serving and candidate, LTM3/5 could be used to trigger the execution of the conditional LTM. RAN2 only further considers the enhancement of L1 measurement event if necessary.

Proposal 1: Event triggering L1 measurement event (e.g., LTM 3/5) could be reused to evaluate the condition for LTM execution.

During the previous discussion, RAN2 already confirmed that the event triggered L1 measurement is designed for LTM early synchronization and LTM cell switch procedure. Considering cell switch procedure of L3 cell switch, L3 measurement is the only pre-operation before the cell switch. L3 evaluation could be performed upon the measurement configuration. For L1/L2 triggered measurement, L1 measurement is one of the important parts in the preparation phase before the LTM cell switch execution. Unlike L3 measurement, L1 measurements could result in more signalling overhead and power consumption. Especially for the CSI-RS based measurement, dynamic activation/de-activation solution is introduced to reduce the overhead from perspectives of both NW and UE. It is not necessary to perform the L1 measurements for all candidates when the serving cell becomes good enough or for all RS types or when the candidate cell is not good enough.

Proposal 2: Event triggered L1 measurement should be designed for the one additional LTM purposes: trigger L1 measurement (e.g., CSI-RS based L1 measurement) for conditional LTM.

During L3 mobility like CHO, UE synchronizes to that candidate cell after detaching the source gNB. For the LTM with less latency, the UE needs to perform the early synchronization (DL and/or UL) before the LTM cell switch execution. During previous discussion, RAN2 already confirmed that the event triggered L1 measurement is designed for LTM early synchronization. This enables that the early synchronization could be obtained earlier than the cell switch by performing synchronization upon the satisfaction of an easily satisfied condition and executing the cell switch upon a stricter condition. What is more, we still need to discuss the early synchronization of C-LTM is triggered by the UE or the NW based on L1 measurement result. On one hand, we could reuse the PDCCH order triggered early TA requisition defined in Release 18 LTM with less specification impacts. But it is possible that UE has not received PDCCH order before it triggers the execution of the LTM based on the L1 measurement results. On the other hand, the early synchronization is also trigger by the UE based on the preconfigured L1 measurement event. For example, UE could send the preamble or perform the UE-based TA measurement with a lower threshold and triggered the execution with a higher threshold.

Proposal 3: RAN2 to discuss based on L1 measurement results, the early synchronization of C-LTM is triggered by the UE or the NW.

3	Conclusions

In this contribution, we discussed the conditional LTM for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Proposal 1: Event triggering L1 measurement event (e.g., LTM 3/5) could be reused to evaluate the condition for LTM execution.

Proposal 2: Event triggered L1 measurement should be designed for the one additional LTM purposes: trigger L1 measurement (e.g., CSI-RS based L1 measurement) for conditional LTM.

Proposal 3: RAN2 to discuss based on L1 measurement results, the early synchronization of C-LTM is triggered by the UE or the NW.

4	References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Rel-19, Sep 9-12, 2024