3GPP TSG-RAN WG2 Meeting #127bis	R2-2408285

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.8.4

Source:	HONOR

Title: 	Discussions on the support of broadcast service

Document for:	Discussion and decision

1	Introduction

During the previous meetings for Rel-19 NTN, RAN2 clarified use cases of the broadcast service in NTN system and made some basic agreements during RAN2#127 meeting as follows.

Agreements: - The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). - RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3). For MBS broadcast service we don’t restrict the work to any satellite constellation type

In this contribution, we would further discuss how to provide the broadcast service via NTN based on the previous agreements.

- Discussion

To understand the mechanism of the intended area for the broadcast service via NTN, RAN2 confirms the expected UE behaviour when it is outside the intended area and when it is inside the intended area. One of the remaining issues is how to provide the area related information to allow the UE to behave differently based on its own location. The area related information is divided into two parts: the geographical region for each intended service area, the mapping between the intended service area and the broadcast services.

During the discussion of RAN2#125bis meeting, RAN2 confirmed the relation between the cell and the intended service area as following.

Agreements: - We will cover at least the case where the indicated intended service area covers a portion of a NTN cell - The intended service area can cover the area of more than one NTN cells (or portions thereof)

Then, we also need to confirm the relation between the service area and the broadcast service. Based on our understanding, multiple broadcast service could be delivered in one intended service area. Thus, the content of the service-related SIBs could be extended by mapping different services with different intended service areas. To avoid signalling burden, area index mapping to the pre-configured geographical area could be one of the information to be carried in the system information.

Proposal 1: For MBS broadcast, one intended service area could be associated with multiple broadcast services and one broadcast service could be associated with multiple intended service areas.

In addition, we still need to discuss the node to perform the association of the intended service area and the broadcast service. Unlike the interest of area, the intended service area is to indicate the region for the broadcast service. For example, only the people in nation A could watch the news from the channel of the nation A. Thus, the higher layer could make the association based on the policy/strategy provided by the service providers. In another hand, since the intended service area is divided by the RAN, the CN could forward the policy/strategy provided by the service providers to RAN and RAN to perform the association.

Proposal 2: RAN2 to discuss whether the CN or RAN performs the association of the intended service area and the broadcast service.

As agreed in the RAN2#126 meeting, service area information might be included in SIB20/ SIB21/ MBSBroadcastConfiguration for MBS broadcast with FFS for ETWS. For PWS, the related system information blocks are SIB6 which contains an ETWS primary notification and SIB7 which contains an ETWS secondary notification. The intended service area related information could be sent along with the broadcast service data or broadcast related configurations. Since the division of the intended service area is more of a static configuration, it could be carried by system information without frequent SI modification.

Proposal 3: The intended service area related information (e.g., the geographical region and the corresponding area index) could be provided to UE via system information (e.g., SIB19), and the association could be provided by specific SIB(s) related to broadcast service (e.g., SIB6/7 for ETWS and SIB21 for MBS).

3	Conclusions

In this contribution, we discussed the issues related to intended service area in NTN. Based on the discussion, the following proposals are concluded:

Proposal 1: For MBS broadcast, one intended service area could be associated with multiple broadcast services and one broadcast service could be associated with multiple intended service areas.

Proposal 2: RAN2 to Proposal 2: RAN2 to discuss whether the CN or RAN performs the association of the intended service area and the broadcast service.

Proposal 3: The intended service area related information (e.g., the geographical region and the corresponding area index) could be provided to UE via system information (e.g., SIB19), and the association could be provided by specific SIB(s) related to broadcast service (e.g., SIB6/7 for ETWS and SIB21 for MBS).

4	References