3GPP RAN WG2 Meeting #127bis	R2-2408288

# Hefei, China, Oct 14th–18th, 2024

Agenda Item:	8.7.4.2

Source:	HONOR

Title:	Discussion on DSR enhancements

Document for:	Discussion and Decision

1. Introduction

In RAN2#127, the following agreements were agreed [1]:

Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In this contribution, we focus on the DSR enhancements.

2 Discussion

In the last meeting, it was agreed that the network can configure multiple remaining time thresholds for each LCG to report multiple pairs of remaining time and buffer sizes. And there will be a single triggering threshold. So, the usage of remaining time thresholds should be clarified. Based on Rel-18 DSR, remaining time thresholds can be used to report more delay information with remaining time below or above triggering threshold. In either case, it is useful for scheduling on the network side. Therefore, the triggering threshold should be configured by the network. Moreover, it can be the one of multiple remaining time thresholds.

Proposal 1: The triggering threshold for enhanced DSR should be configured by the network, which can be the one of multiple remaining time thresholds.

In the rel-18 DSR triggering condition, it is introduced that smallest remaining time of SDUs for the LCH can be used to trigger DSR, and these SDUs have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE. For the EDSR, the same constrictions can be reused and work well.

Proposal 2: The triggering conditions for DSR can be reused for Rel-19 enhanced DSR.

The current delay-critical data is defined as the SDUs whose remaining time becomes below remainingTimeThreshold. Similar as the above analysis, the remaining timer threshold is used to provide more delay information, which may not be suitable to define delay-critical data. The delay-critical data can be defined as the SDUs which remaining time becomes below triggering threshold.

Proposal 3: Delay-critical data can be defined as the SDUs whose remaining time becomes below triggering threshold.

There could be non-delay critical data ahead of delay critical data in the transmission queue. Therefore, the network should consider the data volume of these data when scheduling delay-critical data. For the enhanced DSR, there could be two options to adopt:

- The non-delay critical data ahead of delay critical data is calculated in each pair of information;

- Introduce non-delay critical data volume of non-delay critical data ahead of delay critical data in EDSR.

The above two options can work well for the enhanced DSR. RAN2 considers selecting one of them.

Proposal 4: RAN2 selects one of the two following options for buffer size of enhanced DSR:

- The non-delay critical data ahead of delay critical data is calculated in each pair of buffer size;

- Introduce non-delay critical data volume of non-delay critical data ahead of delay critical data in enhanced DSR.

3 Conclusions

Proposal 1: The triggering threshold for enhanced DSR should be configured by the network, which can be the one of multiple remaining time thresholds.

Proposal 2: The triggering conditions for DSR can be reused for Rel-19 enhanced DSR.

Proposal 3: Delay-critical data can be defined as the SDUs whose remaining time becomes below triggering threshold.

Proposal 4: RAN2 selects one of the two following options for buffer size of enhanced DSR:

- The non-delay critical data ahead of delay critical data is calculated in each pair of buffer size;

- Introduce non-delay critical data volume of non-delay critical data ahead of delay critical data in enhanced DSR.

4. References

[1]	RAN2 #127 Chairman notes