3GPP RAN WG2 Meeting #127bis	R2-2408290

# Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.4.4

Source:	HONOR

Title:	Procedure of LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

1. Introduction

In RAN2#127 meeting, the following agreements on procedures for LP-WUS in RRC_CONNECTED were made [1]:

For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

In RAN1#118 meeting, the following agreements on procedures for LP-WUS in RRC_CONNECTED were made [2]:

For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18. - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration). - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration. - LP-WUS triggers the start of a timer during which UE monitors PDCCH. - FFS the timer is existing timer and/or new timer. - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected. - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL. - No impact on RRM/RLM/BFD measurement requirements is assumed. - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1). - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up. - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not. - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS. From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

In this contribution, we further discuss Option1-2 based on RAN2 agreements

2. Discussion

2.1. Discussion on Option1-2

In agreements for Option 1-2 of the last RAN2 meeting, PDCCH monitoring triggered by LP-WUS is controlled by a timer. Whether the timer is new timer or legacy timer is FFS.

In general, the gNB may send LP-WUS to the UE to trigger PDCCH monitoring when there is DL data to be transmitted to the UE. Thus, the UE is supposed to receive DCI in a very short time after receiving LP-WUS. From this perspective, the timer that controls PDCCH monitoring should be shorter than legacy C-DRX drx-onDurationTimer especially when DL data has a relatively large jitter. A newly defined timer to control PDCCH monitoring triggered by LP-WUS can make the configuration of LP-WUS more flexible.

Proposal 1: RAN2 to define a new timer to control PDCCH monitoring triggered by LP-WUS in Option1-2.

Proposal 2: At least the following configuration for Option 1-2 needs to be provided to the UE.

- LP-WUS cycle and startOffset: parameters used to determine the starting subframe for LP-WUS monitoring.

- The new timer to control the PDCCH monitoring triggered by LP-WUS.

3. Conclusions

In this contribution, we discussed potential RAN2 impact of Option 1-2. The proposals are summarised as below.

Proposal 1: RAN2 to define a new timer to control PDCCH monitoring triggered by LP-WUS in Option1-2.

Proposal 2: At least the following configuration for Option1-2 needs to be provided to the UE

- LP-WUS cycle and startOffset: parameters used to determine the starting subframe for LP-WUS monitoring.

- The new timer to control the PDCCH monitoring triggered by LP-WUS.

4. References

[1] Draft_RAN2_127_Meeting_Report_v1

[2] Draft Report of 3GPP TSG RAN WG1 #118 v0.1.0