3GPP TSG-RAN WG2 Meeting #127bis					R2-2408294

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.5.3

Source:	Samsung

Title:	On-demand SIB1

Document for:	Discussion and decision

# Introduction

RAN2 discussed procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode in RAN2#127 and following agreements were made:

- Scenarios:

  - No consensus for RAN1 case 3 in RAN2.

- OD-SIB1 Validity:

  - We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

  - Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

- WUS configuration of NES cell from NES cell:

  - Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

- MSG3 based OD-SIB1 REQ:

  - Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

- Access restriction for legacy UEs:

  - Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

- Impacts to RRC connected UEs:

  - RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

- OD-SIB1 SI conclusion:

  - RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

In this contribution we discuss further details of on-demand SIB1.

# Discussion

2.1. Timing for SIB1 request configuration acquisition

While the UE is camped on Cell A, at some point in time, UE needs to acquire SIB1 request configuration for Cell B (i.e. cell supporting on-demand SIB1) from Cell A.

- Option 1: UE acquires when UE intends to perform cell reselection to Cell B

- Option 2: UE acquires after it camps to Cell A and reacquires the configuration, if updated (based on SI change notification).

Option 2 seems better in terms of latency to reselect/camp on Cell B.

Proposal 1: RAN2 to discuss and agree on one of the following for acquiring SIB1 request configuration of Cell B (i.e. cell supporting on-demand SIB1) from Cell A:

- Option 1: UE acquires when UE intends to perform cell reselection to Cell B

- Option 2: UE acquires after it camps to Cell A and reacquires the configuration, if updated (based on SI change notification).

2.2. Dynamic switching between periodic broadcast and on demand

UE has acquired the SIB1 request configuration of Cell B from Cell A. Cell B is the highest ranked cell for cell reselection. Before acquiring SIB1 of Cell B for reselection, does UE need to check if SIB1 is currently being broadcasted or provided on demand in Cell B. Note that it is possible that even though SIB1 request configuration of Cell B is received, Cell B may be currently broadcasting SIB1. In legacy, a cell supporting on demand SI request can dynamically switch between periodic broadcast and on demand SI transmission. UE checks the broadcast status bit to determine whether to transmit SI request or not.

Proposal 2: RAN2 to discuss whether a Cell for which SIB1 request configuration is available, can periodically broadcast SIB1 or not.

Proposal 3: If UE has SIB1 request configuration of a Cell, RAN2 to discuss whether UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

2.3. UL carrier for SIB1 request

In legacy SI request is supported for both SUL and NUL. Cell which supports on demand SIB1 can be configured with both SUL and NUL. So SIB1 request should be supported for both SUL and NUL. UE should select the UL carrier before transmitting SIB1 request. Since SIB1 request is based on random access and UL carrier selection is performed at the initiation of random access procedure in legacy, both NUL and SUL can easily be supported for SIB1 request. Not supporting SUL may have impact on random access procedure as exception (i.e. no UL carrier selection for the case random access procedure is initiated for SIB1 request) may need to be defined in MAC spec.

Proposal 4: SIB 1 request is supported for both SUL and NUL.

2.4 SIB1 request configuration

In release 18, RACH configuration for early TA acquisition of neighboring cells was introduced. This can be used as baseline for RACH configuration for SIB1 request procedure. Early TA acquisition is based on PDCCH order and SSB/UL carrier is indicated in PDCCH order, rsrp-ThresholdSSB and rsrp-ThresholdSSB-SUL are not included in RACH configuration for early TA acquisition. However, these are needed for SIB1 request configuration for UE to select SSB/UL carrier. SIB1 request configuration should atleast include the following:

RACH parameters:

- FrequencyInfoUL

- rach-ConfigGeneric

- bwp-GenericParameters

- ssb-PerRACH-Occasion

- prach-RootSequenceIndex

- prach-SubcarrierSpacing

- n-TimingAdvanceOffset

- rsrp-ThresholdSSB

- rsrp-ThresholdSSB-SUL

- ra-SearchSpace

SIB1 request resources (similar to legacy SI request)

  - sib1-RequestPeriod

  - ra-PreambleStartIndex

  - ra-AssociationPeriodIndex

  - ra-ssb-OccasionMaskIndex

Proposal 5a: SIB1 request configuration include SIB1 request resources (similar to legacy SI request): sib1-RequestPeriod, ra-PreambleStartIndex, ra-AssociationPeriodIndex and ra-ssb-OccasionMaskIndex.

Proposal 5b: SIB1 request configuration include following RACH parameters: FrequencyInfoUL, rach-ConfigGeneric, bwp-GenericParameters, ssb-PerRACH-Occasion, prach-RootSequenceIndex, prach-SubcarrierSpacing, n-TimingAdvanceOffset, rsrp-ThresholdSSB, rsrp-ThresholdSSB-SUL and ra-SearchSpace.

2.5 Msg1 repetitions for SIB1 request

Msg1 repetitions is supported for UL coverage in R18 for random access (including Msg1 based SI request). RACH based SIB1 request (similar to Msg1 based SI request) is agreed for on-demand SIB1. So, it is reasonable to support Msg1 repetitions for SIB1 request for better UL coverage.

Proposal 6: Msg1 repetitions is supported for SIB1 request.

2.6 SIB1 acquisition after transmitting SIB1 request

Inorder to acquire SIB1, UE needs to know the search space 0, coreset 0 and Kssb. In a cell not broadcasting SIB1, Kssb in MIB indicates that SIB1 is not broadcasted in cell and search space 0 and coreset 0 are not signalled in MIB. One of the following options can be considered to obtain Kssb, search space 0, coreset 0 to acquire SIB1.

Option 1: Kssb, search space 0, coreset 0 is signalled in SIB1 request configuration

Option 2: Kssb, search space 0, coreset 0 is signalled in SIB1 request ack i.e. RAR

Option 3: Upon transmitting SIB1 request ack, network updates MIB and include Kssb, search space 0, coreset 0 for SIB1 acquisition. UE reacquires MIB.

Option 3 is not efficient and may delay SIB1 acquisition.

Proposal 7: RAN2 to discuss and agree on one of the following options for acquiring SIB1

  - Option 1: Kssb, search space 0, coreset 0 is signalled in SIB1 request configuration

  - Option 2: Kssb, search space 0, coreset 0 is signalled in SIB1 request ack i.e. RAR

2.7 SIB1 request configuration of neighbor cells by NES cell

UE may reselect from NES cell to another cell. So, it is beneficially that NES can also signal SIB1 request configuration of neighbor NES cells similar to Non NES Cell i.e. Cell A. No additional spec support is needed for this.

Proposal 8: NES cell can signal SIB1 request configurations of neighbor NES cells.

2.8 Handling legacy UEs

On-demand SIB1 affects legacy UEs. Legacy UEs (i.e. UEs not supporting on-demand SIB1) cannot camp on cell providing SIB1 on-demand (i.e. not broadcasting SIB1). Legacy UEs can bar the cell based on

    - Option a) cellBarred bit set to barred in MIB or

    - Option b) based on no SIB1 indication via ssb-SubcarrierOffset in MIB or

    - Option c) based on failure to acquire SIB1.

Option a) will require additional barring bit to bar the on-demand SIB1 capable UE. Additionally, cellBarred bit is ignored by some UEs (e.g. R18 NES UEs) in legacy. So, it may not work in all cases.

Option b) will require additional configuration to signal RMSI (SIB1) occasions to receive SIB1 upon sending on demand SIB1 request. For example, search space zero and coreset zero would need to be signalled in SIB1 request configuration.

Option c) delays the barring in case SIB1 is not broadcasted.

RAN2 to consider option b) and/or option c) for barring legacy UEs.

Proposal 9: Legacy UEs can be barred in the cell supporting SIB1 requesting using the following:

  - Option b: Legacy UEs bar the on-demand SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

  - Option c: Network includes cells supporting on demand SIB1 to list of excluded cells

To further reduce burden on legacy UEs, network can also include cells supporting on demand SIB1 to list of excluded cells. This could be more beneficial then barring as legacy UEs can avoid PBCH/MIB decoding and measurements of cells supporting on demand SIB1.

Proposal 10: Network includes cells supporting on demand SIB1 to list of excluded cells for legacy UEs.

2.8 SIB1 request in RRC_CONNECTED

Case 1: RRC Connection re-establishment

If selected cell provides SIB1 on demand and SIB1 request is not supported in RRC_CONNECTED state, UE will bar the selected cell according to green highlighted text. This results in delayed connection re-establishment.

5.2.2.5  Essential system information missing

The UE shall:

1> if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2> if the UE is unable to acquire the MIB:

3> consider the cell as barred in accordance with TS 38.304 [20];

3> perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, or intraFreqReselection-eRedCap for eRedCap UEs, is set to allowed;

2> else if the UE is unable to acquire the SIB1:

3> consider the cell as barred in accordance with TS 38.304 [20];

One of the following can be considered:

- Option 1: Support SIB1 request in RRC_CONNECTED. For RRC connection re-establishment case, UE can send SIB1 request using msg1 based mechanism to the selected cell.

- Option 2: Leave it to UE implementation. UE may not select the NES cell during RRC connection re-establishment.

Case 2: RRC CONNECTED while T311 is not running

UE needs SIB1 to acquire other SIBs (pink highlighted conditions below).

5.2.2.3.1              Acquisition of MIB and SIB1

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period; or

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1, and, the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, si-BroadcastStatus for the required SIB(s) or posSI-BroadcastStatus for the required posSIB(s) is set to notBroadcasting in acquired SIB1 in current modification period; or

1> if the UE is in RRC_CONNECTED while T311 is running:

2> if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3> acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3> if the UE is unable to acquire the SIB1:

4> perform the actions as specified in clause 5.2.2.5;

3> else:

4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2.

2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3> perform the actions as specified in clause 5.2.2.5.

If PCell provides SIB1 on demand and SIB1 request is not supported in RRC_CONNECTED state, UE will not acquire SIB1. No further action is taken by UE as per current spec. UE will not be able to acquire other SIBs.

One of the following can be considered:

- Option 1: UE sends SIB1 request

  - DedicatedSIBRequest or new RRC message can be used for SIB1 request. Alternately, UE can send SIB1 request as in RRC IDLE/INACTIVE. Whether UE can send SIB1 request in RRC_CONNECTED can be signalled by network in RRC message.

- Option 2: UE does not send SIB1 request. UE rely on network to provide the latest SIB1 via RRC message. However, this is not possible for pink highlighted case unless network sends SIB1 in dedicated manner to UE every modification period. We may consider that network sends change notification for si-BroadcastStatus changes if cell supports on demand SIB1. However, this may result in too much overhead.

Supporting SIB1 request in RRC_CONNECTED can be beneficial for both case 1 and case 2.

Proposal 11: RAN2 to consider supporting SIB1 request in RRC_CONNECTED.

# Conclusion

Proposal 1: RAN2 to discuss and agree on one of the following for acquiring SIB1 request configuration of Cell B (i.e. cell supporting on-demand SIB1) from Cell A:

- Option 1: UE acquires when UE intends to perform cell reselection to Cell B

- Option 2: UE acquires after it camps to Cell A and reacquires the configuration, if updated (based on SI change notification).

Proposal 2: RAN2 to discuss whether a Cell for which SIB1 request configuration is available, can periodically broadcast SIB1 or not.

Proposal 3: If UE has SIB1 request configuration of a Cell, RAN2 to discuss whether UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Proposal 4: SIB 1 request is supported for both SUL and NUL.

Proposal 5a: SIB1 request configuration include SIB1 request resources (similar to legacy SI request): sib1-RequestPeriod, ra-PreambleStartIndex, ra-AssociationPeriodIndex and ra-ssb-OccasionMaskIndex.

Proposal 5b: SIB1 request configuration include following RACH parameters: FrequencyInfoUL, rach-ConfigGeneric, bwp-GenericParameters, ssb-PerRACH-Occasion, prach-RootSequenceIndex, prach-SubcarrierSpacing, n-TimingAdvanceOffset, rsrp-ThresholdSSB, rsrp-ThresholdSSB-SUL and ra-SearchSpace.

Proposal 6: Msg1 repetitions is supported for SIB1 request.

Proposal 7: RAN2 to discuss and agree on one of the following options for acquiring SIB1

  - Option 1: Kssb, search space 0, coreset 0 is signalled in SIB1 request configuration

  - Option 2: Kssb, search space 0, coreset 0 is signalled in SIB1 request ack i.e. RAR

Proposal 8: NES cell can signal SIB1 request configurations of neighbor NES cells.

Proposal 9: Legacy UEs can be barred in the cell supporting SIB1 requesting using the following:

  - Option b: Legacy UEs bar the on-demand SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

  - Option c: Network includes cells supporting on demand SIB1 to list of excluded cells

Proposal 10: Network includes cells supporting on demand SIB1 to list of excluded cells for legacy UEs.

Proposal 11: For SIB1 acquisition during RRC Connection re-establishment:

- Option 1: Support SIB1 request in RRC_CONNECTED. For RRC connection re-establishment case, UE can send SIB1 request using msg1 based mechanism to the selected cell.

- Option 2: Leave it to UE implementation. UE may not select the NES cell during RRC connection re-establishment.

Proposal 12: RAN2 to consider supporting SIB1 request in RRC_CONNECTED.

# References