3GPP TSG-RAN WG2 Meeting #127bis	R2-2408301

Hefei, China, Oct 14th – 18th, 2024                            Revision of R2-2406871

Agenda item:		8.8.4

Source:		Lenovo

Title:		Further considerations for broadcast service area indication

Document for:		Discussion

- Introduction

The latest RAN2#127 meeting made the following agreements regarding broadcast services in NTN:

- The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). - RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions. - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2). - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3).

In this contribution we discuss the potential impacts and further issues of introducing broadcast service area indication in NTN.

- Discussion

For MBS broadcast services, the same service and the same specific content data are provided simultaneously to all UEs in a geographical area, and all UEs in the MBS broadcast service area are authorized to receive the data. In TN it is assumed that an MBS broadcast service area is larger than a TN cell coverage so that the service area can be mapped to one or more TN cells. UE can simply judge whether it is in an MBS broadcast service area by its serving cell and the corresponding MBS broadcast configuration it receives. For an NTN cell that could be much larger than an MBS broadcast service area, a UE may not be automatically authorized to receive an MBS broadcast service by the serving NTN cell it is in, i.e., not all UE within the same NTN cell is authorized to receive the same MBS broadcast service. For this reason, RAN2 have agreed to introduce indication of geographical broadcast service area in NTN. In our understanding introducing the geographical broadcast service area indication is just the first step, and further impacts and issues are expected to be resolved as well, including MCCH monitoring and service continuity.

## MCCH monitoring regarding broadcast service area

In legacy MBS broadcast, UE monitors MCCH to receive the MBS configuration for broadcast session. And when an MCCH change notification is received, UE monitors MCCH to update the MBS configuration for broadcast session. MCCH uses a modification period and MCCH contents are only allowed to be modified at each modification period boundary. A notification mechanism is used to announce the change of MCCH contents due to broadcast session start, modification or stop and due to neighbouring cell information modification. It is up to UE implementation how to address a possibility of the UE missing an MCCH change notification.

For the initial MCCH monitoring, in TN a UE starts to monitor upon entering the cell providing SIB20. In an NTN cell which could be much larger than an MBS broadcast service area, UE may not need to start MCCH monitoring upon entering the NTN cell, i.e., whether the UE is in the MBS broadcast area or is authorized to receive MBS broadcast service also affects UE in determining when to start monitoring the MCCH. The previous RAN2#127 meeting agreed that UE may not need to (re)acquire up-to-date MCCH when the UE is not in any intended service area of its interested broadcast services. There were also some discussions on using SIB20 or MCCH to provide the mapping between intended service areas and broadcast services. In our understanding using SIB20 can easily facilitate that UE only monitors MCCH if in interested service area. Besides, it also avoids adding signaling overhead in MCCH and can be easily extended to other broadcast services such as PWS.

Observation 1: The option of using SIB20 to provide the mapping between intended service areas and broadcast services can facilitate area-based UE monitoring on MCCH and can avoid adding overhead in MCCH.

Proposal 1: SIB20 is used to provide the mapping between intended service areas and broadcast services.

Meanwhile, in NTN UE needs to ensure a valid UE location information and a valid ephemeris data (in SIB19) for uplink synchronization. UE re-acquiring SIB19 in NTN may possibly miss the change notification for MCCH. In such case, enhancements may also be considered to accelerate UE in awareness of MCCH change and re-acquire MCCH configuration.

Observation 2: UE may miss a change notification for MCCH when reacquiring SIB19 for UL synchronization.

Proposal 2: RAN2 to discuss whether UE can automatically re-acquire MCCH configuration in SIB20 after re-acquiring SIB19, e.g., when the duration of reacquiring SIB19 covers an MCCH modification period boundary.

## Service continuity regarding broadcast service area

For MBS broadcast service continuity in TN, mobility procedures for MBS reception allow the UE to start or continue receiving MBS service when changing cells. The gNB may indicate in SIB21 the mapping between the current and/or neighboring carrier frequencies and MBS Frequency Selection Area Identities (FSAI). The FSAI is derived from the serviceArea and radiofrequency elements received in the USD (User Service Description) via NAS signalling from network, wherein the serviceArea element declares one or more service areas designated by the SAI (Service Area Identity) which is mapped onto one or more cells.

If UE is receiving or interested to receive an MBS broadcast service and can only receive this MBS broadcast service by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session. The UE may also consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session.

This legacy mechanism works well under the assumption that an MBS broadcast service area is mapped to one or more TN cells. The UE can simply determine the frequency priority based on the receiving or interested MBS broadcast service and the frequency it associates to, and reselects to a cell that supports that service within its whole coverage. When considering that an NTN cell coverage could be larger than an MBS broadcast service area, which cannot be mapped to the cell’s whole coverage, UE cannot simply determine the frequency priority only based on the association between service and frequency, and service area information of neighbour cell may be needed.

Observation 3: In NTN wherein broadcast service area could cover a portion of an NTN cell, UE cannot simply determine the frequency priority only based on the association between service and frequency.

Figure 1 shows such differences between TN and NTN. For TN in a), UE1 from Cell #1 to Cell #2 can continue to receive MBS broadcast service #A as it is supported in Cell #2, and UE2 from Cell #1 to Cell #3 cannot continue to receive MBS broadcast service #A as it is not supported in Cell #3. For NTN in b), MBS broadcast service #A is supported in partial coverage of Cell #2 and Cell #3, and UE1 from Cell #1 to Cell #2 is expected to continue to receive MBS broadcast service #A as UE is still in its service area, while UE2 from Cell #1 to Cell #3 is expected not to continue to receive MBS broadcast service #A as UE is moving out its service area. If the legacy mechanism for TN applies, UE2 will also prioritize a frequency associated to the broadcast service but it will not be authorized to receive as it moves out of service area.

Figure 1. Example of relation between cell coverage and MBS broadcast service area in TN and NTN

Therefore, additional MBS broadcast service area information for neighbour cell may also be needed at UE for service continuity and how to indicate such information needs to be discussed as well.

Proposal 3: RAN2 to discuss how to indicate the broadcast service area information of a neighbour NTN cell to UE for service continuity e.g., frequency priority handling.

- Conclusion

In this contribution we discuss the potential impacts and further issues of introducing broadcast service area indication in NTN. The following observations are given:

Observation 1: The option of using SIB20 to provide the mapping between intended service areas and broadcast services can facilitate area-based UE monitoring on MCCH and can avoid adding overhead in MCCH.

Observation 2: UE may miss a change notification for MCCH when reacquiring SIB19 for UL synchronization.

Observation 3: In NTN wherein broadcast service area could cover a portion of an NTN cell, UE cannot simply determine the frequency priority only based on the association between service and frequency.

And it is proposed that:

Proposal 1: SIB20 is used to provide the mapping between intended service areas and broadcast services.

Proposal 2: RAN2 to discuss whether UE can automatically re-acquire MCCH configuration in SIB20 after re-acquiring SIB19, e.g., when the duration of reacquiring SIB19 covers an MCCH modification period boundary.

Proposal 3: RAN2 to discuss how to indicate the broadcast service area information of a neighbour NTN cell to UE for service continuity e.g., frequency priority handling.