3GPP TSG-RAN WG2 #127bis	R2-2408314

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	   8.1.3

Source:		Lenovo

Title:	Data collection for NW-sided model training in beam management

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss issues related to the training data collection for NW sided model for beam management use case.

# Discussion

## 2.1	RAN3/SA5 related aspects

# RP-242399 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

Currently, RAN3 has started their normative phase discussion on AI for positioning topic. While in the current WID, RAN3/SA5 are not involved in the discussion related to training data collection, therefore we would rely on LS to coordinate with them to trigger any necessary discussion.

RAN3 started their normative phase discussion about AI based Positioning, however RAN3/SA5 are not formally involved in the discussion about NW side model training data collection for AI based beam management.

### 2.1.1	MDT based framework for OAM-centric data collection

RAN2#126 Agreement Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

In the last RAN2 meeting, it was agreed that immediate MDT will be taken as the baseline framework for OAM centric data.

In the meanwhile, the legacy MDT framework, only supports the gNB being requested to collect L3 measurements. In order to request the gNB to collect L1 measurements, some enhancement on network interface signalling may be necessary, e.g., NG-AP interface signalling between AMF and gNB.

Legacy MDT framework only support gNB being requested to collect L3 measurement results. Enhancements to network interface signalling may be needed to support the request of L1 measurement results.

At the same time, there are two kinds of MDT framework, namely management-based MDT and signalling-based MDT. For management-based MDT, OAM configures the MDT configuration to gNB directly without pointing to a specific UE, and it is upon gNB to select the UE according to the configured criteria. Comparatively, for signalling-based MDT, the data collection is pointing to a specific UE via CN, and gNB configures the UE measurement report accordingly.

Comparing to signalling-based MDT, management-based MDT framework seems more suitable for training data collection among a group of UEs that fulfils certain criteria (e.g., cell or frequency or channel condition as discussed for the CSI/BM/Positioning use cases). In addition, management-based MDT is inherently anonymized meaning OAM does not need to understand the data is from which UE, as such it is user privacy friendly.

For management-based MDT, OAM configures the MDT configuration to gNB directly without pointing to a specific UE, and it is upon gNB to select the UE according to the configured criteria.

For signalling-based MDT, the data collection is pointing to a specific UE via CN, and gNB configures the UE measurement report accordingly.

Since management-based MDT and signaling-based MDT would have different implications and specification impacts to RAN3/SA5, it is suggested that RAN2 decides if management-based MDT or signaling-based MDT or both are assumed and inform RAN3/SA5 via LS.

In MDT based framework for OAM-centric data collection, RAN2 is suggested to check the possible impact on RAN3/ SA5 and inform RAN3/SA5 via LS to trigger their discussion.

### 2.1.2	Logged data transition after HO/RLF/RRC state transfer

During the data collection phase, UE may log the L1 measurement results in its buffer and wait for gNB’s request/scheduling to report them. It is possible that before UE reports the logged data to gNB, UE may experience handover, or RLF, or entering RRC inactive/idle. In those cases, UE will not be able to transmit the logged data to the network anymore, and UE may either

- Option 1: Discard the remaining logged data that has not been reported yet

- Option 2: Keep the remaining logged data and wait for the network to fetch after connecting to the new cell or entering RRC connected again.

Upon handover, RLF, or entering RRC Inactive/Idle, RAN2 discusses the handling of the remaining logged data that has not been reported yet:

  - Option 1: Discard the remaining logged data

  - Option 2: Keep the remaining logged data and wait for the network to fetch after HO, RRC reestablishment or entering RRC connected again.

If Option 2 is supported, then the gNB, that fetches the remaining logged data from UE after HO/RLF/RRC state transition, needs to forward the received remaining logged data to either the old gNB or OAM that initiates the data collection at the first place.

If Option 2 is supported, RAN2 is suggested to inform RAN3 via LS to support the transfer of logged data over network interface.

## 2.3	Logging related aspects

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- FFS on event based data collection/logging

Furthermore, in the context of event-driven logging for training data, such as logged L1 measurements, for AIML based BM, we believe RSRP threshold-based event should be considered. For example, a gNB may be interested to train an AIML model that is capable of beam prediction if UE is at the center of the serving cell, or at the edge of the serving cell, or in between. Depending on the gNB’s interest, UE can be configured to start/stop data collection if the measured beam/cell level RSRP is below/above certain threshold.

RAN2 supports event triggered data collection/report, UE may start/stop L1 measurement result logging and reporting according to RSRP threshold-based event, e.g., the measured RSRP is above/below a threshold.

Based on the agreement from last meeting, the UE can be configured to not initiate data collection and logging upon receiving the configuration. In this case, in our perspective the NW can dynamically ask the UE to activate a particular configuration using L1/L2 indication. This dynamic activation by the NW may as well be dependent on some NW-side conditions.

The data collection configurations which are not activated upon configuration can be activated dynamically.

For AI/ML training, the data collection can vary based on factors such as NW-side additional conditions or UE-side additional conditions. Measurements may need to be logged and reported differently; thus, a single data collection configuration may not fit for the different NW-side/UE-side conditions. The network may want to activate or deactivate certain data collection configuration based on some dynamic conditions. In order to reduce the burden of de-configuring/re-configuring, network can provide multiple configurations to the UE, which can be dynamically activated/deactivated as and when required. This will also enable the network to configure the UE with one or more configurations which can be activated/switched upon an event being triggered such as the RSRP threshold.

UE can be configured with one or more data collection configurations.

In the legacy, for logged MDT, a UE variable VarLogMeasReport is used to log the UE measurements in RRC Idle/Inactive state. Similarly, for the logging of L1 measurement results in RRC connected state, we believe a new UE variable different than legacy VarLogMeasReport could be introduced considering the legacy VarLogMeasReport contains information dedicated for logged MDT and cannot be reused.

#### VarLogMeasReport The UE variable VarLogMeasReport includes the logged measurements information. VarLogMeasReport UE variable -- ASN1START -- TAG-VARLOGMEASREPORT-START VarLogMeasReport-r16 ::= SEQUENCE { absoluteTimeInfo-r16 AbsoluteTimeInfo-r16, traceReference-r16 TraceReference-r16, traceRecordingSessionRef-r16 OCTET STRING (SIZE (2)), tce-Id-r16 OCTET STRING (SIZE (1)), logMeasInfoList-r16 LogMeasInfoList-r16, sigLoggedMeasType-r17 ENUMERATED {true}, identityList-r18 CHOICE { plmn-IdentityList-r18 PLMN-IdentityList2-r16, snpn-ConfigIDList-r18 SNPN-ConfigIDList-r18 } } -- TAG-VARLOGMEASREPORT-STOP -- ASN1STOP

In legacy logged MDT, a VarLogMeasReport UE variable is used to log UE measurements in RRC Inactive/Idle state and containing information related to MDT.

RAN2 introduces a new UE variable (similar as VarLogMeasReport) for logging the L1 measurement results in RRC connected state. Then, when transmitting the logged L1 measurement results using RRC MeasurementReport message, each message will convey the logged L1 measurement results stored in the new UE variable.

## 2.2	Reporting related aspects

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- On-demand request from the network is supported.   FFS details on signalling

According to the last time RAN2 meeting, after logging the data, UE can report it to network based on the on-demand request, and FFS if event-based reporting and if data availability indication should be sent beforehand. In the following figure, we describe the overall data reporting procedure that would work efficiently in our view.

- Step 1: UE can be configured with event that would trigger the data report after UE starts logging the required measurement results. For example, some buffer size threshold (e.g., percentage of maximum buffer size) can be configured to trigger the report before the buffer is full.

- Step 2: UE will first send an availability indication to gNB, such that gNB can be prepared to reserve radio resources for scheduling the UL transmission of logged data. Comparing to let UE transmits the logged data straight away once triggered, this is considered a safer option to not interfere with other regular UL transmissions from the same or other UEs, gNB has the flexibility to fetch the logged data when the UL traffic in the cell is low. Since UE may have logged data for different data collection configurations, e.g. data logged for different UE/NW-side conditions, it is beneficial to provide gNB information on the data collection configurations for which UE has some logged data available for transmission, e.g. by signalling an ID identifying the data collection configuration. The availability indication should - similar to a buffer status report - indicate the amount of logged data (e.g. in bytes) for each data collection configuration (identified by an ID) in order to enable gNB to request some corresponding logged data and allocate UE resources accordingly.

- Step 3: Upon reception of the availability indication, gNB can request the UE to transmit some specific, e.g. for some specific data collection configuration(s), or all the logged data stored in its buffer via UEInformationRequest message

- Step 4: UE will generate and transmit UEInformationResponse message that contains the requested logged data stored in its buffer. If one UEInformationResponse is not enough to convey all logged data, UE should be able to generate and transmit multiple UEInformationResponse messages consecutively.

UE can be configured to trigger the data report upon a percentage of its maximum buffer size is reached.

Upon the trigger of the data report, UE will send an availability indication together with data size information for gNB to fetch the logged data with enough radio resource reserved.

UEInformationRequest and UEInformationResponse messages are used for gNB to fetch the logged data from UE.

If one UEInformationResponse is not enough to convey all logged data, UE should be able to generate and transmit multiple UEInformationResponse messages consecutively without waiting for another UEInformationRequest message.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	RAN3 started their normative phase discussion about AI based Positioning, however RAN3/SA5 are not formally involved in the discussion about NW side model training data collection for AI based beam management.

Observation 2	Legacy MDT framework only support gNB being requested to collect L3 measurement results. Enhancements to network interface signalling may be needed to support the request of L1 measurement results.

Observation 3	For management-based MDT, OAM configures the MDT configuration to gNB directly without pointing to a specific UE, and it is upon gNB to select the UE according to the configured criteria.

Observation 4	For signalling-based MDT, the data collection is pointing to a specific UE via CN, and gNB configures the UE measurement report accordingly.

Observation 5	In legacy logged MDT, a VarLogMeasReport UE variable is used to log UE measurements in RRC Inactive/Idle state and containing information related to MDT.

Based on the discussion above, we propose:

Proposal 1	In MDT based framework for OAM-centric data collection, RAN2 is suggested to check the possible impact on RAN3/ SA5 and inform RAN3/SA5 via LS to trigger their discussion.

Proposal 2	Upon handover, RLF, or entering RRC Inactive/Idle, RAN2 discusses the handling of the remaining logged data that has not been reported yet:

a.	Option 1: Discard the remaining logged data

b.	Option 2: Keep the remaining logged data and wait for the network to fetch after HO, RRC reestablishment or entering RRC connected again.

Proposal 3	If Option 2 is supported, RAN2 is suggested to inform RAN3 via LS to support the transfer of logged data over network interface.

Proposal 4	RAN2 supports event triggered data collection/report, UE may start/stop L1 measurement result logging and reporting according to RSRP threshold-based event, e.g., the measured RSRP is above/below a threshold.

Proposal 5	The data collection configurations which are not activated upon configuration can be activated dynamically.

Proposal 6	UE can be configured with one or more data collection configurations.

Proposal 7	RAN2 introduces a new UE variable (similar as VarLogMeasReport) for logging the L1 measurement results in RRC connected state. Then, when transmitting the logged L1 measurement results using RRC MeasurementReport message, each message will convey the logged L1 measurement results stored in the new UE variable.

Proposal 8	UE can be configured to trigger the data report upon a percentage of its maximum buffer size is reached.

Proposal 9	Upon the trigger of the data report, UE will send an availability indication together with data size information for gNB to fetch the logged data with enough radio resource reserved.

Proposal 10	UEInformationRequest and UEInformationResponse messages are used for gNB to fetch the logged data from UE.

Proposal 11	If one UEInformationResponse is not enough to convey all logged data, UE should be able to generate and transmit multiple UEInformationResponse messages consecutively without waiting for another UEInformationRequest message.

# Draft LS to RAN3/ SA5

# Title:	LS on NW side model training data collection for AI based beam management

# Response to:

# Release:	Rel-19

# Work Item:	NR_AIML_Air

Source:	RAN2

To:	RAN3, SA5

Cc:	RAN1, SA2

Contact Person:

#### Name:

#### E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

For the use case of AI based beam management, an AI model could be deployed at gNB to predict beam quality in either spatial domain (“BM-Case1”) or temporal domain (“BM-Case2”).

In order to collect required data for the training of such gNB-sided model, RAN2 agreed to support OAM-centric data collection based on immediate MDT framework. The training data may include L1-RSPR and beam-IDs which are considered new data type comparing to those already supported in the legacy MDT framework.

Therefore, enhancement to the legacy MDT framework is possibly needed and may impact RAN3 and SA5 specification.

2. Actions:

To WG RAN3, WG SA5

ACTION:

RAN2 kindly asks RAN3 and SA5 to take the above progress into account in their future work and provide further feedback if any.

3. Date of Next RAN2 Meetings:

RAN2#128		18th – 22nd Nov 2024	Orlando, USA

RAN2#129		17th – 21st Feb 2025	Athens, GR