3GPP TSG-RAN WG2 Meeting#127bis	R2-2408317

Hefei, China, 10 - 14 October 2024

Agenda item:	 8.10.5

Source:		Lenovo

Title:		Discussion on RACH optimization for SDT

Document for:		Discussion and Decision

1	Introduction

Until now RAN2 have achieved agreements for RACH optimization for SDT including:

For the purpose of SON enhancements for SDT, include RSRP/data volume related information

Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report.

Do not add logging of sdt-RSRP-Threshold, since already agreed by RAN3 to not support it.

UE logs and reports the failure cause for SDT to the network. FFS the details, e.g. if we down select some of the failure causes.

Based on the agreements, in this paper, we would further discuss RACH optimization for SDT.

2	Discussion

SDT is a procedure allowing data transmission while remaining in RRC_INACTIVE without transitioning to RRC_CONNECTED state. SDT is enabled on a radio bearer basis and is initiated by the UE only if less than a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, measured RSRP in the cell is above a configured threshold, and a valid resource for SDT transmission is available.

A SDT procedure may fail due to a low threshold of radio link quality or due to a high threshold of data volume, for example, the UE triggers SDT but data transmission fails because of bad radio link quality or heavy data volume. When SDT fails, the UE may re-trigger SDT transmission or enter RRC connected for data transmission.

To improve SDT performance, it is beneficial to store and report the RSRP when SDT fails, then network can analyse the failure issue e.g. whether the RSRP threshold used to decide whether to trigger SDT is set properly, for example, in the case that the RSRP threshold and RSRP when SDT fails are close, it means that the RSRP threshold is set too low, network may modify with a higher RSRP threshold. Similarly, it is beneficial to store and report the data volume when SDT fails, network can analyse whether the data volume threshold used to decide whether to trigger SDT is set properly, e.g., in the case that the data volume threshold and data volume when SDT fails are close, it means that the data volume threshold is set too high, network may modify with a lower data volume threshold. The detailed informtion to be stored or reported by the UE in the RA report can be further checked with RAN2, and an LS to RAN2 may be needed if RAN3 achieves some conclusions.

Proposal 1: The UE reports RSRP when SDT fails and data volume when SDT fails in the RA report for SDT optimization.

3	Conclusion

In this paper, RACH optimization for SDT are discussed. We have the following proposal:

Proposal 1: RSRP when SDT fails and data volume when SDT fails can be stored and reported in the RA report for SDT optimization.