3GPP TSG-RAN WG2 Meeting #127bis			           	R2- 2408321

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.5.2

Source: 	Lenovo

Title: 	Issues on the procedure of on-demand SSB SCell operation

Document for:	Discussion and decision

# Introduction

In RAN2#127, some agreements were achieved [1].

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

RRC indicates deactivation  no MAC CE indicate activation  RRC reconfig to indicate activation.

MAC CE indicates activation  RRC indicates deactivation

CU or DU to determine the state? CU needs to know the state?

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A. (one new MAC CE even for scenario 2A?)

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration. (SMTC?)

In RAN1#117, some agreements were achieved [2].

Agreement

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

In this contribution, we provide analysis and proposals on procedures and signalling method(s) to support on-demand SSB SCell operation for UE in CONNECTED mode configured with CA, for both intra-/inter-band CA, also we discuss the impact on UE in IDLE/INACTIVE mode.

# Discussion

UE triggered on-demand SSB

Current NR supports activation/deactivation of an SCell based on traffic needs, for example different traffics being transmitted in different serving cells based on the different characteristics of the PHY layer e.g., PCell is deployed in a carrier frequency, and SCell is deployed in another carrier frequency This is beneficial in terms of energy saving in both UE side and gNB side. For UL, the UE can identify the traffic arrival prior to the gNB.

Observation 1: The preferred traffics can be different in PCell and SCell based on the gNB deployment, and the UE can identify the traffic arrival prior to the gNB.

At the same time, it is supported to configure limited number of SSBs to transmit from the spec. and implementation view of point. For example, the SSBs that provide good channel quality can be configured to the UE based on the UE measurement reports. It is beneficial to reduce power consumption comparing with the full set of SSBs transmission.

Observation 2: To reduce power consumption on SSB transmission, the gNB might transmit only limited number of SSBs in the SCell.

There could be a case that the channel quality measured from the configured SSBs changes to a low level. It may be identified by the gNB much later than by the UE because of the possible long-periodicity measurement report.

Therefore, it is beneficial if the UE could send a wake-up signal to request additional or updated SSBs transmission.

Observation 3: In case the channel condition of the SSB(s) is not good enough with the limited number of SSBs transmission, the UE can identify the issue prior to the gNB.

Proposal 1: UE triggered on-demand SSB operation is supported.

gNB triggered on-demand SSB

On the other hand, gNB triggered on-demand SSB SCell operation was agreed in RAN1.

Referring to RAN1 progress, it was agreed to provide at least Frequency of the on-demand SSB, SSB positions within an on-demand SSB burst by using signalling similar to ssb-PositionsInBurst and Periodicity of the on-demand SSB via higher layer RRC signalling. Also, in RAN1#118, it was agreed to configure multiple candidate values by RRC for on-demand SSB at least including Periodicity of the on-demand SSB.Since the existing RRC message can configure cell specific parameters of a UE's serving cell, e.g. when configuring a UE with an SCell or with an additional cell group (SCG), we can reuse the RRC message to configure the on-demand SSB related information.

Proposal 2: Reuse existing RRC message to configure the on-demand SSB related information to UE.

Considering the existing configuration of periodicity for SSB is enumerated which cannot provide multiple values, a new IE may be added to provide multiple candidate values of periodicity of the on-demand SSB.

Proposal 3: A new IE may be added to provide multiple candidate values of periodicity of the on-demand SSB.

L3 measurement

In general, neighbor cell measurement is configured before adding the cell as an SCell. It is not optimal if the UE assumes the SSB in neighbor cell can be measured at any time, which may not exist always. On the other hand, it is a way of energy saving from the perspective of neighbor cell if UE performs neighbor cell measurement when the OD-SSB of the neighbor cell is transmitting and no other RS is transmitted for measurement. Therefore, it is better for the UE to be configured to perform neighbor cell measurement in the same way as the serving cell if the neighbor cell transmits OD-SSB.

It is important that the UE can discern the non-transmission of SSBs from poor radio geometry. Therefore, network (MN or SN) should indicate UE explicitly about Cells (SCells and neighbour cells) not transmitting SSBs regularly.

Proposal 4: UE is informed if a serving and/ or neighbor cell is transmitting SSB on on-demand basis.

Once informed about serving and/ or neighbor cell transmitting SSB on on-demand basis, UE prioritizes measurement of such object: UE may start with the measurement of the said object before any other measurements. Network may use measSequence to achieve this purpose and UE shall prioritize measurement of such an object. Otherwise, UE signals a start time to measure a measurement object (SSB provided on-demand) and the end time for the same can also be indicated accordingly, to enable the network to stop SSB transmission as soon as possible.

Proposal 5: A UE informs the network about the start time and end time of L3 measurements on a frequency with at least one cell providing SSB on-demand.

Impact on IDLE/INACTIVE UE

There could be a scenario that an IDLE/INACTIVE NES-capable UE is performing cell selection/reselection when the OD-SSB is transmitting, and the channel quality is high enough for the UE to camp. Is the UE allowed to camp on the cell or not? According to the WID, it seems the UE shall be barred since the OD-SSB cell is an SCell.

Proposal 6: RAN2 to discuss whether RRC_IDLE/INACTIVE UE is allowed to camp on the on-demand SSB cell when the OD-CD-SSB is transmitted.

# Conclusion

In summary, we have following observations and proposals on the operation of on-demand SSB for SCell:

Observation 1: The preferred traffics can be different in PCell and SCell based on the gNB deployment, and the UE can identify the traffic arrival prior to the gNB.

Observation 2: To reduce power consumption on SSB transmission, the gNB might transmit only limited number of SSBs in the SCell.

Observation 3: In case the channel condition of the SSB(s) is not good enough with the limited number of SSBs transmission, the UE can identify the issue prior to the gNB.

Proposal 1: UE triggered on-demand SSB operation is supported.

Proposal 2: Reuse existing RRC message to configure the on-demand SSB related information to UE.

Proposal 3: A new IE may be added to provide multiple candidate values of periodicity of the on-demand SSB.

Proposal 4: UE is informed if a serving and/ or neighbor cell is transmitting SSB on on-demand basis.

Proposal 5: A UE informs the network about the start time and end time of L3 measurements on a frequency with at least one cell providing SSB on-demand.

Proposal 6: RAN2 to discuss whether RRC_IDLE/INACTIVE UE is allowed to camp on the on-demand SSB cell when the OD-CD-SSB is transmitted.

# References

- RAN2#127 chairman note.

- RAN1#117 chairman note.