3GPP TSG-RAN WG2 Meeting #127bis				        R2- 2408322

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.5.4

Source: 	Lenovo

Title:  	Paging and PRACH adaptation

Document for:	Discussion and decision

# Introduction

In RAN2#125bis (Changsha) [1], paging adaptation was discussed, and following agreements were achieved:

Agreements on paging adaptation: 1. From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle. 2. For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. 3. For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In RAN2#127[2], the following agreements were achieved:

Agreements on paging adaptation

- Option-a) is about the bundling of PF for R19 NES UEs.

- R2 observe that the option-a) and option-b) can be designed to configure the PO:s at same time position.

Companies have already proposed bundling of Paging Frames in the last meetings[2][3], even keeping the total number of Paging occasions same. This was also studied and captured in the TR 38.864.

This paper intends to check if it is possible to achieve better energy saving with respect to paging transmission. Also, PRACH adaptation related issues are discussed in this paper.

# Discussion

## Paging enhancement

Some logs from different cities and operators were previously presented in [4].

From the field logs some clear observations were made:

Observation#1a: The capacity utilization (i.e., the total number of RRC Paging records that could be sent with the used paging configuration) is much less than 1% in all observed cases.

Observation#1b: Average number of paging records contained in one RRC message (when Paging is sent) is close to 2. The real average accounting all configured paging occasions will be much less.

Observation#1c: Maximum number of records included in the same Paging message is much less than 32 (max is 9 as seen in Brazil and 8 in China).

Further, based on the progress made in a post meeting Email Discussion [5], following proposals were made:

Proposal 1: Select option-b as baseline for R19 NES paging enhancement. Proposal 2: R2 further discuss the impact of RRC configuration and the impact of legacy UE for option-b.

In view of the field logs and the email discussion, we also think that option-b is the way to go and further we think that option-b i.e., extended values of N should be used to have increased interval between PFs (e.g. T/64, T/128 …T/N). The value T/N will allow to have just one PF in the Paging DRX cycle.

Proposal 1: RAN2 choose option-b and values of N is extended to have increased interval between PFs (e.g. T/64, T/128 …T/N).

As legacy UEs can’t know about the R19 changes, the legacy paging occasions (PF+POs) are kept as it is and since their number will dwindle with time, the required paging for such UEs will go down further.

Proposal 2: For legacy UEs the legacy paging mechanism can be continued.

## Clarification on the objective

It is captured in TR38.864 that Paging on a NES cell without SIB or a non-anchor NES cell without SSB and SIB is not supported.

Referring to the WID and RAN1 progress, the scenario that can apply paging enhancement is not clear. Can paging enhancement be applied in the case of NES operation without SIB1/SSB? We think the paging enhancement discussed in this agenda item is limited to the paging enhancement of a cell with legacy SIB1 and SSB transmission. As to the paging enhancement for a cell without SIB1 and/or SSB, it shall be discussed in other agenda items e.g., AI8.5.2, AI 8.5.3.

Proposal 3: RAN2 to clarify the paging enhancement is only applied in a cell with legacy/ regular SIB1 and SSB transmission, i.e., a cell without needing an Anchor cell concept.

The discussion on adaptation of common signal/channel transmissions includes adaptation of SSB, PRACH and paging occasions in time domain.

Referring to RAN1 progress on adaption of SSB, it is supported for a cell to serve both legacy UEs and Rel-19 NES-capable UEs. Therefore, we can consider the legacy UE can camp and access a cell applying Rel-19 NES techniques. Then when paging adaptation is also applied in the cell, the legacy UE shall not be prevented to access the cell via cell barring indication. To avoid the potential conflicts, option 1 (Prevent the access of legacy UE via barring) for paging adaptation can be excluded.

Observation 2: Referring to RAN1 progress, option 1 (Prevent the access of legacy UE via barring) may be excluded to support common channel adaptation e.g. adaptation of SSB, PRACH and paging occasions applied in one cell.

Proposal 4: To avoid the impact on legacy UE, Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs) is selected.

## RACH enhancement

In RAN1#116bis discussion on the RACH adaptation for Rel-19 took place with the following agreements:

Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement For adaptation of PRACH in time-domain, support the following: - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any) - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources - FFS: SSB-RO mapping for the additional PRACH resources

According to RAN1 agreement, the configuration of additional PRACH resources is provided by semi-static signaling. From point view of RAN2 specification, the configuration of additional PRACH resources is separately configured from the legacy RACH configuration in SIB 1, wherein the configuration of additional PRACH resources needs to indicate at least the PRACH configuration index, Number of SSBs per RACH occasion. The other config(s) e.g., msg1-FDM, msg1-FrequencyStart, may be also separately configured from the legacy RACH configuration for flexibility. If the other config(s) are not separately configured, UE default reuses the config in legacy RACH configuration.

Proposal 5: RAN2 to agree the additional RACH occasion for adaptation in time domain is separately configured in RACH configuration from legacy RACH configuration in SIB1.

Proposal 6: RAN2 to agree the separate RACH configuration indicates at least RA config Index and Number of SSBs per RACH occasion, the other config(s) e.g., msg1-FDM, msg1-FrequencyStart, may be also separately configured. If the other config(s) are not separately configured, UE default reuses the config in legacy RACH configuration.

# Conclusion

In summary, we have following observations and proposals for adaptation of common signal/channel transmissions:

Observation#1a: The capacity utilization (i.e., the total number of RRC Paging records that could be sent with the used paging configuration) is much less than 1% in all observed cases.

Observation#1b: Average number of paging records contained in one RRC message (when Paging is sent) is close to 2. The real average accounting all configured paging occasions will be much less.

Observation#1c: Maximum number of records included in the same Paging message is much less than 32 (max is 9 as seen in Brazil and 8 in China).

Proposal 1: RAN2 choose option-b and values of N is extended to have increased interval between PFs (e.g. T/64, T/128 …T/N).

Proposal 2: For legacy UEs the legacy paging mechanism can be continued.

Proposal 3: RAN2 to clarify the paging enhancement is only applied in a cell with legacy/ regular SIB1 and SSB transmission, i.e., a cell without needing an Anchor cell concept.

Observation 2: Referring to RAN1 progress, option 1 (Prevent the access of legacy UE via barring) may be excluded to support common channel adaptation e.g. adaptation of SSB, PRACH and paging occasions applied in one cell.

Proposal 4: To avoid the impact on legacy UE, Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs) is selected.

Proposal 5: RAN2 to agree the additional RACH occasion for adaptation in time domain is separately configured in RACH configuration from legacy RACH configuration in SIB1.

Proposal 6: RAN2 to agree the separate RACH configuration indicates at least RA config Index and Number of SSBs per RACH occasion, the other config(s) e.g., msg1-FDM, msg1-FrequencyStart, may be also separately configured. If the other config(s) are not separately configured, UE default reuses the config in legacy RACH configuration.

# References

[1] RAN2#125bis chairman notes.

[2] RAN2#127 chairman notes.

[3] RAN2#126 chairman notes.

[4] R2-2406890 Paging statistics from field and PRACH adaptation Lenovo

[5] Summary of [109] Oppo