3GPP TSG-RAN WG2 Meeting #127bis	R2-2408327

Hefei, China, Oct 14th – 18th, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussions on measurement event prediction

Agenda Item:	8.3.3

Document for: 	Discussion and Decisions

- Introduction

During the RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements, as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

At the RAN2 #125bis meeting, the following were agreed on evaluating measurement event predictions [2].

Agreements:

- At least measurement event evaluation based on RRM measurement prediction result will be studied.   Direct measurement event prediction are is also allowed.

- Clarifications on what is being as input should be provided with results

- Start with A3 as a baseline.

- Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

This paper will discuss the detailed evaluation methodology issues for the measurement event predictions.

- Cases and sub-cases for the study

  - Cases for measurement event predictions

The evaluation studies about RRM measurement predictions have shown the capability of AI/ML-based RRM measurement predictions according to the initial evaluation results. For all 3 high-priority cases of the RRM measurement predictions, AI/ML predictors can provide predictions with acceptable accuracy at least for the typical configurations. The predicted RRM measurement predictions can be used to generate the measurement event predictions when the indirect approaches are considered. Given the state of the art, it is worth studying the performance of measurement event predictions based on the outcome of RRM measurement predictions.

During RAN2 discussions, the priorities of the RRM measurement prediction cases are decided according to the potential performance gain and practical deployment scenarios. We see no reason to revisit them since the situation has not changed. Therefore, we have the following proposal.

Proposal 1

- Study the same cases of RRM measurement prediction at least for the indirect measurement event prediction, and reuse the priorities.

According to the evaluation results from the RRM measurement predictions, the direct L3 measurement prediction (i.e., Sub-Case 2/3), especially Sub-Case 3, provides better performance for measurement predictions. The down selection of the sub-cases can be considered to control the workload of the evaluations.

Proposal 2

- Down-select the sub-cases for the study of measurement event predictions.

  - Sub-case 3 (L1 measurements as inputs to predict the L3 measurements) is used.

  - Usage of the predictions for each use case

Similar to the discussions on the usage of the RRM measurement predictions, the usage of the measurement event predictions should be identified before considering the evaluation methodologies. The usages are discussed case-by-case in the following.

- Case 2: FR1-to-FR1 temporal domain prediction Case B

  - Usage (OH reduction): Generate equivalent measurement events with the reduced sample period.

- Case 3: FR1-to-FR1 inter-frequency prediction

  - Usage (OH reduction): Generate equivalent measurement events related to inter-frequency neighboring cells without actual measurement.

- Case 4: FR2-to-FR2 temporal domain prediction Case A

  - Usage (HO performance): Report the predicted events to enhance the HO performance (avoid too-late HO, suggest HO targets, etc). Proactively adjust the HO parameters (e.g., hys, TTT, etc) to enhance the HO performance.

The predictions are used differently for our first and second study goals. For the measurement overhead reduction, measurement event predictions replace the events generated by actual measurements. For the second study goal, the measurement event predictions will significantly change the mobility procedures for the handover performance enhancements. Both usages should be studied based on different cases.

Proposal 3

- Study the potential performance gain for each case.

  - For measurement overhead reduction (Case 2 and Case 3), study if the predicted measurement events significantly impact the handover performance.

  - For handover performance enhancements (Case 4), study how much gains can be obtained by enhanced mobility procedures with the measurement event predictions.

- Discussions on the evaluations

  - Simulation assumptions and models

Because the measurement event predictions can be obtained based on the RRM measurement predictions, it is straightforward to reuse the simulation assumptions for the RRM measurement predictions. Thus, we propose,

Proposal 4

- Reuse the simulation assumptions of RRM measurement prediction for all cases.

As discussed in Section 2, the impact of the measurement event predictions on the system handover performance should be studied. For such purposes, a system-level simulator should be used for the evaluations. For the RRM measurement predictions, whether and how to set up the system-level simulations are discussed in RAN2. Following the same logic, any outcome from the RRM measurement prediction session about the system-level simulations should be reused for the measurement event predictions.

Proposal 5

- If an HO model is introduced to evaluate the RRM measurement or RLF prediction, it can be reused for the simulation of measurement event prediction.

  - A unified HO model should be used for all system-level AI/ML Mobility SI evaluations.

  - KPIs

In all cases, the accuracy of the predicted measurement events should be investigated. The following intermediate KPIs can be considered for this purpose. For the overhead reduction, if the predicted measurement events have ignorable differences from the ground-truth events after evaluating the missed and false prediction ratio and time accuracy, the study can conclude that the overhead reduction is achieved. For the HO performance enhancements, the following system-level simulations are still necessary to identify the potential gains further.

We have the following proposals on the intermediate KPIs.

Proposal 6

- Use intermediate KPIs to evaluate the accuracy of the measurement event predictions.

- Intermediate KPIs include the miss prediction ratio, false prediction ratio, and average time accuracy based on the following definitions.

  - Correct prediction: The measurement event is predicted within a given time gap relative to the ground-truth event.

  - Missed prediction: The measurement event is not predicted within a given time gap relative to the ground-truth event.

  - False prediction: The ground-truth measurement event does not happen within a given time gap relative to the predictions.

  - Time accuracy: For the correct predictions, the time difference between the ground-truth measurement event and its prediction.

If system-level simulations are adopted, typical handover KPIs, such as HOF, ping-pong, and time-of-stay, should be studied. The exact KPIs should be used across all 3 AI/ML-based mobility study topics.

Proposal 7

- Reuse the KPIs for system-level simulations if defined in other AI/ML Mobility sessions.

- Conclusion

In this contribution, we have the following observations and proposals,

Proposal 1

- Study the same cases of RRM measurement prediction at least for the indirect measurement event prediction, and reuse the priorities.

Proposal 2

- Down-select the sub-cases for the study of measurement event predictions.

  - Sub-case 3 (L1 measurements as inputs to predict the L3 measurements) is used.

Proposal 3

- Study the potential performance gain for each case.

  - For measurement overhead reduction (Case 2 and Case 3), study if the predicted measurement events significantly impact the handover performance.

  - For handover performance enhancements (Case 4), study how much gains can be obtained by enhanced mobility procedures with the measurement event predictions.

Proposal 4

- Reuse the simulation assumptions of RRM measurement prediction for all cases.

Proposal 5

- If an HO model is introduced to evaluate the RRM measurement or RLF prediction, it can be reused for the simulation of measurement event prediction.

  - A unified HO model should be used for all system-level AI/ML Mobility SI evaluations.

Proposal 6

- Use intermediate KPIs to evaluate the accuracy of the measurement event predictions.

- Intermediate KPIs include the miss prediction ratio, false prediction ratio, and average time accuracy based on the following definitions.

  - Correct prediction: The measurement event is predicted within a given time gap relative to the ground-truth event.

  - Missed prediction: The measurement event is not predicted within a given time gap relative to the ground-truth event.

  - False prediction: The ground-truth measurement event does not happen within a given time gap relative to the predictions.

  - Time accuracy: For the correct predictions, the time difference between the ground-truth measurement event and its prediction.

Proposal 7

- Reuse the KPIs for system-level simulations if defined in other AI/ML Mobility sessions.

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #125bis Chair Notes”, Apr., 2024.