3GPP TSG-RAN WG2 Meeting #127bis                                                                  R2-2408331

Hefei, China, 14th – 18th October, 2024

Source: 		  ZTE Corporation, Sanechips

Title:                   Further consideration on the details of paging for AIoT

Agenda item:     8.2.3

Document for:   Discussion and Decision

- Introduction

In RAN#102, a study item on solutions for Ambient IoT (Internet of Things) in NR has been agreed and in RAN#103, the SID is further refined in [1].

In RAN2#125bis meeting, RAN2 has had initial discussion for Ambient IoT. The following agreements related to general aspect and protocol stacks have been achieved:

Agreements (General aspect) - Unless explicitly stated all agreements apply to all device types and for both topologies. - From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2. - RAN2 will support two use cases, “inventory” and “command”. The definition, detailed wording is FFS - Baseline procedure: Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS Step C: The device may perform the data communication with the reader as needed,: Details FFS - We will study the support of both “inventory” and “command” in the same procedure. - FFS if Initial Trigger Message can also include “command”. - RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input. Agreements (Control Plane protocol) - RRC connection management is not supported. FFS how the resource configuration is provided to the device (if needed based on RAN1 progress) - RRM L3 measurement reporting is not supported by Ambient IoT devices. - RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding. - Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals. - RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC Agreements (User Plane protocol) - SDAP is not supported for UP protocol stack. - PDCP layer is not needed. FFS how to handle AS security (if needed pending SA3 dicsussion) and any other really needed functionalities. - RLC layer is not needed. FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size). RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed. - No HARQ and RLC AM - FFS about the level of visibility required by the reader and what information is necessary for AS layer operations. - RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL). FFS how to handle the general QoS requirements from SA2 Agreements (Paging) - Legacy paging message for device will not be supported. - Legacy paging occasion and legacy DRX for the device is not supported. This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well). - RAN2 assumes that the device will not support tracking/RAN area update procedure. - For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message. FFS pending the details from SA2

In RAN2#126 meeting, RAN2 has further achieved the following agreements:

Agreements (Stage 2 General aspects) - As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. - From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command) Agreements (Functionality aspects) - Multiple “AIoT logical channels” for upper layer data are not supported. FFS if AIoT logical channel concept is used depending on final modeling issue. - legacy NR BSR/SR is not needed for A-IoT communication. - FFS whether further indication of device message size/status is needed - AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed) Agreements (A-IoT Paging) - RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. - AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. - From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details. Agreements (A-IoT Random Access) Agreements on “4 step” RA - A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information - A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements - A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request) - The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose. - “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. RAN2 will not use “Msg4” term for further discussion of the random access. Agreements on 2 step CB RA - A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information - A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed - From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

In RAN2#127 meeting, RAN2 has further achieved the following agreements which may be related to the following discussion on Paging-like function:

Agreements (Functionality aspects) - Send an LS to RAN1 regarding TBS size. Ask SA2 on the max and typical upper layer packet size. - RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages. FFS details on signalling - RAN2 will study the need and use of a simple message “size” reporting to the reader - At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional - RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data. Agreements (A-IoT Paging) - Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices. => wait for further RAN1 progress on indication of the start of access occasion. Agreements (A-IoT Random Access) - For 3-step CBRA Support fixed random ID size is 16 bit. The ID is randomly generated. - Failure/success indication of D2R will be studied. FFS if it would be implicit or explicit and for which use case it is needed. FFS whether it is applied only to some cases. - for 2step CBRA, RAN2 design will support msg2. Whether it is needed it is up to the reader. FFS when it is needed. For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2. FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices. - In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2). FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes. - FFS if reader assigns the AS ID for scheduling purposes

In this contribution, based on the progress in both RAN1 and RAN2, we will further discuss the details of paging-like procedure in AIoT air interface and give our proposals.

- Discussion

2.1 Discussion on Device ID

- 2.1.1 Reference from SA2 study: Structured device ID and mask scheme

Generally, the device ID information needs to be provided within the DL paging message to facilitate the device to compare its stored device ID with the provided device ID and determine whether it needs to respond the AIoT service request in AIoT air interface. Please note, here device ID information is just a general saying.

SA2 has agreed that an ambient IoT Device Identifier can be assigned by an operator or by a third party. Some options for device ID structure design mentioned in SA2 study [4] are cited as below:

- EPC-like Device ID: In legacy RFID, Electronic Product Code (EPC) is used to identify the tags which is defined by GS1 in the EPC Tag Data Standard. EPC can be seen as a means for unique identification of physical objects independently of any technology for storing and transmitting such EPC information. So EPCs can also be used independently of existing RFID tags. As shown in the Table A-3 in [4], an EPC starts with a header value that indicates the EPC scheme followed by the bitwise representation of a particular encoding scheme. Some EPC Binary Encoding schemes are listed as examples in the following Table:

| EPC scheme | Header value | Coding scheme specific bits |
| --- | --- | --- |
| Serialised Global Trade Item Number (SGTIN) | 0011 0000 | SGTIN-96 as defined in clause 14.6.1.1 of [10] EPC Header; Filter; Partition; GS1 Company Prefix; Indicator / Item Reference; Serial. 8 bits; 3 bits; 3 bits; 20-40 bits; 24-4 bits; 38 bits. ………. |
| Serial Shipping Container Code (SSCC) | 0011 0001 | SSCC-96 as defined in clause 14.6.2.1 of [10] EPC Header; Filter; Partition; GS1 Company Prefix; Extension / Serial Reference; (Reserved). 8 bits; 3 bits; 3 bits; 20-40 bits; 38-18 bits; 24 bits. ………. |

- AIoT Device ID defined by 3GPP. Such device ID may be similar as 5G-SUPI (Subscription Permanent Identifier) type ID and at least contain country/network code and the subscriber number which is used to uniquely identify a user within the related network:

  - Home Network Identifier + Owner Identifier + Instance Identifier  (Solution#2)

  - Operator ID + Group ID + Instance ID  (Solution#10)

It can be seen that the mentioned options for AIoT device ID are generally kind of structured design. It typically consists of three parts: the first part is for identifying the country and/or network/operator, the second part is for identifying the company or the owner or the service group to which the device belongs, and the third part is for identifying each individual device and should be unique at least within the scope defined by the first two parts.

Such structured design for AIoT device ID will facilitate selecting all devices that belong to a specific network, company, or service category, or just simply choosing a subset of devices, by applying simple mask/filter rules.

Observation 1a: It can be seen from SA2 study that, in some AIoT deice ID designs, e.g., EPC-like or 5G-SUPI like, it typically consists of three parts: a part for identifying the country and/or network/operator, a part for identifying the company or the owner or the service group to which the device belongs, and a part for identifying each individual device uniquely at least within the scope defined by the first two parts. Such structured design for AIoT device ID will facilitate selecting a subset of devices by applying simple mask/filter rules.

- 2.1.2 FFS on multiple device IDs in paging message

In previous meeting, RAN2 has agreed the following possible device ID types which can be contained in paging message:

- an ID of a single A-IoT device:

- a group ID that maps to multiple A-IoT devices.

- without an ID, means all devices that can receive the AIoT message

- (FFS) multiple IDs of A-IoT devices

RAN2 has no consensus on the necessity of multiple ID and suggest to confirm the need for this use case based on SA2 discussion. However, in recent SA2 meetings, no related discussion and agreements can be seen.

We think the scenario for using multiple IDs exists from both CN/AF and AIoT air interface perspectives. For CN/AF, it may need to inventory or read/write a part of or a few specific devices (e.g., not all devices in a certain category/group) based on the business requirement. For AIoT air interface, the reader may need to inventory several remaining devices that have not yet been inventoried after multiple paging rounds and with discontinuous device IDs.

Proposal 1: From RAN2 perspective, multiple IDs can be contained in paging message. The multiple IDs can be used to indicate a part of or a few specific devices (e.g., not all devices in a certain category/group) or multiple devices with discontinuous device IDs.

- 2.1.3 How to indicate group ID or multiple IDs

Both multiple IDs and a group ID can be used to indicate multiple A-IoT devices for AIoT operation. To include a group ID in paging message can be seen as an explicit way to indicate multiple devices if these devices previously have been gathered into a same group and assigned with a group ID and can trigger all the devices within this group to response the paging.

With reference to the Observation 1a, it’s also possible to implicitly indicate a group ID, e.g., by applying a “mask/filter” on structured device ID. For example, the “mask/filter” may have a certain binary pattern for a range of bits [m, n] on the second part of structured device ID, e.g., “company/owner/group” part. A device can perform bitwise AND operation between its device ID and the mask to check whether the bits [m, n] in its own ID are aligned with the binary pattern in the “mask/filter”, if yes, the device can consider being selected for the operation on the group which is represented by the mask pattern.

Observation 1b: For indicating group ID in service request message and further paging message, a group ID can be explicitly included or can be implicitly indicated by applying a “mask/filter” on the “company/owner/group” part of structured device ID.

Also with reference to observation 1a and observation 1b, one way to indicate multiple device IDs is also to apply a “mask/filter” on structured device ID, e.g., on the “instance ID” or “serial number” part. For example, if the lowest “n” bits of the device ID are masked (the mask rule may be setting the lowest “n” bits to 1 and all other bits to 0, and the device performs a bitwise AND operation between its own ID and the mask), then maximum 2n devices can be selected. When serial number part of such structured device IDs of the devices belonging to a same owner are continuous, this way would be signaling-efficient.

There is another way to explicitly indicate multiple IDs, e.g., to include a list of device IDs in the DL service request or paging message, where each entry corresponds to the ID of an individual device. This way is generally used to indicate multiple devices with discrete/discontinuous ID numbers. Just due to the discrete ID numbers, it’s not easy to indicate these devices with a way of “mask/filter”. Given that the device ID might be a long bit string, this way is usually applied when a small number of devices need to be indicated.

Observation 1c: For indicating multiple IDs in service request message and further paging message, a list of device IDs can be explicitly included or can be implicitly indicated by applying a “mask/filter” on the “instance ID” part of structured device ID.

2.2 Detailed Paging scenarios and procedures

In this section, we will discuss several paging scenarios and the related issues/potential solutions.

- 2.2.1 Determination of the start point of D2R access and Q-algorithm

For a simple and very basic scenario that service request from CN/AF indicates to operate (e.g., inventory) a certain number of devices, the reader needs to trigger R2D transmission including a paging message and this paging message would trigger the selected/indicated devices to access network and give response (e.g., to send back its device ID) to the CN/AF.

According to the RAN1 agreements, the R2D transmission containing paging message would at least provide device IDs information, the RA resources and some physical layer modulation/demodulation parameters for the device to perform the D2R transmission. For each information, it’s still FFS whether higher-layer signaling and/or L1 R2D control signaling is used:

RAN21#118: Agreement For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions FFS: other information. FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used. Agreement For R2D reception, the following information potentially can be explicitly/implicitly indicated to the device via PRDCH: - ID associated with device(s) intended for the reception of R2D, potentially including all devices (if supported) - FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

Furthermore, both RAN1 and RAN2 have agreed the slotted ALOHA would be taken as baseline scheme for device to respond the paging and randomly access the network. In last meeting, RAN2 has had some discussion about how a device determine transmission/access occasion to perform the D2R transmission. Per our understanding, the access occasion is more related to the RA resource definition. The format/structure of the RA resources and how to configure them are mainly decided by RAN1. From RAN2 perspective, especially for MAC spec, the main task is access control, e.g., how to control or alleviate the potential access collision caused by that multiple devices are triggered simultaneously to access the network.

To avoid overlapping with the discussion in RAN1, in the following discussion, we will use the saying “determination of the start point of D2R access” instead of “determination of access occasion” as “access occasion” sometimes specifically refers to time-domain RA resource that is still under RAN1 discussion. As for the relationship between our discussion and the discussion in RAN2 RA topic, it lies that after the device determine the start point of D2R access, it still can choose a specific RA resource according to the results of the RAN2 RA topic discussion.

In RA topic discussion, the scheme that has been mentioned more is to randomly select one from the available RA resources set. Per our understanding, since multiple devices have already been discretize when determining the start point of D2R access, another randomization process may be not needed. So another straightforward way is to let device just select the earliest access occasion after the start point of D2R access.

Proposal 2a: For basic paging procedure, RAN2 can mainly discuss how to facilitate device to determine the start point of D2R access with the aim of reducing collision. And RAN2 can assume that after determining the start point of D2R access, the device can choose a RA resource from the available RA resources set by some ways, which depends on the discussion in RA topic.

In the following table, we list some related options mentioned in last RAN2 meeting contributions and give some analysis for the main issues of each option:

| Index | Options |  | Comments |
| --- | --- | --- | --- |
| #1 | Many companies have suggested to refer to the Q-selection algorithm which is used in legacy RFID system. See some cited example proposals in below. So we Q-selection algorithm-like scheme should be one option for determining the start point of D2R access. The detailed analysis on Q-selection algorithm and the proposals for AIoT system can be found in the paragraphs following the table. [R2-2406541]Proposal 5: When shared resources are configured for multiple devices, the following parameter needs to be configured in the AIoT paging message: - a parameter Q (like RFID mechanism); or - a parameter N indicating the number of access occasions [R2-2406609]Proposal 2-3: RAN2 is kindly asked to discuss the resource allocation information in the AIoT paging message for two different RA procedures: - Contention-free RA: information indicating a dedicated time resource/random access occasion - Contention-based RA: a parameter Q indicating 2Q time resource/random access occasions in total [R2-2406856]Proposal 7: Like “QueryRep” command in RFID, a transmission/access occasion start message is supported for determination of transmission/access occasion. Proposal 8: The transmission/access occasion start message instructs devices to decrement their slot counters and, if slot count=0 after decrementing, to backscatter an RN16 to the reader immediately. [R2-2406879]Proposal 4: Device determines access occasion based on the Q-like parameter via random selection, or with further consideration of device energy status. [R2-2406960]Proposal 13: For determining the corresponding D2R transmission/access occasion after A-IoT paging, RAN2 assume there is R2D clock signal, which is similar to QueryRep in RFID until more progress in RAN1. [R2-2407533]Proposal 1: Ambient IoT paging message may include the total number of access occasions for slotted-ALOHA based random access procedure. Proposal 3: Reader may transmit access occasion announcement message. |  |  |
| #2 | RAN1 options [Agreements from RAN1#117]: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max] - Option 2: Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min | Per our understanding, RAN1 option1 just provides a rough time range or sets the latest time by which devices should respond. Meanwhile option1 gives no solution on how to differentiate devices to send their responses within this time range to minimize collision. Given that a certain number of devices will receive paging message simultaneously, without any control, it’s possible that these devices trigger responses at the same time within the time range. RAN1 option2 give no details but our general concern is that as a system without stringent time synchronization (an asynchronous system), AIoT device may not be able to handle the timing point TR2D given by the network/reader. |  |
| #3 | [R2-2406391]With reference to the reception of R2D message, AIoT device can determine its further D2R transmission by the reference time/frequency point with the time/frequency shift from the reference. | This option gives an idea that the start point of D2R transmission can be a point that is offset from end of R2D reception by an offset value: - If this offset is relative to the frequency domain resources of receiving R2D, and if different devices need to have large varied offsets, then the devices would need to support a sufficiently large frequency range, which is too high requirement for AIoT devices. On the contrary, if frequency range supported by the devices is generally small and each device can only offset by a small value, conflicts will remain severe. - If the offset is relative to the time domain resources of receiving R2D, it would be almost infeasible for an asynchronous system as AIoT. |  |
| #4 | [R2-2407508] Proposal 9. For an AIoT paging message that triggers a random access procedure, the reader should schedule multiple D2R access occasions. An intended device generates an RN within the range of the RNs as indicated in the AIoT paging message, and then uses the RN to determine whether and in which D2R access occasion it can send its response. Proposal 10. RAN2 can study both slot-counting (Q algorithm-like) and number-polling (instantaneous number-polling by the reader and number-comparison between the polled number(s) and the RN generated by the device for the device to determine whether it should send a response or not) approaches for the random access procedure. | The slot-counting can be discussed below within Q algorithm-like scheme. For number-polling scheme, per our undemanding, the devices need to determine whether it can start to access based on the comparison results. But it’s unclear what means passing the comparison? Is it equal to or greater than, or less than? Generally, using an equality comparison may be common and simple. However, considering the possible large number of devices, there may be many devices that result in an equal comparison, leading to severe conflicts. If using greater than or less than as the comparison result, it would be difficult to evaluate the probability of conflicts, and it might also be complicated for the reader to set suitable polling number. Overall, this scheme has similarities to Q algorithm-like scheme (e.g., generating a RN vs. generating a random number between 0 and 2Q-1), but it is more complicated to handle. |  |
| #5 | [R2-2406710] Proposal 3: A paging message for contention-based access includes parameters that map the device ID to one or more corresponding resources (e.g., access occasions) in a deterministic way. | The proponent company think the RFID procedure has some drawbacks in that the random number selection is not deterministic, and it can result in wasted occasions (E.g. if no device selects a specific random number). Per our understanding, using device ID to determine the device’s specific access occasion (e.g., similar calculation as the PO calculation by taking the modulus of “instance ID” part of the device ID) may naturally achieve certain degree of evenly spreading the devices to different start points and may have better anti-collision performance. So this scheme can be further studied. |  |
| #6 | Some companies have proposed the scheme that total number of access occasions can be indicated to device by the reader: [R2-2406718]Proposal 3: RAN2 to agree on including a parameter in the AIOT paging message to randomize the access occasions for different AIOT devices and indicate the total number of access occasions in the contention-based access procedures. [R2-2406747]Proposal 3: For time domain access resource, the total number of access occasions within one access round is indicated to device by the reader. [R2-2407283]Proposal 1: A-IoT paging message should include the number of access occasions for the subsequent RA procedure, so that A-IoT devices are aware of the number of access occasions. | It’s unclear to us whether provision of total number of access occasions is related to Q-algorithm like scheme or related to RA resources configuration. If it’s latter, we think the necessity of such information needs to be decided by RAN1. For Q-algorithm like scheme, it may also need further clarification on the necessity of such information. Per our understanding, it may not be needed. The Q value needs to be indicated by reader but it’s mainly used for setting slot counter. The device can select the earliest RA occasion after determining the start point of access. So it seems that the device doesn’t need to care the total number of occasions available. |  |

According to the above analysis, we think at least can be further discussed:

Proposal 2b: RAN2 can further discuss the following schemes for determining the start point of D2R access with the aim of reducing collision:

- Alt1-1: Anti-collision scheme similar as Q-algorithm in RFID.

- Alt1-2: A deterministic way for determining the start point of D2R access and/or access occasion based on device ID.

In the following, we focus the discussion on Q-algorithm like scheme (which has been widely noted by companies) and how to define it for AIoT system. In RFID, Q-selection algorithm is generally used for anti-collision through time domain distribution. Shortly to say, even a device is selected/filtered out via matching with the device ID information provided in paging, it doesn’t means the device can access the reader immediately after receiving the paging message. The main steps of a Q-algorithm can be as below:

- A Q value is broadcast to all the devices by the reader;

- For a certain device, it generates a random number between 0~2Q -1 as its response time slot and loads this random slot number into a slot counter in its side. This random slot number will be further reduced when receiving kind of trigger till it equals to 0.

- The device with the slot counter equal to 0 can trigger random access. So this slot counter can be seen as a length of delay before the device can be allowed to trigger random access. If the device successfully access the reader and responses with its device ID, the device can think that it has been successfully inventoried and can exit the operation. It may be possible that several devices have the slot counter equal to 0 at a certain moment, then all of them can trigger random access at the same time, and this collision can be further addressed by other contention resolution methods, e.g., during the random access procedure.

- The reader can send another DL message to notify the devices to decrease the slot counter. Each time a device receives this DL message, its slot counter is reduced by 1. By this way, for each time, zero, one or multiple devices can reduce their slot counters to 0 and have chance to access the reader.

- Furthermore, if the reader detects some collision in which multiple devices are responding at the same time, it can broadcast a new Q value, and upon receiving this new Q value, all devices will regenerate the random slot number for the slot counter. Usually, in this case, a larger Q value will be provided so that the device can generate a random slot number in a larger value range. On the contrary, it’s also possible that at some point the reader doesn't receive any response from the device. That means some time slots are unnecessary idle, the reader can also provide a new Q value, e.g., with a smaller value, to trigger the devices regenerate random slot number for slot counter that are more tightly distributed.

Per our understanding, in such Q-algorithm, the device does not need to be aware of the exact time position but only needs to decrease a counter according the indication from reader. When the counter is reduced to 0, no matter what the current time it is, the device can trigger access. This way is very suitable to such device as AIoT that cannot have strict time synchronization with the reader.

From RAN2 perspective, the possible impacts on signalling procedure of introducing kind of anti-collision scheme similar as Q-algorithm in RFID, the paging message needs to provide an initial Q value.

Proposal 2c: If anti-collision scheme similar as Q-algorithm in RFID is introduced, an initial Q value needs to be provided via the paging message.

Moreover, the reader further needs to send some subsequent DL messages to reduce the slot counter in device side or even adjust the Q value.

There may be thoughts that the device can reduce the slot counter by itself, e.g., no need of trigger in subsequent DL messages from the reader. We think the issue is that, it’s unclear how long the device will wait before each time reducing the slot counter by itself. And as the device cannot know the current conflict situation, it cannot decide based on the conflict situation whether to wait for a shorter or longer time period. As a result, the effectiveness of conflict avoidance is difficult to predict. On the contrary, as described in the above steps of Q-algorithm, the reader can determine when to trigger a reduction on slot counters and whether to adjust the Q value based on various information.

Observation 2: For anti-collision scheme similar as Q-algorithm in RFID, it may be infeasible to let the device reduce the slot counter by itself. It’s unclear how long the device will wait before each time reducing the slot counter. And as the device cannot know the current conflict situation, it cannot decide based on the conflict situation whether to wait for a shorter or longer time period.

Different from the initial paging message, the DL message from reader for reducing the slot counter has no need to provide device ID information. These subsequent DL messages may be with very simple structure and still can be seen within the current round of paging procedure. The end of current round of paging procedure is generally when all devices in the group are successfully inventoried or the maximum time duration for inventory is reached (the devices which give no response can be marked as failed to inventory).

Proposal 2d: If anti-collision scheme similar as Q-algorithm in RFID is introduced, after the initial paging message, the reader also needs to send subsequent DL messages, e.g., to reduce the slot counter in device side or even adjust the Q value.

- 2.2.2 Multiple paging messages triggered by one service request from CN/AF

In last meeting, RAN2 has agreed that a reader is able to trigger multiple/subsequent AIoT paging messages that are associated with the same service request from CN/AF. During the discussion, some use cases have been mentioned as below:

- Per our understanding, the main usage is to reasonably handle more complicated scenario where a service request from CN/AF aims to a large number of devices. Considering that the resources of the AIoT air interface are very limited relative to the potentially huge number of devices, it would be very difficult or even infeasible for reader to allocate suitable resources or parameter for anti-collision algorithm (e.g., Q value) in one paging message to ensure that all devices can have the opportunities to respond to the service request within a certain time/frequency restraint, then it may be highly possible that only a (small) part of the devices can successfully respond in the first paging round and the others would fail to respond. Furthermore, if the reader does not do any process and just transparently delivers the received responses to the core network, the failed devices can only be known by CN/AF and only the CN/AF can re-initiate the AIoT service request for those failed devices. Such process obviously will be very inefficient and cause waste of resources on end-to-end links. So more efficient way is to allow the reader itself to trigger multiple paging messages for a service request from CN/AF in AIoT air interface.

- Another usage of such process is to handle the scenario that some devices initially turns off or runs out of energy but later turns on, e.g., after the first paging message. Multiple paging messages can provide additional operation opportunities to these devices.

Observation 3a: Allowing the reader to trigger multiple AIoT paging messages that are associated with one service request from CN/AF can achieve the benefits of avoiding waste of resources on end-to-end links caused by re-initiating the AIoT service request from CN/AF and providing additional operation opportunities to those devices that initially turns off or runs out of energy but later turns on, e.g., after the first paging message.

In [5], one way (Alt1) has been mentioned for implementing multiple AIoT paging messages, e.g., to transmit a same AIoT paging message associated with a same service request from CN/AF for multiple times. To avoid the complexity of mixing multiple different issues together, in this section, we do not consider the scenario where several paging messages triggered by different service requests are executed in parallel and that will be discussed in section 2.2.4.

For Alt1, some points and the possible issues have been mentioned as below:

- Each paging message may correspond to a paging round which consists of a series of messages used by the device(s) to respond the service request. Each paging message is targeted at the same batch of devices, but different devices may or may not have opportunities to response the paging in a certain round of paging.

  - Issue#1: In the case that multiple device IDs with discrete/discontinuous ID numbers are in the service request message and further need to be included AIoT air interface messages, it would be a waste to send paging messages with the same device IDs list repeatedly. Considering that some devices may have succeed in previous rounds of paging, if this part of the successful devices is always included in the subsequent paging messages, it is unreasonable. Instead, it makes more sense that all devices that has responded successfully can be eliminated from the paging message of the next round of paging.

- RAN2 has agree to study how to avoid duplicated response from device. A straightforward way is that the devices that have already successfully responded to the request in one round should avoid responding again in another round.

  - Issue#2: The main requirement for such process is that the device need some way to clearly ensure that its response (especially the Msg3) has been successfully received by the reader. Considering that the response may fail to be transmitted to the reader, e.g., due to bad radio condition, only when the device can ensure its response triggered by the first paging message has been successfully received, the device can skip the second and subsequent paging messages. Please note, as Msg2 can be kind of ACK to Msg1, the failure of Msg1 is not in the scope of discussion. In other word, after receiving a paging, if the device just sends Msg1 but receive no Msg2, the device can clearly know the response is failed and it needs to response another paging message. In previous meeting, RAN2 has already had some discussion on the necessity of failure/success indication for Msg3 transmission, but no agreement can be achieved. In the latest email discussion “[POST127][033][AIoT] Random Access”, the discussion is ongoing but the companies’ views are still diverse. In a summary, without an ACK/NACK to the response in Msg3, a certain device cannot determine whether it needs to re-response to the subsequent paging messages associated with a same CN/AF service request since it is always within the list of the targeted devices in all the subsequent paging messages, no matter whether the reader has received the response from this device or not. This issue will make the Alt1 almost infeasible. In other word, Alt1 needs an ACK/NACK scheme for the paging response (e.g., D2R transmission for Msg3).

Observation 3b: For Alt1, transmitting a same AIoT paging message associated with the same service request from CN/AF for multiple times, the following issues can be observed:

- Issue#1: The devices that have already successfully responded to the request in one paging round should avoid responding again in another round. However, without an ACK/NACK scheme for the paging response (e.g., D2R transmission for Msg3), a device cannot determine whether it needs to re-respond to the subsequent paging messages since it’s always within the list of the targeted devices in all the subsequent paging messages, no matter whether the reader has received the response of this device. This issue will make the Alt1 almost infeasible. In other word, Alt1 needs an ACK/NACK scheme for the paging response (e.g., D2R transmission for Msg3).

- Issue#2: In the case that multiple device IDs with discrete/discontinuous ID numbers need to be included in the paging message, it would be a waste to send paging messages with the same device IDs list repeatedly, e.g., without eliminating the devices that have successfully responded from the subsequent paging messages.

For Alt1, if an ACK/NACK scheme can be introduced, the device that has received ACK for Msg3 transmission can internally mark its status as “has been inventoried”. And for the device in such status, it can ignore the subsequent paging messages. Meanwhile, the device that determines the failure of Msg3 transmission according to the NACK scheme can internally mark its status as “has not been inventoried”. For the device in such status, it still needs to response to the subsequent paging messages.

For Alt1, there is an assumption that same device IDs information will be included in these multiple paging messages. However, if an ACK/NACK scheme can be introduced for Alt1, since the device that with status of “has not been inventoried” (e.g., has sent Msg3 but received no ACK) anyway needs to respond the subsequent paging messages, we no longer see the need to provide same device IDs information in the subsequent paging messages. Therefore, the subsequent paging messages can be simplified to just a trigger message without any device IDs information. Any device with status of “has not been inventoried” needs to do response to such simplified paging messages.

Furthermore, there may be a possible rare case that, the reader may want to inventory all the devices even in the subsequent paging message. In such case, including no device IDs information is intentional and means all devices (even the devices that have successfully responded in previous paging round) receiving this paging message are selected. In order to differentiate this case, a delta paging indication (e.g., simple1 bit indication) can be introduced to the subsequent paging messages. For subsequent paging messages including no device ID information but with a delta paging indication, only the devices with status of “has not been inventoried” need to respond. Such option can be seen as Alt2 that can address the Issue#2 of initial Alt1.

For Alt1 and Alt2, an important requirement is to support ACK/NACK scheme to Msg3 transmission. However, if ACK/NACK scheme cannot be supported, there is another way (Alt3) to implement multiple paging messages. That is, it’s no need to include same device IDs information in the subsequent paging messages and the number of entries in the device IDs list gradually decreases. The main point is that the device IDs information in subsequent paging messages will not include the devices that have already responded to the initial or previous paging messages.

This option primarily relies on the reader's judgment of successful Msg3 transmission. In each subsequent paging message, the reader will only include devices whose Msg3 transmission have NOT been successfully received by the reader in the previous paging rounds. Thus for device side, there is no requirement for a device to clearly know whether its Msg3 transmission was successful or not. As long as the device’s ID is included in the subsequent paging messages, the device can interpret this as an unsuccessful Msg3 transmission and can re-respond. Therefore, this solution can operate without an ACK/NACK mechanism and also be better for avoiding issues arising from inconsistency between the reader and the devices (e.g., the reader has not received Msg3 transmission but the device thinks it has succeed to send Msg3 and no longer to respond in certain time period) .

Alt3 can also be seen as a delta paging with that the number of entries in the device IDs list in the subsequent paging messages gradually decreases. For Alt3, a main requirement is that the device ID(s) or device ID(s) list of the target AIoT device(s) needs to be visible to the reader in both DL and UL. The gNB reader can directly match the received AIoT device ID(s) during random access procedure, e.g., via Msg3, with the stored device ID (list) that received from CN/AF, and determine which devices have successfully responded. Based on this, the reader can efficiently trigger subsequent AIoT paging messages by including only the failed AIoT devices

Based on the above discussion, the following proposal is given:

Proposal 3: For more efficient handling a CN/AF service request, it's suggested to discuss the follow options:

- Alt1: Sending multiple identical paging messages and introducing ACK/NACK scheme to Msg3 transmission.

  - The device can mark its status as “has been inventoried” or “has not been inventoried” based on the ACK or NACK of Msg3 transmission. Only the devices with status of “has not been inventoried” need to do response to the subsequent paging messages.

- Alt2: Sending multiple paging messages and introducing ACK/NACK scheme to Msg3 transmission. For the multiple paging messages, only the initial paging message includes device ID information and the subsequent paging messages includes no device ID information but with a delta paging indication.

  - The device can mark its status as “has been inventoried” or “has not been inventoried” based on the ACK or NACK of Msg3 transmission. Only the devices with status of “has not been inventoried” need to do response to the subsequent paging messages with a delta paging indication.

- Alt3: Sending multiple paging messages and in each paging message, the device IDs information of only the devices whose Msg3 transmission have NOT been successfully received by the reader in the previous paging rounds would be included. So the number of entries in the device IDs information in the subsequent paging messages would gradually decrease.

  - As long as the ID information of a device is included in the subsequent paging messages, the device can interpret this as an unsuccessful Msg3 transmission and can re-respond.

In previous meeting, there was a view [R2-2406391] that the device can determine the successful Msg3 transmission by kind of implicit way. For example, if a device, e.g., device-A, receives the paging identified by the same identifier (whether an identifier is needed will be discussed later in other section) as it responded to before, it will think that the current paging is another round of the one it has responded and accessed; and there is no need to respond again. However, sending a subsequent identical paging message (e.g., with same device IDs information and/or message identifier) can also happen in the case that the reader has not received the Msg3 transmission from device-A and the reader still expect the response from this device. So if the device-A no longer performs response, this could lead to device-A not being inventoried for a long time. This is an issue caused by the inconsistency between the reader and the device. Based on the above discussion, we think that implicit ACK/NACK way cannot avoid such issue, but explicit ACK/NACK scheme as described in Alt1 and Alt2, or including gradually updated device list as in Alt3, could resolve this issue.

Observation 3c: An implicit way for determining the successful Msg3 transmission, e.g., receiving another paging identified by the same identifier (whether an identifier is needed will be discussed later in other section) same as it responded to before can means previous Msg3 transmission successful, is unreliable and cannot completely avoid inconsistency between the reader and the device. On the contrary, explicit ACK/NACK scheme as described in Alt1 and Alt2, or including gradually updated device list as in Alt3, could resolve inconsistency issue.

No matter for Alt1, Alt2 or Alt3, there can be an expectation that, in subsequent rounds of paging, the number of devices needing to respond will decrease (either this can be achieved by that fewer and fewer devices determine to re-respond based on their failure status (Alt1 or Alt2), or by that the reader selects fewer and fewer devices (Alt3)). Then accordingly, it’s also expected that the RA resources allocation and parameters of the anti-collision algorithms can be reduced in subsequent paging messages. For Alt3, it might be easy for the reader to set reduced RA resources allocation and reduced value for parameters of the anti-collision as we assume the reader can clearly know how many and which devices have given response based on the visible device ID(s) in Msg3. But for Alt1 and Alt2, it may be not be easy for the reader to do this. Some views may assume the reader can have a rough estimation on how many devices have successfully responded. However, considering there may be Msg3 retransmission (whether and how to support retransmission is still under discussion) from a certain device even within a paging round and the reader cannot distinguish which specific UE a Msg3 comes from, the counted number of received Msg3s by the reader may be greater than the actual number of devices that successfully responded. If the reader estimates the remaining devices based on this counted number and further reduces the resources allocated for the next paging round, there will be a risk of insufficient resources and/or increased conflicts. Therefore, it may be safer for the reader to provide resources and parameter values similar as those carried in the initial paging message, but it is clearly wasteful.

Observation 3d: For air interface resource utilization efficiency, it’s expected that the RA resources allocation and the value for parameters of the anti-collision algorithms can be reduced in subsequent paging messages. For Alt3, it might be easy for the reader to do this as we assume the reader can clearly know how many and which devices have given response based on the visible device ID(s) in Msg3. But for Alt1 and Alt2, it may be not be easy. The reader can only count the number of received Msg3 but cannot distinguish which specific device sends a Msg3. Considering there may be Msg3 retransmission, if the reader reduces the resources allocated for the next paging round based on the counted number of Msg3, there will be a risk of insufficient resources and/or increased conflicts.

- 2.2.3 Multiple paging messages triggered by different service requests from CN/AF

In last RAN2 meeting, when discussing multiple paging messages triggered by one service request from CN/AF (Scenario-1), another scenario was also mentioned where several paging messages can be triggered by different service requests are executed in parallel (Scenario-2).

Generally, we understand if the reader receives another service request from CN/AF before it finishes handling a previous one, it’s possible for the reader to handle this by its implementation, e.g., the reader can serially trigger multiple paging messages for different service requests from CN/AF. But considering that the devices targeted by these two service requests may not be entirely the same, serial processing on these two requests might introduce additional delay for the devices that are targeted by the second request but not the first. Therefore, we understand that allowing the parallel triggering of multiple paging messages for different service requests from CN/AF may be a better choice.

Observation 4a: Considering that the devices targeted by two different service requests from CN/AF may not be entirely the same, serial processing on these two service requests might introduce additional delay for the devices that are targeted by the second request but not the first. So allowing the parallel triggering of multiple paging messages for different service requests may be a better choice.

According to the discussion in section 2.2.2, for Scenario-1, the Alt1 and Alt2 have assumption that the device needs to know that its response has been successfully received by the reader. If the device confirms the Msg3 transmission has been received, the device does not need to response the subsequent paging messages triggered by a same service request. However, if another paging message triggered by different service request from CN/AF can also reach to this device at the same time, the device may become confused on whether it needs to respond another paging message. So in order that the device can differentiate these two paging messages, it’s proposed in [5] that reader can provide an ID in the paging message to associate it with a certain CN request (similar ID is also mentioned in some other contributions and called as session ID or transaction ID or Paging ID. For the convenience of following discussion, we temporarily call it as the session ID). Shortly to say, if a paging message indicates a session ID different from the one indicated in a previous paging message, the device would understand these two paging messages are associated with different requests from the CN and needs to response to both of them. But if the session IDs are same, the device can skip the second paging message if it knows that it has successfully responded to the previous one. We understand such scheme (Alt1) is feasible.

If session ID can be introduced in paging message, there may be further discussion on how to define the value range of the session ID to solve the possible loopback issue, and additionally, the scope in which the uniqueness of the session ID needs to be guaranteed should be discussed.

Observation 4b:  If the device is allowed to skip another paging message when it knows that it has successfully responded to a previous one triggered by same CN/AF service request, a session ID may need to be introduced to facilitate the device to identify a paging triggered by a different CN/AF service request. Fur such session ID, possible loopback issue or the scope in which the session ID can have uniqueness may need to be considered.

Furthermore, if Alt3 is applied to Scenario-1, it is not mandatory for a device to know whether its Msg3 has been successfully received. Also a device does not need to determine whether to skip the subsequent paging messages based on whether it has successfully responded to a previous paging message. The main principle for a device deciding whether to respond to a paging message is whether it is the target device of this paging. Therefore, the device does not need to specifically differentiate whether a paging message is related to the same request from CN/AF or not.

In a summary, if Alt3 is used, regardless of whether a subsequent paging message is triggered by the same or different service request from CN/AF, the device follows the same handling principle for above mentioned Scenario-1 and Scenario-2 —namely, responding as long as the device determines that the device itself is the target by checking the device IDs information in the paging message. Thus, there is no need to introduce additional session ID information in the paging message. We see this scheme as Alt2 and it can also be further studied.

Proposal 4: For handling the case that the reader receives another service request from CN/AF before it finishes handling a previous one, RAN2 can discuss the following schemes:

- Alt1: The reader is allowed to trigger multiple paging messages in parallel and each of them can include an ID, e.g., session ID or transaction ID, to associate with the service request from CN/AF. The device needs to response the paging message as long as it determines that this paging message has not been responded yet by checking the session ID in the paging message.

- Alt2: The reader is allowed to trigger multiple paging messages in parallel and each of them contains the device IDs information received from the corresponding service request from CN/AF. The device needs to response the paging message as long as it determines that it is the target device by checking the device IDs information in the paging message.

- 2.2.4 Further grouping devices and applying RAN mask rule

As mentioned in section 2.2.2, it’s possible for the reader to receive a service request from CN/AF which aims to a large number of devices. Also with reference to the discussion in section 2.1, kind of upper layer filter/mask scheme can be applied to structured device ID to achieve multiple IDs for AIoT operation, e.g., inventory. The upper layer filter/mask is applied generally according to a business requirement (e.g., to filter the devices belonging to a certain company, owner or business type) or it may not even be applied (e.g., upper level just want to do an inventory for all the devices). Please note, when the AF/CN triggers service request for AIoT operation, it generally doesn’t need to consider the resources or capacity etc. factors in AIoT air interface.

According to our analysis in previous meeting [R2-2407148], the AIoT inventory request from upper layer, even for a single business, may reach the scale of hundreds of thousands of devices for one time inventory. There may also be stringent time restriction for the entire inventory operation, e.g., hour level or even shorted. It’s easy to understand such service request may cause heavy load and possible heavy collision during random access procedure at the AIoT air interface.

Observation 5a: Although kind of upper layer filter/mask scheme may be supported, it’s still possible for BS reader to receive service requests, e.g., inventory for a very large number of devices which will further cause heavy load and possible heavy collision for the following random access procedure at the air interface.

Even in section 2.2.2, an option of triggering multiple paging messages have been mentioned, it may be not enough to handle such service request with very large number of target devices.

Therefore, we believe that to apply addition mask/filter in the RAN level is still needed with intention to reasonably split a CN/AF service request into several air interface paging for a smaller group containing fewer devices, which can be helpful to reduce the collision probability within each paging and improves the overall efficiency at AIoT air interface. With reference to Observation 1c in section 2.1.3, if discrete device IDs are contained in the service request from CN/AF, the reader can additionally grouping them into several smaller groups. Or if some upper layer mask is provided to implicitly indicate multiple device IDs, the reader can apply additional “mask/filter”, e.g., on the “instance ID” part of structured device ID.

Proposal 5a: For a CN/AF service request with very large number of target devices, it is also necessary for the RAN to apply the additional mask/filter, e.g., on the “instance ID” part of structured device ID, with intention to reasonably split such service request into several air interface paging messages and each of them will address a smaller number of devices.

It’s easy to understand, to facilitate the reader to apply additional mask rules, it should at least know the structure of the device ID and the mask rules that have already been applied by upper layers.

Proposal 5b: To facilitate the reader to apply additional mask rules when necessary, the reader should at least know the structure of the device ID and the mask rules that have already been applied by upper layers.

- 2.2.5 Device ID visible to the reader

With reference to the discussion for Proposal 3c and Proposal 5b, from RAN2 perspective, we have seen the necessity of making device ID(s) information, at least the structure of the device ID and the mask rules that have already been applied by upper layers, visible to the reader.

In RAN1#118 meeting, there was also much discussion on the R2D scheduling information and D2R scheduling information within corresponding PRDCH transmission. More companies propose that device ID can be included in both R2D scheduling information and D2R scheduling information. RAN1 has achieved the following agreements:

R1-2406846 FL summary#3 on downlink and uplink channel/signal aspects Moderator (Apple) Agreement For the start-indicator part of the R2D time acquisition signal, ON/OFF pattern i.e. high/low voltage transmission is applied - FFS: length/pattern of ON/OFF. - FFS: when TD2R_min is applicable, whether/how the start-indicator part is included in TD2R_min or not. To be discussed in 9.4.2.2. Agreement For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions FFS: other information. FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used. Agreement For R2D reception, the following information potentially can be explicitly/implicitly indicated to the device via PRDCH: - ID associated with device(s) intended for the reception of R2D, potentially including all devices (if supported) - FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

Furthermore, during RAN1 discussion, some companies prefer to provide this device ID information via L1 R2D control signaling or MAC layer PDU, e.g., explicitly in AIoT air interface. One justification has been mentioned in [6] is that an AIoT device can ascertain that the R2D signalling is intended for it by identifying/matching the device ID in the R2D scheduling information as early as possible. If the signaling is not for this AIoT device, the AIoT device can stop receiving and decoding the signaling. This can facilitate early termination of signal/channel detection in device side and reduce device power consumption.

Observation 6: RAN1 has agreed that ID associated with device(s) can be indicated to the device via R2DCH, along with scheduling information for corresponding uplink D2R transmission. There is also view in RAN1 that device ID information can be provided via L1 R2D control signaling or MAC layer PDU, e.g., explicitly in AIoT air interface. One justification is that an AIoT device can ascertain that the R2D signalling is intended for it as early as possible by identifying/matching the device ID in the R2D scheduling information. This can facilitate early termination of signal/channel detection in device side and reduce device power consumption.

Generally, when CN/AF needs to indicate device ID information to the devices for a kind of AIoT operation, e.g., inventory, such device ID information is mainly used by the device to determine whether it’s the target device of the upper layer operation. For this purpose, it would be fine that the device ID information is encapsulated in the upper layer user data and with end-to-end security protection.

However, based on the above discussion, e.g., Proposal 3c, Proposal 5b and Observation 6b, we also recognize the necessity of making the device IDs information visible to the reader or explicitly transmitting in AIoT air interface. The main intention is to enhance the signalling scheduling/resource allocation in AIoT air interface or enable the device to parse the ID earlier.

In previous meeting discussion, some companies think before discussing the encapsulation ways for AIoT information or service data transmitted over the interface, SA3 should first confirm the security requirements for the information and data, including the device ID information. However, till now, SA3 gives no clear agreements on this aspect. According to the above discussion, we see in many scenarios, the device ID information can be achieved by applying “mask/filter” on structured device ID. If the structure of the device ID is defined in 3GPP specification and can be known to the reader, the security risk of mainly including the mask rules directly in the air interface are negligible. Moreover, to explicitly include a group ID or one or a few discrete device IDs directly in the air interface may also have negligible security risk.

In a summary, we suggest that the necessity and solutions for explicitly indicating the device ID information over the AIoT air interface should be studied in SID or later WID stage.

Proposal 6: It’s suggested to study necessity and solutions for explicitly indicating the device ID information directly in DL/UL AIoT air interface. And this is also means the device ID information can be visible to the reader.

- 2.2.6 Summary for the discussed paging scenarios and solutions

In the following table, we give a summary for the scenarios, issues and solutions discussed in section 2.2.2~2.2.4:

| Cases | Main issues discussed | The potential solutions |
| --- | --- | --- |
| CaseA: Paging for a very small number of devices triggered by a service request from CN/AF | How to schedule the multiple devices to access the reader with the aim of reducing collision. | Three options can be further studied: - AltA-1: Anti-collision scheme similar as Q-algorithm in RFID - AltA-2: A deterministic way for determining the start point of D2R access and/or access occasion based on device ID Based on the analysis, AltA-1 is preferred. |
| CaseB: Paging for a large number of devices triggered by a service request from CN/AF | CaseB1: According to the discussion and progress in last meeting, a service request from CN/AF can trigger multiple AIoT paging messages. The details are still FFS | Three options can be further studied: - AltB1-1: Sending multiple identical paging messages + introducing ACK/NACK scheme to Msg3 transmission to avoid repeated responses from device - AltB1-2: Sending multiple paging messages (Initial paging message with device ID info + subsequent paging messages without device ID info but with delta paging indication) + introducing ACK/NACK scheme to Msg3 transmission - AltB1-3: Sending multiple different paging messages and in each paging message, the device IDs information of only the devices whose Msg3 transmission have NOT been successfully received would be included. Based on the analysis, AltB1-3 is preferred. |
|  | CaseB2: If the number of devices is very large, it can consider grouping a large number of devices (e.g., thousands of devices) into several AIoT paging messages, with each message containing only a medium or small number of devices (e.g., hundreds of devices). Here for each resulted AIoT paging message, the solutions for above CaseB1 can also be applied to. Furthermore, the resulted several AIoT paging messages can be equivalent to the CaseC that multiple paging messages associated with different service requests from CN/AF. So the solution for CaseC can also be applied to. | One option can be further studied: - AltB2-1: To make device ID information visible to the reader and in the air interface + The reader can apply additional mask rule on structured device ID Based on the analysis, AltB2-1 is preferred. |
| CaseC: Multiple paging messages triggered by different service requests from CN/AF in parallel | The device needs to response the paging messages triggered by different service requests from CN/AF, and meantime, the device needs to avoid to response the paging messages triggered by a same paging message | Two options can be further studied: - AltC-1: A kind of session ID can be introduced to paging message to represent different service requests from CN/AF + Device only responses the paging message containing different session IDs - AltC-2: Device always responses the paging messages as long as the device determines that it is the target device by checking the device IDs information in the paging message Based on the analysis, AltC-2 is preferred. |

2.3 More appropriate inventory area for the devices

Considering one use case of flower auction mentioned in SA study [2], there may be totally about 2,2Million devices in the auction ground of 1.7km2. Then 5~6 macro base stations may need to be deployed in the area and there may be about 300,000 ~ 500,000 devices within the coverage area of a base station. Obviously, if to inventory all the devices in the entire auction ground in one time, it will mean a very large inventory load.

So an ideal way is that, each inventory can only be performed on the devices within a certain area, e.g., an area naming inventory area. This inventory area may be similar as the tracking area in legacy paging scheme, but the difference is that, as RAN2 has agreed, no tracking/RAN area update procedure will be supported by the devices, then it may be more possible to rely on network node to setup and maintain such inventory area, e.g., based on the inventory response from AIoT devices during inventory procedure.

Proposal 7a: A concept of inventory area can be introduced with intention that an inventory is mainly performed for the devices within a certain inventory area.

For Topology 1, since AIoT device communicates with BS reader directly, it can be assumed that an AIoT inventory area in TP1 may include at least one BS reader. And the association between devices and a BS reader can be set up by network node, e.g., BS reader or CN node. From RAN perspective, one base station (BS reader) can be associated with multiple AIoT devices.

Proposal 7b: It can be assumed that an AIoT inventory area for TP1 may include one or more BS readers. The association between devices and a BS reader can be set up by network node, e.g., BS reader or CN node. From RAN perspective, one base station (BS reader) can be associated with multiple AIoT devices.

For Topology 2, AIoT device communicates with intermediate node (e.g. NR UE), and the intermediate node will further communicates with the base station. Therefore, it can be assumed that an AIoT inventory area in TP2 may include at least one UE readers. And the UE should associate with a paging area, which should also be known by the base station. The association between a certain device with the UE reader and further with the attached base station can also be set up by network node, e.g., base station or CN node. From RAN perspective, one base station can be associated with multiple UE readers and each of UE reader can be associated with multiple AIoT devices.

Proposal 7c: It can be assumed that an AIoT inventory area for TP2 may include one or more UE readers. The association between a certain device with the UE reader and further with the attached base station can also be set up by network node, e.g., base station or CN node. From RAN perspective, one base station can be associated with multiple UE readers and each of UE reader can be associated with multiple AIoT devices.

The above process for setting up association between a certain device with the reader (and may further with the BS for TP2) can be done during inventory procedure. Taking TP2 as example, the UE reader node that receives the inventory response from AIoT devices, can further deliver the device ID information to the next level network node, e.g., the base station, and attach its own node information (UE id or UE reader id), so that the base station node can record the association between each device and the UE reader. Please note, here the base station node may receive Uu responses from multiple UE readers which are in its coverage and each Uu response may include responses from multiple AIoT devices. Similarly, the base station node can also deliver the association between AIoT devices and the UE readers, and additionally its own node information (base station id) to the AIoTF node in core network in order that the core network node can record the association among the AIoT devices, the UE readers and the base station.

Proposal 7d: The setting up association between a certain device with the reader (and may further with the attached base station) can be done during inventory procedure.

Furthermore, it needs to discuss whether the device is capable to record the AIoT inventory area information, e.g., the BS reader or UE reader where it was successfully inventoried in last time. If yes, the device can only response the inventory from the BS reader or UE reader in the AIoT inventory area. This would be very helpful for reducing the unnecessary response for air interface inventory in device side.

In this way, a reader ID may need to be provided in the paging message of the inventory procedure.

Proposal 7e: RAN2 can further discuss whether the device is capable to record the BS reader or UE reader where it was successfully inventoried in last time. If yes, the reader ID may need to be provided to the device in the paging message of inventory procedure.

Another related issue has been mentioned in previous meeting, e.g., whether AIoT devices need to differentiate paging messages triggered by same or different readers. A corresponding case has been mentioned that “one distinct AIoT service request may be sent to multiple AIoT readers”. We see no clear justification for such case and it may also cause heavy load for the interface between BS and device. We assume this case can be avoided based on correct association among [CN, reader, device].

- Conclusion

According to above discussion for paging procedure in air interface, the following proposals are given:

Proposal 1: From RAN2 perspective, multiple IDs can be contained in paging message. The multiple IDs can be used to indicate a part of or a few specific devices (e.g., not all devices in a certain category/group) or multiple devices with discontinuous device IDs.

Proposal 2a: For basic paging procedure, RAN2 can mainly discuss how to facilitate device to determine the start point of D2R access with the aim of reducing collision. And RAN2 can assume that after determining the start point of D2R access, the device can choose a RA resource from the available RA resources set by some ways, which depends on the discussion in RA topic.

Proposal 2b: RAN2 can further discuss the following schemes for determining the start point of D2R access with the aim of reducing collision:

- Alt1-1: Anti-collision scheme similar as Q-algorithm in RFID.

- Alt1-2: A deterministic way for determining the start point of D2R access and/or access occasion based on device ID.

Proposal 2c: If anti-collision scheme similar as Q-algorithm in RFID is introduced, an initial Q value needs to be provided via the paging message.

Proposal 2d: If anti-collision scheme similar as Q-algorithm in RFID is introduced, after the initial paging message, the reader also needs to send subsequent DL messages, e.g., to reduce the slot counter in device side or even adjust the Q value.

Proposal 3: For more efficient handling a CN/AF service request, it's suggested to discuss the follow options:

- Alt1: Sending multiple identical paging messages and introducing ACK/NACK scheme to Msg3 transmission.

  - The device can mark its status as “has been inventoried” or “has not been inventoried” based on the ACK or NACK of Msg3 transmission. Only the devices with status of “has not been inventoried” need to do response to the subsequent paging messages.

- Alt2: Sending multiple paging messages and introducing ACK/NACK scheme to Msg3 transmission. For the multiple paging messages, only the initial paging message includes device ID information and the subsequent paging messages includes no device ID information but with a delta paging indication.

  - The device can mark its status as “has been inventoried” or “has not been inventoried” based on the ACK or NACK of Msg3 transmission. Only the devices with status of “has not been inventoried” need to do response to the subsequent paging messages with a delta paging indication.

- Alt3: Sending multiple paging messages and in each paging message, the device IDs information of only the devices whose Msg3 transmission have NOT been successfully received by the reader in the previous paging rounds would be included. So the number of entries in the device IDs information in the subsequent paging messages would gradually decrease.

  - As long as the ID information of a device is included in the subsequent paging messages, the device can interpret this as an unsuccessful Msg3 transmission and can re-respond.

Proposal 4: For handling the case that the reader receives another service request from CN/AF before it finishes handling a previous one, RAN2 can discuss the following schemes:

- Alt1: The reader is allowed to trigger multiple paging messages in parallel and each of them can include an ID, e.g., session ID or transaction ID, to associate with the service request from CN/AF. The device needs to response the paging message as long as it determines that this paging message has not been responded yet by checking the session ID in the paging message.

- Alt2: The reader is allowed to trigger multiple paging messages in parallel and each of them contains the device IDs information received from the corresponding service request from CN/AF. The device needs to response the paging message as long as it determines that it is the target device by checking the device IDs information in the paging message.

Proposal 5a: For a CN/AF service request with very large number of target devices, it is also necessary for the RAN to apply the additional mask/filter, e.g., on the “instance ID” part of structured device ID, with intention to reasonably split such service request into several air interface paging messages and each of them will address a smaller number of devices.

Proposal 5b: To facilitate the reader to apply additional mask rules when necessary, the reader should at least know the structure of the device ID and the mask rules that have already been applied by upper layers.

Proposal 6: It’s suggested to study necessity and solutions for explicitly indicating the device ID information directly in DL/UL AIoT air interface. And this is also means the device ID information can be visible to the reader.

Proposal 7a: A concept of inventory area can be introduced with intention that an inventory is mainly performed for the devices within a certain inventory area.

Proposal 7b: It can be assumed that an AIoT inventory area for TP1 may include one or more BS readers. The association between devices and a BS reader can be set up by network node, e.g., BS reader or CN node. From RAN perspective, one base station (BS reader) can be associated with multiple AIoT devices.

Proposal 7c: It can be assumed that an AIoT inventory area for TP2 may include one or more UE readers. The association between a certain device with the UE reader and further with the attached base station can also be set up by network node, e.g., base station or CN node. From RAN perspective, one base station can be associated with multiple UE readers and each of UE reader can be associated with multiple AIoT devices.

Proposal 7d: The setting up association between a certain device with the reader (and may further with the attached base station) can be done during inventory procedure.

Proposal 7e: RAN2 can further discuss whether the device is capable to record the BS reader or UE reader where it was successfully inventoried in last time. If yes, the reader ID may need to be provided to the device in the paging message of inventory procedure.

- References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, RAN#103, March, 2024

- TR 22.840 v19.0.0: "Study on Ambient power-enabled Internet of Things", 3GPP SA1, Dec. 2023

- TR 38.848 v18.0.0: "Study on Ambient IoT (Internet of Things) in RAN (Release 18)", RAN#101, Sep. 2023

- TR 23.700-13 v1.0.0 (2024-09), Study on Architecture support of Ambient power-enabled Internet of Things (Release 19), September. 2024

- R2-2407244, Discussion on DL messages for Ambient IoT UEs, Ericsson, RAN2#127, August 2024

- R1-2406407, Discussion on channel and signal for Ambient IoT, ZTE Corporation, Sanechips, RAN1#118, August 2024

- R2-240xxxx, Further consideration on TP2 for AIoT, ZTE Corporation, RAN2#127bis, October 2024