3GPP TSG-RAN WG2 Meeting #127bis	R2-2408332

Hefei, China, 14th – 18th October, 2024

Agenda Item:	8.2.5

Source:	ZTE Corporation, Sanechips

Title:	Further consideration on TP2 for AIoT

Document for:	Discussion and Decision

Introduction

In RAN#102, a study item on solutions for Ambient IoT (Internet of Things) in NR has been agreed and in RAN#103, the SID is further refined in [1].

In RAN2#127 meeting, RAN2 has had initial discussion for TP2 in Ambient IoT. The following agreements have been achieved:

Agreements (Topology 2 considerations) - RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2. - A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device. - RAN2 assumes that intermediate UE authorization is performed by upper layers. RAN2 impact can be studied after further SA/RAN3 progress. - RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied) - NW is in control of resources to be used by AIoT air interface. - Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO). May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

In this paper, we will give further analysis on the Topology 2 and give our thoughts for the RAN related aspects, especially for the interface between the gNB and UE reader.

Discussion

  - General considerations

For Topology 2 (referred to as TP2 in the following text), besides reusing as much as possible the new AIoT air interface which is under discussion for Topology 1 (referred to as TP1 in the following text), we firstly assume that the interface between UE reader and the base station should try to leverage legacy 5G NR air interface, e.g., Uu interface. Then we will focus on identifying issues on the (legacy) Uu interface for supporting TP2 and the potential enhancements that are necessary. Unless specific issue is identified, we will not discuss enhancements to the AIoT air interface between AIoT device and the UE reader in the context of specific discussion of TP2.

Furthermore, according to the SID, the additional network control on the UE reader also needs to be studied.

Proposal 1a: For TP2, the interface between the UE reader and the base station should try to leverage legacy Uu interface, with additional network control provision.

In order to identify the possible RAN2 impacts of, we think it may be needed for RAN2 to firstly give an assumption on the general steps for triggering and performing AIoT service in TP2.

Proposal 1b: If needed, RAN2 can firstly agree the following assumed steps for triggering and performing AIoT service in TP2:

Step1. UE(s) is authorized/de-authorized to be able to perform as reader;

Step2. When CN/AF initiates AIoT service request, it can select the involved UE reader(s) from the authorized ones;

Step3. The CN/AF provides AIoT service data to the UE reader(s) via gNB (explicitly or transparently);

Step4. The gNB allocates AIoT air interface resources to UE reader(s) to facilitate it perform AIoT service (Step4 may occur before or after Step3);

Step5. UE reader performs AIoT service for the relevant devices, using the same or similar procedure as that in TP1.

Step6. UE reader provides the collected AIoT service responses/data to CN/AF via gNB (explicitly or transparently).

  - Selection of UE reader

In last meeting, RAN2 has agreed the assumption that intermediate UE authorization is performed by upper layers.  From RAN2 perspective, one straightforward way can be that, UE is authorized/de-authorized to perform as reader by CN during registration/de-registration procedure. Furthermore, such Authorization/de-authorization should be based on UE relevant capability.

Per our understanding, each time CN/AF initiates an AIoT service request, it can select the involved UE readers from the authorized ones and may also with reference to the association among CN mode, reader and the devices. However, as the CN may not be able to know timely whether a UE is currently suitable for being a reader (for example, a UE may want to refuse to act as reader due to insufficient battery power), purely relying on the CN to select UE reader may be insufficient. Meanwhile, considering that in some scenarios the base station can have more up-to-date information on the connection existence or radio situation of a UE, or even UE’s preference, it should also allow the base station to involve or adjust UE reader selection.

Proposal 2a: As the base station may have more up-to-date information on the connection existence or radio situation of a UE, or even UE’s preference, it should also allow the base station to involve the UE reader selection.

If the involvement of base station on UE reader selection, the base station at least needs to know whether a UE has been authorized. So this information may need to be passed from the CN to the base station or reported by the UE to the base station itself, e.g., via UE assistance information report.

Proposal 2b: The information about whether a UE has been authorized to perform as reader may need to be passed from the core network to the base station or reported by the UE to the base station.

  - Resources allocation to UE reader for AIoT air interface operations

    - General aspects

As mentioned in Proposal 1b, one of the main steps for performing AIoT service in TP2 is to allocate AIoT air interface resources to UE reader(s). Although per our understanding, this step generally occurs after the step of delivery AIoT service data from CN to UE reader, as there are some thinking that it may be also possible to occur earlier, we discuss this aspect first.

As background, for TP1 and for the basic AIoT service procedure, e.g., inventory, RAN2 has agreed that paging-like and random access-like procedures will be introduced on AIoT air interface for triggering multiple devices to respond to the network.

One of the important discussion is the contents that need to be sent by the reader to AIoT devices via DL R2D message on AIoT air interface. Although there are not many relevant agreements yet, the following information has been widely mentioned/discussed in previous meetings, some of which are also mentioned in our contribution [5]:

- The device ID information of the AIoT device(s)

- Selection criteria: generally Mask/Filter/Group information. It can be combined with device ID information to be used to indicate which individual, which group, or which discrete AIoT device(s) need to reply to the service request from CN/AF

- Radio resources information or random access control info (if RACH is needed):

  - Scheduling information for D2R transmission (e.g., time/frequency domain resources, MCS etc.);

  - Information for determining the start point of D2R transmission (or another saying, access occasion): e.g. Q value for Q-selection algorithm, or the total number of access occasions N.

For TP2, at least above information need to be consider to provide to UE reader for performing AIoT operations over AIoT air interface.

Proposal 3a: With reference to the discussion on AIoT air interface for TP1, at least the following information needs to be considered when discussing AIoT air interface resources allocation to UE reader:

- The device ID information of the AIoT device(s)

- Selection criteria

- Radio resources information for R2D transmission and/or random access control info (if RACH is needed):

For TP1, the above information can either be acquired by gNB reader from core network (device ID information, (part of) selection criteria) or determined by gNB reader itself (e.g., (part of) selection criteria, radio resources, Q value). However, for TP2, we need further discuss how the UE reader can acquire such information.

For TP2, we assume that the UE reader can be a regular UE but is capable of perform AIoT services. In order to avoid radio interference among different UE readers, and also for a certain UE reader, avoid possible interference between UE’s own data transmission over legacy Uu interface and AIoT operation over AIoT air interface (if the assumption that UE reader can operate on only one interface at a certain time is agreed, it can naturally avoid such interference), the AIoT air interface resources for a UE reader to perform AIoT operations (e.g., the radio resources and/or random access control info mentioned above) should be allocated by the base station. We think this is straightforward and also very important in the context of 3GPP (licensed spectrum-based system).

In last RAN2 meeting, many companies gave similar understandings that as licensed spectrum is to be used in D2R/R2D transmission in AIoT air interface, it’s naturally coordination would be needed between the network and the UE reader on which radio resources are to be used for D2R/R2D transmission. Therefore, based on most companies’ preference, RAN2 has made a high level agreement that NW is in control of resources to be used by AIoT air interface.

Proposal 3b: The following information that has been agreed to be obtained from CN and should be visible to the reader in TP1 is also needed to the base station in TP2 to assistant resources allocation for AIoT air interface:

- The service type of A-IoT (e.g. inventory, command). The more finer information on command type (e.g. read/write/disable) would also be useful;

- Targeted for one or more than one devices;

- Approximate number of target devices (if available).

    - Whether to consider sidelink resource allocation schemes

In 5G NR, two sidelink resource allocation modes are supported: mode 1 and mode 2. In mode 1, the sidelink resource allocation is provided by the network. In mode 2, UE decides the SL transmission resources in the resource pool(s).

For sidelink mode 1 (refer as SL-mode-1 in below), the resources allocation is generally controlled by the serving gNB. But it’s still requires the assistance information from the TX/RX UEs to the gNB. Moreover, it may also require the coordination or assistance information exchanging between TX UE and RX UE.

For sidelink mode 2 (refer as SL-mode-2 in below), the gNB mainly provides resource pool configuration to the UE and UE can decide usage of the allocated resources by its own decision-making. The following additional requirements can also be observed:

- UE needs to autonomously select resources from the resource pool, which requires the UE to have certain decision-making capabilities. UE may do the selection based on local measurements such as channel quality and interference levels.

- Considering that it’s possible to configure same or shared resource pool (e.g., via SIB) to several UEs and it means multiple UEs can access the same resource block, two mechanisms have been supported due to the nature of autonomous scheduling and the need to avoid interference among UEs:

  - Sensing: Sensing involves a UE listening to the channel to detect whether it is occupied by other transmissions before it selects a resource block for its own transmission. This helps prevent collisions with other ongoing transmissions.

  - Inter-UE Coordination: This mechanisms allow UEs to coordinate their transmissions among themselves, either directly or indirectly, to avoid interfering with each other. This could be achieved through explicit signaling between UEs or through implicit means, such as adhering to a common set of rules for resource selection.

Based on above background information, we think SL-mode-1 has the similar concept as above BS-controlled resources allocation options. Furthermore, if the base station can reasonably configure AIoT resources, it also doesn’t need the collaboration between UEs that may cause operation complexity and power consumption for the UE reader. Therefore we do not need to specifically discuss this SL-mode-1 and can just focus on the BS-controlled resources allocation.

Observation 1a: SL-mode-1 scheme has the similar concept as the BS-controlled resources allocation options. And the relevant scheme for collaboration between UEs may not be needed and may cause operation complexity and power consumption for UE reader.

Proposal 4a: RAN2 doesn’t need to specifically discuss the SL-mode-1 scheme and can just focus on the BS-controlled resources allocation.

For SL-mode-2, the proponent companies who suggests to apply it to AIoT resources allocation in TP2 think this scheme can be applicable to the UE reader out-of-coverage scenario and also suitable for quick local decisions on D2R/R2D communication. However, per our understanding, SL-mode-2 may be mainly suitable for scenarios like V2X where the UE's service may be frequent, predictable, and may persist for a period of time. Moreover, adjacent UEs may have similar services and therefore continuous collaboration is needed. Conversely, in our scenario like AIoT operation in TP2, the services might be dynamic, bursty or happen infrequently. So on one hand, purely pre-configuring resources may be very possible to result in low resource utilization. On the other hand, it’s highly uncertain which UE is selected as reader and whether adjacent UEs are all selected to execute such service. Therefore requiring UE readers to perform sensing or inter-UE coordination before using allocated resources may be unnecessary in much time, adding undue operational complexity and power consumption to the UE reader. Therefore, we believe that SL-mode-2 is not suitable for the AIoT resources allocation for UE reader in TP2.

Observation 1b: SL-mode-2 scheme may be mainly suitable for scenarios like V2X where the UE's service may be frequent, predictable, and may persist for a period of time. Conversely, for AIoT operation in TP2, the services might be dynamic, bursty or happen infrequently. So purely pre-configuring resources may be very possible to result in low resource utilization. Furthermore, the sensing or inter-UE coordination required by SL-mode-2 may be unnecessary in much time, adding undue operational complexity and power consumption to the UE reader

Proposal 4b: RAN2 doesn’t need to take SL-mode-2 as a candidate scheme for BS-controlled resources allocation.

    - The ways for configuring AIoT air interface resources to UE reader

In last RAN2 meeting, some detailed solutions on how to allocate/configure AIoT air interface resources for UE reader have been mentioned in the contributions. In below we give a simple summary.

Please note, according to above discussion on SL-mode-1 and SL-mode-2, we no longer list them for further discussion.

Moreover, with reference to the discussion for TP1, there is already some discussion on what resources need to be allocate and what assistant information needs to be visible to the base station in section 2.3.1 General aspects, see Proposal 3a and 3b, and so not mention them again in this section.

| Index | Description | Comments |
| --- | --- | --- |
| Alt1 | gNB configures AIoT air interface resources to the UE reader when it in connected mode, e.g., via dedicated signaling as RRC Reconfiguration | How to allocate the resources can be further according to the assistance information reported by UE reader or from CN/AF. Furthermore, even such dedicated configuration is provided to UE reader in connected mode, it’s also possible for UE reader to use it in idle state, depending on NW control. |
| Alt2 | gNB pre-configures AIoT air interface resources which can be shared by all the involved UE readers, e.g., via SIB signaling: - With additional NW indication on which specific part of the shared resources can be used by a certain UE reader (the indication on when to start and to stop using the resources can also covered). | As discussion in section 2.3.2, we think the scheme that UE reader autonomously selects resource from shared resources without confirmation from gNB (like SL-mode-2) would not be suitable to AIoT scenario. But from reducing signaling overhead perspective, we think it’s still possible to provide shared resources via SIB. Additionally, the NW/BS needs to indicate which specific part of the shared resources can be used by a certain UE reader. |
| Alt3 | Triggered by request from UE reader, gNB configures AIoT air interface resources to it. | One point that Alt3 differs from the previous Alt1 is that the resource allocation of Alt3 must be triggered by a request from the UE reader, while in Alt1, information from the UE reader only act as an assistant input, and resource allocation can proceed even without this assistant input. Alt3 may be mainly needed by the User Plane-based AIoT data delivery solution discussed in section 2.4 |

Based on above summary, the following proposals are given:

Proposal 5a: UE reader in either RRC_IDLE/RRC_INACTIVE or RRC_CONNECTED can be provided with AIoT radio resource allocation/configuration by the base station through RRC signaling.

Proposal 5b: the following options on how to allocate/configure AIoT air interface resources for UE reader can be further discussed:

- gNB configures AIoT air interface resources to the UE reader when it in connected mode, e.g., via dedicated signaling;

- gNB pre-configures AIoT air interface resources which can be shared by all the involved UE readers.

  - With additional NW indication on which specific part of the shared resources can be used by a certain UE reader

- Triggered by request from UE reader, gNB configures AIoT air interface resources to it.

    - CW control aspect

There is a specific thing, e.g., CW transmission, which may be relevant to the discussion on AIoT air interface resources allocation but mainly discussed by RAN1 in previous meetings. From RAN2 perspective, it may be a general view that CW transmission is mainly used for energy harvesting and how UE harvests energy is not in the study scope of this SID, and also may have no impacts on specifications.

Even in case of TP1, RAN2 will not specify any signaling for the CW control. Thus, the same paradigm should be adopted also for TP2. This is feasible and can be supported as long as the network is in control of the radio resources that are used for the D2R and R2D transmissions in TP2. Since the network knows which exact resources are used, the network can also ensure that the energy harvesting mechanisms are in place for these radio resources (again same assumption as in TP1). Thus, there is no need to discuss anything specific for CW control for TP2.

Proposal 6: For TP2, RAN2 will not discuss anything about CW control.

    - Control on AIoT resources when UE reader is “temporarily” out-of-connection

In last RAN2 meeting, there is much discussion on whether a UE can (still) act as UE reader when it is out-of-coverage. RAN2 has agreed to support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  Furthermore, RAN2 may consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

With reference to above discussion on resources allocation for UE reader, there may be a possible case that a UE in connected mode is provided with AIoT service request and allocated some AIoT air interface resources which are intentionally to be used after the UE reader is released to idle/inactive state (e.g., it may like the PUR resources for IoT). We see this as a normal case where UE with “out-of-connection” can act as reader to perform AIoT operations on the allocated resources. In this case, NW and UE reader can have consistent assumption on the usage of the allocated resources (e.g., when to start the usage and when to stop) and so no issue can be seen.

There may be another case that a UE reader (e.g., UE_A) in connected mode is allocated AIoT air interface resources that can only be used when UE_A is in connected mode. It’s possible that later UE_A is “temporarily” out-of-connection (short or long). In such case, as the BS may detect the disconnection of UE_A, may release the resources previously allocated to it, and further re-assign the resources to other normal UE or UE reader (e.g., UE_B), if UE_A keeps using thi allocated AIoT air interface resource to continue the AIoT services, it is likely to cause interference between UE_A and UE_B, causing both of them to fail to operate properly. Therefore, for this case, some coordination/control may be needed on usage of the AIoT air interface resources after UE reader is disconnected.

A general way may be that the BS can give kind of indication to the UE reader when allocating AIoT air interface resources. For example, such indication can indicate whether the previously allocated AIoT air interface resources can be used after UE reader disconnects from the base station, or can further indicate a time period in which UE reader can keep using this previously allocated AIoT air interface resources (similar as the concept of validity duration).

Proposal 7a: RAN2 discuss whether AIoT air interface resources which are allocated for a UE reader in connected mode can be continued to use after the UE reader disconnects from the NW. If yes, RAN2 further discuss whether a validity duration-like information needs to be introduced for the AIoT air interface resources.

We see the scheme mentioned in Proposal 7a can also be applied to the case that UE reader is “temporarily” out-of-coverage (e.g., shortly). It’s suggested RAN2 will not discuss other more complex scenarios, which can be discussed in future release (e.g., based on the Topology 4 in [3, TR 38.848]) or be addressed by non-standardized UE implementation (e.g., by using unlicensed spectrum).

Proposal 7b: With the intention that we will introduce scheme for supporting UE reader “temporarily” out-of-connection and make it also feasible to UE reader “temporarily” out-of-coverage, RAN2 will not further discuss the support for out-of-coverage case.

  - AIoT service data transmission to UE reader

    - NAS(-like) layer for AIoT device

According to the progress in RAN2 study, a simplified and compacted protocol stack for the AIoT air interface between AIoT device and the Reader will be designed. In such protocol stack, at least physical layer and MAC layer are involved. Meanwhile, RAN2 has agreed to assume no support of AS security until SA3 provides further requirements.

With reference to the SA1 requirements study [2, TR 22.840], for many use cases, it mentions that mechanisms are needed to prevent the services data of AIoT device from being obtained by other untrusted or unauthorized user. Meanwhile, energy efficient security mechanism is needed for lightweight devices like AIoT devices and it can include authentication, encryption and data integrity. From RAN2 perspective, we understand that, under the strict constraints for the AIoT devices such as working based on harvested power and very low complexity, the security we mainly need to achieve is to protect the security of the data of the items to which the AIoT device is attached. In other word, we assume the end-to-end protection of Commands (e.g., read, write) and/or the responses to the Commands (e.g., the information stored in the Ambient IoT devices) would be the main security requirement.

Observation 2a: With reference to the SA1 requirements, RAN2 can assume the end-to-end protection of the service data of the AIoT devices (e.g., Commands (read, write etc.) and/or the responses to the Command) would be the main security requirement in AIoT system.

Meanwhile, from RAN2 perspective, we understand that the basic AIoT service, e.g., the inventory is mainly to check the presence or absence of the AIoT devices in the concerned area. As response to the inventory request, the AIoT devices send the ID information to the CN/AF and CN/AF can authorize whether the devices are the subscriber indeed based on the subscription data. Furthermore, if needed, CN/AF can trigger other services (e.g., Command) to the AIoT devices that are confirmed to exist and have properly subscribed to the network. Such process can also help to avoid security risk of sending Command service to the devices directly. Based on such understanding, we don’t think it is necessary to encrypt the identity information (e.g., device ID) of the AIoT devices, especially on the transmission links.

Observation 2b: Based on the inventory procedure, CN/AF can check the presence or absence of the AIoT devices and authorize whether they have properly subscribed to the network before sending other AIoT services to the devices. So we see no additional necessity to end-to-end protect the identity information (e.g., device ID) of the AIoT devices.

According to the above Observation 2a, we see the necessity of an upper layer to provide security for the service data of the AIoT service devices. With reference to the legacy LPWA technology, e.g., CP solution for NB-IoT/eMTC, it can be seen that if light-weight security is needed for protecting the IoT data, to only support NAS (-like) layer security is enough. Therefore, within 5G network, it’s also suggested to support NAS (-like) layer security to provide end-to-end protection for the AIoT service data. This makes it feasible to reuse legacy IoT security mechanisms to the extent and potentially reduce the standardization work for AIoT. Please note this suggestion is applicable to both TP1 and TP2. An example of protocol stack for TP1/TP2 is as below.

Figure 1. Example of protocol stack with AIoT NAS layer for TP1

Proposal 8a: From RAN2 perspective, for both TP1 and TP2, NAS (-like) layer security can be considered to provide end-to-end protection of the AIoT service data for the AIoT devices (e.g., Commands and/or responses to the Commands) within 5G network. An AIoT NAS layer which can provide NAS (-like) layer security can be introduced.

From RAN2 perspective, we mainly need to ensure that AS layer protocols can support the transmission of AIoT NAS layer data.

Proposal 8b: AS layer protocols need to support the transmission of AIoT NAS layer data.

    - The protocol stack for AIoT data transmission to/from UE reader

As mentioned in Proposal 1b, one of the main steps for performing AIoT service in TP2 is how to transfer the DL AIoT service data from core network to the UE reader in order to facilitate UE reader to trigger AIoT devices in the AIoT air interface, and how the UE reader deliver the received UL AIoT service data from AIoT devices to the core network.

In the previous discussion in SA2 [4, TR 23.700-13] and RAN3, there is already much discussion on the architecture aspects and protocol stack for Ambient IoT for TP2, and also on the delivery of AIoT data. There are mainly two types of schemes, one can be seen as Control Plane-based solutions (e.g., Solution 5, Solution 8, Solution 17, Solution 21, Solution 30, Solution 33 in [4]) while the other can be seen as User Plane-based solutions (e.g., Solution 3, Solution 22, Solution 32 in [4]). The examples for the mentioned solutions can be found in the folloinwg table:

| User Plane-based solution | Alt1: The example is Solution 3 in [4, TR 23.700-13]. The assumed protocol stack for TP2 is as below: In such solution, UE reader establishes a PDU session to send AIoT service data as a GTP-U data to UPF which forwards it to the AIoT aware CN node. The same encapsulation and transmission is performed in the DL direction. In this solution, the AIoT service delivery between AIoT devices and core network can be transparent to the base station. |
| --- | --- |
| Control Plane-based solutions | Alt2: The example is Solution 21 in [4, TR 23.700-13]. The assumed protocol stack for TP2 is as below: In such solution, UE reader uses the NAS between UE reader and AMF and further interface between AMF and AIoT CN node to send AIoT service data (which is already encapsulated in AIoT NAS PDU) to the AIoT aware CN node. The same encapsulation and transmission is performed in the DL direction. Per our understanding, the AIoT service data (encapsulated in AIoT NAS PDU) would be further encapsulated in the legacy NAS PDU between UE reader and AMF. |
|  | Alt3: The example is Solution 33 in [4, TR 23.700-13]. The assumed protocol stack for TP2 is as below: Per our understanding, this solution is also a Control Plane-based solution. The main point in this solution which is different from that in the NAS-based solution 21 is that AIoT service data (which is already encapsulated in AIoT NAS PDU) could be directly delivered via the RRC signaling between UE reader and the base station. |

From RAN2 perspective, especially focusing on the efficient resources allocation and management when leveraging legacy Uu interface to deliver AIoT service data between CN and UE reader, a brief comparison on several aspects between the two type scheme categories is as below. Please note, to make it easier to check, the better-performing option is marked with green text:

| Aspects for comparison | UP-based solution | CP-based solution |
| --- | --- | --- |
| Delivery of DL data for command | For UP-based solution (Alt1), the command services data aimed to AIoT devices will be encapsulated in GTP-U data (possibly together with other user data aimed to UE reader itself) and directly provided to UE reader. It can be under the control of UE reader to firstly trigger inventory and then transmit command data. However, considering that the concerned AIoT devices may not be in the coverage of a UE Reader (e.g., the inventory may fail), directly providing the command data to the UE Reader may cause unnecessary resource waste on legacy Uu interface between UE Reader and the base station. | The better way to deliver DL data for command may be to let the base station hold the command data till it gets inventory results from UE reader/AIoT devices. It’s easy to see CP-based solution (Alt2/3) is more feasible to support such step-by-step inventory and command delivery procedures. |
| Delivery of UL data for command responses | When UE reader receives the UL data from devices for responding the command, the UE reader needs to deliver the UL data to CN. For UP-based solution, the UL data from the devices for command responses will be encapsulated in GTP-U data and in one possibility can be together with other user data from UE reader itself. However, as the UL user data from different terminal nodes (device or UE) may need to be delivered to different CN nodes, such merged encapsulation way would not feasible. Therefore, anyway new or different bearer for the user data from devices would be needed. And more differentiation work need to be done in base station. | With CP-based solution, it’s easy for base station to differentiate the UL data from different terminal nodes (device or UE) based on different control plane signaling. |
| Feasibility for assisting base station to allocate AIoT air interface resources | As discussed in the section 2.3, in the context of 3GPP and with licensed spectrum applied, in order to avoid the radio interferences among different UE readers, RAN2 has agreed the assumption that the AIoT air interface resources should be controlled by the base station. In order to facilitate the base station to allocate suitable resources for AIoT air interface, some information needs to be visible to the base station. RAN2 also agreed some assistant information can be provided to BS from CN, e.g., service type of A-IoT (e.g. inventory, command), approximate number of target devices (if available)。 However, for UP-based solution (Alt1), it’s highly possible that all the AIoT service related information, even the device ID(s) information, will be encapsulated in the user plane data and therefore not easy to be exposed to the base station. Also CN node may need some additional effort to extract the needed assistant information. | With CP-based solution, it would be feasible or easier to explicitly provide the device ID(s) information from the CN to the base station and further provide to UE reader and AIoT devices. Therefore, the BS can know not only the rough number of the devices but also know more information. That information can further facilitate the BS/UE reader to do delta paging in AIoT air interface (see discussion in [5]). Shortly to say, CP-based solution would be better for efficiently controlling AIoT air interface resources allocation and scheduling. |
| UE reader selection | It generally assumes the CN can select suitable UE reader to perform AIoT service according to the association among the AIoT device(s) and network nodes established previously. We also see possibility that the base station may have more up-to-data information about UE reader and may need to adjust the UE reader selection (see discussion in section 2.3). For UP-based solution, it is almost impossible for the base station to adjust the list of devices sent to a UE reader (e.g., to stop UE-A as reader and let another near UE-B to do the inventory for the devices which previously belong to UE-A). | With CP-based solution, it’s feasible or easier for BS to adjust the UE reader selection, e.g., to adjust or even re-assign the list of devices from a certain UE reader to another UE reader based on the assistance information from CN and acquired by BS itself. |

Observation 3: Based on the above comparison in aspects such as Delivery of DL data for command, Delivery of UL data for command responses, Feasibility for assisting base station to allocate AIoT air interface resources and UE reader selection, it can be seen that CP-based solution outperforms UP-based solution.

For CP-based options, from RAN2 perspective, both of them are feasible and efficient. There are still some differences between the two alternatives and each of them has pros and cons. For example, Alt2 may be more in line with the existing Control Plane 5GS Optimization for CIoT mechanism for LPWA, but it seems to cause redundant NAS layer encapsulation. Meanwhile, RRC-involved scheme, e.g., Alt3, may have simple NAS encapsulation, but may involve more signaling transition between different protocol layers. Therefore, how to choose between them can be decided later, maybe till WID stage based on more progress in all the RAN2, RAN3 and SA2.

Based on the above discussion, the following proposals are given:

Proposal 9a: From RAN2 perspective, for AIoT service data delivery between UE reader and core network in TP2, control plane-based solutions (NAS-based and RRC-based schemes) should be adopted for further study.

Proposal 9b: Send LS to RAN3 and SA2 to indicate RAN2’s preference on control plane-based solution.

  - The state of UE reader when performing AIoT operations

In previous meeting, there are some proposals about what the state of UE reader should be when it performs AIoT service. The following diverse views have been mentioned:

- As baseline, the UE reader can keep in RRC_CONNECTED state during the AIoT procedure.

- At least RRC_CONNECTED and RRC_INACTIVE UE can be selected as an intermediate UE.

- RAN2 assumes that the intermediate UE is not supposed to be idle/inactive state, when the UE reader is performing the A-IoT procedure.

- If the UE can collect and aggregate device data for later transmission, it is unnecessary to require the UE to remain in the RRC_CONNECTED state. In other word, the UE can allow data/signaling exchanges on the AIoT air interface and store the device data in RRC_IDLE or RRC_INACTIVE state. Later the UE can switch to RRC_CONNECTED state for data reporting.

- The UE Reader can be in any RRC State while performing R2D/D2R activities.

Per our understanding, a UE reader firstly is a normal UE and can act as a reader after being authorized and selected by core network. So the UE reader should have the capability to operate over legacy Uu interface and on AIoT air interface. However, considering that for either legacy Uu interface or AIoT air interface, there will be both uplink and downlink transmission for a certain service operation (for legacy Uu interface, DL: BS->UE, UL: UE->BS; for AIoT air interface: DL: UE->device, UL: device->UE), and also considering limited UE capability and transmission power, we believe that requiring UE reader to perform full operations (UL and DL) simultaneously on both legacy Uu interface and AIoT air interface would be too much to ask for and may be difficult to achieve.

Observation 4: Considering that for either legacy Uu interface or AIoT air interface, there will be both uplink and downlink transmission for a certain service operation, and also considering limited UE capability and transmission power, we believe that requiring UE reader to perform full operations (UL and DL) simultaneously on both legacy Uu interface and AIoT air interface would be too high a requirement.

Therefore, we think a more reasonable assumption may be that at any given time, the UE reader can operate on only one of the interfaces, e.g., either legacy Uu interface or AIoT air interface. In other word, the UE reader cannot perform full operations (UL and DL) simultaneously over both legacy Uu interface and AIoT air interface. If needed, RAN2 can further check with RAN1 and/or RAN4 for this aspect.

Proposal 10a: From RAN2 perspective, a reasonable assumption can be that at any given time, a UE reader can work on only one of the interfaces, e.g., either legacy Uu interface or AIoT air interface.

With Proposal 10a, we see no clear justification to give any restriction on UE state:

- If UE reader is currently in idle or inactive state and knows that an AIoT operation needs to be performed (this can be according to some previous configuration and it’s possible that the AIoT resources have been provided to UE reader at an earlier time point), UE reader can start the AIoT operation over AIoT air interface. Based on the capability of UE reader (e.g., for storing the received AIoT service responses), the UE can choose a suitable time point (e.g., after finishing part or all the service operations) to initiate legacy Uu interface connection establishment and send the stored AIoT service responses to CN/AF. Additionally, considering that the AIoT operation may take a kind of long time, in order to minimize the impact on legacy Uu interface, some mechanisms need to be considered to allow the UE reader to intermittently listen for paging on the legacy Uu interface and even perform some measurements.

- Even if a UE reader is currently in connected mode over legacy Uu interface, when there is a trigger (e.g., based on kind of network control or configuration) for this UE reader to perform AIoT operation, it’s also feasible for the UE reader to keep in connected mode and just stop the current data transmission over legacy Uu interface (may need some coordination with network), and further start the AIoT operation over AIoT air interface. After finishing part or all the service operations in AIoT air interface, the UE reader can back to active and continue the data transmission over legacy Uu interface (can include the stored AIoT service responses to transmit to CN/AF). As an easily conceivable implementation way, AIoT operation can be configured to perform during the inactive phase of Connected-mode DRX. More generally, it is also possible that the UE reader can stop legacy Uu interface transmissions at any time (again, this may need some coordination with network or according to some configuration) and start AIoT operation. However, similar to that for idle/inactive state, in order to minimize the impact on legacy Uu interface, some mechanisms need to be considered to allow the UE reader to intermittently back to active on legacy Uu interface, e.g., for monitoring PDCCH or performing some measurements.

Proposal 10b: Based on network control, UE reader in any state, e.g., idle/inactive/connected, is allowed to perform AIoT operations over AIoT air interface. In order to minimize the impact on legacy Uu interface, when performing AIoT operations:

- For UE reader in idle or inactive state, some mechanisms need to be considered to allow it to intermittently perform (some) idle mode tasks on legacy Uu interface, e.g., monitoring paging or performing measurements.

- For UE reader in connected state, some mechanisms need to be considered to allow it to intermittently back to active to perform (some) connected mode tasks on legacy Uu interface, e.g., monitoring PDCCH or performing measurements.

Conclusion

This contribution gives the following observations and proposals for supporting TP2 for Ambient IoT:

Proposal 1a: For TP2, the interface between the UE reader and the base station should try to leverage legacy Uu interface, with additional network control provision.

Proposal 1b: If needed, RAN2 can firstly agree the following assumed steps for triggering and performing AIoT service in TP2:

Step1. UE(s) is authorized/de-authorized to be able to perform as reader;

Step2. When CN/AF initiates AIoT service request, it can select the involved UE reader(s) from the authorized ones;

Step3. The CN/AF provides AIoT service data to the UE reader(s) via gNB (explicitly or transparently);

Step4. The gNB allocates AIoT air interface resources to UE reader(s) to facilitate it perform AIoT service (Step4 may occur before or after Step3);

Step5. UE reader performs AIoT service for the relevant devices, using the same or similar procedure as that in TP1.

Step6. UE reader provides the collected AIoT service responses/data to CN/AF via gNB (explicitly or transparently).

Proposal 2a: As the base station may have more up-to-date information on the connection existence or radio situation of a UE, or even UE’s preference, it should also allow the base station to involve the UE reader selection.

Proposal 2b: The information about whether a UE has been authorized to perform as reader may need to be passed from the core network to the base station or reported by the UE to the base station.

Proposal 3a: With reference to the discussion on AIoT air interface for TP1, at least the following information needs to be considered when discussing AIoT air interface resources allocation to UE reader:

- The device ID information of the AIoT device(s)

- Selection criteria

- Radio resources information for R2D transmission and/or random access control info (if RACH is needed):

Proposal 3b: The following information that has been agreed to be obtained from CN and should be visible to the reader in TP1 is also needed to the base station in TP2 to assistant resources allocation for AIoT air interface:

- The service type of A-IoT (e.g. inventory, command). The more finer information on command type (e.g. read/write/disable) would also be useful;

- Targeted for one or more than one devices;

- Approximate number of target devices (if available).

Proposal 4a: RAN2 doesn’t need to specifically discuss the SL-mode-1 scheme and can just focus on the BS-controlled resources allocation.

Proposal 4b: RAN2 doesn’t need to take SL-mode-2 as a candidate scheme for BS-controlled resources allocation.

Proposal 5a: UE reader in either RRC_IDLE/RRC_INACTIVE or RRC_CONNECTED can be provided with AIoT radio resource allocation/configuration by the base station through RRC signaling.

Proposal 5b: the following options on how to allocate/configure AIoT air interface resources for UE reader can be further discussed:

- gNB configures AIoT air interface resources to the UE reader when it in connected mode, e.g., via dedicated signaling;

- gNB pre-configures AIoT air interface resources which can be shared by all the involved UE readers.

  - With additional NW indication on which specific part of the shared resources can be used by a certain UE reader

- Triggered by request from UE reader, gNB configures AIoT air interface resources to it.

Proposal 6: For TP2, RAN2 will not discuss anything about CW control.

Proposal 7a: RAN2 discuss whether AIoT air interface resources which are allocated for a UE reader in connected mode can be continued to use after the UE reader disconnects from the NW. If yes, RAN2 further discuss whether a validity duration-like information needs to be introduced for the AIoT air interface resources.

Proposal 7b: With the intention that we will introduce scheme for supporting UE reader “temporarily” out-of-connection and make it also feasible to UE reader “temporarily” out-of-coverage, RAN2 will not further discuss the support for out-of-coverage case.

Proposal 8a: From RAN2 perspective, for both TP1 and TP2, NAS (-like) layer security can be considered to provide end-to-end protection of the AIoT service data for the AIoT devices (e.g., Commands and/or responses to the Commands) within 5G network. An AIoT NAS layer which can provide NAS (-like) layer security can be introduced.

Proposal 8b: AS layer protocols need to support the transmission of AIoT NAS layer data.

Proposal 9a: From RAN2 perspective, for AIoT service data delivery between UE reader and core network in TP2, control plane-based solutions (NAS-based and RRC-based schemes) should be adopted for further study.

Proposal 9b: Send LS to RAN3 and SA2 to indicate RAN2’s preference on control plane-based solution.

Proposal 10a: From RAN2 perspective, a reasonable assumption can be that at any given time, a UE reader can work on only one of the interfaces, e.g., either legacy Uu interface or AIoT air interface.

Proposal 10b: Based on network control, UE reader in any state, e.g., idle/inactive/connected, is allowed to perform AIoT operations over AIoT air interface. In order to minimize the impact on legacy Uu interface, when performing AIoT operations:

- For UE reader in idle or inactive state, some mechanisms need to be considered to allow it to intermittently perform (some) idle mode tasks on legacy Uu interface, e.g., monitoring paging or performing measurements.

- For UE reader in connected state, some mechanisms need to be considered to allow it to intermittently back to active to perform (some) connected mode tasks on legacy Uu interface, e.g., monitoring PDCCH or performing measurements.

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, RAN#103, March, 2024

[2] 3GPP TR 22.840 v19.0.0: "Study on Ambient power-enabled Internet of Things", 3GPP SA1, Dec. 2023

[3] 3GPP TR 38.848 v18.0.0: "Study on Ambient IoT (Internet of Things) in RAN (Release 18)", RAN#101, Sep. 2023

[4] 3GPP TR 23.700-13 v1.0.0 (2024-09), “Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)”, September. 2024

[5] R2-2408331 Further consideration on the details of paging for AIoT, RAN2#127bis, October 2024