3GPP TSG-RAN WG2 Meeting #127bis	R2-2408337

Hefei, China, 14 - 18 Oct, 2024

Source: 	Huawei, HiSilicon, Turkcell

Title: 	Discussion on DL coverage enhancements

Agenda Item:	8.8.2

Document for:	Discussion and decision

Introduction

According to the latest WID, the objective of DL coverage enhancements contains system level enhancements (including SSB periodicity extension) and link level enhancements (to be studied and concluded by RAN1 first) [1]:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

In RAN2#127, some agreements on SSB periodicity extension have been made, and it was also agreed to further consider SMTC impacts due to beam hopping / larger SSB periodicity:

Agreements:

1.	From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

2.	From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms.

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

4.	If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

In addition to the above agreements, RAN2 also discussed cell/beam-level DTX/DRX mechanism and identified some critical issues, an LS was sent to RAN1 [2] and RAN1 reply LS has been received [3]:

Question1 : Can RAN1 provide the information on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values?

Answer to Q1:

According to the updated WID (RP-241667), SSB enhancement other than SSB periodicity extension is not considered. Thereby, RAN1 understanding is that enhancements to the existing SSB patterns, e.g. SSB position in burst, SSB index number, are not within the scope.

The extension of the SSB periodicity is still under RAN1’s discussion.

Question4 : RAN2 would like to remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status (i.e. "off", "common messages only" and "active traffic") will likely have to relate to beams visible to the UE (e.g. SSB beams). RAN2 would also like to know whether RAN1 intends to define beam status for beams not visible to the UE or to define new beam status for beams visible to the UE.

Answer to Q4:

Beam footprint status in terms of "off", "common messages only" and "active traffic” in one of the RAN1 agreements is only defined for the sake of system-level evaluation methodology.

In this contribution, we further discuss the possible system level enhancements from RAN2 perspective.

Discussion

Cell level vs. beam level

Earlier in RAN2#125bis meeting, RAN2 initially provided the insights on RAN2 aspects of system-level downlink coverage enhancements. The corresponding agreement is listed below:

Agreements:

- With regard to link level enhancement, RAN2 waits for RAN1 agreement on the DL channels to enhance before starting any RAN2 work.

- We will continue the discussion on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.) in the next meetings, trying to identify questions to RAN1 for aspects where we need their input

RAN1 discussed the system level DL coverage enhancement for the dynamic power sharing among satellite beams, i.e. operating in the granularity of satellite beam. However, only NR cells and NR SSB beams are visible to UEs. In order to transform the satellite beam, we need to figure out the correlation among satellite beam, NR beam, and cell. The following 5 candidate options for the mapping relationship has been provided in [4]:

Option 1: Single NR cell per satellite beam and single NR beam per NR cell;

Option 2: Multi-Satellite beams per NR cell and single NR beams per NR cell;

Option 3: Multi-Satellite beams per NR cell and multi NR beams per NR cell;

Option 4: Multi NR cells per satellite beam;

Option 5: Multi NR cells split across satellite beams.

The mapping issue has remained open for several RAN1 meetings and no conclusion has been reached yet. In our understanding, how the satellite beams map to NR beams is up to network implementation. Unless further down-scoping made in RAN1, RAN2 needs to consider all the above candidate options when analyzing the pros and cons of the cell level Cell DTX/DRX and beam level Cell DTX/DRX.

For Option 1 and Option 4 (where the satellite beam is equal to or larger than an NR cell), the cell level Cell DTX/DRX mechanism is a better choice compared with beam level DTX/DRX, since the satellite beam operation is common to the UEs within the cell coverage. On the other hand, for option 2/3/5 (where the satellite beam is smaller than an NR cell), the satellite beam operation affects a portion of an NR cell, which makes the cell level cell DTX/DRX unsuited for these options.

Beam level DTX/DRX has a finer granularity, and the beam level pattern can take a non-uniform distribution of users and of traffic load into account by setting the active/inactive duration and cycle. Therefore, a beam level DTX/DRX is suitable to address all scenarios. To support beam level DTX/DRX, the network should provide the UE with multiple DTX/DRX patterns associated with each beam. When the UE switches its DL SSB beam to a new DL SSB beam, the UE needs to switch to the DTX/DRX pattern associated with the new DL SSB beam. It is noteworthy that the RAN1 is also discussing the Cell/beam level cell DTX/DRX. Therefore, we propose RAN2 to wait for sufficient RAN1 progress before further RAN2 discussion.

Proposal 1: RAN2 waits for RAN1 input before further discussion on cell level and beam level Cell DTX/DRX pattern.

Implications of SSB periodicity extension

As the revised WID states, the SSB periodicity extension is further considered. RAN1 is now discussing whether to extend the SSB periodicity to be larger than 160ms, e.g. 320ms and 640ms. Apart from the IEs confirmed by the last RAN2 meeting to be impacted by SSB periodicity larger than 160ms, there is also a discussion on the SMTC design [5].

The intention of SMTC enhancement in [5] is to extend the number of SMTCs per frequency (e.g. up to 6 SMTCs) and let the UE select the SMTCs among them (e.g. consider only the SMTCs corresponding to the closet neighbour cells). For Idle/Inactive UEs, it is proposed to broadcast reference location of neighbour cells for location-based SMTC selection; for Connected mode UEs, it is also questioned whether the NW can have precise UE location for SMTC configuration and proposed to provide reference location of neighbour cells for location-based SMTC selection.

From our perspective, for RRC_CONNECTED UEs, the general rule is that SMTC is up to NW configuration and cannot be selected autonomously by the UE. The reason is that the NW needs to configure gaps based on the SMTC pattern, and gaps will affect the NW scheduling. Even in case of gap-less measurements, there could still be scheduling availability issues during SMTC occasions (e.g. UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on the serving cell). Therefore, the NW and UE must have an aligned understanding on the SMTC pattern. We think the propagation delay reporting mechanism introduced in R17 is sufficient for SMTC configuration RRC_CONNECTED.

For RRC_IDLE/RRC_INACTIVE UEs, it is up to UE implementation to autonomously adjust the SMTC (there is no RAN4 requirement yet). Since each SMTC is associated with one or multiple cell IDs (PCIs), based on the ephemeris info for each neighbour cell, the UE is capable of calculating the distance to neighbour cells. It is up to UE implementation to monitor all or a subset of SMTCs for each frequency.

Without further enhancement on location-based SMTC selection, it does not make sense to extend the number of SMTCs per frequency. Note that the 4 SMTCs per frequency has been controversial in R17 and a UE capability is added as a compromise, introducing more SMTCs will further increase UE complexity and power consumption hence is not desirable.

Proposal 2: RAN2 do not increase the number of SMTCs per frequency.

Proposal 3: RAN2 do not consider location-based SMTC selection.

Conclusion

This contribution discussed the DL coverage enhancements in Rel-19 NR NTN. Proposals are summarized as follows:

Proposal 1: RAN2 waits for RAN1 input before further discussion on cell level and beam level Cell DTX/DRX pattern.

Proposal 2: RAN2 do not increase the number of SMTCs per frequency.

Proposal 3: RAN2 do not consider location-based SMTC selection.

References

- RP-241789, Non-Terrestrial Networks (NTN) for NR Phase.

- R2-2405762, LS on DL coverage enhancements, CMCC

- R1-2407538, Draft Reply LS on DL coverage enhancements, CMCC

- R1-2400303, Discussion on NR NTN Downlink coverage enhancements, Thales

- R2-2407532, Downlink coverage enhancement SMTC impacts, Sequans Communications