3GPP TSG-RAN WG2 #127bis meeting	R2-2408343

# Hefei, China, 14th Oct – 18th Oct, 2024

Source:		NEC

Agenda item:		8.7.4.1	LCP enhancements

Title:		Considerations on LCP enhancements for XR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#127 meeting, agreements [1] on scheduling enhancements for XR were made as below:

RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

In this contribution, we will provide our views on the enhancement of LCP procedure.

# 2. Discussion

2.1 LCP enhancements for delay-critical data.

LCP enhancements

In the RAN2#126 meeting, it was agreed that an additional priority will be configured to LCHs with delay-critical data, but it hasn’t been decided whether the priority only applies to delay-critical data within the LCH or for the whole LCH.

From our understanding, the intention of delay-awareness LCP is to prioritize the delay-critical data over non-delay-critical data to avoid blockage of delay-critical data. If the additional priority applies to the whole LCH, non-delay-critical data of the LCH shall be prioritized as well. For example, if additional priority of LCH A is higher than that of LCH B, even with just one delay-critical packet in LCH A, its non-delay-critical data shall also be multiplexed in a MAC PDU before the delay-critical data of LCH B, which may lead to insufficient resources for the delay-critical data of LCH B. Hence, we suggest that the additional priority only applies to delay-critical data within an LCH, that is, delay-critical and non-delay-critical data of an LCH shall be associated with different priority values when performing LCP.

Proposal 1: The additional priority only applies to delay-critical data within a LCH, that is, delay-critical and non-delay-critical data of an LCH shall be associated with different priority values when performing LCP.

Furthermore, the concept of PDU Set has been introduced in Rel-18, and all PDUs belonging to a PDU Set may be needed for the decoding of the corresponding information unit. In this situation, all PDUs belonging to a PDU Set shall be discarded as a whole if at least one PDU is discarded. Similarly, as all PDUs belonging to a PDU Set share the same PSDB, to avoid only part of a PDU Set is multiplexed in a MAC PDU but all PDUs are needed for decoding, which still brings extra delay for a PDU Set, RAN2 should consider maximizing the transmission of a PDU Set during delay-awareness LCP procedure.

Proposal 2: Maximize the transmission of a PDU Set during delay-awareness LCP procedure when all PDUs are needed for a PDU Set.

In the RAN2#125bis meeting, it was agreed to consider potential impact of delay-awareness LCP on traffic from SRBs. From our understanding, we should discuss SRB 0 and SRBs other than SRB 0 separately.

RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied: - Alternative 1: Enhance LCP restrictions/LCH selection. - Alternative 2: Enhance LCH prioritization. RAN2 should consider potential impact on traffic from SRBs.

For SRB0 (i.e., data from UL-CCCH), it always have a higher priority than delay-critical data (i.e., data from any Logical Channel, except data from UL-CCCH), which has already been specified by the priority order of logical channels in current specification [2]. There is no need to break the rules.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation; … - MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment; - data from any Logical Channel, except data from UL-CCCH; … NOTE 2: Prioritization among MAC CEs of same priority is up to UE implementation.

For SRBs other than SRB 0, a priority value is associated with the corresponding logical channel, either by applying the default configuration or an explicit configuration. From our understanding, by configuring a proper priority value for the logical channel with delay-critical data, the network can control the impact and the priority order between logical channels with delay-critical data and logical channels for SRBs other than SRB 0. So it can left to the network’s implementation and there should be no specification impact.

Proposal 3: It can be left to network’s implementation to configure the priority for delay-critical data of a LCH and logical channels for SRBs other than SRB 0.

Delay-awareness LCP was ever discussed during Rel-18 XR, and it was agreed in the RAN2#120 meeting that delay-awareness LCP should be able to be turned on/off.

If delay-aware LCP is introduced, need the ability to turn it off. SRBs not impacted. Not considered further unless fundamental issues are identified.

Based on it, to reduce the impacts on the legacy LCP procedure and avoid RAN1 impact, we propose RAN2 to introduce a MAC CE-based mechanism for the activation/deactivation of delay-awareness LCP.

Proposal 4: RAN2 to introduce a MAC CE-based mechanism for the activation/deactivation of delay-awareness LCP.

Regarding the configured UL grant originally for XR traffic, delay-awareness LCP mechanism can be applied to multiplex the delay-critical data with a higher priority. However, there exist other types of periodic traffic (e.g., voice or URLLC service) that are configured with configured UL grant as well. For these services, if delay-awareness LCP is applied to the configured UL grants, delay-critical data of XR traffic may occupy the resources originally reserved for the periodic voice or URLLC traffic, which shall bring extra delay for these services and cause a bad user experience. For this, we propose that different configured UL grant configurations can apply different strategies of whether to apply delay-awareness LCP mechanism.

Proposal 5: Different configured UL grant configurations can apply different strategies of whether to apply delay-awareness LCP mechanism.

# Summary

This contribution provides our analysis on the enhancement of LCP procedure, and has the following proposals:

Proposal 1: The additional priority only applies to delay-critical data within a LCH, that is, delay-critical and non-delay-critical data of an LCH shall be associated with different priority values when performing LCP.

Proposal 2: Maximize the transmission of a PDU Set during delay-awareness LCP procedure when all PDUs are needed for a PDU Set.

Proposal 3: It can be left to network’s implementation to configure the priority for delay-critical data of a LCH and logical channels for SRBs other than SRB 0.

Proposal 4: RAN2 to introduce a MAC CE-based mechanism for the activation/deactivation of delay-awareness LCP.

Proposal 5: Different configured UL grant configurations can apply different strategies of whether to apply delay-awareness LCP mechanism.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Holland

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.3.0