3GPP TSG-RAN WG2 #127bis meeting	R2-2408344

# Hefei, China, 14th Oct – 18th Oct, 2024

Source:		NEC

Agenda item:		8.7.4.2	DSR enhancements

Title:		Considerations on DSR enhancements for XR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#127 meeting, agreements [1] on scheduling enhancements for XR were made as below:

RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

In this contribution, we will provide our views on the enhancements of DSR.

# 2. Discussion

The legacy DSR in Rel-18 does not inform the network about the status of non-delay-critical data, as a result, the network cannot schedule it accordingly. For the resources scheduled based on the current DSR, if non-delay-critical data of LCH1 has a higher priority than delay-critical data of LCH2, non-delay-critical data of LCH1 shall be multiplexed first and the remaining resources may not be sufficient for the transmission of delay-critical data. Although BSR reports the total buffer size, the timing gap between DSR and BSR reports may lead to outdated and inaccurate information when scheduling delay-critical data.

Moreover, it was agreed in the RAN2 #127 meeting that the network shall configure multiple remaining time thresholds for reporting multiple pairs of remaining time and buffer sizes per LCG. It is evident that packets with remaining time above the maximum configured threshold can also be reported, which may encompass both non-delay-critical and low-importance data. Based on it, the network can obtain the comprehensive insights into the entire buffer and consider both delay-critical and non-delay-critical data when allocating resources.

Proposal 1: The buffer size of non-delay-critical data (e.g., packets with remaining time above the maximum configured threshold) is reported in the enhanced DSR.

Currently, PDU Set Importance (i.e., PSI) is only used during PDCP discard operation when congestion happens. If the congestion does happen, PDU Sets with low importance shall be discarded to guarantee the transmission of PDU Sets with high importance. If no congestion happens, there should be sufficient resources for the uplink transimssion. Based on the analysis, we don't see how low-importance data will adversely affect high-importance data. Since the benefit of reporting PSI-related information is not clear, we suggest PSI-related information is not carried in the enhanced DSR.

Proposal 2: PSI-related information is not reported in the enhanced DSR.

The legacy DSR in Rel-18 is triggered when a remaining time of the running PDCP discardTimer is below a configured threshold (i.e., remainingTimeThreshold).

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

Since the delay information reported by legacy DSR and enhanced DSR is mainly different in granularity, we think the trigger condition of the legacy DSR can be reused by enhanced DSR, that is, enhanced DSR is triggered when a remaining time of the running PDCP discardTimer is below a configured threshold (e.g., remainingTimeThreshold or a new threshhold).

Proposal 3: The trigger condition of the legacy DSR may be reused by enhanced DSR. FFS whether a new threshold should be introduced for the trigger.

A MAC PDU should contain at most one type of DSR MAC CE, either legacy DSR MAC CE or enhanced DSR MAC CE. Furthermore, enhanced DSR with finer granularity should be prioritized over legacy DSR. For example, if an enhanced DSR is triggered or configured while a legacy DSR is pending for an LCH, an enhanced DSR should be reported and the legacy DSR should be cancelled.

Proposal 4: enhanced DSR with finer granularity should be prioritized over legacy DSR.

# Summary

This contribution provides our analysis on the enhancement of DSR, and has the following proposals:

Proposal 1: The buffer size of non-delay-critical data (e.g., packets with remaining time above the maximum configured threshold) is reported in the enhanced DSR.

Proposal 2: PSI-related information is not reported in the enhanced DSR.

Proposal 3: The trigger condition of the legacy DSR may be reused by enhanced DSR. FFS whether a new threshold should be introduced for the trigger.

Proposal 4: enhanced DSR with finer granularity should be prioritized over legacy DSR.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Holland

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.3.0