3GPP TSG-RAN WG2 Meeting #127bis	R2-2408350

# Hefei, China,  14th - 18th October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1934 | Rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Random Access Preamble group B for 2-step RA and 4-step RA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, NR_2step_RACH-Core |  |  |  |  |  | Date: |  |  | 2024-10-4 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 5.1.1, Random Access Preambles group B is separately defined for 2-step RA type and 4-step RA type as below: - if groupBconfigured is configured, then Random Access Preambles group B is configured for 4-step RA type. - if groupB-ConfiguredTwoStepRA is configured, then Random Access Preambles group B is configured for 2-step RA type. In the current specification for random access procdure, it is clearly specified in most of the condition checks for Random Access Preambles group B that whether it is for 4-step RA type or 2-step RA type. However, there are two condition checks for Random Access Preambles group B without specifying whether it is for 4-step RA type or 2-step RA type. Case 1: A statement in clause 5.1.1 states that, if Random Access Preambles group B is configured, PCMAX for serving cell is assumed to be available for UE. For the statement, there are 2 interpretations: (1) The statement applies for 4-step RA. (2) The statement applies for both 4-step RA and 2-step RA. In our understanding, interpretation 2 is the correct understanding, as PCMAX is used in both 4-step RA and 2-step RA to determine the use of Random Access Preambles group B. Case 2: A statement that appeared multiple times in clause 5.1.2 specifies the random access preamble group selection based on whether Random Access Preambles group B is configured. In our understanding, the wording “Random Access Preambles group B” in clause 5.1.2 associates with 4-step RA, and the wording “Random Access Preambles group B for 2-step RA” in clause 5.1.2a associates with 2-step RA. For case 1 and case 2, the same statement “if Random Access Preambles group B is configured” are used for different meanings. Therefore, we suggest to clarify the wording for these different cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify the wording for Random Access Preambles group B that refers to 4-step RA to “Random Access Preambles group B for 4-step RA type” and Random Access Preambles group B that refers to either 4-step RA or 2-step RA to “Random Access Preambles group B for 4-step RA type or 2-step RA type”. Impact analysis Impacted 5G architecture options: NR-SA, NR-DC, (NG)EN-DC, NE-DC Impacted functionality: Random Access procedure Inter-operability: If the network implements the CR and the UE does not, there is no interoperability issue. If the UE implements the CR and the network does not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It’s not clear the wording "Random Access Preambles group B" is associated with 2-step, 4-step or both, which may cause misalignment in implementation. In clause 5.1.1, the condition check may not pass when groupB-ConfiguredTwoStepRA is configured. In clause 5.1.2, the condition check may pass when groupB-ConfiguredTwoStepRA is configured and groupBconfigured is not configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of 1st Change

5.1.1	Random Access procedure initialization

…

When a Random Access procedure is initiated, UE selects a set of Random Access resources as specified in clause 5.1.1b and initialises the following parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources:

…

-	if groupBconfigured is configured, then Random Access Preambles group B is configured for 4-step RA type.

-	Amongst the contention-based Random Access Preambles associated with an SSB (as defined in TS 38.213 [6]), the first numberOfRA-PreamblesGroupA included in groupBconfigured Random Access Preambles belong to Random Access Preambles group A. The remaining Random Access Preambles associated with the SSB belong to Random Access Preambles group B (if configured).

-	if groupB-ConfiguredTwoStepRA is configured, then Random Access Preambles group B is configured for 2-step RA type.

-	Amongst the contention-based Random Access Preambles for 2-step RA type associated with an SSB (as defined in TS 38.213 [6]), the first numberOfRA-PreamblesGroupA included in GroupB-ConfiguredTwoStepRA Random Access Preambles belong to Random Access Preambles group A. The remaining Random Access Preambles associated with the SSB belong to Random Access Preambles group B (if configured).

NOTE 3:	If Random Access Preambles group B is supported by the cell Random Access Preambles group B is included for each SSB.

-	if Random Access Preambles group B is configured for 4-step RA type:

-	ra-Msg3SizeGroupA: the threshold to determine the groups of Random Access Preambles for 4-step RA type;

-	msg3-DeltaPreamble: ∆PREAMBLE_Msg3 in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection included in groupBconfigured;

-	numberOfRA-PreamblesGroupA: defines the number of Random Access Preambles in Random Access Preamble group A for each SSB included in groupBconfigured.

-	if Random Access Preambles group B is configured for 2-step RA type:

-	msgA-DeltaPreamble: ∆MsgA_PUSCH in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection included in GroupB-ConfiguredTwoStepRA;

-	numberOfRA-PreamblesGroupA: defines the number of Random Access Preambles in Random Access Preamble group A for each SSB included in GroupB-ConfiguredTwoStepRA;

-	ra-MsgA-SizeGroupA: the threshold to determine the groups of Random Access Preambles for 2-step RA type.

-	the set of Random Access Preambles and/or PRACH occasions for SI request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for beam failure recovery request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for reconfiguration with sync, if any;

-	ra-ResponseWindow: the time window to monitor RA response(s) (SpCell only);

-	ra-ContentionResolutionTimer: the Contention Resolution Timer (SpCell only);

-	msgB-ResponseWindow: the time window to monitor RA response(s) for 2-step RA type (SpCell only).

In addition, the following information for related Serving Cell is assumed to be available for UEs:

-	if Random Access Preambles group B is configured for 4-step RA type or 2-step RA type:

-	if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5], and SUL carrier is selected for performing Random Access Procedure:

-	PCMAX,f,c of the SUL carrier as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16].

-	else:

-	PCMAX,f,c of the NUL carrier as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16].

End of 1st Change

Start of 2nd  Change

5.1.2	Random Access Resource selection

If the selected RA_TYPE is set to 4-stepRA, the MAC entity shall:

…

1>	else (i.e. for the contention-based Random Access preamble selection):

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	if the RA_TYPE is switched from 2-stepRA to 4-stepRA:

3>	if a Random Access Preambles group was selected during the current Random Access procedure:

4>	select the same group of Random Access Preambles as was selected for the 2-step RA type.

3>	else:

4>	if Random Access Preambles group B is configured for 4-step RA type; and

4>	if the transport block size of the MSGA payload configured in the rach-ConfigDedicated corresponds to the transport block size of the MSGA payload associated with Random Access Preambles group B:

5>	select the Random Access Preambles group B.

4>	else:

5>	select the Random Access Preambles group A.

2>	else if Msg3 buffer is empty:

3>	if Random Access Preambles group B is configured for 4-step RA type:

4>	if the potential Msg3 size (UL data available for transmission plus MAC subheader(s) and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – preambleReceivedTargetPower – msg3-DeltaPreamble – messagePowerOffsetGroupB; or

4>	if the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC subheader is greater than ra-Msg3SizeGroupA:

5>	select the Random Access Preambles group B.

4>	else:

5>	select the Random Access Preambles group A.

3>	else:

4>	select the Random Access Preambles group A.

2>	else (i.e. Msg3 is being retransmitted):

3>	select the same group of Random Access Preambles as was used for the Random Access Preamble transmission attempt corresponding to the first transmission of Msg3.

2>	select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB and the selected Random Access Preambles group;

2>	set the PREAMBLE_INDEX to the selected Random Access Preamble.

End of 2nd Change