3GPP TSG-RAN WG2 Meeting #127bis		       R2-2408353

Hefei, China: 14th October – 18th October 2024

Title:	Discussion on LCM for UE-sided models

Source:	ASUSTeK

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

- Introduction

The WI NR_AIML_air agreed to provide specification support for LCM for functionality selection, activation, deactivation, switching and fallback. In this contribution, we would like to discuss some issues based on RAN2 progress.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models

- Discussion

In RAN2 #127 meeting, the following procedure is agreed for applicable functionality reporting and functionality configuration for UE side models.

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

From the agreements, UE may decide the applicable functionalities based on UE-side additional conditions (internally known by UE), and report applicable functionalities upon change of applicable functionality via UAI.

After the UE has reported the applicable functionalities, the network could activate the functionality of the UE. And the UE may continue to evaluate the applicable functionalities when performing inference, and decide a functionality to be not applicable after the functionality is activated due to UE-side additional condition changes (e.g., internal issues), that the UE cannot perform inference. After determining a functionality to be not applicable, the UE updates the applicable functionalities through UAI reporting to NW.

Observation 1: When the functionality becomes not applicable when activated, the UE is not able to perform inference, and the UE reports updated applicable functionalities to NW.

After NW receives the updated applicable functionalities report, NW may reconfigure the UE or deactivate the functionality as a response. In the meantime, the UE is not able to perform inference. This may cause unexpected issues.

Figure: An example of set A prediction when the AIML functionality becomes not applicable when the functionality is activated.

For Beam Management, the UE predicts set A of beams by measuring set B of beams. When the UE is not able to perform inference, that is, the UE measures set B of beams but cannot predict set A of beams, there may be no set A measurements to report during the time gap between NW response and UE detection of internal issues.

Based on the difference between the set A and the set B of beams, there could be 3 following cases:

Case 1: The set B of beams is different from set A (i.e., not a subset of set A)

When the set B of beams is different from set A, stopping the inference may affect the ongoing beam management as measurement of set A of beams will be completely missing.

Case 2: The set B of beams is a subset of set A:

When the set B of beams if a subset of set A, stopping the inference may affect the ongoing beam management as the number of beams measured may not be sufficient.

Case 3: The set B of beams is the same as set A:

When the set B of beams is the same as set A of beams, UE may have the measurements of set A of beams but not reported to NW.

Observation 2: There is a time gap between a functionality becomes not applicable and NW responds to the updated applicability.

In order to avoid the issues, the UE should be able to fallback to legacy operation autonomously before NW responds to seamlessly continue to perform Beam Management. The UE may stop the AIML functionality upon detecting internal issues, performance monitoring below a threshold, after the evaluation of applicable functionalities … etc. After the UE stops the AIML functionality, the UE may switch to legacy (non-AIML) Beam Management and perform measurements on set A of beams. The UE may report the measured set A of beams to NW following the existing procedure.

Proposal 1: The UE supports autonomous fallback to legacy or autonomous deactivation before NW responds to an updated applicable functionalities report.

- Conclusion

Observation 1: When the functionality becomes not applicable when activated, the UE is not able to perform inference, and the UE reports updated applicable functionalities to NW.

Observation 2: There is a time gap between a functionality becomes not applicable and NW responds to the updated applicability.

Proposal 1: The UE supports autonomous fallback to legacy or autonomous deactivation before NW responds to an updated applicable functionalities report.

References

[1] RP-240774, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] RAN2 Chair’s Notes of 3GPP TSG RAN WG2 #127

Annex