3GPP TSG-RAN WG2 Meeting #127bis	R2-2408355

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:		8.2.2

Source:				ASUSTeK

Title:		Discussion on energy status report and data status indication

Document for: 	Discussion and Decision

- Introduction

In RAN2 #127 meeting, the following agreements have been made for energy status report and message size/status indication:

Agreements

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

In this contribution, we would like to discuss more details of energy status report and message status indication for A-IoT device.

- Discussion

In the last meeting, it has been agreed that RAN2 will study the need and use of energy status report. Since the A-IoT device may not always be able to perform transmission with limited energy capacity, and the exact stored energy at the A-IoT device is invisible to the network, it would be beneficial to provide the energy status report to the reader.

If the network schedules an A-IoT device which does not have enough energy to perform or complete the access procedure, transmission failure would occur during the access procedure. Such failure not only impacts the network performance but also creates unnecessary collisions, data delay and resources inefficiency. Additionally, considering the distance between each A-IoT device and the reader is different, the energy harvesting for the A-IoT devices could not be predicted by the network, even the A-IoT devices may harvest energy from the same carrier wave. As a result, to help the network schedule the random access for the large amount of A-IoT devices with resources and transmission efficiency, the energy status report is needed.

When the remaining energy of the A-IoT device is not enough for more R2D reception or D2R transmission in the access procedure, the A-IoT device could provide the energy status report in the last D2R transmission which is expected to be successfully transmitted based on the remaining energy. And the energy status report could include the remaining energy of the A-IoT device or a (minimum) required time that the A-IoT device could harvest enough energy for the next transmission. Before then, the access procedure could be suspended or stopped at the device side. And the reader could wait until the A-IoT device has sufficient energy, based on the energy status report it receives.

Proposal 1: If the energy of the A-IoT device is not enough to perform remaining R2D/D2R transmission(s), the A-IoT device provides the energy status report in the last D2R transmission which is expected to be transmitted based on remaining energy.

Proposal 2: The energy status report includes remaining energy storage of the A-IoT device or (minimum) required time of energy accumulation for the next transmission.

Proposal 3: The access procedure could be suspended or stopped if the A-IoT device does not have enough energy and the energy status report has been sent.

On the other hand, the data status report for A-IoT device has been discussed in the previous meetings. The legacy BSR mechanism was excluded and it was agreed to study the need and use of a “simple message size reporting”. We propose that the A-IoT device could simply provide an indication on whether there is pending D2R data for transmitting. This allows the network to allocate subsequent resources to the A-IoT device or receive subsequent D2R transmission from the A-IoT device at the next access occasion.

Regardless of whether AS segmentation would be performed or not, the A-IoT device would have additional data to transmit for a particular operation (e.g., read command). For an instance, some D2R data would become available after the A-IoT device receives the command from the reader. The received command would induce some D2R data coming in the A-IoT device. And the A-IoT device may not be able to transmit all the data over one allocated or selected resource. The knowledge of A-IoT device’s data status would be useful for the network to schedule proper resources for A-IoT devices.

Proposal 4: The A-IoT device could provide an indication on whether there is pending D2R data for transmitting.

- Conclusion

In this contribution, we have following proposals for reporting information of the A-IoT device:

Proposal 1: If the energy of the A-IoT device is not enough to perform remaining R2D/D2R transmission(s), the A-IoT device provides the energy status report in the last D2R transmission which is expected to be transmitted based on remaining energy.

Proposal 2: The energy status report includes remaining energy storage of the A-IoT device or (minimum) required time of energy accumulation for the next transmission.

Proposal 3: The access procedure could be suspended or stopped if the A-IoT device does not have enough energy and the energy status report has been sent.

Proposal 4: The A-IoT device could provide an indication on whether there is pending D2R data for transmitting.