3GPP TSG-RAN WG2 Meeting #127bis	R2-2408357

Hefei, China, Oct 14th – 18th, 2024	Revision of R2-2406522

Agenda Item:	8.2.5

Source:			ASUSTeK

Title:	Consideration on information and scenario in Topology 2

Document for: 	Discussion and Decision

- Introduction

In TR 38.848 [1], connectivity topologies for Ambient IoT networks and devices are defined. Among the connectivity topologies, topology 2 is specified as below:

### 4.2.1 Connectivity topologies #### 4.2.1.0 Introduction The following connectivity topologies for Ambient IoT networks and devices are defined for the purposes of the study. In all these topologies, the Ambient IoT device may be provided with a carrier wave from other node(s) either inside or outside the topology. The links in each topology may be bidirectional or unidirectional. BS, UE, assisting node, or intermediate node could be multiple BSs or UEs, respectively. The mixture of indoor and outdoor placement of such nodes is regarded as a network implementation choice. Account would need to be taken of potential impact on device or node complexity. In the connectivity topologies, this does not imply the existence of multi-hop assisting or intermediate nodes. #### 4.2.1.2 Topology 2: BS ↔ intermediate node ↔ Ambient IoT device Figure 4.2.1.2-1: Topology 2 In Topology 2, the Ambient IoT device communicates bidirectionally with an intermediate node between the device and basestation. In this topology, the intermediate node can be a relay, IAB node, UE, repeater, etc. which is capable of Ambient IoT. The intermediate node transfers Ambient IoT data and/or signalling between BS and the Ambient IoT device.

In RAN2#127 meeting [2], the following agreements on Ambient IoT topology 2 were made:

Agreements

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

- A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

- RAN2 assumes that intermediate UE authorization is performed by upper layers.  RAN2 impact can be studied after further SA/RAN3 progress.

- RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

- NW is in control of resources to be used by AIoT air interface.

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

In this contribution, we would like to discuss the information required in Topology 2 and study different scenarios.

- Discussion

- UE report

As specified in TR 38.848 [1], an intermediate node is introduced to support data or signalling transmission between the base station and the device in Topology 2 of Ambient IoT. The Ambient IoT device communicates with the intermediate node, which transfers the Ambient IoT data/signalling to BS (or gNB). Compared to Topology 1, the intermediate node is involved to play a role of the reader in a D2R transmission or a R2D transmission with the Ambient IoT device.

For a UE to become an intermediate node, it should be useful for the UE to provide some information to the gNB. For example, the UE should at least report its capability to perform functionalities of intermediate node before being configured. For another example, the UE could provide its location information to the network, and the network would configure the UE to become an intermediate node if there are Ambient IoT devices nearby. The information could assist the network to determine whether a UE should become an intermediate node.

Proposal 1: The UE should be able to provide information to network to assist determining whether the UE should become an intermediate node.

- Scheduling Information

After receiving the information from the candidate UE, the gNB determines whether the UE could become an intermediate node. And if it is the case, as shown in RAN2#127 agreement [2], the gNB is in control of the resource used in AIoT air interface. That is, the gNB should indicate the scheduling information for the R2D transmission or D2R transmission to the intermediate node. For example, when the gNB informs the intermediate node to initiate a R2D transmission or to trigger a D2R transmission, the gNB could also provide the scheduling information to the intermediate node.

Proposal 2: The gNB provides scheduling information to the intermediate node for the transmissions between the device and the reader.

- Scenarios in Topology 2

On the other hand, in RAN1#116bis meeting [2], several scenarios for topology 2 are defined as following:

| Scenario | CW Inside/outside topology | Diagram of the scenario | Description of the scenario | Device 1/2a/2b | CW spectrum | D2R spectrum |
| --- | --- | --- | --- | --- | --- | --- |
| D2T2-A1 | CW inside topology |  | - CW node inside topology 2 - ‘CW’ in CW2D and ‘R2’ in D2R are different - ‘CW’ in CW2D and ‘R1’ in R2D are same - ‘R1’ in R2D and ‘R2’ in D2R are different - BS communicates with R1 and R2 | Device 1, 2a | Case 2-2 (inside topology, UL) | Same as CW |
| D2T2-A2 |  |  | - CW node inside topology 2 - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D - BS communicates with R |  | Same as D2T2-A1 | Same as CW |
| D2T2-B | CW outside topology |  | - CW node outside topology 2 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same - BS communicates with R |  | Case 2-3 (outside topology, DL) Case 2-4 (outside topology, UL) | Same as CW |
| D2T2-C | No CW |  | - No CW Node. - BS communicates with R | Device 2b | N/A | FFS |

When the carrier wave is inside the topology, D2T2-A1 refers to the case that the intermediate node for D2R transmission and the one for R2D transmission are different. On the contrary, D2T2-A2 stands for the case that the intermediate node for D2R transmission and the one for R2D transmission are the same.

Observation 1: Based on RAN1’s agreement, the intermediate node for R2D transmission and D2R transmission could be the same or different.

Since the intermediate node (e.g. a UE) is assigned by the gNB (or the network), the operation and configuration should be under network control. For the case where there are two intermediate nodes to serve a device, there should be alignment for the gNB to make. For example, the resource used between gNB and each intermediate node, and the resource used between each intermediate node and the devices should be allocated without collision or interference. The gNB provides scheduling information for R2D transmission to R2D intermediate node; furthermore, it provides scheduling information for D2R transmission to R2D intermediate node (e.g. to include it in the PRDCH) and D2R intermediate node (e.g. to know where to receive the D2R transmission). Additionally, the required information used to assist corresponding R2D transmission should be aware to the R2D intermediate node after receiving at the D2R intermediate node. As a result, RAN2 could study the impact to procedures with different intermediate nodes for R2D and D2R transmissions.

Proposal 3:  For the case where there are two intermediate nodes in Topology 2, the gNB provides scheduling information for R2D transmission to R2D intermediate node. In addition, the gNB provides scheduling information for D2R transmission to R2D intermediate node and D2R intermediate node.

- Conclusion

Proposal 1: The UE should be able to provide information to network to assist determining whether the UE should become an intermediate node.

Proposal 2: The gNB provides scheduling information to the intermediate node for the transmissions between the device and the reader.

Observation 1: Based on RAN1’s agreement, the intermediate node for R2D transmission and D2R transmission could be the same or different.

Proposal 3:  For the case where there are two intermediate nodes in Topology 2, the gNB provides scheduling information for R2D transmission to R2D intermediate node. In addition, the gNB provides scheduling information for D2R transmission to R2D intermediate node and D2R intermediate node.

- Reference

[1] 3GPP TR 38.848 v18.0.0 (2023-09), ”Study on Ambient IoT (Internet of Things) in RAN”

[2] Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands

[3] RAN1#116bis meeting minute