3GPP TSG-RAN WG2 Meeting #127bis		       R2-2408359

Hefei, China: 14th October – 18th October 2024

Title:	Discussion on measurement event prediction

Source:	ASUSTeK

Agenda Item:	8.3.3

Document for:	Discussion and Decision

- Introduction

The SI FS_NR_AIML_Mob agreed to study measurement event prediction. In this contribution, we would like to discuss and share our understanding on the modeling of measurement event prediction.

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2]

- Discussion

In RAN2 #125-bis meeting, the following agreements are made:

Agreements 1 At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction are is also allowed. 2 Clarifications on what is being as input should be provided with results 3 Start with A3 as a baseline. 4 Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

From the agreements made in RAN2 #125-bis meeting, both direct and indirect approaches are supported. From our understanding, indirect approach implies in addition to actual measurements, the UE also utilizes measurements predicted by the RRM prediction functionality for the evaluation of measurement event triggering conditions. For direct approach, currently there is no clear modeling.

In RAN2 #127 meeting, the following agreements are made for RLF prediction:

Agreements - Both direct and indirect are allowed. Companies should indicate what they used and what inputs they are using - Output for indirect: predicted SINR. Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models. - Output for direct: probability of RLF within an window - Companies should report the prediction time window they have used in the simulations

From the agreements made in RAN2 #127 meeting, the output of direct RLF prediction is probability of RLF within a time window. We see some commonality between the modelling of direct RLF prediction and direct measurement event prediction, and thus the similar agreements for RLF prediction could be adopted for measurement event prediction. That is, the output for direct measurement event prediction could at least include the probability of measurement event happening within a time window.

Observation 1: Direct approach for RLF prediction and measurement event prediction share some commonalities.

Proposal 1: For direct measurement event prediction, at least output the probability of measurement event happening within a time window.

From the agreements made in RAN2 #125-bis meeting, Event A3 (Neighbour becomes offset better than SpCell) was agreed as the baseline to study. Event A3 includes comparison between SpCell and neighbouring cell measurements, and is triggered when neighbouring cell becomes better than the serving SpCell.

RAN2 is currently studying the feasibility of inter-cell RRM prediction from serving cell measurements, which may benefit the prediction of Event A3. For example, the future measurement results of neighbouring cell(s) can be predicted from serving cell measurement results. The RRM measurement prediction scenarios include temporal domain prediction, spatial domain prediction and frequency domain prediction, while inter-cell temporal domain prediction are currently de-prioritized.

Observation 2: Temporal domain prediction of neighbouring cell(s) from serving cell measurements have low priority.

Proposal 2: RAN2 start with using at least SpCell and neighbouring cell measurement(s) as input for direct measurement event prediction.

Proposal 3: Whether neighbouring cell measurement(s) are required for direct measurement event prediction may wait until further progress in RRM measurement prediction is made.

The timing to perform direct measurement event prediction may also affect the input for direct measurement event prediction, and 2 options are described below:

-	Option 1: Measurement event prediction may be performed before the entering condition of an event is fulfilled

-	Option 2: Measurement event prediction may be performed upon or after the entering condition of an event is fulfilled.

Figure: An example of prediction window for option 1 and option 2.

Option 1 may be more beneficial to handover performance as it can trigger the measurement report earlier. If option 1 is used, the input to the model may be cell measurements from all possible cells. As for option 2, the modelling is simpler and more feasible, but the performance gain may be limited. If option 2 is used, the input may be the cells that triggered the entering condition of the event.

Observation 3: The timing to perform measurement event prediction affects the input for direct measurement event prediction.

Proposal 4: RAN2 to select between option 1 and option 2 to study for the timing to perform measurement event prediction

Option 1: Measurement event prediction may be performed before the entering condition of an event is fulfilled

Option 2: Measurement event prediction may be performed upon or after the entering condition of an event is fulfilled.

- Conclusion

Observation 1: Direct approach for RLF prediction and measurement event prediction share some commonalities.

Observation 2: Temporal domain prediction of neighbouring cell(s) from serving cell measurements have low priority.

Observation 3: The timing to perform measurement event prediction affects the input for direct measurement event prediction.

Proposal 1: For direct measurement event prediction, at least output the probability of measurement event happening within a time window.

Proposal 2: RAN2 start with using at least SpCell and neighbouring cell measurement(s) as input for direct measurement event prediction.

Proposal 3: Whether neighbouring cell measurement(s) are required for direct measurement event prediction may wait until further progress in RRM measurement prediction is made.

Proposal 4: RAN2 to select between option 1 and option 2 to study for the timing to perform measurement event prediction

Option 1: Measurement event prediction may be performed before the entering condition of an event is fulfilled

Option 2: Measurement event prediction may be performed upon or after the entering condition of an event is fulfilled.

References

[1] RP-240082, “Revised SID on AIML for mobility in NR”

[2] RAN2 Chair’s Notes of 3GPP TSG RAN WG2 #125-bis

[3] RAN2 Chair’s Notes of 3GPP TSG RAN WG2 #127

Annex