3GPP TSG-RAN WG2 Meeting #127bis	R2-2408364

Hefei, China, 14th– 18th October, 2024

Agenda Item:	8.11.2

Source:	Huawei, HiSilicon

Title:	Impacts on the random access by the evolution of duplex operation

Document for: Discussion and Decision

# 1	Introduction

In the last meeting, RAN2 achieved the following working assumptions and agreements about the impacts on random access operation by the evolution of duplex operation:

- Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. - RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. - RAN2 to strive for a common SBFD CBRA framework independent of RRC state. - FFS whether/how early indication is used during a SBFD RA procedure. - RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

In this contribution, we will further analyse the RAN2 impacts to support random access in SBFD symbols by UEs in RRC CONNECTED mode and in RRC_IDLE/INACTIVE mode.

# 2	Discussion

## 2.1 RACH events

In the last RAN2 meeting, there was working assumption on the appliable RACH events for SBFD resource. Considering that random access in SBFD symbols aims to reduce the random access latency, to reduce the PRACH collision probability and to improve the coverage of PRACH and Msg3 transmission, we do not see strong reason to drop any of the RACH events and all of them should benefit from the advantage of RACH operation in SBFD resource.

Proposal 1: Confirm the working assumption "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events".

## 2.2 RACH Configuration

In the last RAN2 meeting, there was agreement about RACH configuration with detailed signalling FFS. It is understood that RAN2 can start working on the signalling design while RAN1 is progressing on the RACH resource configuration.

We notice that in previous RAN1 meeting, there were some agreements about framework of SBFD RACH configuration as following, and RAN1 was also discussing the issue when two RACH configuration options are enabled:

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

Firstly, based on the agreement of RAN1 that enabling both options at the same time for a UE is not supported and that UE is not required to support both options, we understand there are two types of UE with different capabilities, i.e. one supporting Option 1 RACH configuration only and one supporting Option 2 RACH configuration only.

For SBFD-aware UEs supporting Option 1 configuration only, if the UE receives RACH configuration with option 2, it will ignore the SBFD RACH resource configured by Option 2; while for the UE supporting Option 2 configuration, since the legacy RACH resources are also configured by the existing signalling same as in Option 1, the UE can determine the legacy ROs from the legacy signalling. If the gNB configures SBFD RACH resources using both options simultaneously targeting two UE types e.g. in SIB1, it is unclear what the UE behaviour would be. It may need to ignore the SBFD ROs configured in Option1. From RAN2 point of view, it is ambiguous what is meant by UE only supporting Option 2.

|  | gNB uses Option 1 only | gNB uses Option 2 only | gNB uses both options |
| --- | --- | --- | --- |
| UE supports Option 1 only | Follow Option 1 config | No SBFD RACH config | Follow Option 1 config |
| UE supports Option 2 only | No SBFD RACH config | Follow Option 2 config | Follow Option 2 config? |

We think it is preferrable to define a default configuration UE capability for the support of performing RACH procedure using SBFD resource. For simplicity, considering that Option 1 is to use the existing signalling, we propose to define supporting Option 1 configuration as the default UE capability for the support of performing RACH operation using SBFD resource. When the gNB configures SBFD RACH resources using both options simultaneously, some rules should be specified to UEs, e.g. Option 2-only UE always ignores the additional ROs configured by Option 1 in this scenario, or an explicit indicator can be introduced to indicate whether Option 1 is enabled, or an explicit indicator to indicate following which option when there are two sets of additional ROs.

Proposal 2: Define supporting the RACH configuration Option 1 as the default UE capability for performing RACH operation using SBFD resource.

Proposal 3: RAN2 to discuss the UE behaviour when the gNB enables two RACH configuration options simultaneously.

## 2.3 RACH resource selection

In the last meeting, RAN2 discussed the RACH resource selection issue, and RAN1 also discussed the issue of PRACH resource selection for PRACH transmission attempt and re-attempt in one random access procedure and achieved the following agreements:

Agreement For SBFD-aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs Agreement For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-Ros Agreement For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Agreement For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

From RAN2 perspective, we think for both RACH configuration Option 1 with Alt1-1 and RACH configuration Option 2, the PRACH resource in SBFD symbols (new PRACH resource) and the PRACH resource in non-SBFD symbols (legacy PRACH resource) are valid for SBFD-aware UEs. When the new PRACH resource and the legacy PRACH resource are both provided, the behavior of SBFD-aware UEs on initial PRACH resource selection and the re-attempt PRACH resource selection should be clarified.

For initial PRACH resource selection, two solutions were mentioned in both last RAN1 and last RAN2 meeting:

- Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations

- Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs

We understand Option 2 that based on the rule controlled by network can align the behaviour of UE and network. The network can control the UE to use which PRACH resource in the certain situation with this option. As an example, the network may guide the UE to use legacy PRACH resource when the interference on SBFD symbols is high. For Option3, it could lead to ambiguous understanding of UE behaviour on the network side, if the resource selection is up to UE implementation. From RAN2 perspective, if choose between two solutions, RAN2 may assume option2 that based on certain specified/configured conditions/prioritizations can be preferred.

Proposal 4: For the initial PRACH resource selection, RAN2 prefers to select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations by SBFD-aware UEs.

Another aspect to discuss on resource selection is the UE fall-back behaviour. For example, after multiple times of RACH failures with resource in SBFD symbols, UE shall be allowed to switch from the PRACH resources in SBFD symbols to the PRACH resources in non-SBFD symbols at least. The fall-back from non-SBFD symbols to SBFD symbols can also be discussed, preferably together with RAN1.

Proposal 5: Confirm that the UE is allowed to switch from the PRACH resources in SBFD symbols to the PRACH resources in non-SBFD symbol. FFS switch from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols.

For re-attempt PRACH resource selection, three solutions were mentioned in the last RAN1 meeting:

- Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure

- Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure

- Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

Comparing above three options for PRACH resource selection, Option 1 is simpler for both gNB and UE implementation but it imposes restriction of no-switching, which may reduce resource utilization efficiency and increases RACH latency. In contrast, the SBFD-aware UEs have better flexibility to switch PRACH resources with Option 2. Further, if an SBFD-aware UE always selects the PRACH resources in SBFD symbols even if they have failed several times, this will cause severe UE-to-UE CLI, adjacent-cell interference and resource wasting. To avoid those problems, SBFD-aware UEs shall be allowed to switch from the PRACH resources in SBFD symbols to the PRACH resources in non-SBFD symbols. For Option3, there is understandably more complexity from UE implementation perspective, i.e., the UE need to perform more decisions than Option1 and Option2.

Proposal 6: For re-attempt PRACH resource selection, RAN2 prefers to support Option 2 that first use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure.

## 2.4 Early indication in SBFD RA procedure

In the last RAN2 meeting, there is one FFS issue about the early indication in SBFD RA procedure:

- FFS whether/how early indication is used during a SBFD RA procedure.

Based on the discussion in the last meeting, there are three options for the early indication:

- Option 1: resource-based solution

- Option 2: preamble partitioning based solution

- Option 3: msg3 based early identification solution

Comparing above three options for early indication, we think both Option 1 and Option 2 are helpful for enabling SBFD-aware UEs to use SBFD resource in advance and for network to know the UE capability earlier. Option1 is naturally supported when SBFD-aware UEs transmit PRACH on the additional-ROs, it is understood that there are no additional specification impacts. The only concern is for the scenario that when the UE chooses legacy PRACH resource from two types of PRACH resources, the gNB can not recognize whether the UE is a SBFD-aware UE. We think there is no strong requirement for early identification in such case: when the UE, under certain condition, chooses the legacy PRACH resource for MSG1, it is highly likely that the SBFD resources may also be unsuitable for the subsequent parts of the RACH procedure.

Option 2 is based on the preamble partitioning. Currently the preamble partitioning has been applied to the features such as RedCap, SDT, NSAG and Msg3 Repetition. We understand that SBFD as being lower layer function which provides more transmission resources than the legacy system, does not necessarily alter the service requirements, priorities etc. for UE higher layer applications. Additionally, a SBFD related UE capabilities may not be significantly different from legacy UE capabilities: SBFD-aware UEs need to read resource configuration and perform transmission on SBFD resources. Further there would be additional complexity introduced by defining SBFD as one new feature. From these considerations, we think it is not suitable to define SBFD as a new feature or part of a new feature combination.

Regarding Option 3, we think it may be useful for network to know the UE capability earlier so that can schedule subsequent transmissions/receptions in SBFD symbols.

Observation 1: Resource-based early indication solution is naturally supported with no specification impact.

Proposal 7: There is no need to define SBFD as a new feature for early indication, i.e., there is no need to enhance the preamble partitioning mechanism for SBFD.

Proposal 8: Msg3 based early identification solution is supported.

## 2.5 RACH-RNTI calculation

In the last RAN2 meeting, some contributions mentioned the RA-RNTI calculation issue caused by SBFD RACH resource configuration.

In the RAN1#116bis meeting, the configuration related to legacy-ROs and additional-ROs was discussed with the following agreements:

Agreement For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, down-select (in RAN1#117) from the following alternatives: - Alt 2-3: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. - FFS: The case where the additional-ROs partially overlap with non-SBFD symbols - Alt 2-4: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs. For the legacy-ROs configured by legacy RACH configuration, the legacy RO validation rules and the legacy SSB-RO mapping rules are followed for SBFD aware UEs.

In the RAN1#118 meeting, there was further agreement about the RACH configuration:

Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

According to these agreements, there shall be no need to enhance the calculation of RACH-RNTI. For RACH configuration Option1, the legacy ROs and additional ROs share the same space of RO index, hence no RA-RNTI collision issue. For RACH configuration Option2 with Alt2-3, the additional ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs, i.e., the "valid" SBFD ROs and legacy RO cannot overlap at the same time point. Thus there would be no RA-RNTI collision issue as well.

Proposal 9: No need to enhance the RA-RNTI formula for SBFD RACH configuration.

# 3	Conclusion

Based on the above discussion we have the following observation and proposals:

Observation 1: Resource-based early indication solution is naturally supported with no specification impact.

Proposal 1: Confirm the working assumption "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events".

Proposal 2: Define supporting the RACH configuration Option 1 as the default UE capability for performing RACH operation using SBFD resource.

Proposal 3: RAN2 to discuss the UE behaviour when the gNB enables two RACH configuration options simultaneously.

Proposal 4: For the initial PRACH resource selection, RAN2 prefers to select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations by SBFD-aware UEs.

Proposal 5: Confirm that the UE is allowed to switch from the PRACH resources in SBFD symbols to the PRACH resources in non-SBFD symbol. FFS switch from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols.

Proposal 6: For re-attempt PRACH resource selection, RAN2 prefers to support Option 2 that first use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure.

Proposal 7: There is no need to define SBFD as a new feature for early indication, i.e., there is no need to enhance the preamble partitioning mechanism for SBFD.

Proposal 8: Msg3 based early identification solution is supported.

Proposal 9: No need to enhance the RA-RNTI formula for SBFD RACH configuration.

# 4	Reference

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)