3GPP TSG-RAN WG2 Meeting #127bis	R2-2408380

Hefei, China, 14 – 18 October 2024

Agenda item:	8.10.2

Source:	Nokia

Title:	MRO for LTM

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core

Document for:	Discussion and Decision

# Introduction

The following agreements were reached on the topic of MRO for LTM at RAN2#125bis and RAN2#126:

- For LTM MRO, RAN2 considers the following three connection failure cases:

- -	Too late LTM

- -	Too early LTM

- -	LTM to wrong cell

- For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

- -	Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell.

- -	Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

- For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

- -	Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

- LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

- -	Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

- -	Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

- RAN2 considers SHR, RA report and RLF for MCG LTM SON.

- RAN2 will start work on MCG LTM.

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

This paper discusses clarifications of the agreed LTM MRO scenarios and further LTM failure cases referring to the other important phases (like preparation or early synchronization phase) beyond the LTM cell switch execution phase.

# 2	Discussion on LTM MRO scenarios

## 2.1	Clarification on the already agreed LTM MRO scenarios

The agreed scenarios from RAN2#125bis represent an adaptation of classic MRO scenarios as defined in the past for other mobility mechanisms. However, the current wording is at times unclear or lacks essential details. In the following we will attempt to clarify the overall goals and propose rewording of each scenario.

If legacy detection mechanisms for Too Late, Too Early and Wrong Cell are to be followed, then it is our understanding that only the cell where the UE first attempts the radio connection re-establishment to, either via re-establishment, reconnection or LTM recovery, is of interest. A parallel with the scenarios/detection mechanism for CHO can be drawn with the remark that, for LTM, the configuration is kept after successful cell switch and the source cell may also be configured as a candidate cell for LTM recovery.

Proposal 1: RAN2 to agree that detection mechanism is based on the cell where the UE first attempts radio connection re-establishment to, either via re-establishment, reconnections or LTM recovery.

### 2.1.1	Too Late LTM

The defining characteristic of a too late handover is that the UE was long enough in a cell as to be configured with a possible handover or it was configured but the handover was not executed. As a result of running out of coverage of the serving cell, an RLF is detected, and re-establishment is attempted to a different cell other than the source cell. This holds true also in the case that an LTM recovery is attempted after the initial failure and prior to the (second) re-establishment.

Agreed scenario 1a seems to cover this exact scenario. However, without specifying to which cell the re-establishment is attempted, it cannot be clearly classified as a too late handover. Therefor we propose the following re-wording:

- Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and attempts re-establishment in a cell other than the source cell.

Proposal 2: RAN2 to agree the update of case 1a description as follows: 
Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and attempts re-establishment in a cell other than the source cell.

Case 1b mentions that the UE ‘selects an LTM candidate cell’, which means the UE immediately attempts LTM recovery to a candidate cell, but the recovery fails due to T304 expiry. The current definition of the scenario is however incomplete as after LTM recovery failure, the UE may also attempt re-establishment.

Observation 1: The current definition of case 1b ends with a failed LTM fast recovery without knowing the final connection state of the UE.

To clarify this scenario, we propose the following rewording that also helps to distinguish between regular/initial LTM execution and LTM recovery:

Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell other than the source cell and attempts LTM recovery, which fails, then attempts re-establishment.

Proposal 3: RAN2 to agree the update of case 1b description as follows: 
Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell other than the source cell and attempts LTM recovery, which fails, then attempts re-establishment.

Case 1c is similar to case 1b with the slight difference that the LTM recovery to the selected LTM candidate cell is successful. In order to further clarify the scenario definition, we propose the following rewording:

Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM recovery with an LTM candidate cell other than the source cell.

Proposal 4: RAN2 to agree the update of case 1c description as follows: 
Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM recovery with an LTM candidate cell other than the source cell.

### 2.1.2	Too early LTM

Case 2a follows the legacy definition and we propose the following clarification of the text:

Case 2a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in the source cell.

Proposal 5: RAN2 to agree the update of case 2a description as follows: 
Case 2a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in the source cell.

In our understanding, case 2b, should reflect a similar scenario as case 1b: first mobility related failure which is HOF or RLF in the target cell shortly after the cell switch followed by a failed LTM fast recovery and ending up with a re-establishment.

We propose the following correction of the case definition:

Case 2b: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell which is the initial source cell and attempts LTM recovery, which fails, then attempts re-establishment.

Proposal 6: RAN2 to agree the update of case 2b description as follows:
Case 2b: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell which is the initial source cell and attempts LTM recovery, which fails, then attempts re-establishment

Case 2c is complementary to the case 2b but here the LTM recovery is successful. Only minor additions for clarity are needed:

Case 2c: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the source cell which is also an LTM candidate cell.

Proposal 7: RAN2 to agree the update of case 2c description as follows: 
Case 2c: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the source cell, which is also an LTM candidate cell.

### 2.1.3	Wrong cell LTM

Cases 3a and 3b needs only minor clarification to reflect the classic scenario.

Case 3a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in a cell other than the source cell and the target cell.

Proposal 8: RAN2 to agree the update of case 3a description as follows: 
Case 3a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in a cell other than the source cell and the target cell.

- Case 3b: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell other than source or target cell and attempts LTM recovery, which fails, and then attempts re-establishment.

Proposal 9: RAN2 to agree the update of case 3b description as follows: 
Case 3b: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell other than source or target cell and attempts LTM recovery, which fails, and then attempts re-establishment.

Finally, scenario 3c, complementary to scenario 3b, needs only minor additions:

Case 3c: the UE detects HOF during LTM cell switch or /RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the selected LTM candidate cell which is different from the source or target one.

Proposal 10: RAN2 to agree the update of case 3b description as follows: 
Case 3c: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the selected LTM candidate cell which is different from the source or target one.

## 2.2	 Early TA acquisition optimization

The main rational behind and strength of LTM is the RACH-less cell change, which is only possible with knowing the timing advance (TA) when cell switch is triggered. One critical part of the LTM is therefore the phase called early TA acquisition, which the UE may be instructed to perform before receiving the cell switch command. If the ETA is successful, the interruption time during the cell switch will be significantly reduced as RA can be skipped. If the UE received the cell switch command that does not include a valid TA previously acquired for the target cell, the UE can still access the cell via regular RA procedure, but the benefits of LTM compared to the legacy L3 handover types are lost.

In case of the fully defined network-triggered early TA acquisition, the two trigger criteria for early TA acquisition and cell switch command need to be properly synced for a successful RACH-less LTM. Firstly, the TA once acquired may become invalid because of changed UE position or changed propagation conditions with respect to the candidate cell and the network will need to trigger early TA acquisition again. Secondly, the early TA acquisition procedure should not be performed too often as it is associated with some interruption time. It is thus important that the early TA acquisition trigger criterion is cell-pair individually and mutually with the LTM cell switch command trigger criterion optimized. For example, the network should avoid unnecessary early TA acquisition (e.g. obtain the TA for cells that are then not used for cell switch) and also identify instances where the TA was acquired but became invalid by the time cell switch command was sent.

Observation 2 Timely and correct early TA acquisition is a crucial part of LTM to guarantee the benefits of LTM, namely a RACH-less cell switch for minimum handover interruption time.

Proposal 11: RAN2 to study what additional information needs to be logged that could enable the optimization of early TA acquisition, e.g.  minimize instances where the TA was unnecessarily acquisitioned or became invalid.

## 2.3	 Enhancement of SHR

Successful HO report (SHR) was introduced in Rel-17 to capture near failure events that happen during a successful HO. SHR is triggered based on running timers (T310/T312/T304) exceeding a preconfigured threshold. In some cases, additional SHR triggering conditions have been defined. For example, in case of DAPS HO, SHR is also generated if the UE experiences an RLF on the source link during the DAPS HO as this is not the desired behaviour.

One optimization aspect with respect to LTM is the unnecessary configurations. As a reminder, in LTM, the UE may receive multiple configurations but will only execute one of them once it received the cell switch command. Moreover, the UE does not release the configuration and the prepared candidate PCells after LTM cell switch execution or LTM recovery execution. This enables fast triggering of cell changes and reduces the signalling overhead that is associated with the preparation of the LTM configurations.

There is currently however no way for the network to detect any unnecessary configurations and implicitly blocked resources. Information that would help the network to identify these cases could be added to the SHR.

Proposal 12: RAN2 to enhance SHR with new information and triggering criteria that allow the network to identify unnecessary LTM configurations.

## 2.4	LTM configuration

An open point from the last RAN2 meeting deals with the logging of the LTM candidates

RAN2#127 - Some info will be logged to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not (TBD if explicit/implicit)

In Rel-17, the measurement report (MeasResultNR) that may be embedded in RLF report/ SHR/ CEF report/ SPR included a flag indicating if a neighbour is CHO candidate or not. The limitation however was that this approach does not work for RLF after successful HO as the CHO configuration is released after handover completion. Thus, a mixed solution was agreed where both UE and network can log/deduce the UE CHO configuration.

In LTM however, since the configuration is retained after successful cell switch, this is no longer an issue.

Observation 3: in LTM, since the configuration is retained after successful cell switch, it is feasible for the UE to indicate if a cell was LTM candidate or not under all failure and near failure scenarios.

Despite this, we think it is better to have a unique solution for determining the LTM configuration that also reduces the signalling load on the UE side.

Proposal 13: RAN2 to agree on network-based solution for optimization of LTM configuration/candidate list and send LS to RAN3. (See draft LS in the Annex.)

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: RAN2 to agree that detection mechanism is based on the cell where the UE first attempts radio connection re-establishment to, either via re-establishment, reconnections or LTM recovery.

Proposal 2: RAN2 to agree the update of case 1a description as follows: 
Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and attempts re-establishment in a cell other than the source cell.

Observation 1: The current definition of case 1b ends with a failed LTM fast recovery without knowing the final connection state of the UE.

Proposal 3: RAN2 to agree the update of case 1b description as follows: 
Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell other than the source cell and attempts LTM recovery, which fails, then attempts re-establishment.

Proposal 4: RAN2 to agree the update of case 1c description as follows: 
Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM recovery with an LTM candidate cell other than the source cell.

Proposal 5: RAN2 to agree the update of case 2a description as follows: 
Case 2a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in the source cell.

Proposal 6: RAN2 to agree the update of case 2b description as follows:
Case 2b: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell which is the initial source cell and attempts LTM recovery, which fails, then attempts re-establishment

Proposal 7: RAN2 to agree the update of case 2c description as follows: 
Case 2c: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the source cell, which is also an LTM candidate cell.

Proposal 8: RAN2 to agree the update of case 3a description as follows: 
Case 3a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in a cell other than the source cell and the target cell.

Proposal 9: RAN2 to agree the update of case 3b description as follows: 
Case 3b: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell other than source or target cell and attempts LTM recovery, which fails, and then attempts re-establishment.

Proposal 10: RAN2 to agree the update of case 3b description as follows: 
Case 3c: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the selected LTM candidate cell which is different from the source or target one.

Observation 2 Timely and correct early TA acquisition is a crucial part of LTM to guarantee the benefits of LTM, namely a RACH-less cell switch for minimum handover interruption time.

Proposal 11: RAN2 to study what additional information needs to be logged that could enable the optimization of early TA acquisition, e.g.  minimize instances where the TA was unnecessarily acquisitioned or became invalid.

Proposal 12: RAN2 to enhance SHR with new information and triggering criteria that allow the network to identify unnecessary LTM configurations.

Observation 3: in LTM, since the configuration is retained after successful cell switch, it is feasible for the UE to indicate if a cell was LTM candidate or not under all failure and near failure scenarios.

Proposal 13: RAN2 to agree on network-based solution for optimization of LTM configuration/candidate list and send LS to RAN3. (See draft LS in the Annex.)

# Annex A: Draft LS to RAN3

3GPP TSG-RAN WG2 Meeting #127bis	R2-240xxxx

Hefei, China, 14 – 18 October 2024

Title:	[DRAFT] LS on LTM configuration

Response to:	-

Release:	Release 19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG3

Cc:

Contact Person:

#### Name:	First Last

E-mail Address:	first.last@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description

Both RAN2 and RAN3 groups have been discussing both UE and network-based solutions for determining the LTM configuration, similar to solutions agreed in Rel-17. It is important to remark that the limitations that apply to CHO and lead to have both UE and network-based solutions do not apply to LTM, as in LTM the configuration is retained after successful cell switch. Thus, RAN2 see no blocking point to have only a network-based solution for LTM case in Rel-19.

2. Actions:

To XX group.

ACTION: 	RAN2 respectfully asks RAN3 to agree and specify a network-based solution for the optimization of LTM configuration/candidate list.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#128	from 2024-11-18	to 2024-11-22		Orlando, USA

RAN2#129	from 2025-02-17	to 2025-02-21		Athens, GR